3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213687

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.12.3

Source: 	ZTE Corporation

Title: 	Discussion on ATG BS and UE RF requirements

Document for:	Discussion

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some initial views on coexistence study for ATG BS and UE RF requirements.

- Specify features to core specifications for ATG BS and UE [RAN4]

  - Scenario:

    - BS on the ground, and the CPE type of UE mounted in the aircraft

    - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft

    - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion.

  - Specify core requirements for coexistence between ATG and IMT terrestrial network

    - Example bands include n1, n78 and n79.

    - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS)

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

    - Specify RRM core requirements for ATG UE

      - Taking into account identified differences between ATG and fully ground based systems

      - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

Discussion

## 2.1. ATG BS type

As shown in the following Figure, ATG BS need to track the aircraft to provide the corresponding traffic service, therefore BS type 1-H or BS type 1-O should be more straight forward method to provide the beam tracking capability.

Figure 2.2.1-1 ATG network coexisting with terrestrial network [Rural area]

Proposal 1: to define the BS type 1-H and BS type 1-O for ATG BS.

## 2.2. ATG BS requirements

In the following table 1, we share some initial views on the ATG BS and its details could be found as following:

Table 1. summary of the proposals for ATG BS requirements

| Tx requirements | Applicability notes |
| --- | --- |
| gNB output power | ATG BS should belong to the Wide area BS type and its maximum output power should be left up to the declaration. In addition, MCL definition for the legacy wide area BS is not applicable for ATG BS anymore. |
| Output power dynamics |  |
| RE power control dynamic range | To reuse the existing requirement defined in TS 38.104 |
| Total power dynamic range | To reuse the total power dynamic range of TS 38.104 |
| Transmit ON/OFF power | To reuse the existing requirement defined in TS 38.104 |
| Transmitted signal quality |  |
| Frequency error | To reuse the same requirement defined in TS 38.104 |
| Modulation quality | To reuse the same requirement defined in TS 38.104 |
| Time alignment error | This is not applicable for ATG BS since its channel characteristic is difficult to support the MIMO which is similar as NR over NTN. |
| Unwanted emissions |  |
| Occupied bandwidth | To reuse the same requirement defined in TS38.104 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | This depends on the outcome of coexistence study. |
| Operating band unwanted emissions | This depends on the outcome of coexistence study. |
| Transmitter spurious emissions | To reuse the same spurious emission requirement defined in TS 38.104 |
| Transmitter intermodulation | Not applicable since it’s supposed to have no surrounding interfering gNB next to gNB |
| Rx requirement |  |
| Reference sensitivity level | To reuse the same reference requirements for Wide Area BS defined in TS 38.104 |
| Dynamic range | Since IoT of ATG BS is supposed to quite low compared with IoT level of the legacy TN BS, it’s sufficient to reuse the existing requirement for Wide Area BS defined in TS 38.104. |
| ACS | This depends on the outcome of coexistence study. |
| In-band blocking | This depends on the outcome of coexistence study. |
| Out of band blocking | To reuse OOBB power level as -15dBm for ATG BS which should be sufficient enough. |
| Receiver spurious emission | To reuse the requirements defined in TS 38.104 |
| Receiver intermodulation | Similar as transmitter intermodulation requirements, not applicable since it’s supposed to have no surrounding interfering gNB next to gNB |
| In-channel selectivity | Similar as dynamic range requirements, the IoT of ATG BS is supposed to quite low compared with IoT level of the legacy TN BS, it’s sufficient to reuse the existing requirement for Wide Area BS defined in TS 38.104. |

## 2.3. ATG UE requirements

In the following table 2, we share some initial views on the ATG UE RF requirement and its details could be found as following:

Table 2. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Transmitter Characteristics |  |  |
| General | No | Generic requirements from TN should apply. |
| Tx power | Yes | Option 1: more preferred way not to define the power class for ATG UE similar as IAB-MT |
| MPR | No | No MPR requirement defined similar as IAB-MT |
| A-MPR | Yes | No A-MPR requirement defined similar IAB-MT; |
| Configured Tx power | No | No configured Tx power defined similar as IAB-MT |
| Output Power Dynamics | No |  |
| Transmit signal quality |  |  |
| - Frequency error | No | To use the existing requirement defined for TN UE in TS 38.101-1 as baseline. |
| - Transmit modulation quality | No | To follow the existing requirements defined for TN UE in TS 38.101-1. |
| Output RF spectrum emissions |  |  |
| - Occupied bandwidth | No | Expect the same as TN in TS 38.101-1. |
| - Out of band emission |  |  |
| - SEM | No | This depends on the coexistence study. |
| - Additional SEM | Yes | Not applicable |
| - ACLR | No | This depends on the coexistence study. |
| - Spurious emission |  |  |
| - General | No | Generic requirements from TN in TS 38.101-1. should apply |
| - For UE coexistence | Yes | Not applicable since it’s far away from TN UEs on the ground. |
| Transmit intermodulation | No | Not applicable |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | Only 2 Rx branches are needed and it’s not necessary to mandate 4Rx for band n1/n78 and n79 since antenna assumptions for ATG UE and handheld smartphone is different. |
| Reference sensitivity | Yes | - To follow the existing Reference requirements of band n1/78/79 for 2Rx in TS 38.101-1. |
| Maximum input level | No | This depend on the simulation results input to verify how this requirement could be further relaxed similar as NR over NTN in Rel-17. |
| ACS | No | This depends on the coexistence study. |
| Blocking characteristics |  |  |
| - In-band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Narrow band | No | To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| Spurious response | No | Expect the same as TN and its power level of interfering signal could be further discussed similar as in-band blocking and out of band blocking. |
| Intermodulation | No | Not applicable |
| Spurious emissions | No | To reuse General requirements defined in TS 38.101-1 |

Conclusions

In this contribution, we shared some initial views on coexistence study for ATG BS and UE RF requirements and proposals are made as following:

Proposal 1: to define the BS type 1-H and BS type 1-O for ATG BS.

Proposal 2: to the proposals in table 2 as baseline for ATG BS RF requirements;

Proposal 3: to the proposals in table 3 as baseline for ATG UE RF requirements;

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.