3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213688

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.14.2

Source: 	ZTE Corporation

Title: 	Discussion on CA based positioning enhancement

Document for:	Discussion

Introduction

In RAN#94e meeting, one study item [RP-213588] on expanded and improved NR positioning was approved as one of Rel-18 RAN package which is led by RAN1 and it was further updated in [RP-221814] during the latest RAN#96 meeting. In this SID, RAN4 is tasked to study the following use case to improve the positioning accuracy. In this contribution, we want to share initial views on the feasibility to support the intra-band CA based PRS/SRS positioning enhancement.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary.

Discussion

As indicated in the objective for intra-band based PRS/SRS positioning enhancement, the following RF factors e.g.timing errors, phase coherency, frequency errors, power imbalance would have the impacts on the positioning accuracy. Therefore in this contribution, we want to share some initial analysis from BS RF perspective to investigate its feasibility and its achieved performance based on the existing state of art.

First of all, for intra-band scenario defined in the current NR/LTE specification, it consist of both intra-band contiguous CA case and intra-band non-contiguous CA case. In this contribution, we want to prioritize intra-band contiguous CA case for PRS/SRS based positioning enhancement firstly and further discuss the intra-band non-contiguous CA case at the later phase.

## 2.1. RF architecture for Intra-band contiguous CA case

In general for the BS hardware supporting the intra-band contiguous CA as shown in Figure 2.1-1, multiple aggregated contiguous carriers could be supported by the same Tx/Rx transceivers at BS side. In other words, physical signals generated from baseband module will go through the same IF and RF Tx/Rx chain, then the timing difference/phase coherency among candidate carriers would be mainly impacted by group delay of PA and analog filtering in different frequency ranges. In fact, these kind of group delay difference imposed on different candidate carriers within intra-band contiguous CA scenario should be quite limited.

For frequency errors among multiple aggregated contiguous carriers, this should be also negligible since the sync source/PLL would be used for different contiguous carriers.

For power imbalance among multiple aggregated contiguous carriers, this should be also negligible since intra-band contiguous CA scenario is always assumed as co-located scenario in which all carrier are transmitted from the same transmitter hardware or received by the same receiver hardware. Its amplitude response in the frequency domain should be limited which have negligible impacts on the positioning accuracy.

Figure 2.1-1. The basic diagram for AAU/RRU for NR BS

Observation 1:

From BS hardware supporting the intra-band contiguous CA case, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source which might impact positioning accuracy e.g. frequency error or power imbalance, these contributing factor in intra-band contiguous CA case is negligible.

From UE hardware perspective, its baseline hardware architecture should be similar as BS side. In other words, multiple aggregated contiguous carriers could be supported by the same Tx/Rx transceivers at UE side generally. Indeed it should be noted during the discussion of intra-band contiguous CA TAE requirement back to Release 15,  to define TAE requirements 260ns for FR1 and 130ns for FR2, the main technical motivation for strict TAE requirements in NR would be the possibility to use a single RF/FFT/baseband in the UE Rx receiver which is Option A in Figure 6.5.3.2.2-2 of TR38.817-02. However it should be known that this is just the minimum RF requirements for the reception of intra-band contiguous CA carrier where the orthogonality among component carriers should be assured, indeed its achieved timing alignment errors among different carriers could be much better than the specified minimum requirement as its dominant factors impacting the timing alignment error is mainly coming from group delay difference of PA and analog filtering.

Observation 2:

From UE hardware perspective, just similar as BS side, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source which might impact positioning accuracy e.g. frequency error or power imbalance, these contributing factor in intra-band contiguous CA case is negligible.

## 2.2. Performance evaluation for intra-band contiguous CA

In the following section, some initial simulation results are provided for intra-band contiguous CA based positioning enhancement. Basically due to the increasing aggregated carrier bandwidth, then its positioning accuracy would be improved with certain degree, however it should be also noted that increasing timing alignment error among the contiguous aggregated carriers due to the RF imperfection, then its achievable performance under the intra-band contiguous CA case could be also jeopardized compared with single carrier case. In general, RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fullfil the minimum requirements.

Proposal 1: RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fulfill the minimum requirements

### 2.2.1 FR1 intra-band contiguous CA

For FR1 intra-band contiguous CA case, the simulation results are shown in the following Figure 2.2.1-1 – Figure 2.2.1-3 and its detailed values at different tile of CDF curve are summarized in the following Table 2.2.1-1. The detailed simulation assumption can be found in Appendix. From these simulation results, it can be observed that, with the increasing timing error between two CCs (timing error is modeled as STD [standard normal distribution]), the positioning accuracy would decrease correspondingly. For the case of 50MHz+50MHz (InF_SH), as in the Figure 2.2.1-1 below, when the relative timing alignment error exceeds 4ns, then the positioning accuracy of intra-band contiguous CA would be worse than that of single carrier. For the case of 100MHz+100MHz (InF_SH), as in the Figure 2.2.1-2 below, when the relative timing alignment error exceeds 2ns, then the positioning accuracy of intra-band contiguous CA is worse than that of single carrier (100MHz).

Figure 2.2.1-1 . Simulation results for 50MHz+50MHz carrier aggregation (InF_SH)

Figure 2.2.1-2 . Simulation results for 100MHz+60MHz carrier aggregation (InF_SH)

Figure 2.2.1-3 . Simulation results for 100MHz+100MHz carrier aggregation (InF_SH)

Table 2.2.1-1. Summary of positioning accuracy in Figure 2.2.1-1 and Figure 2.2.1-2

|  |  | Position Accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Case | Delay (ns) | CDF=50% | CDF=67% | CDF=80% | CDF=90% |
| 50MHz+50MHz (InF_SH) | 0 | 0.22 | 0.29 | 0.36 | 0.49 |
|  | 1 | 0.25 | 0.33 | 0.43 | 0.54 |
|  | 2 | 0.33 | 0.47 | 0.63 | 0.98 |
|  | 3 | 0.47 | 0.73 | 0.89 | 1.26 |
|  | 4 | 0.57 | 0.85 | 1.13 | 1.48 |
|  | 5 | 0.75 | 1.03 | 1.52 | 2.21 |
|  | 6 | 0.90 | 1.27 | 2.02 | 2.87 |
|  | Single FL (50MHz) | 0.55 | 0.75 | 0.95 | 1.46 |
| 100MHz+60MHz (InF_SH) | 0 | 0.112 | 0.145 | 0.184 | 0.230 |
|  | 1 | 0.182 | 0.247 | 0.318 | 0.400 |
|  | 2 | 0.337 | 0.466 | 0.585 | 0.733 |
|  | Single FL (100MHz) | 0.217 | 0.267 | 0.329 | 0.421 |
| 100MHz+100MHz (InF_SH) | 0 | 0.091 | 0.126 | 0.146 | 0.186 |
|  | 1 | 0.139 | 0.192 | 0.246 | 0.314 |
|  | 2 | 0.245 | 0.350 | 0.432 | 0. 615 |
|  | Single FL (100MHz) | 0.217 | 0.267 | 0.329 | 0.421 |

### 2.2.2 FR2 intra-band contiguous CA

For FR2 intra-band contiguous CA case, for the case of 100MHz+100MHz (InF_SH), as in the Figure 2.2.2-1 below, when the relative timing alignment error exceeds 2ns, then the positioning accuracy of intra-band contiguous CA would be worse than that of single carrier similar as FR1 observation.

Figure 2.2.1-1 . Simulation results for 50MHz+50MHz carrier aggregation (InF_SH)

Table 2.2.2-1-1. Summary of positioning accuracy in Figure 2.2.1-1

|  |  | Position Accuracy (m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Case | Delay (ns) | CDF=50% | CDF=67% | CDF=80% | CDF=90% |
| 100MHz+100MHz (InF_SH) | 0 | 0.081 | 0.110 | 0.136 | 0.187 |
|  | 1 | 0.119 | 0.170 | 0.236 | 0.284 |
|  | 2 | 0.215 | 0.313 | 0.424 | 0.569 |
|  | Single FL (100MHz) | 0.264 | 0.349 | 0.413 | 0.507 |

## 2.3. Measurement results for intra-band contiguous CA TAE

As mentioned in section 2.2, RAN4 need to study its feasibility to fulfill the minimum requirements. In the following section, we want to provide some initial measurement results to further discuss its feasibility.

### 2.3.1 FR1 intra-band contiguous CA

Some very initial field testing via an existing commercial smart phone are performed on two contiguous carriers that carry SRS transmission as the following Figure 2.3.1-1. More field tests with advanced UE/gNB are ongoing.

From these figures, it can be observed that the timing difference between the CC with 100MHz and the CC with 60MHz only is around 4Tc = 586 – 582 = 2ns for the first path.

Figure 2.3.1-1. Field test of SRS timing errors between one 100MHz CC and one 60MHz CC

Note: More testing data for downlink intra-band contiguous CA would be provided. Based on the state of the art, achieved timing alignment error between different bands would be around 2ns which consist of group delay response of PA and analog filtering in different frequency range and transmitter calibration error, however for intra-band contiguous CA, it should only consist of group delay response of PA and analog filtering in different frequency range, therefore it could be expected that less than 2ns timing alignment error for intra-band CA is also feasible.

Observation 3: for FR1 intra-band contiguous CA scenario, extremely small timing difference (e.g. less than 2ns) in UL between contiguous CCs is feasible.

### 2.3.2 FR2 intra-band contiguous CA

As mentioned in section 2.2, for intra-band contiguous CA, physical signals generated from baseband module will go through the same IF and RF Tx/Rx chain, then the timing difference/phase coherency among candidate carriers would be mainly impacted by group delay of PA and analog filtering in different frequency ranges. In the following figure, group delay response for 26GHz is provided, it could be found that group delay between 24.25GHz, 26.263GHz and 27.5GHz, its group delay difference should be less than 0.2ns. For other RF components’ group delay response could be provided later on.

Observation 4: for FR2 intra-band contiguous CA scenario, extremely small timing difference between contiguous CCs due to RF filtering is around 0.2ns.

Conclusions

In this contribution, we want to share some initial consideration on intra-band contiguous CA based positioning enhancement and proposals are made as following:

Observation 1:

From BS hardware supporting the intra-band contiguous CA case, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source which might impact positioning accuracy e.g.frequency error or power imbalance, these contributing factor in intra-band contiguous CA case is negligible

Observation 2:

From UE hardware perspective, just similar as BS side, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source which might impact positioning accuracy e.g.frequency error or power imbalance, these contributing factor in intra-band contiguous CA case is negligible.

Proposal 1: RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fullfil the minimum requirements

Observation 3: for FR1 intra-band contiguous CA scenario, extremely small timing difference (e,g,2ns) in UL between contiguous CCs is feasible from the field test.

Observation 4: for FR2 intra-band contiguous CA scenario, extremely small timing difference between contiguous CCs due to RF filtering is around 0.2ns.

References

- RP-221814,Study on expanded and improved NR positioning, Intel, approved

- RAN4#104e meeting agenda

Appendix

Table 2  Simulation assumption for FR1-1

| Parameter | Setting |
| --- | --- |
| Scenario | InF-SH |
| Carrier frequency | 3.5GHz |
| Bandwidth | 50+50MHz 100+100MHz |
| SCS | 30kHz |
| Description of measurement algorithm | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm |

Table 3  Simulation assumption for FR2-1

| Parameter | Setting |
| --- | --- |
| Scenario | InF-SH |
| Carrier frequency | 28GHz |
| Bandwidth | 200+200MHz |
| SCS | 120kHz |
| Description of measurement algorithm | MUSIC |
| Description of positioning technique | TDOA, Gauss-Newton algorithm |