3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213689

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.14.3

Source: 	ZTE Corporation

Title: 	Discussion on carrier phase based positioning

Document for:	Discussion

Introduction

In RAN#94e meeting, one study item [RP-213588] on expanded and improved NR positioning was approved as one of Rel-18 RAN package which is led by RAN1 and it was further updated in [RP-221814] during the latest RAN#96 meeting. In this SID, RAN1 and RAN4 is tasked to study solutions for accuracy improvement based on NR carrier phase measurements. In this contribution, we want to share some initial understandings from RAN4 perspective.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary.

Discussion

For the carrier phase based positioning, it could be expected that the phase assumption of TRP would directly have impacts on the positioning performance at UE side. For the practical TRP node, the carrier phase or timing of the specific TRP or BS will be affected by many factors (such as sync signal quality, internal interface delay uncertainty, clock module jitter, sampling rate and the stability of the components). If there is not calibration process for different TRP or BS, then the phase error has to be considered for phase based positioning. Even with calibration process among the serving TRPs, there would be still residual phase errors among different TRPs.

Figure 1. Illustration of BS/TRP’s hardware framework

In the following section, we will provide some initial analysis how the carrier phase error of TRP transmitter have the impacts on the positioning accuracy.

The carrier phase error is modeled as following:

ΦT=Φ+Φerrormod 1.0                              (Equ. 1)

where ΦT is the practical transmitted signal phase within range of 0 to 1.0, Φ is the ideal phase which is determined by the distance between UE and TRP in the LOS channel condition within the range of 0 to 1.0, and Φerror is the initial random phase error within a range of 0 to 1.0. It is noted that Φerror values are assumed to independent for each serving TRP.

With this above carrier phase error model, the initial simulation results are provided in the following figure 1 to show its impact on the positioning accuracy. It can be observed that even the initial random phase error is limited to 0 ~ 0.001*2π, the performance is still degraded significantly (esp., for a large scope of integer for searching).

Observation 1: For the initial random phase error is limited to 0 ~ 0.001*2π, the performance is still degraded significantly (esp., for a large scope of integer for searching).

Figure 2   Positioning accuracy with random phase error at TRPs (with N±4, no phase calibration, i.e., no PRU)

In summary, we have the following proposal.

Proposal 1: RAN4 need to study the timing error and phase errors among different TRPs and provide the corresponding feedback to RAN1.

Conclusions

In this contribution, we want to share some initial consideration on carrier phase based positioning and proposals are made as following:

Observation 1: for the initial random phase error is limited to 0 ~ 0.001*2π, the performance is still degraded significantly (esp., for a large scope of integer for searching).

Proposal 1: RAN4 need to study the timing error and phase errors among different TRPs and provide the corresponding information to RAN1.

References

- RP-221814,Study on expanded and improved NR positioning, Intel, approved

- RAN4#104e meeting agenda