3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213691

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.13.2.1

Source: 	ZTE Corporation

Title: 	Discussion on full duplex coexistence in adjacent channel scenario

Document for:	Discussion

Introduction

In RAN#94e meeting, the work item [RP-221352] on study on evolution of NR duplex operation was approved as one of Rel-18 RAN1 package. In this contribution, we want to share some initial views on from the coexistence of full duplex BS in the adjacent channel scenario.

Discussion

## 2.1. Coexistence in adjacent channel scenario

In general, when the new 3GPP based RAT is introduced, RAN4 need to study the coexistence with the legacy system to make sure without causing much performance degradation. The detailed coexistence scenario would be found in the following Figure 2 and Figure 3. In addition, full duplex BS coexisting with full duplex BS in the adjacent channel should be also considered since this is also typical scenario in which both operators upgrades its legacy BS to full duplex BS.

Figure 1. full duplex BS coexisting with full duplex BS in the adjacent channel

Figure 2. full duplex BS coexisting with legacy TDD DL in the adjacent channel

Figure 3. full duplex BS coexisting with with legacy TDD UL in the adjacent channel

Proposal 1: to study the following three coexistence scenario in the adjacent channel:

- Full duplex BS coexisting with full duplex BS in the adjacent channel;

- Full duplex BS coexisting with legacy TDD DL in the adjacent channel;

- Full duplex BS coexisting with legacy TDD UL in the adjacent channel.

## 2.2.The impacts on the legacy system

In the following table,we provided some initial analysis based on the coexistence study of CLI in Rel-16. For the full duplex BS, we assumed that slot with 50% PRB for DL and 50% PRB for UL. Based on the initial analysis, only the following coexistence scenarios highlighted in orange should be further checked its feasibility considering the potential lager frequency gap between DL and UL

Proposal 2: to further check the feasibility of following coexistence scenarios highlighted in orange in Table 2.2-1 and Table 2.2-2.

Table 2.2-1: Summary of simulation scenarios for FR1 (4 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor in CLI in Rel-16 | Aggressor in CLI in Rel-18 | Victim | Note for Rel-18 full duplex |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Macro → Macro | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, DL | No UE-UE coexistence issue expected |
| 2 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, UL | FFS for BS-BS coexistence issue expected |
| 3 | Macro → Indoor | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, DL | FFS for UE-UE coexistence issue expected |
| 4 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, UL | no BS-BS coexistence issue expected |
| 5 | Indoor → Macro | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, DL | No UE-UE coexistence issue expected |
| 6 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, UL | no BS-BS coexistence issue expected |
| 7 | Indoor → Indoor | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, DL | No UE-UE coexistence issue expected |
| 8 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% (worst case) | NR, 50 MHz, DL NR, 50 MHz, UL | NR, 100 MHz, UL | no BS-BS coexistence issue expected FFS for higher BS transmission power if assumed, |

Table 2.2-2: Summary of simulation scenarios for FR2 (30 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor in CLI in Rel-16 | Aggressor in CLI in Rel-18 | Victim | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Macro → Macro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, DL | No UE-UE coexistence issue expected |
| 10 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, UL | FFS for BS-BS coexistence issue expected |
| 11 | Micro → Micro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, DL | No UE-UE coexistence issue expected |
| 12 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, UL | FFS for BS-BS coexistence issue expected |
| 13 | Indoor → Macro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, DL | No UE-UE coexistence issue expected |
| 14 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, UL | No BS-BS coexistence issue expected |
| 15 | Indoor → Indoor | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, DL | No UE-UE coexistence issue expected |
| 16 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 100 MHz, DL NR, 100 MHz, UL | NR, 200 MHz, UL | FFS for BS-BS coexistence issue expected |

## 2.3.The impacts on full duplex BS

Similar as full duplex BS interfering legacy NR BS, vice versa, the legacy NR BS would also interfering full duplex BS. Different from the legacy NR BS, for full duplex BS, it need to consider the internal self interference and also intra-carrier inter-sub-band CLI. For the full duplex BS, we assumed that slot with 50% PRB for DL and 50% PRB for UL.

Proposal 3: to further study the coexistence scenario in Table 2.3-1 and Table 2.3-2 to investigate the impacts on full duplex BS.

Table 2.3-1: Summary of simulation scenarios for FR1 (4 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor | Victim Baseline | Victim in CLI in Rel-18 |
| --- | --- | --- | --- | --- |
| 1 | Macro → Macro | NR, 100 MHz, DL | NR, 100 MHz, DL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 2 |  | NR, 100 MHz, UL | NR, 100 MHz, UL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 3 | Macro → Indoor | NR, 100 MHz, DL | NR, 100 MHz, DL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 4 |  | NR, 100 MHz, UL | NR, 100 MHz, UL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 5 | Indoor → Macro | NR, 100 MHz, DL | NR, 100 MHz, DL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 6 |  | NR, 100 MHz, UL | NR, 100 MHz, UL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 7 | Indoor → Indoor | NR, 100 MHz, DL | NR, 100 MHz, DL | NR, 50 MHz, DL NR, 50 MHz, UL |
| 8 |  | NR, 100 MHz, UL | NR, 100 MHz, UL | NR, 50 MHz, DL NR, 50 MHz, UL |

Table 2.3-2: Summary of simulation scenarios for FR2 (30 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Victim Baseline | Victim in CLI in Rel-18 |
| --- | --- | --- | --- | --- |
| 9 | Macro → Macro | NR, 200 MHz, DL | NR, 200 MHz, DL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 10 |  | NR, 200 MHz, UL | NR, 200 MHz, UL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 11 | Micro → Micro | NR, 200 MHz, DL | NR, 200 MHz, DL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 12 |  | NR, 200 MHz, UL | NR, 200 MHz, UL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 13 | Indoor → Macro | NR, 200 MHz, DL | NR, 200 MHz, DL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 14 |  | NR, 200 MHz, UL | NR, 200 MHz, UL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 15 | Indoor → Indoor | NR, 200 MHz, DL | NR, 200 MHz, DL | NR, 100 MHz, DL NR, 100 MHz, UL |
| 16 |  | NR, 200 MHz, UL | NR, 200 MHz, UL | NR, 100 MHz, DL NR, 100 MHz, UL |

## 2.4. Simulation assumptions for full duplex coexistence study

For the coexistence study for Rel-18 full duplex coexistence study, the simulation assumption in TR 38.828 could be used as baseline. In addition, considering the separate antenna pattern assumption for Tx and Rx to enable the self-interference mitigation, then antenna pattern for FR1 BS and FR2 should be further discussed.

Proposal 4: to use the simulation assumption in TR 38.828 as baseline for full duplex coexistence study and further discuss the antenna pattern and related power for FR1 BS and FR2 BS due to the separate Tx and Rx antenna array assumption.

Conclusions

In this contribution, we want to share some initial views on from the coexistence of full duplex BS in the adjacent channel scenario and proposals are made as following:

Proposal 1: to study the following three coexistence scenario in the adjacent channel:

- Full duplex BS coexisting with full duplex BS in the adjacent channel;

- Full duplex BS coexisting with legacy TDD DL in the adjacent channel;

- Full duplex BS coexisting with legacy TDD UL in the adjacent channel;

Proposal 2: to further checked the feasibility of following coexistence scenarios highlighted in orange in Table 2.2-1 and Table 2.2-2.

Proposal 3: to further study the coexistence scenario in Table 2.3-1 and Table 2.3-2 to investigate the impacts on full duplex BS.

Proposal 4: to use the simulation assumption in TR 38.828 as baseline for full duplex coexistence study and further discuss the antenna pattern and related power for FR1 BS and FR2 BS due to the separate Tx and Rx antenna array assumption.

References

- RP-221352, Study on evolution of NR duplex operation, CMCC, Samsung, Approved

- TR38.828,Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)