3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213692

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	11.13.2.2

Source: 	ZTE Corporation

Title: 	Discussion on self-interference and CLI for full duplex BS

Document for:	Discussion

Introduction

In RAN#94e meeting, the work item [RP-221352] on study on evolution of NR duplex operation was approved as one of Rel-18 RAN1 package. In this contribution, we want to share some initial views on from the self-interference and CLI for full duplex BS.

Discussion

## 2.1. RF architecture for full duplex BS

First of all, before the discussion for the details of self interference of full duplex BS, it’s necessary to have the overview of RF architecture of full duplex BS. As shown in the following figure 1 for FR1 full duplex BS and FR2 full duplex BS.

Figure 1. general illustration of FR1 full duplex BS

As shown in the above Figure 1 for FR1 full duplex BS, to handle the self-interference from the transmitter to its own receiver, there would be following potential approaches to mitigate the interference to ensure its receiver performance:

- Antenna isolation from transmitter to receiver;

- Sub-band filtering of transmitter to further reject the leakage into the receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- Sub-band filtering of receiver to reject the power from the transmitter;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

- In addition, it should be noted that for the practical full duplex BS, it’s not necessary to consider the combination of all approaches to suppress the interference from the transmitter to receiver, only part of the combination could be also enough to ensure its receiver performance. The details for the feasibility of FR1 full duplex BS could be found in section 2.2.

- Observation 1: for FR1 full duplex BS, the following approach could be used to handle the self-interference:

- Antenna isolation from transmitter to receiver;

- Sub-band filtering of transmitter to further reject the leakage into the receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- Sub-band filtering of receiver to reject the power from the transmitter;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

Figure 2. general illustration of FR2 full duplex BS

As shown in the above Figure 2 for FR2 full duplex BS, similar as FR1 full duplex BS, to handle the self-interference from the transmitter to its own receiver, there would be following potential approaches to mitigate the interference to ensure its receiver performance:

- Antenna isolation from transmitter to receiver;

- Sub-band filtering of transmitter to further reject the leakage into the receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- Sub-band filtering of receiver to reject the power from the transmitter;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

- In addition, it should be noted that sub-band filtering has been crossed out since it might be not typical and feasible to suppress the interference from transmitter to receive due to its slow rolling off slope of its filtering response.

- Observation 2: for FR2 full duplex BS, the following approach could be used to handle the self-interference:

- Antenna isolation from transmitter to receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

## 2.2. Self interference analysis and RF requirement impacts

In the following section, we want to share some initial views on feasibility analysis for self interference of full duplex BS for both FR1 and FR2.

### 2.2.1. FR1 full duplex BS

For FR1 full duplex BS, we want to use the Medium range BS as example case and could further discuss other BS types supporting full duplex operation. For Medium rane BS supporting full duplex, it’s assumed to operate at 4.9GHz, with 100MHz, 30kHz, and 50MHz DL sub-band, 50MHz UL sub-band. To ensure the enough self interference mitigation between transmitter and receiver, the following combination of mitigation approaches has been considered and open for other combination of mitigation approaches.

- Free space isolation:

Firstly of all, as indicated in section 2.1, it’s quite typical implementation to have the separate transmitter and receiver to provide the certain level of isolation. Here we assumed that separation distance between transmitter antenna and receiver antenna is 0.12m which will result in 28dBc free space isolation.

- Antenna isolation

- Secondly considering isolation between sub-array of transmitter and sub-array of receiver and sub-array is placed on the plane and side by side with separation distance as 0.12m, then the Antenna isolation between transmitter and receiver considering the free space pathloss would be

- Antenna isolation=28dBc+12(Tx gain)+12 (Rx gain)=54.2dBc

It should be noted that without considering the other blockage material between transmitter and receiver. With increasing the separation distance and other blockage material placed between the transmitter and received, it could be expected that higher coupling loss could be achieved between transmitter and receiver.

- Figure 2.2.1-1. antenna element pattern for full duplex BS [3dB beamwidth 90 degree]

- Figure 2.2.1-2. antenna array pattern for full duplex BS [3dB beamwidth 90 degree]

For other self interference mitigation factor are listed in the following table, it could be found that referense degradation due to self-interference would be around 0.8dB which seems acceptable from RAN4 perspective which is much less than the impact of the legacy ACS/blocking impacts.

Table 2.2.1-1. self interference analysis for FR1 Medium range BS supporting full duplex operation

| Self interference mitigation factors | Value [dB] |
| --- | --- |
| Antenna isolation from transmitter to receiver; ① | 50dBc |
| Sub-band filtering of transmitter to further reject the leakage into the receiver; ② | [30] |
| ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL; ③ | 45dBc |
| Sub-band filtering of receiver to reject the power from the transmitter; ④ | 45dB |
| ACS of receiver to reject the power from the transmitter by digital filtering;⑤ | 46dB |
| Digital interference cancellation at receiver;⑥ | [30] |
| RF interference cancellation; ⑦ | NA |
| Self interference calculation at receiver baseband (Note 1) | Value [dBm] |
| From transmitter leakage perspective ① | 31dBm-45(ACLR)-50(antenna isolation)-30(digital cancellation/Tx sub-band filtering)-10*log10(50*10) =-121dBm/100kHz |
| From receiver channel selectivity perspective: ② | 31dBm-50(antenna isolation)-45 (Rx sub-band filter)-46dB (ACS)-10*log10(50*10) = -136.9897dBm/100kHz |
| Total interference of ①+② |  |
| Self interference calculation at receiver LNA input (Note 2) | Value [dBm] |
| Received power within freq range of wanted signal ③ | 31dBm-45(ACLR)-50(antenna isolation) =-64dBm>-67.1dBm inerference level of dynamic range requirement of 50MHz |
| Received power within freq range of DL signal ④ | 31dBm-50(antenna isolation)-45 (sub-band filter) = -54dBm<-50dBm for ACS requirement and -38dBm for IBB requirements of 50MHz |
| Note 1: If referense degradation due to self-interference of full duplex BS is expected to 0.8dB, then the total interference received should be -174dBm/Hz+10*log10(100*10^3)+10dB-7dB= -121dBm/100kHz Note 2: the receiver power at the receiver LNA should be taken into account, otherwise this might be blocked due to the high input power. |  |

Observation 3: it seems feasible to support the full duplex operation for Medium range BS.

Proposal 1 : for FR1 full duplex BS, to consider the self interference mitigation approaches as mentioned in table 2.2.1-1 to different BS class supporting the full duplex operation and its detailed value could be further studied.

Proposal 2: for FR1 full duplex BS, to check the feasibility from both refesens degradation and LNA blocking perspective.

### 2.2.2. FR2 full duplex BS

For FR2 full duplex BS, we want to use the Medium range BS as example case and could further discuss other BS types supporting full duplex operation. For Medium range BS supporting full duplex, it’s assumed to operate at 30GHz, with 200MHz, 120kHz, and 100MHz DL sub-band, 100MHz UL sub-band. To ensure the enough self interference mitigation between transmitter and receiver, the following combination of mitigation approaches has been considered and open for other combination of mitigation approaches.

- Free space isolation:

Firstly of all, as indicated in section 2.1, it’s quite typical implementation to have the separate transmitter and receiver to provide the certain level of isolation. Here we assumed that separation distance between transmitter antenna and receiver antenna is 0.2m which will result in 44dBc free space isolation.

- Antenna isolation

- Secondly considering isolation between sub-array of transmitter and sub-array of receiver and sub-array is placed on the plane and side by side with separation distance as 0.2m, then the Antenna isolation between transmitter and receiver considering the free space pathloss would be

- Antenna isolation==48dBc+23(Tx gain)+23 (Rx gain)+2.2=96.2dBc

It should be noted that without considering the other blockage material between transmitter and receiver. With increasing the separation distance and other blockage material placed between the transmitter and received, it could be expected that higher coupling loss could be achieved between transmitter and receiver.

- Figure 2.2.2-1. antenna element pattern for full duplex BS [3dB beamwidth 65 degree]

- Figure 2.2.2-2.. antenna array pattern for full duplex BS [3dB beamwidth ]

For other self interference mitigation factor are listed in the following table, it could be found that referense degradation due to self-interference would be around 0.8dB which seems acceptable from RAN4 perspective which is much less than the impact of the legacy ACS/blocking impacts.

Table 2.2.2-1. self interference analysis for FR2 Wide area BS supporting full duplex operation

| Self interference mitigation factors | Value [dB] |
| --- | --- |
| Antenna isolation from transmitter to receiver; ① | 96dBc |
| Sub-band filtering of transmitter to further reject the leakage into the receiver; ② | N/A |
| ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL; ③ | 28dBc |
| Sub-band filtering of receiver to reject the power from the transmitter; ④ | N/A |
| ACS of receiver to reject the power from the transmitter by digital filtering;⑤ | 24dB |
| Digital interference cancellation at receiver;⑥ | N/A |
| RF interference cancellation; ⑦ | NA |
| Self interference calculation at receiver baseband (Note 1) | Value [dBm] |
| From transmitter leakage perspective ① | 26dBm-28(ACLR)-96(antenna isolation)-10*log10(100*10) =-128dBm/100kHz |
| From receiver channel selectivity perspective: ② | 26dBm-96(antenna isolation)-24dB (ACS)-10*log10(100*10) = -123.9897dBm/100KHz |
| Total interference of ①+② |  |
| Self interference calculation at receiver LNA unit input (Note 2) | Value [dBm] |
| Received power within freq range of wanted signal ③ | 26Bm-28 ACLR-96(antenna isolation) =-98dBm Note: there is no FR2 Rx dynamic range requirements defined . |
| Received power within freq range of DL signal ④ | 26dBm-96(antenna isolation) = -70dBm <-83dBm (refesens)+27.7 ACS |
| Note 1: If referense degradation due to self-interference of full duplex BS is expected to 0.8dB, then the total interference received should be -174dBm/Hz+10*log10(100*10^3)+10dB-7dB= -121dBm/100kHz Note 2: the receiver power at the receiver LNA should be taken into account, otherwise this might be blocked due to the high input power. |  |

Observation 4: it seems feasible to support the full duplex operation for Wide area BS with only the antenna isolation considered.

Proposal 3 : for FR2 full duplex BS, to consider the self interference mitigation approaches as mentioned in table 2.2.2-1 with the removal of sub-band filtering to different BS class supporting the full duplex operation and its detailed value could be further studied.

Proposal 4: for FR2 full duplex BS, to check the feasibility from both refesens degradation and LNA blocking perspective.

## 2.3. CLI for Co-channel scenario

In the following section, we provide some initial overview for co-channel CLI from both gNB to gNB and UE to UE perspective.

### 2.3.1. Interference analysis from gNB perspective

- Figure 2.3.1-1. inter-site gNB-gNB co-channel inter-subband CLI

- Figure 2.3.1-2. co-site inter-sector gNB-gNB co-channel inter-subband CLI

- For inter-site gNB-gNB co-channel intra-subband CLI and co-site inter-sector gNB-gNB co-channel intra-subband CLI, this should be further discussed in RAN4 whether this scenario is feasible or not. At least from our understanding, it’s quite difficult to mitigate the co-channel intra-subband interference.

- Figure 2.3.1-3. inter-site gNB-gNB co-channel intra-subband CLI [ Low priority, this should be discussed in RAN1 firstly]

- Figure 2.3.1-4.co-site inter-sector gNB-gNB co-channel intra-subband CLI [ Low priority, this should be discussed in RAN1 firstly]

### 2.3.2. Interference analysis from UE perspective

- Figure 2.3.2-1. inter-site UE-UE co-channel inter-subband CLI

- Figure 2.3.2-2. co-site inter-sector UE-UE co-channel inter-subband CLI

- Similar as inter-site gNB-gNB co-channel intra-subband CLI and co-site inter-sector gNB-gNB co-channel intra-subband CLI, the following two scenarios (e.g.inter-site UE-UE co-channel intra-subband CLI and co-site inter-sector UE-UE co-channel intra-subband CLI ) should be further discussed in RAN4 whether this scenario is feasible or not. At least from our understanding, it’s quite difficult to mitigate the co-channel intra-subband interference.

- Figure 2.3.2-3. inter-site UE-UE co-channel intra-subband CLI [ Low priority, this should be discussed in RAN1 firstly]

- Figure 2.3.2-4. co-site inter-sector UE-UE co-channel intra-subband CLI [ Low priority,this should be discussed in RAN1 firstly]

Conclusions

In this contribution, we want to share initial views on from the self-interference and CLI for full duplex BS and observations and proposals are made as following:

- Observation 1: for FR1 full duplex BS, the following approach could be used to handle the self-interference:

- Antenna isolation from transmitter to receiver;

- Sub-band filtering of transmitter to further reject the leakage into the receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- Sub-band filtering of receiver to reject the power from the transmitter;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

- Observation 2: for FR2 full duplex BS, the following approach could be used to handle the self-interference:

- Antenna isolation from transmitter to receiver;

- ACLR of transmitter which is mainly determined by the PA performance and digital filtering implemented for DL;

- ACS of receiver to reject the power from the transmitter by digital filtering;

- Digital interference cancellation at receiver;

- RF interference cancellation;

Observation 3: it seems feasible to support the full duplex operation for Medium range BS.

Proposal 1 : for FR1 full duplex BS, to consider the self interference mitigation approaches as mentioned in table 2.2.1-1 to different BS class supporting the full duplex operation and its detailed value could be further studied.

Proposal 2: for FR1 full duplex BS, to check the feasibility from both refesens degradation and LNA blocking perspective.

Observation 4: it seems feasible to support the full duplex operation for Wide area BS with only the antenna isolation considered.

Proposal 3 : for FR2 full duplex BS, to consider the self interference mitigation approaches as mentioned in table 2.2.2-1 with the removal of sub-band filtering to different BS class supporting the full duplex operation and its detailed value could be further studied.

Proposal 4: for FR2 full duplex BS, to check the feasibility from both refesens degradation and LNA blocking perspective.

References

- RP-221352, Study on evolution of NR duplex operation, CMCC, Samsung, Approved

- TR38.828,Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)