3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213693

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	12.5.1

Source: 	ZTE Corporation

Title: 	Discussion on spec drafting for TS 36.108

Document for:	Discussion

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some initial views on spec skeleton for TS 36.108.

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

Discussion on spec drafting

The spec skeleton for 36.108 is proposed as following. Basically speaking, the skeleton is reused from TS 38.108 with some updates for system parameter part since sync raster is not needed for IoT over NTN. For requirements for eMTC over NTN and NB-IoT over NTN, the corresponding requirement could be drafted in the corresponding table just similar as TS 36.104.

3	Definitions, symbols and abbreviations	8

3.1	Definitions		8

3.2	Symbols		8

3.3	Abbreviations		9

4	General		9

4.1	Relationship with other core specifications	9

4.2	Relationship between minimum requirements and test requirements	9

4.3	Requirement reference points	9

4.3.1	Hybrid AAS SAN	9

4.3.2	OTA AAS SAN	10

4.4	Satellite Access Node classes	11

4.5	Regional requirements	11

4.6	Applicability of minimum requirements	11

5	Operating bands and channel arrangement	12

5.1	General		12

5.2	Operating bands	13

5.3	Satellite Access Node channel bandwidth	13

5.4	Channel arrangement	15

5.4.1	Channel spacing	15

5.4.2	Channel raster	15

5.4.3	Carrier frequency and EARFCN	16

6	Conducted transmitter characteristics	16

6.1	General		16

6.2	Satellite Access Node output power	17

6.2.1	General		17

6.2.2	Minimum requirement for hybrid AAS SAN	17

6.3	Output power dynamics	17

6.3.1	General		17

6.3.2	RE power control dynamic range	17

6.3.2.1	General		17

6.3.2.2	Minimum requirement for hybrid AAS SAN	17

6.3.3	Total power dynamic range	18

6.3.3.1	General		18

6.3.3.2	Minimum requirement for hybrid AAS SAN	18

6.4	Transmit ON/OFF power	18

6.5	Transmitted signal quality	18

6.5.1	Frequency error	18

6.5.1.1	General		18

6.5.1.2	Minimum requirement for hybrid AAS SAN	19

6.5.2	Modulation quality	19

6.5.2.1	General		19

6.5.2.2	Minimum Requirement for hybrid AAS SAN	19

6.5.3	Time alignment error	19

6.6	Unwanted emissions	19

6.6.1	General		19

6.6.2	Occupied bandwidth	20

6.6.2.1	General		20

6.6.2.2	Minimum requirement for hybrid AAS SAN	20

6.6.3	Adjacent Channel Leakage Power Ratio	20

6.6.3.1	General		20

6.6.3.2	Minimum requirement	21

6.6.4	Operating band unwanted emissions	22

6.6.4.1	General		22

6.6.5	Transmitter spurious emissions	22

6.6.5.1	General		22

6.6.5.2	Basic Limits	22

6.6.5.2.1	General transmitter spurious emissions requirements	22

6.6.5.2.2	Protection of the own Satellite Access Node receiver	23

6.6.5.2.3	Additional spurious emissions requirements	23

6.6.5.2.4	Co-location with other Satellite Access Nodes	24

6.7	Transmitter intermodulation	24

7	Conducted receiver characteristics	24

7.1	General		24

7.2	Reference sensitivity level	25

7.2.1	General		25

7.2.2	Minimum requirements for hybrid AAS SAN	25

7.3	Dynamic range	26

7.3.1	General		26

7.3.2	Minimum requirements for hybrid AAS SAN	26

7.4	In-band selectivity and blocking	27

7.4.1	Adjacent Channel Selectivity (ACS)	27

7.4.1.1	General		27

7.4.1.2	Minimum requirements for hybrid AAS SAN	27

7.4.2 	In-band blocking	28

7.5	Out-of-band blocking	29

7.5.1	General		29

7.5.2	Minimum requirements for hybrid AAS SAN	29

7.6	Receiver spurious emissions	30

7.6.1	General		30

7.6.2	Minimum requirement for hybrid AAS SAN	30

7.7	Receiver intermodulation	30

7.8	In-channel selectivity	30

7.8.1	General		30

7.8.2	Minimum requirements for hybrid AAS SAN	31

8	Conducted performance requirements	31

8.1	General		31

8.2	Performance requirements for PUSCH	32

8.3	Performance requirements for PUCCH	32

8.4	Performance requirements for PRACH	32

9	Radiated transmitter characteristics	32

9.1	General		32

9.2	Radiated transmit power	32

9.3	OTA Satellite Access Node output power	32

9.4	OTA output power dynamics	33

9.5	OTA transmit ON/OFF power	33

9.6	OTA transmitted signal quality	33

9.7	OTA unwanted emissions	33

9.8	OTA transmitter intermodulation	33

10	Radiated receiver characteristics	33

10.1	General		33

10.2	OTA sensitivity	33

10.3	OTA reference sensitivity level	33

10.4	OTA dynamic range	33

10.5	OTA in-band selectivity and blocking	33

10.6	OTA out-of-band blocking	34

10.7	OTA receiver spurious emissions	34

10.8	OTA receiver intermodulation	34

10.9	OTA in-channel selectivity	34

11	Radiated performance requirements	34

11.1	General		34

11.2	Performance requirements for PUSCH	34

11.3	Performance requirements for PUCCH	34

11.4	Performance requirements for PRACH	34

Annex A (normative): Reference measurement channels	35

Annex B (normative): Error Vector Magnitude 	36

Annex D (informative): Change history	38

Conclusions

In this contribution, we want to share some initial views on spec skeleton for TS 36.108 and proposals are made as following:

Proposal 1: to use the proposed spec skeleton as baseline for TS 36.108;

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN