3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213696

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	12.5.3

Source: 	ZTE Corporation

Title: 	Discussion on SAN RF requirements for IoT over NTN

Document for:	Discussion

Introduction

In RAN#95e meeting, the work item [RP-221556] on NB-IoT/eMTC core & performance requirements for NTN was approved as one of Rel-18 RAN4 package. In this contribution, we want to share some initial views on SAN RF requirement for IoT over NTN.

- Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]:

- Specification of a 200 kHz channel raster in bands where this is feasible.

- In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses.

- Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary.

- Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL)

  - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL)

Discussion

## 2.1. SAN type

As indicated the objective [1], it’s agreed to leverage the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands). For Rel-17 SAN types for NR over NTN, there are two SAN types defined, namely SAN type 1-H and SAN type 1-O. For IoT over NTN in Rel-18, we propose to define the same SAN types unless there is any specific reasons to preclude these types.

Proposal 1: to define SAN type 1-H and type 1-O for IoT over NTN;

## 2.2. eMTC

In the following table 1, we share some initial views on the SAN RF requirements for SAN supporting eMTC and details could be found as following:

Table 1. summary of the proposals for SAN RF requirement for eMTC over NTN

| Tx requirements | Applicability |
| --- | --- |
| Satellite Access Node output power | Reuse the same requirement for NR over NTN in TS38.108 |
| Output power dynamics |  |
| RE power control dynamic range | Reuse the same requirement of TS36.104 with modulation order up to 64QAM |
| Total power dynamic range | Reuse the Total power dynamic range of TS 36.104 |
| Transmit ON/OFF power | This is not applicable for FDD bands |
| Transmitted signal quality |  |
| Frequency error | Reuse the same requirement for NR over NTN in TS38.108 |
| Modulation quality | Reuse the same requirement for NR over NTN in TS38.108 with 64QAM as optional feature. |
| Time alignment error | This is not applicable similar as NR over NTN in TS38.108 |
| Unwanted emissions |  |
| Occupied bandwidth | Reuse the same requirement for NR over NTN IN TS38.108 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | Reuse the same requirement for NR over NTN in TS 38.108. |
| Operating band unwanted emissions | Reuse the same requirement for NR over NTN in TS 38.108. Note: to follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements |
| Transmitter spurious emissions | Reuse the same requirement for NR over NTN in TS38.108 Note: to follow ITU-R recommendation SM.329-12 (Unwanted emissions in the spurious domain), with the measurement bandwidth of 4 kHz. |
| Transmitter intermodulation | Not applicable similar as NR over NTN in TS 38.108 |
| Rx requirement |  |
| Reference sensitivity level | Alt 1: (more preferred since only Cat M1 is supported in Rel-17/18 eMTC over NTN ) - Reuse the same FRC A1-1 for 1.4MHz LTE BW for all carriers since only Cat M1 is supported in Rel-17/18 eMTC over NTN; - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Ruse the implementation margin as 2dB; Alt 2: (less spec impacts compared with Alt1, however less preferred since only Cat M1 is supported for eMTC over NTN) - Reuse the same FRC A1-1 for 1.4MHz, FRC A1-2 for 3MHz and and FRC A1-3 for 5MHz, 10MHz, 15MHz and 20MHz - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Ruse the implementation margin as 2dB; |
| Dynamic range | For GEO, to follow the same agreement without requirement defined. For LEO, Alt 1: (more preferred since only Cat M1 is supported in Rel-17/18 eMTC over NTN ) - Reuse the same FRC A2-1 for 1.4MHz LTE BW for all carriers since only Cat M1 is supported in Rel-17/18 eMTC over NTN; - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse IoT 15dB for NR over NTN - Ruse the implementation margin as 2.5dB; Alt 2: (less spec impacts compared with Alt1, however less preferred since only Cat M1 is supported for eMTC over NTN) - Reuse the same FRC A2-1 for 1.4MHz, FRC A2-2 for 3MHz and and FRC A2-3 for 5MHz, 10MHz, 15MHz and 20MHz - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse IoT 15dB for NR over NTN - Ruse the implementation margin as 2.5dB; |
| ACS | To reuse 38dBc for both GEO and LEO. For wanted signal power level PREFSENS+6dB FRC for wanted signal should follow the agreement of Reference sensitivity level |
| In-band blocking | No requirement defined similar as NR over NTN in TS38.108 |
| Out of band blocking | To reuse OOBB power level as -44dBm for both GEO and LEO. Note: the worst coexistence case (i.e. SpaceOps TC station at satellite Nadir) is applicable for both NR over NTN and IoT over NTN. For wanted signal power level PREFSENS+6dB FRC for wanted signal should follow the agreement of Reference sensitivity level |
| Receiver spurious emission | To reuse the requirements for NR over NTN in TS38.108 and follow the updates of TS 38.108 if necessary since this is under the discussion. |
| Receiver intermodulation | No requirement defined similar as NR over NTN in TS38.108 |
| In-channel selectivity | For GEO and LEO, to reuse ICS level as 12 dB for GEO, 24dB for LEO used for NR over NTN in Rel-17. For FRCs for eMTC over NTN, Alt 1: (more preferred since only Cat M1 is supported in Rel-17/18 eMTC over NTN ) - Reuse the same FRC A1-4 for 1.4MHz LTE BW for all carriers since only Cat M1 is supported in Rel-17/18 eMTC over NTN; - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Reuse the implementation margin as 2dB; Alt 2: (less spec impacts compared with Alt1, however less preferred since only Cat M1 is supported for eMTC over NTN) - Reuse the same FRC A1-4 for 1.4MHz, FRC A1-5 for 3MHz, FRC A1-2 for 5MHz and FRC A1-3 for 10MHz, 15MHz and 20MHz - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO |

## 2.3. NB-IoT

In the following table 2, we share some initial views on the SAN RF requirements for SAN supporting NB-IoT and details could be found as following:

Table 2. summary of the proposals for SAN RF requirement for NB-IoT over NTN

| Tx requirements | Applicability |
| --- | --- |
| Satellite Access Node output power | Reuse the same requirement for NR over NTN in TS38.108 |
| Output power dynamics |  |
| RE power control dynamic range | Not applicable, there is not requirement defined in TN BS. |
| Total power dynamic range | Not applicable, there is not requirement defined in TN BS. |
| Transmit ON/OFF power | This is not applicable for FDD bands |
| Transmitted signal quality |  |
| Frequency error | Reuse the same requirement for NR over NTN in TS38.108 |
| Modulation quality | Reuse the same requirement for NB-IoT defined in TS6.104 in principle and further discuss the 16QAM supported in Rel-17 should be also supported for NB-IoT over NTN. |
| Time alignment error | This is not applicable similar as NR over NTN in TS38.108 |
| Unwanted emissions |  |
| Occupied bandwidth | Reuse the same requirement for NR over NTN IN TS38.108 which is following ITU-R Recommendation SM.328. |
| Adjacent Channel Leakage Power Ratio | This depends on the outcome of coexistence study. |
| Operating band unwanted emissions | to follow Annex 5 of ITU recommendation SM.1541-6 to define SAN OBUE requirements and there might be some further adjustment based on the ACLR requirement |
| Transmitter spurious emissions | Reuse the same requirement for NR over NTN in TS38.108 Note: to follow ITU-R recommendation SM.329-12 (Unwanted emissions in the spurious domain), with the measurement bandwidth of 4 kHz. |
| Transmitter intermodulation | Not applicable similar as NR over NTN in TS 38.108 |
| Rx requirement |  |
| Reference sensitivity level | - Reuse the same FRC A14-1 for 15kHz and FRC A14-2 for 3.75kHz - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Ruse the implementation margin as 2dB; |
| Dynamic range | For GEO and LEO, to further study the IoT level to identify whether it’s necessary to define the dynamic range requirements; For FRC and other parameters, - Reuse the same FRC A15-1 for 15kHz and FRC A15-2 for 3.75kHz - Reuse the NF for NR over NTN. e.g. 7.4dB for GEO and 4.3dB for LEO - Ruse the implementation margin as 2.5dB; |
| ACS | This depends on the outcome of coexistence study. |
| In-band blocking | No requirement defined similar as NR over NTN in TS38.108 |
| Out of band blocking | To reuse OOBB power level as -44dBm for both GEO and LEO. Note: the worst coexistence case (i.e. SpaceOps TC station at satellite Nadir) is applicable for both NR over NTN and IoT over NTN. For wanted signal power level PREFSENS+6dB |
| Receiver spurious emission | To reuse the requirements for NR over NTN in TS38.108 and follow the updates of TS 38.108 if necessary since this is under the discussion. |
| Receiver intermodulation | No requirement defined similar as NR over NTN in TS38.108 |
| In-channel selectivity | No requirement should be defined similar as TN NB-IoT due to its narrow bandwidth. |

Conclusions

In this contribution, we want to share some initial views on SAN RF requirement for IoT over NTN and proposals are made as following:

Proposal 1: to define SAN type 1-H and type 1-O for IoT over NTN;

Proposal 2: to further discuss the SAN supporting eMTC over NTN as summarized in table 1;

Proposal 3: to further discuss the SAN supporting eMTC over NTN as summarized in table 2.

References

- RP-221556, WID on NB-IoT/eMTC core & performance requirements for NTN

- R1-2200042, LS on DL synchronization enhancements for IoT NTN, RAN1 (Jan-22)