3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213716

E-meeting,15th Aug – 26th Aug,2022

Agenda item:	9.5.5.1.3

Source: 	ZTE Corporation

Title: 	Discussions on NR repeater test cases

Document for:	Approval

Introduction

Based on the approved Rel-17 RAN4 led repeater WID RP-212129, the conformance testing for both NR repeater should be completed in Sep, 2022. In this contribution, we want to provide some further views how to select the RF channels for NR repeater similar as NR BS.

Discussion

In the last meeting, we reached the following agreement for RF channels of repeater conformance testing, in the following table, we elaborate this RF channel configurations in more detailed way.

| Issue 3-2-1: Further regarding the RF channels tested it has been proposed: Similar as the legacy NR BS conformance testing, we propose the following RF channels for NR repeater conformance testing where similar principle as NR BS is used for NR repeater (i.e. BMT etc) Tentative proposal: The RF channel tested can be further discussed when the outcome of the test model configuration is beter known GTW Agreement: - For the definition on RF channels, using the same principle of NR BS as baseline. |  |  |  |
| --- | --- | --- | --- |
|  | RF channel |  |  |
|  | Single carrier | Multi-carrier | Multi-carrier, Multi-band |
| Repeater output power | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Frequency stability Error Vector Magnitude | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Out of band gain | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Unwanted emissions, ACLR, spurious emission | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Input intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Output intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Adjacent Channel Rejection Ratio (ACRR) | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Transmit ON/OFF power | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Note: since there is no carrier related definition in repeater, carrier should be placed by nominal channel bandwidth of pass band. |  |  |  |

Conclusions

In this contribution, we want to provide some initial analysis how to select the test case for NR repeater similar as NR BS and proposals are made as following:

Proposal 1: propose to adopt the RF channels in above table for NR repeater conformance testing.

References

- TS 38.106 17.0.0

- TS 36.143 15.0.0

- TS 38.176-1 16.3.0

- TS 38.176-2 16.3.0

- R4-1813878, WF on RF channel for OTA testing, Nokia, approved.