3GPP TSG-RAN WG4 Meeting#104-e                               R4-2213723

E-meeting,15th Aug – 26th Aug,2022

Source:	ZTE Corporation

Title:	TP for TS 38.115-2: Section 6.8

Agenda Item:	9.5.5.3

Document for:	Approval

- Introduction

In terms of the work split in the approved WF[1]. This contribution provides a text proposal to TS38.115-2 [2] on sub-clause 6.8.

- Reference

- R4-2211153, WF on NR FR1/FR2 repeater test specs draft rules and TP splits, CATT, approved.

- R4-2210846,Spec skeleton for TS 38.115-2 v.0.0.1, ZTE Corporation. Approved.

- Text Proposal

----- Start of TP -----

6.8	OTA Adjacent Channel Rejection Ratio (ACRR)

<Text will be added.>

### 6.8.1	Definitions and applicability

OTA Adjacent Channel Rejection Ratio (ACRR) is the ratio of the average gain over a carrier of the repeater in the passband to the average gain of the repeater over an adjacent channel outside the repeater passband. The requirement shall apply to the uplink and downlink of the Repeater. The bandwidth of the channel inside the passband and the adjacent channel are assumed to be minimum {400MHz, passband BW}.

The requirement is differentiated between downlink and uplink.

The requirement shall apply during the transmitter ON state.

The ACRR is a ratio of gain in the adjacent channel to gain in the wanted channel. The gain in each case is defined as the ratio of TRP output power to directional input power.

### 6.8.2	Co-existence with NR

This requirement shall be applied for the protection of NR signals in geographic areas in which NR Repeater and NR BS are deployed so that they serve adjacent channels. The reference carrier is a NR carrier.

#### 6.8.2.1	Minimum requirements

The minimum requirement is in TS 38.106 [2] sub-clause 7.8.1.1.

#### 6.8.2.2	Test purpose

To verify that the Repeater OTA ACRR requirement is met as specified in sub-clause 6.8.2.1.

#### 6.8.2.3	Method of test

##### 6.8.2.3.1	Initial conditions

Test environment: 	normal; see Annex A2.

A measurement system set-up is shown in annex D.

1)	Connect the signal generator equipment to the Repeater input port.

2)	Connect the power measuring equipment to the Repeater output port.

3)	The measurement device characteristics shall be:

-	measurement filter bandwidth: defined in sub-clause 6.8.1;

-	detection mode: true RMS voltage or  true average power.

##### 6.8.2.3.2	Procedure

1)	Place the repeater with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.2.7.

2)	Align the manufacturer declared coordinate system orientation of the repeater with the test system.

3)	Align the repeater with the test antenna in the declared direction to be tested.

4)	Align the repeater to that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)	Set the signal generator to transmit a signal modulated with test model XX for downlink and  test model XX for uplink as defined in section 4.9 at the first or last channel with channel offset from frequency range of passband defined in section 6.8.2.3.3 within the pass band.

6)	Adjust the input power to the Repeater to create the maximum nominal Repeater output power at maximum gain

7)	Measure the RRC filtered mean power at the RF output port over a certain slot.

8)	Set the signal generator to transmit the same signal and the same input power at one of the channel offsets outside the repeater pass band according to Tables in section 6.9.2.3.3.

9)	Measure the RRC filtered mean power at the RF output port over a certain slot.

10)	Calculate the ratio of the measured power in the pass band to the measured power at the channel offset.

11)	Repeat step 8) to 10) until all channel offsets in Tables in section 6.9.2.3.3 are measured.

##### 6.8.2.3.3	Test Requirements

The requirement shall apply at the RIB when the AoA of the incident wave of a received signal in the passband and a received signal on an adjacent channel outside repeater passband is from the same direction and are the same as the TX reference direction for the opposite DL/UL setting.

For a repeater operating at passband operating in FR2, the ACRR requirements in table 6.8.2.3.3-1 shall apply in downlink. In normal conditions the ACRR for downlink shall be higher than the value specified in the Table 6.8.2.3.3-1.

Table 6.8.2.3.3-1: Repeater Downlink ACRR

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit (dB) |
| --- | --- | --- | --- |
| NR | Wide Area repeater | minimum {400MHz, passband BW}/2 | [28] (Note 2) [26] (Note 3) |
|  | Medium Range repeater | minimum {400MHz, passband BW}/2 | [28] (Note 2) [26] (Note 3) |
|  | Local Area repeater | minimum {400MHz, passband BW}/2 | [28] (Note 1, 2) [26] (Note 1, 3) |
| NOTE 1: This requirement is not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |

For a repeater operating at passband operating in FR2, the ACRR requirements in table 6.8.2.3.3-2 shall apply in uplink. In normal conditions the ACRR for uplink shall be higher than the value specified in the Table 6.8.2.3.3-2.

Table 6.8.2.3.3-2: Repeater Uplink ACRR

| Co-existence with other systems | Repeater Class | Channel offset from frequency edge of passband (MHz) | ACRR limit (dB) |
| --- | --- | --- | --- |
| NR | Wide Area repeater | minimum {400MHz, passband BW}/2 | [28] (Note 2) [26] (Note 3) |
|  | Local Area repeater | minimum {400MHz, passband BW}/2 | [17] (Note 1, 2) [16] (Note 1, 3) |
| NOTE 1: This requirement is not applicable if the passband occupies the entire operating band. NOTE 2: Applicable to bands defined within the frequency spectrum range of 24.25 – 33.4 GHz. NOTE 3: Applicable to bands defined within the frequency spectrum range of 37 – 52.6 GHz |  |  |  |

----- End of TP -----