3GPP TSG-RAN4 WG4 Meeting #104-e	R4-2213731

# Electronic meeting, August 15 - August 26, 2022

Source: 	Huawei, HiSilicon

Title: 	RF aspects of carrier phase measurements

Agenda Item:	11.14.3

Document for:	Approval

# Introduction

In RAN#96, the revised SID on Study on expanded and improved NR positioning was approved in RP-221814 [1], which contains the following study objective.

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

On carrier phase measurement, RAN1 already reached the following agreements in RAN1#109-e [2], which are are related to RF.

Agreement - For the purposes of discussion, for NR downlink and/or uplink carrier phase positioning, the carrier phase (CP) at a RF frequency at a receiver is a phase that is a function of the signal propagation time from an Tx antenna reference point of a transmitter (e.g., a TRP or a UE) to a Rx antenna reference point of the receiver (e.g., a UE or a TRP). - The propagation time can be expressed in a fractional part of a cycle of the RF frequency and a number of integer cycles, but the CP may be independent of the number of integer cycles. Agreement - In addition to the evaluation assumptions of NR Rel-16/17, the following error sources may also be considered during the evaluation: - Phase noise (FR2) - CFO/Doppler - Oscillator-drift - Transmitter/receiver antenna reference point location errors - Transmitter/receiver initial phase error - Phase center offset - Note: Other error sources are not precluded - Note: UE mobility can be considered in the evaluations - Note: one or more error sources can be evaluated jointly - Note: companies should provide the error sources model with their evaluations

# Discussion

## Carrier phase modeling

The mathematical derivation of carrier phase positioning can be expressed in the following using complex signal model. The Appendix also provides an equivalent derivation using the I/Q modulation.

The transmitted signal is given by:

|  | xt=stexpj2πfct+jϕ0 | (1) |
| --- | --- | --- |

where s(t) denotes the baseband signals, fc denotes the carrier frequency, ϕ0 denotes the RF initial phase at t=0. After passing the channel, the transmitted signal xt can be received at the receiver side.

The received signal is given by:

|  | yt=st-τ0expj2πfct-τ+jϕ0 | (2) |
| --- | --- | --- |

where τ0 denotes the propagation delay. By down-converting yt to the baseband with receive LO of exp-j2πfct-jϕ1, we have

|  | rt=ytexp-j2πfct-jϕ1=st-τexp-j2πfcτ+j(ϕ0-ϕ1) | (3) |
| --- | --- | --- |

where ϕ1 denotes the receiver RF initial phase at t=0.

The baseband phase can be measured as Φ=-2πfcτ0+ϕ0-ϕ1, where the term -2πfcτ is of interest because τ=dc with d being the distance between the transmit antenna and the receive antenna. However, given the phase measurement wraps every 2π, only the factional part, i.e. fcτ0-floorfcτ0 can be observed.

Due to the presence of the initial phase difference ϕ0-ϕ1 between the transmitter and receiver, Φ itself cannot be used to determine the UE location, differential methods and double differential methods are used to mitigate the impact of the initial phase difference between the transmitter and receiver, which are under RAN1 discussion.

Observation 1: The receive baseband phase can represent RF carrier phase measurement.

## Practical RF impairments

In practical, apart from the delay and the initial phase difference, other RF impairments may also be considered.

From the transmitter side, the signal after up-conversion needs to pass the filter and the PA, and the transmitted to the antenna. Additional RF group delay may be added to the intended signal when emitting from the Rx antenna.

-Pi/2

Σ

cos2πfct+ϕ0

-sin2πfct+ϕ0

Figure 1 Additional RF group delay at Tx side

The actual emitting signal can be expressed as

xTxAntt=st-tgTXexpj2πfct-tgTX+jϕ0

Likewise, in the Rx side, the filtering and the LNA may add additional RF group delay to the intended signal after received from the Rx antenna.

Observation 2: Tx RF and Rx RF may add additional group delay, which will further add the phase offset, resulting in the observed baseband Rx phase being the lump sum of the following phase components

- Phase corresponding to the fractional part of a wavelength in the propagation distance.

- Initial phase difference in the oscillators between Tx and Rx

- Additional RF delay caused by filters and amplifiers at both Tx and Rx

According to RAN1 analysis in R1-2205870 [3], those RF impairments incurred by the Tx and Rx can be mitigated using double difference methods with the form of

dd=mt1r1-mt2r1-mt1r2-mt2r2

where

- mtx,ry corresponds to the measurement for the link between Tx x and Rx y.

This is due to the fact that the impairments for a Tx is common to both Rx, and the impairments for a Rx is common to both Tx.

With the above understanding, we do not see any need to further define any RF requirements for carrier phase positioning, since the baseband measurement combination (via double difference methods) can overcome the impairment.

Proposal 1: The practical RF impairments can be mitigated by double difference methods with the phase measurement in the baseband, and thus will not define the RF requirement for carrier phase measurements.

- The carrier phase measurement requirement should be covered by RRM.

## Phase continuity over time

Typically, there should be no assumption on the phase continuity across slots except for some coverage enhancement scenarios, which do not apply to carrier phase positioning.

For the TDD spectrum, the interruption due to switching from DL to UL and back to DL may cause the random initial phase shift between the two DL transmission occasions.

For the FDD spectrum, unless the dedicated RF components are used for transmitting the reference signal for positioning purpose, it is not likely that the transmission phase continuity can be maintained, due to e.g. fluctuation of transmitting power across slots, which can be quite dynamic, subject to the cell load or UE load.

Proposal 2: No need to define the requirement of phase continuity across slots for carrier phase positioning for both TDD and FDD spectrum.

# Conclusion

In this contribution we discussed the RF impacts on positioning frequency aggregation, according to the analysis, we have the following observations and proposals:

Observation 1: The receive baseband phase can represent RF carrier phase measurement.

Observation 2: Tx RF and Rx RF may add additional group delay, which will further add the phase offset, resulting in the observed baseband Rx phase being the lump sum of the following phase components

  - Phase corresponding to the fractional part of a wavelength in the propagation distance.

  - Initial phase difference in the oscillators between Tx and Rx

  - Additional RF delay caused by filters and amplifiers at both Tx and Rx

Proposal 1: The practical RF impairments can be mitigated by double difference methods with the phase measurement in the baseband, and thus will not define the RF requirement for carrier phase measurements.

  - The carrier phase measurement requirement should be covered by RRM.

Proposal 2: No need to define the requirement of phase continuity across slots for carrier phase positioning for both TDD and FDD spectrum.

# References

- RP-221814, Revised SID on Study on expanded and improved NR positioning, Intel, CATT, Ericsson, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6 - 9, 2022.

- RAN1, Chair’s Notes RAN1#109-e v19, RAN1#109-e, e-Meeting, 9th – 20th May, 2022.

- R1-2205870, Evaluation and solutions for NR carrier phase positioning, Huawei, HiSilicon, RAN1#110, Toulouse, France, 22nd – 26th Aug., 2022.

# Appendix

DAC

DAC

-Pi/2

Σ

I

Q

xItcos2πfct+ϕ0-xQtsin2πfct+ϕ0

ADC

ADC

I

Q

xIt-τcos2πfct-τ+ϕ0-xQt-τsin2πfct-τ+ϕ0

Delay τ

cos2πfct+ϕ1

-sin2πfct+ϕ1

12xIt-τcos2πfcτ+ϕ1-ϕ0+xQt-τsin2πfcτ+ϕ1-ϕ0+j12-xIt-τsin2πfcτ+ϕ1-ϕ0+xQt-τcos2πfcτ+ϕ1-ϕ0=12xIt-τe-j2πfcτ+ϕ0-ϕ1+jxQt-τe-j2πfcτ+ϕ0-ϕ1=12xIt-τ+jxQt-τexp-j2πfcτ+ϕ0-ϕ1

12xIt-τcos2πfcτ+ϕ1-ϕ0+xQt-τsin2πfcτ+ϕ1-ϕ0

12-xIt-τsin2πfcτ+ϕ1-ϕ0+xQt-τcos2πfcτ+ϕ1-ϕ0

cos2πfct+ϕ0

-sin2πfct+ϕ0

-Pi/2

xIt

xQt

Figure 3 Illustration of carrier phase measurement using I/Q branches