3GPP TSG-RAN WG4 Meeting #104-e                                      R4-2213737

Electronic Meeting, August 15 – 26, 2022

Source: 	Huawei, HiSilicon

Title: 	On UE RF requirement for FR2 multi-Rx chain DL reception

Agenda Item:	11.8.2

Document for:	Approval

Introduction

As the outcome of RAN#96 discussion, the latest approved Rel-18 WI for FR2 multi-Rx chain DL reception [1] could be found as below:

- 4.1 Objective of SI or Core part WI or Testing part WI - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - The starting point is the enhancements related to L1-RSRP measurement enhancements - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTE: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited. - 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - RRM Performance requirements and test cases corresponding to the new core requirements - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused - UE demodulation requirements: - Specify performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier. - Demod requirements on up to 4 DL MIMO layers - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined NOTE: − For the dual-TCI demodulation requirement case, focus on UEs supporting “simultaneousReceptionDiffTypeD-r16” UE capability, and for the 2- or 4-layer downlink MIMO reception, focus on UEs supporting the basic mTRP CSI reporting capability (FG 23-7-1/1b of NR FeMIMO).

In this contribution, we would like to share our views on this topic for the first meeting of this WID.

Discussion

Initial considerations on requirements

Principle for the discussion

Since Rel-15, RAN4 shares the understanding that one assumption for FR2 RF requirements derivation is single activated panel. Meanwhile, there is no limitation on the total number of panel in UE. For instance, there could be a single panel UE, or a multiple panels UE with only one activated panel at a time. As long as such UE can satisfy those RF requirements.

Observation 1: The assumption for current RAN4 RF requirements is single activated panel, but there is no specific limitation of the UE implementation, e.g. total panel number and layout.

Although this Rel-18 WID is established for enabling UE simultaneously reception of two different QCL Type-D RSs, we think all discussion shall carefully consider the existing design and product. For example, a legacy multi-panel equipped UE could support the new feature only by software upgrade, rather than hardware stacking.

Proposal 1: All existing implementation shall be carefully considered when the general RF requirements will be defined for simultaneous DL reception with two different QCL Type-D RSs on single component carrier.

On the “up to 4 layer MIMO”

We understand for a UE which is capable of simultaneous reception from two different Rx beam (QCL Type-D), 4 layer MIMO can be supported since there could be 2 panels in total with 4 independent polarization directions. But based on the following agreements in Rel-15, new Demod requirements and test cases, instead of RF requirements, shall be considered for 4 layer MIMO. So we would like to emphasize the following proposal:

Agreements: - In Rel-15 UE in the 28GHz mmWave range NR - REFSENS requirement shall be based on DL MRC diversity (rank 1 with two receivers). - FFS whether this apply to all directions. - UE architecture shall support minimum dual layer (same as LTE rank 2) for demodulation performance requirement. - FFS whether this apply to all directions. - RF architecture support for higher order DL layers is optional.

Proposal 2: For simultaneous DL reception with two different QCL Type-D RSs on single component carrier,	the legacy spherical coverage requirement for reception from a single direction shall be kept.

- Demod requirements and test cases for 4 layer MIMO could be further considered.

About the “spherical coverage requirement”

We think there are mainly two factors impacting the “spherical coverage requirement”.

Factor 1. Angular offset between two AoAs

According to the objective of this WID, the spherical coverage requirement will be defined based on 2 AoAs. Apparently the relative angular offset between those 2 AoAs has strong correlation with the definition of this requirement and even the test method design.

Observation 2: The correlation between the following two factors is very strong:

- relative angular offset between 2 AoAs

- definition of the spherical coverage requirement and the test method design

Because from cellular network topology perspective, TRP location is fixed. So the relative angular offset between two LOS links from different TRP to a single UE must be distributed within a certain range, although the UE could be randomly distributed within each cell. Besides, given that the UE’s fixed panel layout, it must have different spherical coverage performance towards different AoA pair.

Unfortunately, there seems no agreed assumption during RAN1 related discussion about m-TRP operation in Rel-16/17 for the aforementioned offset. To better converge the RAN4 discussion by figuring out the reasonable angular offset range between 2 AoAs, that obvious gain from simultaneous reception from m-TRP can be observed, we think it is better to ask RAN1 to provide more information on valid range of UE distribution which can support m-TRP with up to 4 layers based on their SLS evaluations.

Proposal 3: Send an LS to RAN1 for asking more background info at least about:

- All necessary SLS assumptions to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on.

- The valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS.

Factor 2. Reception power imbalance between the DL signals from 2 AoAs

Currently, the EIS spherical coverage requirement for single band is defined as below (taking PC3 as an example):

It means that a qualified UE needs to meet (or exceed) the specified EIS in dBm at 50th percentile of the CCDF composed of all EIS values measured over the full sphere around the UE.

While we think the 50% percentile could still be applied as the baseline, more details on the potential form of the new spherical coverage requirement could be further discussed. The following figures are provided for better illustration of potential spherical coverage issues:

Fig. 1 Better spherical coverage but with large power imbalance

Fig. 2 Worse spherical coverage but with small power imbalance

As depicted in above two figures, with the same angular offset between 2 AoAs (the TRP topology and UE position remain unchanged), the DL performance could be different since the antenna gain could be different towards the 2 AoAs.

For the case in Fig. 1, the UE can achieve very good spherical coverage (very small uncovered area around the whole sphere). Such UE can achieve almost peak EIS on the direction towards TRP 2, while for TRP 1 direction, UE has low antenna gain comparing to the direction towards TRP 2. Consequently, the DL performance will not be good since the reception power imbalance can be quite large judging from the fact that the 50% EIS spherical coverage requirement is about 10dB lower than the peak EIS.

For the case in Fig. 2, the UE who has worse spherical coverage compared to case in Fig 1 due to the relative larger uncovered area, may achieve better simultaneous DL reception performance because peak EIS can be achieved on both directions towards two TRPs, respectively.

Fig. 3 The same spherical coverage as in Fig. 2 but with worst simultaneous DL reception performance

Another example which could exist during handover procedure is provided in Fig. 1c). In this case, the worse simultaneous DL reception performance can be expected since the reception power on any of two directions is low.

As we can see, the new requirement which will be introduced for this feature shall be discussed further considering the angular offset and the reception power imbalance of the DL signals between two AoAs, which is also related to the UE implementation with multiple panels.

Proposal 4: Taking the current 50%-tile EIS spherical coverage requirement (for PC3) for single band as the baseline, the new spherical coverage requirement shall be further discussed at least considering the following two factors:

- angular offset between two AoAs

- reception power imbalance of the DL signals between two AoAs

From our understanding, a UE may be capable of DL reception bases on 4 polarization direction within a Y%-tile subset of the 50%-tile EIS spherical coverage area, and logically a (negative) delta REFSENS dBm could be introduced depends on how much the polarization diversity or MIMO gain will be. But how to derive the Y%-tile subset and the delta value shall be further analyzed which may involve some SLS level evaluation.

Proposal 5: Simulation is needed to discuss whether and how new spherical coverage requirement can be specified.

Initial consideration on the test method for 2 AoAs

As defined in the TR 38.810, during the existing EIS spherical coverage test procedure, TE will traverse all 266 grids around the sphere to obtain the CDF curve. Though it is still early to touch the details of the new test method, since it is obviously cannot be decoupled with the definition of the new RF requirements, we think it is very intuitive that the procedure will become quite time consuming. The reason is that TE will measure the EIS 266*265 times if no simplification will be introduced. Regardless how the potential RF requirements along with the new test method for 2 AoAs will be defined, it is necessary to consider how to simplify the new test method.

Proposal 6: The simplification of the new test method for 2 AoAs shall be further considered.

Conclusion

In this contribution we discussed on the WI for FR2 multi-Rx chain DL reception. According to the analysis, we have the following observations and proposals:

Observation 1: The assumption for current RAN4 RF requirements is single activated panel, but there is no specific limitation of the UE implementation, e.g. total panel number and layout.

Observation 2: The correlation between the following two factors is very strong:

- relative angular offset between 2 AoAs

- definition of the spherical coverage requirement and the test method design

Proposal 1: All existing implementation shall be carefully considered when the general RF requirements will be defined for simultaneous DL reception with two different QCL Type-D RSs on single component carrier.

Proposal 2: For simultaneous DL reception with two different QCL Type-D RSs on single component carrier,	the legacy spherical coverage requirement for reception from a single direction shall be kept.

- Demod requirements and test cases for 4 layer MIMO could be further considered.

Proposal 3: Send an LS to RAN1 for asking more background info at least about:

- All necessary SLS assumptions to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on.

- The valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS.

Proposal 4: Taking the current 50%-tile EIS spherical coverage requirement (for PC3) for single band as the baseline, the new spherical coverage requirement shall be further discussed at least considering the following two factors:

- angular offset between two AoAs

- reception power imbalance of the DL signals between two AoAs

Proposal 5: Simulation is needed to discuss whether and how new spherical coverage requirement can be specified.

Proposal 6: The simplification of the new test method for 2 AoAs shall be further considered.

- References

- R4-221753, “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #96, Budapest, Hungary, June 6 – 9, 2022.

- R4-1706067, “WF on UE Reference Architecture”, Sony, Skyworks Solutions Inc., LG Electronics, 3GPP TSG-RAN4 Meeting #83, Hangzhou, China, May 25 – 29, 2017.