3GPP TSG RAN WG4 Meeting #104-e 		R4-2213740

Electronic Meeting, August 15 – 26, 2022

Title:	[Draft] Reply LS reply on the UE power limitation for STxMP in FR2

Response to:	R1-2205639

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:	-

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1	Overall description

RAN4 would like to thank RAN2 for the LS and the following questions:

Regarding UE power control for STxMP in FR2, RAN1 has two following assumptions on power limitation so far: - Assumption 1: Power limitation per panel for STxMP - Assumption 2: A total power limitation per UE over all UE panels used for STxMP Above power limitation includes both total radiated power and EIRP, and scenarios of these assumptions include at least single carrier scenario in FR2. RAN1 seeks a few answers from RAN4 on the following questions in order to proceed further on the study of UE power control for STxMP. Question 1: From RAN4 perspective, is Assumption 1 is feasible? Question 2: From RAN4 perspective, is Assumption 2 is feasible? Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)?

Regarding the single carrier scenario in which RAN1 is interested, the response from RAN4 is:

Answer to Question 1 & Question 2:

The power limitation mechanism, e.g. P-MPR reporting for MPE is defined per UE, which has been discussed and not changed in Rel-17 WI FeMIMO. Furthermore, the basic assumption for RAN4 requirements definition is single panel and they will be verified per band. So for the single carrier scenario, ONLY assumption 2 is feasible from RAN4 perspective.

Since only Assumption 2 is feasible, the answer to Question 3 could be provided as below:

As explained above, for single carrier scenario, the total power limitation per UE over all UE panels used for STxMP shall not exceed the existing power limitation for a given power class.

- 2	Actions

To RAN1

ACTION: 	RAN4 respectfully asks RAN1 to consider the above responses during their discussion.

- 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #104-e		August 15 – August 26, 2022		Electronic Meeting

TSG RAN WG4 Meeting #104bis-e		October 10 – October 19, 2022		Electronic Meeting