3GPP TSG-RAN WG4 Meeting # 104-e      	R4-2213745

Electronic Meeting, August 15 – August 26, 2022

Agenda Item:	12.5.5

Source: 	CMCC

Title: 			 Discussion on RRM core requirements for LTE NB-IoT NTN and eMTC NTN

Document for:	Discussion

- Introduction

In RAN#95-e meeting, the work item on NB-IoT/eMTC core & performance requirements for NTN has been approved. According to the latest WID [1], the RRM core requirements should be specified. We copy the objective as below for information:

- The objective of this Work Item is to specify RF and RRM requirements for NB-IoT and eMTC operation over NTN functionality defined in the Rel-17 LTE_NBIOT_eMTC_NTN work item, covering the following aspects: - Specification of RF requirements for Satellite Access Node (SAN) and UE including the following [RAN4]: - Specification of a 200 kHz channel raster in bands where this is feasible. - In bands where it is not feasible to define a 200 kHz channel raster, the specification of a 100 kHz channel raster to be used in conjunction with signalling of the “part-of EARFCN” indication on MIB, with multiple EARFCN hypotheses. - Verification of co-existence of IoT NTN with TN, re-using or extrapolating from existing coexistence results (from NR NTN or other) where appropriate, and considering additional simulations as necessary. - Leveraging the studies and requirements (where applicable) of NTN NR bands n256 and n255 (and any relevant E-UTRA bands), specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation: - S-band (1980-2010 MHz in UL, and 2170-2200 MHz in DL) - L band (1626.5 MHz – 1660.5 MHz in UL, and 1525 MHz – 1559 MHz in DL) - Specification of Radio Resource Management core requirements for the UE [RAN4]. - All requirements specified as part of this WI shall be Release-independent from Rel-17, and generic requirements defined for the above frequency bands shall be used as a basis for definition (in a later Release-independent work item) of requirements for future E-UTRA bands for IoT NTN operation. - The specification work of this WI shall: - Be limited to FDD requirements applicable for NB1/NB2 and Cat-M1 UE categories (HD-FDD and FD-FDD for Cat-M1). - Re-use the framework and requirements from NB-IoT/eMTC, as well as NR NTN requirements, where applicable. - NOTE: Rel-17 IoT NTN specifications do not cover non-NTN NB-IoT/eMTC functionality defined later than Rel-16.

In this contribution, we identify the RRM impact considering the agreements in Release 17 Work Item LTE_NBIOT_eMTC_NTN.

- RRM impact of R17 IOT NTN feature

Scenario

Based on the WID, R17 IOT-NTN ATG work item focus on FR1 FDD scenario. Therefore, the TDD related requirements are not applicable to R17 IOT-NTN.

Proposal 1: TDD related requirements are not applicable to R17 IOT-NTN.

UE pre-compensation of UL timing offset

According to the agreement in RAN1#106-e, UE perform UL timing pre-compensation is supported.

Agreement: The following agreement from NR NTN are re-used for IoT NTN as working assumption - In Rel-17 IoT-NTN, at least support UE which can compute timing advance and frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes Agreement: The following agreements from NR NTN are re-used for IoT NTN as working assumption. - Timing Advance formula can be transposed to IoT-NTN with Ts used instead of Tc The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Ts Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance.

Therefore, the reference point for the UE initial transmit timing control requirement and timing advanced adjustment requirement shall be revised. In addition, the additional error due to GNSS error and satellite position calculation error should be introduced to Te. We think the Te extended value can reuse NTN assumption in each case.

Proposal 2: The reference point for the UE initial transmit timing control requirement and timing advanced adjustment requirement shall be revised

Proposal 3: Te should be relaxed, the extended value can reuse NTN assumption in each case

Cell stop time-assisted cell reselection

According to the agreement in RAN2#116-e, cell stop time-assisted cell reselection is supported.

Agreement: - For quasi-earth fixed cell, UE should start measurements on neighbour cells before the broadcast stop time of the serving cell, i.e the time when the serving cell stops covering the current area, and the exact time to start measurements (inter and intra-frequency) is up to UE implementation. FFS to what extent this need to be covered in the TS. - The timing information on when a serving cell is going to stop serving the area is broadcast in the same SIB as the ephemeris information. - Broadcast of the timing information on when a serving cell is going to stop serving the area is only applicable to quasi earth fixed cell (not to moving cell). - Location-assisted cell reselection (e.g. as for NR NTN) is not supported for IoT NTN in rel 17.The following agreement from NR NTN are re-used for IoT NTN as working assumption

Similar to NR NTN, when the case of cell stop time is broadcasted and applicable, the UE cell-reselection behavior and other restrictions should be further decided and specified.

Proposal 4: For cell-reselection requirement, when the case of cell stop time is broadcasted and applicable, the UE cell-reselection behavior and other restrictions should be decided and specified.

Unknow cell case in paging reception requirement.

The unknow cell case is also valid for IOT-NTN, since IOT-NTN can operate in NGSO deployment and the cell stop time-assisted cell reselection approach is introduced. We think the definition can reuse NR NTN’ as follow:

The target cell is considered as known if it has been detectable during Tdetect, and the time span between SIB broadcasting cell stop time and the cell stop time is not less than Ttrigger. Otherwise, the cell is considered as unknown

The maximum interruption in paging reception should be relaxed under the unknow cell case. The extended time can reuse NTN’s assumption which is 3Ttarget_cell_SMTC_period.

Proposal 5: The maximum interruption in paging reception should be relaxed under the unknow cell case.

Measurements for eMTC-NTN

For eMTC-NTN, UE is capable of measure on neighboring cells. For measurements of cells belonging to different satellite as the serving cell and performed outside the MG, if UE don’t support parallel measurements capability, for L1 RLM measurements, a scaling factor should be used to account overlapping between L1 resources and SMTC for L3 measurements. For L3 measurements, a factor should be introduced to account overlapping between the associated SMTC and L1 resources

Proposal 6: For eMTC-NTN, if the measurements of cells belonging to different satellite as the serving cell and performed outside the MG, and UE don’t support parallel measurements capability

- For L1 RLM measurements, a scaling factor should be used to account overlapping between L1 resources and SMTC for L3 measurements.

- For L3 measurements, a factor should be introduced to account overlapping between the associated SMTC and L1 resources

- Conclusion

In this contribution, we identify potential RRM impact R17 IOT NTN features. The proposals are:

Proposal 1: TDD related requirements are not applicable to R17 IOT-NTN.

Proposal 1: TDD related requirements are not applicable to R17 IOT-NTN.

Proposal 2: The reference point for the UE initial transmit timing control requirement and timing advanced adjustment requirement shall be revised

Proposal 3: Te should be relaxed, the extended value can reuse NTN assumption in each case

Proposal 4: For cell-reselection requirement, when the case of cell stop time is broadcasted and applicable, the UE cell-reselection behavior and other restrictions should be decided and specified.

Proposal 5: The maximum interruption in paging reception should be relaxed under the unknow cell case.

Proposal 6: For eMTC-NTN, if the measurements of cells belonging to different satellite as the serving cell and performed outside the MG, and UE don’t support parallel measurements capability

- For L1 RLM measurements, a scaling factor should be used to account overlapping between L1 resources and SMTC for L3 measurements.

- For L3 measurements, a factor should be introduced to account overlapping between the associated SMTC and L1 resources

- Reference

[1] RP-220938, New WID on NB-IoT/eMTC core & performance requirements for NTN, MediaTek Inc.