3GPP TSG-RAN WG4 Meeting #104-e	R4-2213746

Electronic Meeting, August 15th - August 26th, 2022

Agenda Item:	9.23.1

Source:	MediaTek Inc.

Title:	Discussion on the remaining issues for SDT

Document for:	Discussion

# Introduction

In the last RAN4 meeting, the agreements on the RRM requirements for SDT WI [1] were achieved and captured in the WF [2]. In this contribution, we discuss the remaining issues as a part of the maintenance, which are captured below:

Issue 1-3-1: Does UE required to meet the EMR measurement requirements during subsequent SDT transmission? - Proposals - Option 1: Yes - Option 2: No Agreement: FFS Option 2. However, RAN4 seek for RAN2’s confirmation on whether or not Section 5.7.8 of RRC specs covers EMR requirements for the sake of correct interpretation on the RAN2 agreements “The UE is not required to perform Idle/inactive measurements in section 5.7.8 of RRC during SDT. Check the details of this requirements”. Issue 1-4-6: In RAN4 understanding, for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation. Should this be captured into specs? - Option 1: Yes - Option 2: No 2nd round discussion: No consensus reached. Moderator suggests to continue discussion in the next meeting and focus on other more important issues in this meeting. Agreement: Further discuss in next meeting

# Discussion

Issue 1-3-1 in the WF is related to the UE measurement requirements during subsequent SDT transmission. In the previous meetings, RAN4 has reached the agreement that UE is not required to meet inter-frequency or inter-RAT measurements during subsequent SDT transmission. However, the remaining issue here is whether to include EMR measurements to the previous agreement, i.e., whether UE is also not required to meet EMR measurements during subsequent SDT transmission.

In our understanding, RAN4 previous agreement should also apply to EMR measurements and to be aligned with RAN2 previous agreement that “UE is not required to perform Idle/inactive measurements in section 5.7.8 of RRC during SDT transmission” as shown below. It is noticed that section 5.7.8 already includes EMR measurements. However, RAN4 in the last meeting, agreed to seek RAN2 confirmation in [3] regarding whether section 5.7.8 covers EMR measurements.

Agreements

- When CG-TAT expires, MAC shall release the CG resources. RRC keeps the CG configuration (for delta signalling).

- Add a condition that RNAU is only initiated if Txxx (i.e. the new SDT timer) is not running

- The UE is not required to perform/log measurements during SDT

- The UE is not required to perform Idle/inactive measurements in section 5.7.8 of RRC during SDT.  Check the details of this requirements

- Delta signalling is based on the previous SDT configuration (i.e. only applicable to SDT operation and will be released when the UE moves to connected and hence delta configuration based on connected mode CG configuration is not supported).  FFS other details

- for ROHC continuity, update the running CR as follows:

- “the cell for ROHC continuity is the PCell where the UE receives the RRCRelease message”

Observation #1: RAN2 has agreed that UE is not required to perform measurements in section 5.7.8 of RRC during SDT transmission, which already includes EMR measurements.

Proposal #1: RAN4 can either agree on that UE is not required to meet EMR measurements requirements during subsequent SDT transmission or wait for RAN2 confirmation.

The other remaining issue in the WF is Issue 1-4-6 which is regarding whether to capture the following RAN4 understanding in the specs: for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation.

This is a common understanding in RAN4, where the specs don’t capture it for the legacy requirements either (e.g., cell reselection). Therefore, we don’t think we need to capture this in the specs for SDT.

Observation #2: The following is a common understanding in RAN4, and the specs don’t capture it for the legacy requirements either: for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation.

Proposal #2: No need to capture the following understanding in the specs for SDT since it is a common understanding in RAN4, where the specs don’t capture it for the legacy requirements either (e.g., cell reselection): for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation.

# Summary

In this contribution we have discussed the remaining issues from the previous WF on NR SDT. The following observations were approached:

Observation #1: RAN2 has agreed that UE is not required to perform measurements in section 5.7.8 of RRC during SDT transmission, which already includes EMR measurements.

Observation #2: The following is a common understanding in RAN4, and the specs don’t capture it for the legacy requirements either: for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation.

Furthermore, the following proposals have been introduced:

Proposal #1: RAN4 can either agree on that UE is not required to meet EMR measurements requirements during subsequent SDT transmission or wait for RAN2 confirmation.

Proposal #2: No need to capture the following understanding in the specs for SDT since it is a common understanding in RAN4, where the specs don’t capture it for the legacy requirements either (e.g., cell reselection): for UE performing Rx beam sweeping, the UE should select the largest RSRP value from multiple measured samples from Rx beam sweeping for the same SSB to perform TA validation.

# References

- RP-212594, Revised WID on NR small data transmissions in INACTIVE state, ZTE.

- R4-2210622, “Way forward on RRM requirements for NR SDT”, ZTE.

- R4-2211122, “reply LS on TA validation for CG-SDT”, ZTE.