3GPP TSG-RAN WG4 Meeting #104-e	R4-2213747

Electronic Meeting, August 15th - August 26th, 2022

Agenda item:		9.8.2.2

Source:	MediaTek Inc.

Title:	Discussion on the testability of FR1+FR2 TCs for HO with PSCell

Document for:		Discussion

# 1	Introduction

In the last RAN4 meeting, the agreements on the RRM requirements for HO with PSCell were achieved and captured in the WF [1]. In this paper, we provide our view on the remaining issue in the performance part regarding the testability of FR1+FR2 TCs as captured in the WF below:

<Way forward/Agreement>: Issue 2-1-1: Test cases design principle - FR1+FR2 test cases - Option 1: Test case design is delayed until testability issues are solved - Option 2: Introduce the test case and define the applicability - Option 3: Do not introduce the test

# 2	Discussion

In the last meeting, RAN4 has agreed on six different TCs for HO with PSCell in the performance part, which are all in FR 1 as shown below [1]:

Issue 2-5: Test cases list for HO with PSCell - Option 1 - TC #1: From SA to EN-DC (All cells are in FR1) - PCell: unknown target PCell - PSCell: unknown target PSCell - Purpose: To verify delay requirements for sequential processing - TC #2: From SA to EN-DC (All cells are in FR1) - PCell: known target PCell - PSCell: known target PSCell - Purpose: To verify delay requirements for parallel processing - TC #5: From NE-DC to NE-DC (All cells are in FR1) - PCell: Intra-frequency handover from FR1 to FR1, known target PCell - PSCell: known target PSCell - Purpose: To verify delay requirements for known PCell and known PSCell combination - TC #6: From NE-DC to NE-DC (All cells are in FR1) - PCell: Inter-frequency handover from FR1 to FR1, unknown target PCell - PSCell: unknown target PSCell - Purpose: To verify delay requirements for unknown PCell and unknown PSCell combination - TC #9: From EN-DC to EN-DC (All cells are in FR1) - PCell: E-UTRA intra-frequency handover, known target PCell - PSCell: known target PSCell - Purpose: To verify delay requirements for known PCell and known PSCell combination - TC #11: From EN-DC to EN-DC (All cells are in FR1) - PCell: E-UTRA inter-frequency handover, unknown target PCell - PSCell: unknown target PSCell - Purpose: To verify delay requirements for unknown PCell and unknown PSCell combination

However, the TCs for FR1+FR2 are still open for discussion as shown in the WF:

Issue 2-1-1: Test cases design principle - FR1+FR2 test cases - Option 1: Test case design is delayed until testability issues are solved - Option 2: Introduce the test case and define the applicability - Option 3: Do not introduce the test

In our view, FR1+FR2 TCs can be postponed until the testability issue is resolved.

Proposal 1: For FR1+FR2 test cases, the test case design is delayed until testability issues are solved.

# 3	Summary

In this contribution, the following proposal is introduced:

Proposal 1: For FR1+FR2 test cases, the test case design is delayed until testability issues are solved.

# Reference

- R4-2210583, “WF on further RRM enhancement for NR and MR-DC – HO with PSCell”, vivo.