3GPP TSG-RAN WG4 Meeting #104-e	R4-2213748

Electronic Meeting, August 15th - August 26th, 2022

Agenda Item:	11.17.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for MUSIM gaps

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. The absence of RRM requirements in the current MUSIM implementations may create unpredictable performance for both Network A and Network B on SIM A and SIM B, respectively. In order to guarantee network performance for MUSIM operation, it is important to define the RRM requirements for it. This was therefore captured in the objective of Rel-18 MUSIM WID [1] shown below:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2]. 2. Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2] - The following MUSIM gap requirements are considered - Measurements in Network A - Measurements in Network B in RRC idle/inactive - Note: it is up to RAN4 decision whether to define requirements for Network B. - Identify and specify, if needed, solutions for MUSIM gap collision handling for the following cases [RAN4, RAN2] - Case 1: Collisions between MUSIM gap and legacy measurement gap (i.e., Rel-15 to Rel-17 measurement gaps) - Case 2: Collisions between MUSIM gap and SMTC - Case 3: Collisions between different MUSIM gaps - Note: RAN2 work can be triggered by RAN4 LS only, if needed - Identify impacts on L1 measurements, RLM/BFD and L3 measurements and specify corresponding UE requirements, if necessary, when MUSIM gap(s) are configured, for the following scenarios [RAN4] - Only MUSIM gap(s) are configured - MUSIM gap(s) and legacy measurement gap are configured - Note: requirements are applicable to MUSIM gaps defined in Rel-17 MUSIM WI (LTE_NR_MUSIM)

# Discussion

The first objective of the MUSIM WID in Rel-18 is to enhance MUSIM procedures to operate in RRC_CONNECTED state simultaneously in Network A (NW A) and Network B (NW B), for which RAN4 impact will be identified in RAN#99. Therefore, in this meeting, we focus on the second objective of the MUSIM WID, which is that to define RRM requirements for Rel-17 MUSIM gaps.

## RRM requirements for MUSIM gaps

When UE implementation supports MUSIM capability, in which SIM A would be on NW A and SIM B would be on NW B, UE can request MUSIM gaps from NW A to monitor NW B activities (paging, measurements, SI reading, etc.). Figure 1 shows MUSIM procedures when the UE is in RRC_CONNECTED state on NW A and in RRC_IDLE/_INACTIVE state on NW B.

NW A

NW B

Paging/measurements/ SI reception

2. Reconfigure gap

1. Request gap

0. Data

SIM A

SIM B

Figure 1: MUSIM procedures.

Based on the UE observations for NW B configurations, UE can request from NW A appropriate MUSIM gap patterns (up to 4 MUSIM gaps, 3 periodic and 1 aperiodic) as defined in Table 9.1.10-1 of 38.133 [2]. Then NW A may comply with the UE request and provide the required configurations for MUSIM gaps.

Observation #1: NW A can reconfigure the UE with up to 4 MUSIM gaps (3 periodic and 1 aperiodic).

The new configured MUSIM gaps may collide with the legacy measurement gaps (MGs) and/or SMTC window configured for NW A when the UE is in RRC_CONNECTED state on NW A. This collision could lead into unpredictable measurement delay and performance on NW A. As a result, new requirements are required to be introduced for the measurements performed in NW A, which can include:

- Intra-frequency measurements

- Inter-frequency measurements

- Inter-RAT measurements.

Proposal #1: Introduce new requirements for intra-/inter-frequency and inter-RAT measurements in NW A when the UE is configured with MUSIM gaps.

On the other hand, when the UE is in RRC_IDLE/_INACTIVE state on NW B, UE will not be configured with dedicated measurement gaps for RRM measurements while in RRC_IDLE/_INACTIVE state. In other words, there are no strict requirements on when/where the UE should perform these measurements in RRC_IDLE/_INACTIVE state. As a result, measurements in NW B can be performed within any available MUSIM gap occasion without the need to introduce new requirements (when the UE is configured with the requested MUSIM gaps). However, the impact on the measurement requirements in NW B can be further studied considering different scenarios (e.g., when NW A rejects UE’s request for MUSIM gap, in which measurement behaviour in NW B might be impacted by NW A decision).

Proposal #2: No new requirements to be introduce for NW B measurements in RRC_IDLE/_INACTIVE state, however, further study the impact on NW B measurement requirements considering different scenarios.

## MUSIM gap collision handling

As discussed earlier, MUSIM gap could collide with other measurements, which include the following cases:

- Case 1: legacy measurement gaps

- Case 2: SMTC

- Case 3: different MUSIM gaps

Due to these collisions, measurements on both NW A and NW B can be affected if no proper collision handling mechanism is introduced. However, before discussing a mechanism, lets first define “collision” for MUSIM gap. MUSIM gap collision can be defined based on the same definition used for Rel-17 Concurrent MGs collision [2].

Observation #2: In Rel-17, when the UE is configured with Concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4] ms.

Similarly, we can define the collision for MUSIM gaps as follows:

Proposal #3: MUSIM gap is considered colliding with the legacy measurement gaps or other MUSIM gaps if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4] ms.

### Collision between MUSIM gaps and legacy measurement gaps

In this section, MUSIM gap collision with the legacy measurement gaps (i.e., Rel-15 to Rel-17 measurement gaps) as well as with other MUSIM gaps are discussed together (i.e., Case 1 and 3). Note that:

- Legacy measurement gaps can occur as a single gap (e.g., Rel-15/16 MG) or multiple gaps (e.g., Rel-17 Concurrent MG).

- For MUSIM, UE can also be configured with up to 4 MUSIM gaps (3 periodic and 1 aperiodic).

Therefore, MUSIM gaps collision with the legacy MGs (single or multiple MG) could happen as demonstrated in Figure 2.

Figure 2: MUSIM gaps collision with (a) single legacy MG, (b) multiple legacy MGs.

As discussed before, measurements on both NW A and NW B can be affected if no proper collision handling mechanism is defined. MUSIM gaps collision with the legacy measurement gaps can be handled by applying priority rule. This is also inspired by the handling mechanism used in the collision of the Concurrent MGs in Rel-17 [2]. However, MUSIM gap collision in this WI can be more sophisticated since it can happen between different MUSIM gaps and/or legacy MGs (single or multiple MG) and more gaps to be considered.

Observation #3: MUSIM gaps could collide with a single legacy MG (e.g., Rel-15/16 MG) or multiple legacy MGs (e.g., Rel-17 Concurrent MGs).

Proposal #4: Apply priority rule for handling MUSIM gaps collision with the legacy MGs, where:

- UE only performs the measurements associated to a higher priority gap.

- The lower priority gap occasions are considered as dropped.

- Data scheduling is resumed on the dropped gap occasions.

The priority rule in Proposal #4 can be applied in two different methods: either as a gap-group priority or as individual priority.

Method 1: Gap-group priority can be applied to handle collisions between MUSIM gaps and legacy MGs by defining a high-level priority for the gaps that belong to a specific group (e.g., MUSIM gaps-group and legacy MGs-group) as shown in Figure 3 (left). Then, within each gap group, individual priorities can be defined to handle the collision between different gaps in the same group as shown in Figure 3 (right). It is worth mentioning that this solution can also coexist with the existing requirements in the specs, specifically, the defined requirements for collision handling of Concurrent MGs in Rel-17. In other words, this solution extends the application of priority rule used in Rel-17 Concurrent MGs to include MUSIM gaps. However, this priority should first apply to handle the collision between different gap-group (MUSIM gaps and legacy MGs), and then to handle collision within the same gap group. The advantage of this method is that if the group priority is given first, UE can try to optimize the MUSIM gap request (MGRP, MGL, offset) based on the known group priority.

Figure 3: MUSIM gaps collision handling (Method 1).

Method 2: An alternative solution to handle the collision between MUSIM gaps and legacy MGs is to define individual priorities for all the gaps, regardless their related gaps group, as shown in Figure 4. With this method, a single list of priorities is required to configure all the gaps (MUSIM gaps and legacy MGs) for the UE. In our understanding this method does not require much RRC change. Probably, extending the current gap priority signalling to MUSIM gap is the only change needed.

Figure 4: MUSIM gaps collision handling (Method 2).

Proposal #5: Method 1: First, apply gap-group priority to handle collisions between different gaps groups (i.e., MUSIM gaps group and legacy MGs group). Then, within each gap group, apply different priorities to handle the collision between the gaps within the same group.

Proposal #6: Method 2: Define individual priorities for all the gaps, regardless of their related gaps-group (i.e., MUSIM gaps group or legacy MGs group). Only a single list of priorities is required.

RAN4 may still need to discuss some follow-up problem, even after introducing priority to MUSIM gap. The problem is that the MUSIM gaps were used for NW B activities. It depends on the NW B configuration which is completely unknown to NW A. However, under the current signalling framework, the priority is all assigned by NW A. This may create some problems. For example, if NW A assigns a higher priority to MUSIM Gap#1 than MUSIM Gap#2, and the UE exploits MUSIM Gap#2 to receive paging from the serving cell of NW B while uses MUSIM Gap#1 to measure a different frequency layer in NW B, UE may miss paging from NW B due to collision between MUSIM gaps (which are prioritised blindly by NW A).

Proposal #7: RAN4 to study the issue when the priority is all assigned by NW A, under the current signalling framework, which might lead to missing significant activities in NW B due to MUSIM gap collision handling (e.g., reading the paging in NW B, which are unknown to NW A).

### Collision between MUSIM gaps and SMTC

Besides the collision with the legacy MGs, MUSIM gaps may also collide with the SMTC window of NW A. As a result, this collision needs to be handled. It is known that the periodicity of MUSIM gaps (ranging from 20 to 5120 ms) can be larger than the SMTC window periodicity (ranging from 5 to 160 ms). In other words, SMTC occasions can occur more frequently than the MUSIM gaps. Therefore, whenever there is a collision between a MUSIM gap and an SMTC occasion, the SMTC occasion can be punctured (only when colliding with the non-dropped MUSIM gaps) as shown in Figure 5. Note that, this approach is also followed when collisions happen between the legacy MGs and the SMTC window.

Figure 5: MUSIM gaps collision with the SMTC window.

Observation #4: MUSIM gaps periodicity can be larger than SMTC window periodicity, i.e., SMTC occasions can occur more often than MUSIM gaps occasions.

Proposal #8: MUSIM gaps collision with the SMTC window can be handled by puncturing the collided SMTC occasions with the non-dropped MUSIM gaps.

# Summary

In this contribution we have discussed the RRM requirements for MUSIM gaps, and whether new requirements are needed for NW A and NW B. In addition, collision handling mechanisms among MUSIM gaps, legacy MGs and SMTC were discussed. The following observations were approached:

Observation #1: NW A can reconfigure the UE with up to 4 MUSIM gaps (3 periodic and 1 aperiodic).

Observation #2: In Rel-17, when the UE is configured with Concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4] ms.

Observation #3: MUSIM gaps could collide with a single legacy MG (e.g., Rel-15/16 MG) or multiple legacy MGs (e.g., Rel-17 Concurrent MGs).

Observation #4: MUSIM gaps periodicity can be larger than SMTC window periodicity, i.e., SMTC occasions can occur more often than MUSIM gaps occasions.

Furthermore, the following proposals have been introduced:

Proposal #1: Introduce new requirements for intra-/inter-frequency and inter-RAT measurements in NW A when the UE is configured with MUSIM gaps.

Proposal #2: No new requirements to be introduce for NW B measurements in RRC_IDLE/_INACTIVE state, however, further study the impact on NW B measurement requirements considering different scenarios.

Proposal #3: MUSIM gap is considered colliding with the legacy measurement gaps or other MUSIM gaps if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4] ms.

Proposal #4: Apply priority rule for handling MUSIM gaps collision with the legacy MGs, where:

- UE only performs the measurements associated to a higher priority gap.

- The lower priority gap occasions are considered as dropped.

- Data scheduling is resumed on the dropped gap occasions.

Proposal #5: Method 1: First, apply gap-group priority to handle collisions between different gaps groups (i.e., MUSIM gaps group and legacy MGs group). Then, within each gap group, apply different priorities to handle the collision between the gaps within the same group.

Proposal #6: Method 2: Define individual priorities for all the gaps, regardless of their related gaps-group (i.e., MUSIM gaps group or legacy MGs group). Only a single list of priorities is required.

Proposal #7: RAN4 to study the issue when the priority is all assigned by NW A, under the current signalling framework, which might lead to missing significant activities in NW B due to MUSIM gap collision handling (e.g., reading the paging in NW B, which are unknown to NW A).

Proposal #8: MUSIM gaps collision with the SMTC window can be handled by puncturing the collided SMTC occasions with the non-dropped MUSIM gaps.

# References

- RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo.

- 3GPP TS 38.133, V17.6.0, “NR; Requirements for support of radio resource management”.