3GPP TSG-RAN WG4 Meeting #104-e	R4-2213750

Electronic Meeting, August 15th - August 26th, 2022

Agenda item:		9.19.2.1

Source:	MediaTek Inc.

Title:	Open issue in the performance part of NR positioning - Rx/Tx TEG

Document for:		Discussion

# 1	Introduction

In the last RAN4 meeting, the agreements on the ePOS performance requirements were achieved and captured in the WF [1]. In this paper, we provide our view on the remaining issue in the performance part regarding the RxTx TEG values as captured in the WF below:

### Issue 2-1-3 The candidate timing error margins for RxTx TEGs Candidate options: - Option 1: - (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc. - Option 2: - Use the same candidate values as Rx TEG.

# 2	Discussion

In the last meeting, RAN4 has agreed on the following values for Rx TEGs and Tx TEGs:

### Issue 2-1-1 The candidate timing error margins for Rx TEGs Agreements: - Candidate timing error margins: - 0Tc, 2 Tc, 4 Tc, 6 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 56Tc, 64 Tc, 72 Tc, 80 Tc. - FFS: The applicable timing error margin values that can be selected by the UE are the pre-defined values that are not larger than the sum of the Rel-16 group delay margin (dependent on PRS/SRS BW) and frequency drift margin. ### Issue 2-1-2 The candidate timing error margins for Tx TEGs Agreements: - Use the same candidate timing error margins as Rx TEG.

Therefore, for the remaining issue on RxTx TEG values shown below:

### Issue 2-1-3 The candidate timing error margins for RxTx TEGs Candidate options: - Option 1: - (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc. - Option 2: - Use the same candidate values as Rx TEG.

In our view, it is reasonable to give a larger margin for a RxTx TEG, since it contains 2 group delay error.

Proposal 1: Define a larger margin for RxTx TEGs than Rx TEGs and Tx TEGs:

- (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc.

# 3	Summary

In this contribution, the following proposal is introduced:

Proposal 1: Define a larger margin for RxTx TEGs than Rx TEGs and Tx TEGs:

- (16 values): 1/2 Tc, 1 Tc, 2 Tc, 4 Tc, 8 Tc, 12 Tc, 16 Tc, 20 Tc, 24 Tc, 32 Tc, 40 Tc, 48 Tc, 64 Tc, 80 Tc, 96 Tc, 128 Tc.

# Reference

R4-2210602, “WF on NR Positioning Enhancements (Part 2)”, CATT.