3GPP TSG-RAN WG4 Meeting #104-e                 	R4-2213775

Electronic Meeting, August 15 – August 26, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results for TRS measurement Accuracy

Agenda item:	9.21.2.2

Document for:	Discussion

Introduction

In this paper, for the propagation delay compensation with TRS-based RTT method, we present our simulation results for TRS measurement accuracy.

Discussion

UE Rx-Tx time difference measurement requirement for TRS

Based on the agreed TRS simulation assumption from RAN4#103-e (attached in the annex with typos corrected for the BW definition and propagation conditions, the channel condition is aligned with the PRS requirement in TS 38.133), we have the following simulation results for 90%-ile of the TUE-RX errors for the serving cell

- FR1 results

Table 1 TUE-RX accuracy with TRS, 1 sample, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±64 | 24 | 15kHz |
| ± 32 | 52 |  |
| ± 16 | 104 |  |
| ±32 | 24 | 30kHz |
| ±16 | 52 |  |
| ±8 | 132 |  |
| ±16 | 24 | 60 kHz |
| ±8 | 66 |  |
| ±4 | 132 |  |

Table 2 TUE-RX accuracy with TRS, 1 sample, TDL-A, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±144 | 24 | 15kHz |
| ± 96 | 52 |  |
| ± 64 | 104 |  |
| ±96 | 24 | 30kHz |
| ±68 | 52 |  |
| ± 40 | 132 |  |
| ±64 | 24 | 60 kHz |
| ±40 | 66 |  |
| ±32 | 132 |  |

Table 3 TUE-RX accuracy with TRS, 4 samples, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±64 | 24 | 15kHz |
| ± 32 | 52 |  |
| ± 16 | 104 |  |
| ±32 | 24 | 30kHz |
| ±16 | 52 |  |
| ±8 | 132 |  |
| ±16 | 24 | 60 kHz |
| ±8 | 66 |  |
| ±4 | 132 |  |

Table 4 TUE-RX accuracy with TRS, 4 samples, TDL-A, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±144 | 24 | 15kHz |
| ± 88 | 52 |  |
| ± 64 | 104 |  |
| ±88 | 24 | 30kHz |
| ±68 | 52 |  |
| ± 40 | 132 |  |
| ±64 | 24 | 60 kHz |
| ±40 | 66 |  |
| ±32 | 132 |  |

- FR2 results

Table 5 TUE-RX accuracy with TRS, 1 sample, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±20 | 24 | 60kHz |
| ±8 | 66 |  |
| ±4 | 132 |  |
| ±8 | 32 | 120kHz |
| ±4 | 66 |  |
| ±2 | 132 |  |

Table 6 TUE-RX accuracy with TRS, 1 sample, TDL-C, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±96 | 24 | 60kHz |
| ±64 | 66 |  |
| ±36 | 132 |  |
| ±64 | 32 | 120kHz |
| ±36 | 66 |  |
| ±26 | 132 |  |

Table 7 TUE-RX accuracy with TRS, 4 samples, AWGN, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±20 | 24 | 60kHz |
| ±8 | 66 |  |
| ±4 | 132 |  |
| ±8 | 32 | 120kHz |
| ±4 | 66 |  |
| ±2 | 132 |  |

Table 8 TUE-RX accuracy with TRS, 4 samples, TDL-C, TDD

| Accuracy (Tc) | TRS bandwidth RB | SCS |
| --- | --- | --- |
| ±88 | 24 | 60kHz |
| ±62 | 66 |  |
| ±36 | 132 |  |
| ±60 | 32 | 120kHz |
| ±34 | 66 |  |
| ±26 | 132 |  |

Given the above simulation results, we have the following observations:

- The TRS measurement sample number has minor effect on the TUE-RX  error.

- The TRS TUE-RX error changes significantly between AWGN fading channels in both FR1 and FR2.

Based on the above observations, we have the following proposals:

Conclusion

For UE Rx-Tx time difference measurement accuracy with TRS, we have the following observations and proposals:

- The TRS measurement sample number has minor effect on the TUE-RX  error.

- The TRS TUE-RX error changes significantly between AWGN fading channels in both FR1 and FR2.

Annex A: Simulation assumption for TRS

Table 1 General parameters (TRS)

| Parameter | FR1 | FR2 |
| --- | --- | --- |
| Cell layout | 1 serving cell |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TTD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) |  |
| Data and CCH load in non-TRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 5 MHz, 10 MHz, 20 MHz - 15 kHz - FDD, TDD - 4 GHz - 10 MHz, 20 MHz, 50 MHz - 30 kHz - FDD, TDD - [4] GHz - 20 MHz, 60 MHz, 100 MHz - 60 kHz - FDD, TDD | - [30] GHz - 20 MHz, 50 MHz, 100 MHz - 60 kHz - TDD - [30] GHz - 50 MHz, 100 MHz, 200 MHz - 120 kHz - TDD |
| Propagation conditions [TS 38.101-4] | AWGN, [TDL-A (30 ns delay spread, 5Hz)] | AWGN, [TDL-C (60 ns delay spread, 30Hz)] |
| Es/Iot [dB] | -3 |  |
| Number of UE receive antennas | 2 rx (uncorrelated with equal gain, no rx beamforming) |  |
| UE measurement bandwidth | Full carrier bandwidth |  |

Table 2 CSI-RS for tracking transmission configuration parameters (TRS)

| Parameter | Value |
| --- | --- |
| Number of transmit TRS antennas | 1 |
| Cell ID, TRP ID, CSI-RS Resource Set ID | 1 |
| Number with CSI-RS Resource sets with trs-Info | 1 |
| TRS transmission bandwidth (in PRBs) - full carrier BW | - 15 kHz: 24(5MHz), 52 (10MHz), 104 (20MHz) - 30 kHz: 24(10MHz), 52 (20MHz),132 (50MHz) - 60 kHz: 24(20MHz), 64 (50MHz),132 (100MHz) - 120 kHz: 32(50MHz), 64 (100MHz), 128 (200MHz) |
| Sample rate (Tc) | - 15 kHz: 256(5MHz), 128(10MHz), 64 (20MHz) - 30 kHz: 128(10MHz), 64 (20MHz), 32 (50MHz) - 60 kHz: 64 (20MHz), 32 (50MHz), 16 (100MHz) - 120 kHz: 32 (50MHz), 16 (100MHz), 8 (200MHz) |
| TRS periodicity | 40ms |
| TRS resources number | 4 |
| TRS symbol location | {4,8} |
| TRS frequency density | 3 |
| TRS samples | 1, 4 |
| Evaluation period | 40 ms, 200 ms |

At least the following performance characteristics are to be provided for TUE-RX:

90%-ile of the TUE-RX errors for each cell

and,

TUE-RX error = abs(estimated TUE-RX – ideal TUE-RX ) (based on perfect channel and UE location knowledge).