3GPP TSG-RAN WG4 Meeting # 104 -e	R4-2213779

# Electronic Meeting, Aug.15th –Aug.26th ,2022

Title: 	Discussions on remain issues on test setup for CRS-IM

Source: 	Huawei, HiSilicon

Agenda item:	9.10.1.3

Document for:	Discussion

Background

In last meeting RAN 4 agreed to introduce test cases with inter-RAT MO configured. But there are some remain issues left. In this paper we provide our views on these open issues.

We captured the open issues as following:

For time period length before PDSCH scheduling - Option 1: - The time period length before PDSCH scheduling is set to 4.34s including two periods: - Period 1: TIdentify, E-UTRAN FDD for FDD and TIdentify, E-UTRAN TDD for TDD defined in clause 9.4.2.2 of TS 38.133. - Period 2: PBCH decoding time which is set to 500ms. - Where - TIdentify, E-UTRAN FDD - No DRX is assumed - TIdentify, E-UTRAN FDD= TBasicIdentify(480ms)*480/TInter1(60ms)*CSSFinterRAT (1)=3840ms - Where - TIdentify, E-UTRAN TDD - No DRX is used - Configuration 0 in Table 9.4.3.2-1 of TS 38.133 - TIdentify, E-UTRAN TDD= TBasicIdentify(480ms)*480/TInter1(60ms)*CSSFinterRAT (1)=3840ms - Other options are not precluded Test applicability - If a UE supports both without NWA and NWA based CRS-IM for 15kHz SCS (i.e., UE Capability #2 and #3), - Option 1: UE is only required to pass performance requirements without NWA signalling based test setup, i.e. UE capability#2. - Option 2: UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#3. - If a UE supports both without NWA and NWA based CRS-IM for 30kHz SCS (i.e., UE Capability #4 and #5), - Option 1: UE is only required to pass performance requirements without NWA signalling based test setup, i.e. UE capability#4. - Option 2: UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#5.

Discussions

Time period length before PDSCH scheduling

As analysed in lase meeting, in the time period before PDSCH scheduling, UE should perform neighbouring cell identifying and PBCH decoding. According to [1], 500ms is enough for UE to decode PBCH correctly and the time of identifying LTE cells should meet the RRM requirements defined in 38.133. The time period in option 1 is derived based on the procedure of 38.133. Furthermore, we don’t see any difference of measurement between NR 15kHz and 30kHz. Hence, we confirm option 1 as time period before PDSCH scheduling for both 15kHz and 30kHz SCS

Proposal 1: Confirm option 1 as time period before PDSCH scheduling for both 15kHz and 30kHz SCS.

Measurement Gap configuration

In order to guarantee UE have enough time to perform returning, we suggest to make LTE’s PBCH in the middle of measurement gap. Considering we agreed to use gap pattern ID0, (MGL=6ms, MGRP=40ms), we suggest to configure gap offset to 7 for FDD (See Figure 2-1) and 0 for TDD (See Figure2-2).

Figure 2-1: Measurement gap pattern for 15kHz FDD

Figure 2-1: Measurement gap pattern for 15kHz TDD

Proposal 2: Configure gap offset to 7 for FDD and 0 for TDD to make LTE’s PBCH in middle of measurement gap.

Test applicability

The intention of   introducing test cases with inter-RAT measurement is to guarantee such type UE can be tested . From point of test complexity, test with inter-RAT MO need longer test time and additional test complexity. Hence we slightly support option 2.

Proposal 3: If a UE supports both without NWA and NWA based CRS-IM for 15kHz SCS (i.e., UE Capability #2 and #3),  UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#3.

Proposal 4: If a UE supports both without NWA and NWA based CRS-IM for 30kHz SCS (i.e., UE Capability #4 and #5),  UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#5.

Others

In previous meetings, RAN 4 reached agreement that UE make some default assumptions when performing CRS-IM such as same bandwidth, CRS port of neighbouring cells as configured in serving cells. However, if default assumptions is invalid and UE doesn't know that, UE will perform CRS-IM with wrong assumptions and system performance degradation will be observed. We would like to invite companies to give some feedback that how to solve this problem.

Proposal 5: Invite companies to give some feedback how to solve the problem that if default assumptions is invalid and UE doesn't know that, UE will perform CRS-IM with wrong assumptions and system performance degradation will be observed.

Conclusion

In this paper we provide our views on test setup for CRS-IM receiver with scenario2. The proposals are:

Proposal 1: Confirm option 1 as time period before PDSCH scheduling for both 15kHz and 30kHz SCS.

Proposal 2: Configure gap offset to 7 for FDD and 0 for TDD to make LTE’s PBCH in middle of measurement gap.

Proposal 3: If a UE supports both without NWA and NWA based CRS-IM for 15kHz SCS (i.e., UE Capability #2 and #3),  UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#3.

Proposal 4: If a UE supports both without NWA and NWA based CRS-IM for 30kHz SCS (i.e., UE Capability #4 and #5),  UE is only required to pass performance requirements with NWA signalling based test setup, i.e. UE capability#5.

Proposal 5: Invite companies to give some feedback how to solve the problem that if default assumptions is invalid and UE doesn't know that, UE will perform CRS-IM with wrong assumptions and system performance degradation will be observed.

Reference

[1] R4-2209815 Discussion on test setup on CRS-IM with 15 kHz SCS