3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213807

# Electronic Meeting, Aug.9th –20th ,2022

Title: 	Discussions on open issues for PDCCH and PBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.2.2

Document for:	Discussion

Background

As per [1], there are some open issues left for PBCH and PDCCH. In this paper, we provide our views on these open issues

Discussions

PDCCH requirements

Whether to define requirements for multi-slot PDCCH monitoring

The options for these issue are listed as follows:

- Option 1: Yes - Option 1a: Define one requirement for both UE supporting monitor one slot and two slots every four slots. - Option 2: No - Option 2a: Implicitly test this in PDSCH requirements.

Based on our understanding, 4-slot PDCCH monitoring has been verified in PDSCH requirements which can also apply for PDCCH test. However, 2-slot PDCCH always means 2-TB scheduling for one PDCCH. RAN 4 should define separate PDCCH feedback and scheduling pattern for 2-slot PDCCH monitoring which may bring much work burden. Moreover, monitoring capability has no performance impact. Therefore, we propose the following:

Proposal 1: Implicitly test UE supporting 4-slots monitoring in PDCCH and PDSCH requirements

Test cases

One issue is whether to consider large bandwidth. I.e. 120kHz/400MHz and 480kHz/1600MHz. Firstly, we suggest to preclude 480kHz/1600MHz since it is optional. As for 120kHz/400MHz, we understanding some companies’ intention that large CORESET RB will be more flexible for PDCCH transmission with interleaving and more diversity gain can be get. Hence, we can separate different CORESET RB allocation into cases with different AL configurations but not increase the test number. For example, we can use 120kHz/400MHz for AL16 and use 120kHz/100MHz for AL2,4 and 8.

For 480kHz SCS, considering it is optional feature, we suggest to limit it to some cases.

We capture proposed cases in Table 2-1:

Table 2-1: Proposed PDCCH test cases

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA10-200 | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low |
| 4 | 400/120 | 2 | 240 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 2x2 Low |
| Note: Interested companies can provide simulation results for other cases not included in above table, as needed. |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Use Table 2-1 as final PDCCH test cases

PBCH requirements

One issue is SSB index assumption, the options are captured as follows:

SSB index assumption - Option 1: Only with not known SSB index - Option 2: Both known and not known SSB index - Option 3: Only known SSB index for 120kHz and 480kHz SCS

Based on our understanding,  RAN 4 agreed to define requirements for both FR2-2 single carrier operation and FR1+FR2-2 CA/DC operation. For FR2-2 single carrier operation, UE should perform SSB searching in initial access which means SSB with unknown index should be considered. For CA/DC scenario, UE may have knowledge of SSB index of FR2-2 carrier which means SSB with known index should be considered. Hence, we propose to define PBCH requirements for both known and not known SSB index.

Proposal 3: Define FR2-2 PBCH requirements for both known SSB index and unknown index.

Conclusion

In this paper, we provide our views on PDCCH and PBCH requirements. The proposals are:

Proposal 1: Implicitly test UE supporting 4-slots monitoring in PDCCH and PDSCH requirements

Proposal 2: Use Table 2-1 as final PDCCH test cases

Proposal 3: Define FR2-2 PBCH requirements for both known SSB index and unknown index.

Reference

[1]   R4-2210665 WF on UE demodulation performance requirements definition for 52.6 - 71 GHz. Qualcomm Incorporated