3GPP TSG-RAN WG4 Meeting # 104e	R4-2213812

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions on remain issues on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.3.1

Document for:	Discussion

Background

As per [1], RAN 4 left some open issues on FR2-2 PUSCH demodulation requirements, in this paper, we provide our views on these open issues

Discussions

It is noted that we captured the simulation results with corresponding simulation assumptions in appendix.

MCS

In RAN 4 102-e meeting, we reached following agreements:

Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. - No Tx phase noise is modelled - Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) - 70 GHz carrier frequency is assumed

Based on our simulation results, performance loss for MCS20 due to phase noise is 1.1dB for 120kHz SCS and 1.2dB for 480kHz SCS. Therefore, MCS20 is not feasible. In order to minimum the phase noise impact, we propose to select MCS 18  instead of MCS20

Proposal 1: Define PUSCH performance requirements with MCS 4, 16 and 18.

Antenna configuration

1T2R Low and 2T2R Low are typical configurations, hence we propose to use 1T2R Low and 2T2R Low.

Proposal 2: Use 1T2R Low and 2T2R Low

Bandwidth

For FR2-2 120 kHz SCS, 100MHz and 400MHz bandwidth are supported. But based from our understanding, 400MHz bandwidth need higher testable SNR and have more phase noise impact. Hence, we propose to only consider 100MHz. For 480kHz SCS, 1600MHz bandwidth is optional, hence we propose to only consider 400MHz bandwidth.

Proposal 3: Only consider 100MHz bandwidth for 120kHz SCS and 400MHz bandwidth for 480kHz SCS.

Doppler spread

200Hz corresponding to 3km/h and 650Hz corresponding to 10km/h are listed as two options for PUSCH Doppler spread. Based on our simulation results for PDSCH, cases with two Doppler spread have the same performance.  We don’t see any need to define case with another Doppler spread repeatedly.

Proposal 4: Define PUSCH requirements with 200Hz Doppler spread.

RMS delay spread

One issue is how to select RMS delay spread. Based on our understanding, 10ns is feasible because 10ns is used for evaluation from RAN 1.

Proposal 5: Define PUSCH requirements with 10ns RMS delay spread.

Propagation conditions

Based on our understanding, higher MCS is always scheduled for TDL-D and low MCS is always scheduled for TDL-A.  To be practical, we propose to use MCS 4 and MCS 16 for TDL-A and MCS 18 for TDL-D

Proposal 6: Use MCS 4 and 16 for TDL-A and MCS 18 for TDL-D.

Proposed cases

The propose cases are captured in Table 2-1:

Table 2-1: Proposed simulation assumptions for FR2-2 PUSCH

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |

Proposal 7: Use Table 2-1 for FR2-2 PUSCH performance requirements definition.

Conclusion

In this paper we provide our simulation results for FR2-2 PUSCH requirement. The proposals and observations are:

Proposal 1: Define PUSCH performance requirements with MCS 4, 16 and 18.

Proposal 2: Use 1T2R Low and 2T2R Low

Proposal 3: Only consider 100MHz bandwidth for 120kHz SCS and 400MHz bandwidth for 480kHz SCS.

Proposal 4: Define PUSCH requirements with 200Hz Doppler spread.

Proposal 5: Define PUSCH requirements with 10ns RMS delay spread.

Proposal 6: Use MCS 4 and 16 for TDL-A and MCS 18 for TDL-D.

Proposal 7: Use Table 2-1 for FR2-2 PUSCH performance requirements definition.

Appendix

Table 4-1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| SCS/Bandwidth |  | 120kHz/100MHz, 120kHz/400MHz and 480kHz/400MHz |
| Antenna configuration |  | 1T2R Low |
| MCS |  | MCS 4,16,20 |
| Propagation conditions |  | TDLA10-200; TDLA10-650 and TDLD10-200 |
| PN model |  | Set 2 in TR 38.808 |
| PN compensation |  | - No PN - For cases except 120kHz/400MHz with MCS20, CPE; - For case with 120kHz/400MHz with MCS20, ICI de-filter with 1, 2 and 4 tap. |
| Receiver type |  | MMSE |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric | Normalized throughput | 70% |

The simulation results are captured in Figure 4-1:

Figure 4-1: Simulation results for FR2-2 PUSCH requirements

The target SNR@70% of maximum TP are captured in Table 4-2:

Table 4-2: Summary of simulation results for PUSCH

| SCS | CBW | MCS | Channel model | PN compensation | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | No PN | -2.2 |
|  |  |  |  | CPE | -2.1 |
| 120 | 100 | 16 | TDLA10-200 | No PN | 8.6 |
|  |  |  |  | CPE | 9.1 |
| 120 | 100 | 20 | TDLD10-200 | No PN | 9.9 |
|  |  |  |  | CPE | 11.0 |
| 120 | 400 | 4 | TDLA10-650 | No PN | -2.3 |
|  |  |  |  | CPE | -2.2 |
| 120 | 400 | 16 | TDLA10-200 | No PN | 8.9 |
|  |  |  |  | CPE | 9.4 |
| 120 | 400 | 20 | TDLD10-200 | No PN | 10.0 |
|  |  |  |  | CPE | 11.2 |
|  |  |  |  | ICI u=1 | 11.7 |
|  |  |  |  | ICI u=2 | 11.7 |
|  |  |  |  | ICI u=3 | 11.7 |
| 480 | 400 | 4 | TDLA10-650 | No PN | -2.0 |
|  |  |  |  | CPE | -1.9 |
| 480 | 400 | 16 | TDLA10-200 | No PN | 8.4 |
|  |  |  |  | CPE | 9.0 |
| 480 | 400 | 20 | TDLD10-200 | No PN | 9.9 |
|  |  |  |  | CPE | 11.1 |

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation

[2]   R4-2209842 Discussion on PDSCH requirements for FR2-2. Huawei, HiSilicon