3GPP TSG-RAN WG4 Meeting # 104e	R4-2213812

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Simulation results on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       9.14.8.3.1

Document for:	Discussion

Background

As per [1], RAN 4 listed some initial cases for FR2-2 PUSCH for simulation, in this paper, we provide our simulation results.

Simulation results

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| SCS/Bandwidth |  | 120kHz/100MHz, 120kHz/400MHz and 480kHz/400MHz |
| Antenna configuration |  | 1T2R Low |
| MCS |  | MCS 4,16,20 |
| Propagation conditions |  | TDLA10-200; TDLA10-650 and TDLD10-200 |
| PN model |  | Set 2 in TR 38.808 |
| PN compensation |  | - No PN - For cases except 120kHz/400MHz with MCS20, CPE; - For case with 120kHz/400MHz with MCS20, ICI de-filter with 1, 2 and 4 tap. |
| Receiver type |  | MMSE |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric | Normalized throughput | 70% |

The simulation results are captured in Figure 2-1:

Figure 2-1: Simulation results for FR2-2 PUSCH requirements

The target SNR@70% of maximum TP are captured in Table 2-2:

Table 4-2: Summary of simulation results for PUSCH

| SCS | CBW | MCS | Channel model | PN compensation | Target SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | No PN | -2.2 |
|  |  |  |  | CPE | -2.1 |
| 120 | 100 | 16 | TDLA10-200 | No PN | 8.6 |
|  |  |  |  | CPE | 9.1 |
| 120 | 100 | 20 | TDLD10-200 | No PN | 9.9 |
|  |  |  |  | CPE | 11.0 |
| 120 | 400 | 4 | TDLA10-650 | No PN | -2.3 |
|  |  |  |  | CPE | -2.2 |
| 120 | 400 | 16 | TDLA10-200 | No PN | 8.9 |
|  |  |  |  | CPE | 9.4 |
| 120 | 400 | 20 | TDLD10-200 | No PN | 10.0 |
|  |  |  |  | CPE | 11.2 |
|  |  |  |  | ICI u=1 | 11.7 |
|  |  |  |  | ICI u=2 | 11.7 |
|  |  |  |  | ICI u=3 | 11.7 |
| 480 | 400 | 4 | TDLA10-650 | No PN | -2.0 |
|  |  |  |  | CPE | -1.9 |
| 480 | 400 | 16 | TDLA10-200 | No PN | 8.4 |
|  |  |  |  | CPE | 9.0 |
| 480 | 400 | 20 | TDLD10-200 | No PN | 9.9 |
|  |  |  |  | CPE | 11.1 |

Conclusion

In this paper we provide our simulation results for FR2-2 PUSCH requirement.

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation