3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213820

# Electronic Meeting, Aug.15th - 26th ,2022

Title: 	Discussions on PUCCH requirements for Rel-17 enhanced IIOT and URLLC support

Source: 	Huawei, HiSilicon

Agenda item:       9.21.3

Document for:	Discussion

Background

As per [1], in last meeting, RAN4 reached the agreements that not introducing performance for PHY channels except PUCCH. In this paper, we provide our views on PUCCH performance requirements

Discussions

Whether to introduce enhanced PF0 requirements

We have following options for open issues on enhanced PF0:

Agreements: Option 2a as starting point and further confirm by RAN4#104-e meeting. - Option 2a: Develop PUCCH requirements for PF0, with sub slot repetition and inter sub-slot hopping, but condition introduction in the specification on the availability of at least 2 simulation inputs

Based on our understanding, compared to Rel-16’s procedure that slot-based PUCCH inter-slot repetition is supported, sub-slot based PUCCH repetition have stricter requirements on processing delay. Furthermore, PF0 repetition requirements was not defined in previous release. Hence we think it is feasible to define requirements for enhanced PF0 with sub-slot based repetition.

Simulation assumptions

Similar to Rel-16 PF1 multi-slot repetition requirements definition, we suggest to limit the test number to one case to reduce the simulation work and CR work. We suggest following simulation assumptions as start point for discussion:

Table 2-1: Test Parameters

| Parameter | Test |
| --- | --- |
| SCS(kHz)/Bandwidth(MHz) | 30/20 |
| Propagation conditions | TDLC300-100 Low |
| Antenna condition | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| Number of symbols | 2 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Disabled |
| Inter-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 |
| Sub slot length | 2 symbols |
| Repetition number | 2 |
| Test metric | Prob(ACK miss)<1% Prob(DTX to ACK)<1% |

We provide our simulation results based on assumptions in Table2-1 in Figure 2-1.

Figure 2-1: Simulation results for PF0 with sub-slot repetition

The target SNR is -2.08dB

Proposal 1: Introduce PF0 performance requirements with sub slot repetition.

Proposal 2: Use test assumptions in Table 2-1:

Conclusion

In this paper we provide our views on PF0 requirements with sub slot repetition. The proposals are:

Proposal 1: Introduce PF0 performance requirements with sub slot repetition.

Proposal 2: Use test assumptions in Table 2-1:

Reference

[1]  R4-2210673 WF on enhanced IIoT and URLLC support demodulation and CSI requirements. Nokia, Nokia Shanghai Bell