3GPP TSG-RAN WG4 Meeting #104-e	R4-2213831

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion and simulation results on BS coverage enhancement demod PUCCH

Source: 	Huawei, HiSilicon

Agenda item:	9.16.2.2

Document for:	Discussion

Background

During RAN4#103-e meeting, WF [1] on PUCCH demodulation performance of Rel-17 NR coverage enhancement was approved. In this contribution, we share our views and provide our updated simulation results about the BS coverage enhancement PUCCH demodulation requirements.

Discussion

DMRS configuration for PUCCH format 3

- Option 1: only cover without additional DMRS - Option 2: Cover both with and without additional DMRS - Make decision on this issue in the next meeting. - Encourage companies to bring simulation results for both DMRS1+1 and DMRS1+0 for the next meeting

The only remained issue is that whether to consider additional DMRS or not for PUCCH format 3. From our point of view, with JCE enabled scenario, the additional DMRS cannot improve performance obviously since that there are already more than one DMRS in different slots used for channel estimation. In addition, as per simulation results in clause 3, DMRS 1+1 has nearly same performance as DMRS 1+0 for PUCCH JCE but with more resource overhead. Also to reduce the test effort, we prefer to only consider the typical DMRS 1+0 configuration for JCE PUCCH format 3.

Only consider disabled additional DMRS for BS PUCCH format 3 demodulation requirements with JCE.

Simulation

PUCCH demodulation with Joint Channel Estimation (JCE)

Table 3.1-1 Simulation assumption for PUCCH JCE (common)

| Parameter | Value |
| --- | --- |
| Antenna configuration | 1T2R |
| PUCCH format | PF1 and PF3 |
| Actual TDW length | 8 for FDD, 2 for TDD |
| Configured TDW number | 8 for FDD, 2 for TDD |
| Number of repetitions | 8 for FDD, 2 for TDD |
| Frequency hopping | Disabled |
| Phase and power offset modelling | N/A |
| Bit payload | 2bits for PF1, 16 bits for PF3 |
| Resource allocation | 14 symbols and 1 PRB |
| DMRS configuration for PF3 | 1+0/1+1 |
| Propagation condition | TDLC300-100 Low |
| Test metric | PF1: NACK to ACK probability < 0.1%, ACK missed detection probability < 1% PF3: UCI block error probability < 1% |

PUCCH format 1

Here we provide the initial simulation results for PUCCH format 1 JCE for alignment, as shown in Table 3.1.1-2 based on simulation assumption as shown in Table 3.1.1 and Table 3.1.1-1.

Table 3.1.1-1 Simulation assumption for PUCCH JCE (PF1)

| Parameter | Test |
| --- | --- |
| First PRB prior to frequency hopping | 0 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 3.1.1-2 Simulation results for PUCCH JCE (PF1)

| Case Number | SCS/kHz | SNR@1% 1st BLER |  |
| --- | --- | --- | --- |
|  |  | FDD | TDD |
| 1 | 15 | -15.00 | -8.34 |
| 2 | 30 | -13.51 | -7.23 |
| 3 | 60 | -14.41 | -9.40 |
| 4 | 120 | -13.64 | -7.77 |

PUCCH format 3

Here we provide the initial simulation results for PUCCH format 3 JCE for alignment, as shown in Table 3.1.2-2 based on simulation assumption as shown in Table 3.1-1 and Table 3.1.2-1.

Table 3.1.2-1 Simulation assumption for PUCCH JCE (PF3)

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 0 |

Table 3.1.2-2 Simulation results for PUCCH JCE (PF3)

| Case Number | SCS/kHz | SNR@1% 1st BLER |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | FDD |  | TDD |  |
|  |  | DMRS 1+0 | DMRS 1+1 | DMRS 1+0 | DMRS 1+1 |
| 1 | 15 | -11.61 | -12.04 | -2.72 | -2.95 |
| 2 | 30 | -10.28 | -10.85 | -2.23 | -2.04 |
| 3 | 60 | -6.29 | -6.44 | -0.11 | -0.64 |
| 4 | 120 | -7.01 | -7.04 | -0.65 | -0.70 |

There is negligible performance difference between DMRS 1+0 and DMRS 1+1 for PUCCH JCE.

Proposals

In this contribution, we discuss on BS PUCCH demodulation requirements for coverage enhancement. Our observations and proposals are:

- There is negligible performance difference between DMRS 1+0 and DMRS 1+1 for PUCCH JCE.

- Only consider disabled additional DMRS for BS PUCCH format 3 demodulation requirements with JCE.

Reference

R4-2210667, WF on PUCCH demodulation performance of Rel-17 NR coverage enhancement, RAN4#103-e, Nokia, Nokia Shanghai Bell