3GPP TSG-RAN WG4 Meeting #104-e	R4-2213837

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion and simulation results on UE FeMIMO CSI mTRP

Source: 	Huawei, HiSilicon

Agenda item:	9.17.4.2.1

Document for:	Discussion

Background

During RAN4#103-e meeting, WF [1] on CSI requirement for Rel-17 FeMIMO was approved. In this contribution, we share our views and provide our updated simulation results about the UE FeMIMO mTRP CSI requirements.

Discussion

Test cases for CSI reporting enhancement for m-TRP transmission

Number of CSI-RS Ports

Agreement: - 8 ports per TRP as baseline - Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting

The only remained issue is the number of CSI-RS ports per TRP. As per RAN1 feature list, the number of Tx ports per TRP can be {2, 4, 8, 12, 16, 24, 32}. Some companies propose to define higher number of CSI-RS ports to observe more gains. From our understanding, considering that we are defining the minimum requirements, more test coverage should be considered firstly to ensure UE such that support only 4 ports can also perform the test. Also, we think it is feasible to define cases with lower gain since in Rel-15 RI requirements, some cases are even defined with TP ratio of 0.9. Therefore, we prefer to select 4 ports per TRP for CSI reporting requirements for mTRP.

Select 4 ports per TRP for CSI reporting requirements for mTRP.

Simulation

Here we provide the updated simulation results for the FeMIMO mTRP CSI requirements for alignment, as shown in Table 3-2 and Figure 3-1, based on simulation assumption as shown in Table 3-1.

Table 3-1 Simulation assumption for FeMIMO mTRP CSI reporting enhancement

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30 kHz SCS |
| CBW | 10 MHz | 40 MHz |
| Scheme | Single-DCI based SDM |  |
| Antenna configuration | 2x2 XPL High, 2x4 XPL High |  |
| Propagation condition | TDLA30-10 |  |
| Number of ports per TRP | 4, 8 |  |
| DMRS type | Type 1 |  |
| Number of DMRS symbols | 1+1 |  |
| Number of layers | 2 (1MIMO layer per TRP) |  |
| Maximum number of HARQ transmission | 4 |  |
| TDD pattern | N/A | 7D1S2U, S: 6D 4G 4U |
| NZP CSI-RS resource | 4port: Row 4 8port: Row 8 |  |
| ZP CSI-RS resource | Row 5 |  |
| (N1, N2, O1, O2) | 4port: (2, 1, 4, 1) 8port: (4, 1, 4, 1) |  |
| CSI configuration | 5slot periodicity, 1 slot offset for all CSI resources | 10slot periodicity, 1 slot offset for all CSI resources |
| CSI delay | 4port: 6ms 8port: 8ms | 4port: 5.5ms 8port: 6.5ms |
| TRS configuration | 20ms periodicity, 10ms offset, 2 slot patterns, symbol 4 and 8 |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| CSI reporting | Wideband, Mode1, X=0 |  |
| Time/frequency offset between two TRxP | N/A |  |
| Codebook type | typeI-SinglePanel |  |
| Test metric | TP ratio between following PMI and Random PMI |  |
| Test point | 90% |  |

Table 3-2 Simulation results for FeMIMO mTRP CSI reporting enhancement

| Case Number | Duplex | Antenna configuration | Number of ports | γ |
| --- | --- | --- | --- | --- |
| 1 | FDD | 2x2 | 4 | 1.31 |
| 2 | FDD | 2x2 | 8 | 2.06 |
| 3 | FDD | 2x4 | 4 | 1.61 |
| 4 | FDD | 2x4 | 8 | 2.39 |
| 5 | TDD | 2x2 | 4 | 1.44 |
| 6 | TDD | 2x2 | 8 | 2.06 |
| 7 | TDD | 2x4 | 4 | 1.70 |
| 8 | TDD | 2x4 | 8 | 2.38 |

Figure 3-1 Simulation results for FeMIMO mTRP CSI reporting enhancement

Proposals

In this contribution, we discuss and provide our updated simulation results on UE CSI requirements for FeMIMO mTRP. Our observations and proposals are:

- Proposal 1: Select 4 ports per TRP for CSI reporting requirements for mTRP.

Reference

R4-2210669, WF on CSI requirement for Rel-17 FeMIMO, RAN4#103-e, Samsung