3GPP TSG-RAN WG4 Meeting #104-e	R4-2213840

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion on UE FeMIMO demod mTRP

Source: 	Huawei, HiSilicon

Agenda item:	9.17.4.1.2

Document for:	Discussion

Background

During RAN4#103-e meeting, WF [1] on demodulation requirement for Enhancement on Multi-TRP was approved. In this contribution, we share our views about the UE FeMIMO mTRP demodulation requirements.

Discussion

Test scope

Whether to define PDCCH requirement for multi-TRP repetition transmission schemes

- Introduce PDCCH requirements for multi-TRP repetition transmission - Option 1: With UE soft-combining - Option 2: Without UE soft-combining

For non-SFN scheme, PDCCH can be transmitted with two TCI state corresponding to two SS sets associated with corresponding CORESETs. Encoding / rate matching is based on one repetition, and the same coded bits are repeated for the other repetition. Each repetition has the same number of CCEs and coded bits, and corresponds to the same DCI payload. Two PDCCH candidates are explicitly linked together (UE knows the linking before decoding).

UE can report a value of BD from {2,3}. From the demodulation perspective, for UE reporting the value of 2, UE can decode the first repetition and then perform soft-combining of two repetition if failed to decode the first repetition. For UE reporting the value of 3, UE can decode the first repetition and then decode the second repetition and then perform soft-combining of two repetition if failed to decode both the first and the second repetition.

As per our contribution [2], there is great gain by performing soft-combining for non-SFN PDCCH enhancement. In our view, this feature is very important to improve PDCCH reliability. Considering the great gain by performing soft-combining, we propose to define performance requirements with UE soft-combining for enhancements on Multi-TRP for PDCCH.

Define PDCCH requirement with UE soft-combining for multi-TRP repetition transmission schemes.

Test setup for PDCCH requirement

Aggregation level

- Aggregation level - Option 1: AL=2 - Option 2: AL=4 - Decide aggregation level base on simulation results in next meeting. - Operating SNR condition for selection AL - Option 1: SNR > [-4] dB - Other options are not precluded

Based on our simulation results in [3], for the 4Rx configuration after considering the impairment margin, the SNR at 1% BLER for AL2 is larger than -4dB at the same time that for AL4 is smaller than -4dB. So we propose to select AL2 for PDCCH with FDM repetition scheme.

Select AL2 for PDCCH with FDM repetition scheme.

SNR setting for each TRP

- SNR setting for each TRP - Option 1: Balance SNR (no blocking considered) - The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP - Option 2: Blocked TRP testing - Blocked TRP testing for Option 2 in Issue 1-1-1 (without UE soft-combining): Probability of blocking p=10% with X=10 dB blocking as a starting point - Other options are not precluded

In last meeting, blocked TRP testing are proposed by some companies considering that RAN1 has similar setup during the evaluation. From our understanding, the test purpose of multi-TRP PDCCH enhancement is to verify properly processing under multi-TRP enhancement scheme, current test setup is enough for the testing. Also to align with Rel-16 multi-TRP PDSCH requirements, we don’t think TRP blocking should be considered.

Do not consider TRP blocking for multi-TRP PDCCH enhancement.

Proposals

In this contribution, we discuss on UE demodulation requirements for FeMIMO mTRP. Our observations and proposals are:

- Define PDCCH requirement with UE soft-combining for multi-TRP repetition transmission schemes.

Select AL2 for PDCCH with FDM repetition scheme.

Do not consider TRP blocking for multi-TRP PDCCH enhancement.

Reference

R4-2210671, WF on demodulation requirement for Enhancement on Multi-TRP, RAN4#103-e, Huawei, HiSilicon

R4-2205775, Discussion on UE FeMIMO demod mTRP, RAN4#102-e, Huawei, HiSilicon

R4-2213842, Simulation results on UE FeMIMO demod mTRP, RAN4#104-e, Huawei, HiSilicon