3GPP TSG-RAN WG4 Meeting #104-e	R4-2213856

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Discussion and simulation results on satellite NTN demod PRACH

Source: 	Huawei, HiSilicon

Agenda item:	9.11.7.2.3

Document for:	Discussion

Background

During RAN4#103-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we discuss and provide our initial simulation results about the NTN PRACH demodulation requirements.

Discussion

Preamble Format

Agreements - Including format 0, B4 and C2 - FFS for format 2 and A2

For the preamble format, down scope is needed to reduce the test effort. Considering that there is no preamble format 2 introduced for the legacy BS PRACH requirements, we prefer to not introduce preamble format 2 for NTN SAN requirements. Also we don’t think format 2 is typical in the NTN scenario.

Do not introduce preamble format 2 for NTN SAN requirements.

Simulation

PRACH

Here we provide the initial simulation results for PRACH for alignment, as shown in Table 3.1-3 and Table 3.1-4, based on simulation assumption as shown in Table 3.1-1 and Table 3.1-2.

Table 3.1-1 Simulation assumption for PRACH (Part1)

| Parameter | Value |
| --- | --- |
| Preamble format | 0/B4/C2 |
| Antenna configuration | 1Rx/2Rx |
| SCS | 1.25kHz for long preamble format 15kHz and 30kHz SCS for short preamble format (LRA=139) |
| Propagation | NTN-TDLA100-200 |
| Time error tolerance | 0.522μ+Tdelay (Same formula as legacy BS requirements) |
| Frequency offset | 200Hz |

Table 3.1-1 Simulation assumption for PRACH (Part2)

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| B4,C2 | 15 | 23 | 0 | 0 |
|  | 30 | 46 | 0 | 0 |

Table 3.1-3 Ideal simulation results for PRACH with long format

| Case Number | Antenna configuration | SNR (dB) |
| --- | --- | --- |
|  |  | 0 |
| 1 | 1x1 | 0.05 |
| 2 | 1x2 | -8.57 |

Table 3.1-4 Ideal simulation results for PRACH with short format

| Case Number | SCS (kHz) | Bandwidth (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | B4 | C2 |
| 1 | 15 | 5 | 1x1 | -4.87 | 0.18 |
| 2 | 15 | 5 | 1x2 | -10.26 | -6.29 |
| 4 | 30 | 10 | 1x1 | -5.53 | -1.95 |
| 5 | 30 | 10 | 1x2 | -10.98 | -7.54 |

Conclusion

In this contribution, we discuss and provide our initial simulation results on NTN PRACH demodulation requirements.

Reference

R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon