3GPP TSG-RAN WG4 Meeting #104-e	R4-2213862

# Electronic Meeting, 15th – 26th Aug, 2022

Title: 	Simulation results on satellite NTN demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	9.11.7.2.1

Document for:	Information

Background

During RAN4#103-e meeting, WF [1] on NTN SAN demodulation requirements was approved. In this contribution, we provide our updated simulation results about the NTN PUSCH demodulation requirements.

Simulation

PUSCH

Here we provide the initial simulation results for PUSCH for alignment, as shown in Table 2.1-2, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Channel model |  | NTN-TDL-A100-200 and NTN-TDLC100-200 |
| MCS |  | MCS4 |
| Transform precoding |  | Disabled and Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0, group hopping and sequence hopping are disabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |

Table 2.1-2 Ideal simulation results for PUSCH (Transform precoding disabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA |  | NTN-TDLC |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 1.21 | 1.22 | 2.33 | 2.38 |
| 2 | 15 / 5 | 1x2 | -2.89 | -2.89 | -2.48 | -2.44 |
| 3 | 30 / 10 | 1x1 | 0.89 | 0.91 | 2.33 | 2.32 |
| 4 | 30 / 10 | 1x2 | -3.06 | -3.06 | -2.61 | -2.60 |

Table 2.1-2 Ideal simulation results for PUSCH (Transform precoding enabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTN-TDLA |  | NTN-TDLC |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 1.81 | 1.81 | 2.35 | 2.36 |
| 2 | 15 / 5 | 1x2 | -2.64 | -2.62 | -2.47 | -2.45 |
| 3 | 30 / 10 | 1x1 | 1.73 | 1.73 | 2.33 | 2.30 |
| 4 | 30 / 10 | 1x2 | -2.77 | -2.76 | -2.61 | -2.62 |

UL timing adjustment

Here we provide the initial simulation results for UL timing adjustment for alignment, as shown in Table 2.2-3, based on simulation assumption as shown in Table 2.2-1 and Table 2.2-2.

Table 2.2-1 Simulation assumption for UL timing adjustment (part 1)

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15 kHz SCS: 5MHz 30 kHz SCS: 10MHz |
| MCS |  | MCS4 |
| Channel model |  | NTN-TDL-A100-200 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 5 MHz CBW/15kHz SCS: 12 RB for each UE 10MHz CBW/30kHz SCS: 12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 12 for 5MHz for SCS 15kHz, and 12 for 10MHz for SCS 30kHz |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | slot #1 in radio frames |
|  | SRS resource allocation | 15 kHz SCS: CSRS = 5, BSRS =0, for 20 RB 30 kHz SCS: CSRS =5, BSRS =0, for 20 RB |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

Table 2.2-1 Simulation assumption for UL timing adjustment (part 2)

| Parameter | Scenario X |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: NTN-TDLA100-200 |
| UE speed | 120 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s 30 kHz: 5 s |
|  | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 |

Table 2.2-3 Ideal simulation results for UL timing adjustment

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | 2.56 | 2.58 |
| 2 | 15 / 5 | 1x2 | -1.88 | -1.86 |
| 3 | 30 / 10 | 1x1 | 1.66 | 1.92 |
| 4 | 30 / 10 | 1x2 | -2.29 | -2.10 |

PUSCH repetition type A

Here we provide the initial simulation results for PUSCH repetition type A for alignment, as shown in Table 2.3-2, based on simulation assumption as shown in Table 2.3-1.

Table 2.3-1 Simulation assumption for PUSCH repetition type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel model |  | NTN-TDLA100-200 |
| MCS |  | 5 in Table 3 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 2] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 2: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 2.3-2 Ideal simulation results for PUSCH repetition type A

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | -3.73 | -3.76 |
| 2 | 15 / 5 | 1x2 | -7.56 | -7.59 |
| 3 | 30 / 10 | 1x1 | -4.72 | -4.71 |
| 4 | 30 / 10 | 1x2 | -8.25 | -8.23 |

Conclusion

In this contribution, we provide our initial simulation results on NTN PUCCH demodulation requirements.

Reference

R4-2210662, WF on NTN SAN demodulation requirements, RAN4#103-e, Huawei, HiSilicon