3GPP TSG-RAN WG4 # 104-e	R4-2213907

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.10.1.1

Source:	MediaTek Inc.

Title:	Discussion for PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC

Document for:	Discussion

- Introduction

In RAN4#103-e, a way forward on PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC was agreed [1]. In this contribution, we provide our view for remaining issues.

- Discussion

Interference modelling in PDCCH region

- Option 1: All the REs in control region filled with QPSK randomly modulated symbols with random precoding for the number of antenna ports. - Option 2: Assume PDCCH transmission from interference cells, and for each cell, OCNG signal is transmitted on each RE that is not occupied by the PDCCH of this cell. - Option 3: Assume PDCCH transmission from interference cells which is non-overlapping with serving cell.

For the interference modelling in PDCCH region, some companies provide analysis [2] and simulation [3] showed that the colliding PDCCH transmission does not introduce testability issue for PDSCH. Therefore, we prefer to use Option1 or Option 2 to better reflect the realistic scenario.

Proposal 1: Prefer Option 1 and Option 2, assuming overlapping PDCCH among serving cell and interference cell.

T-put gain for 2Rx and 4Rx

Issue 2-1-3: T-put gain for 2 Rx - Option 1: 2 - Option 2: 1.5 Issue 2-1-5: T-put gain for 4 Rx - Option 1: 2.5 - Option 2: 2 - Option 3: 1.5

The remaining issue for CQI requirements is the throughout gain for 2Rx and 4Rx. According to our simulation results, we prefer to use γ=2 for both 2Rx and 4Rx.

Table 1. Summary for the simulation results at SINR = -2dB

|  | FDD 2T2R | FDD 2T4R | TDD 2TR | TDD 2T4R |
| --- | --- | --- | --- | --- |
| TP ratio (γ) | 2.65 | 2.72 | 2.81 | 2.56 |

Proposal 2: Define CQI reporting test metric with γ=2 for both 2Rx and 4Rx.

- Conclusion

In this contribution, we provide our views for remaining issues for PDSCH demodulation and CSI requirements for inter-cell interference MMSE-IRC [1]. We have the following proposals.

Proposal 1: Prefer Option 1 and Option 2, assuming overlapping PDCCH among serving cell and interference cell.

Proposal 2: Define CQI reporting test metric with γ=2 for both 2Rx and 4Rx.

- Reference

- R4-2210681, “WF on PDSCH demod and CSI requirements for inter-cell interference MMSE-IRC”, Ericsson

- R4-2209414 , “On PDSCH requirements for UE MMSE-IRC receiver for inter-cell interference suppression”, China Telecom

- R4-2204376 “Discussion on PDSCH demodulation MMSE-IRC requirements for scenario with inter-cell interference”, Intel