3GPP TSG-RAN WG4 # 104-e	R4-2213926

Electronic Meeting, August 15 – August 26, 2022

Agenda item:	9.18.5.1.2

Source:	MediaTek Inc.

Title:	Simulation results for RedCap PDCCH and PBCH

Document for:	Discussion

- Introduction

In this contribution, we provide simulation results for PDCCH and PBCH requirements based on the simulation assumptions in [1].

- Simulation Results

PDCCH

In Table 1 and 2, we provide the summary of the simulation results for the agreed PDCCH requirements in RAN4#103-e.

1Rx UE: - FR1 FDD 15kHz: Define AL8 only - FR1 TDD 30kHz: Define AL4 only 2Rx UE: - FR1 FDD 15kHz: Define both AL4 and AL8 - FR1 TDD 30kHz: Define both AL4 and AL8 FR2 TDD 120kHz: Define both AL4 and AL8

Table 1. Summary of PDCCH simulation results for FR1 FDD

| Rx number | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10 | 48 | 1 | 8 | TDLA30-10 | 2x1 low | 1% | 5.3.2.1.2-1 Test 3 | 2.9 | 4.9 |

Table 2. Summary of PDCCH simulation results for FR1 TDD

| Rx number | CBW (MHz) | RB size | duration | AL | Propagation condition | Antenna configuration | Metric (Pm-dsg) | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20 | 48 | 1 | 4 | TDLC300-100 | 1x1 | 1% | 5.3.2.2.1-1 Test 2 | 6.5 | 8.5 |

PBCH

We summarize PBCH simulation results for the case of unknown SS/BPCH block index in Table 3

Table 3. Summary of PBCH simulation results.

| 1Rx UE or 2Rx UE | BW / SSB SCS | Propagation condition | Antenna configuration | Metric (Pm-bch) | SS/PBCH block index | Reference from TS38.101-4 | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | TDLC300-100 | 1x1 | 1% | Unknown | 5.4.2.1-2 Test 1 | -6.1 dB | -4.1 dB |
| 1Rx UE | 20MHz / 30kHz | TDLA30-10 | 1x1 | 1% | Unknown | 5.4.2.2-2 Test 1 | -4.0 dB | -2.0 dB |

- Conclusion

In this contribution, we provide simulation results for RedCap PDCCH and PBCH based on the agreed simulation assumption in [1].

- Reference

- R4-2210672, “WF on RedCap demodulation and CQI reporting requirements”, Ericsson