# 3GPP TSG-RAN WG4 Meeting # 104-e	R4-2213956

# E-Meeting, 15 Aug. 2022 – 26 Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TC for PUCCH SCell activation and deactivation delay requirements of FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh2- Perf |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TC for PUCCH SCell activation and deactivation delay requirements needs to be specified based on the core requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test case for PUCCH SCell activation and deactivation delay for one FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case for PUCCH SCell activation will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

#### A.4.5.3.X4	SCell Activation and deactivation of one FR1 unknown PUCCH SCell and one FR1 unknown SCell (All NR cells in FR1) with valid TA and invalid TA

##### A.4.5.3.X4.1	Test Purpose and Environment

The purpose of this test is to verify that when a PUCCH SCell and DL SCell are activated using the same MAC CE command, the PUCCH SCell, DL SCell activation and deactivation delays are within the requirements stated in clause 8.3.13 and 8.3.15, when the PUCCH SCell in FR1 and DL SCell in FR1 are unknown to the UE at the time of activation.

The supported test configurations are shown in table A.4.5.3.X4.1-1 below. The test parameters are given in Tables A.4.5.3.X4.1-2 and cell-specific test parameters are given in A.4.5.3.X4.1-3.

The test consists of two subtests, one with valid timing advance (TA), and other with invalid TA. The test consists of six successive time periods, with duration of T1, T2, T3, T4, T5 and T6, respectively. There is one E-UTRA carrier and three NR carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 and Cell 2 but is not aware of Cell3 (PUCCH SCell) and Cell4 (DL SCell). Cell1 is on to E-UTRA carrier and is configured on primary timing advance group (pTAG). Cell2 is on NR carrier and is configured on primary timing advance group (sTAG). AT the start of the test, the UE is only monitoring the SpCell on MCG and SCG. The UE shall be continuously scheduled on the SpCell(s) throughout the whole test. PCC of Cell 2, and SCC of Cell 3 and SCC of Cell 4 are on different bands.

At the beginning of T1, the UE receives an RRC message by which the PUCCH SCell (Cell 3) and DL SCell (Cell 4) becomes configured on radio channel 3 and 4 respectively. The UE starts monitoring the SCC1(Cell 3 CC) and SCC2 (Cell 4 CC) but do not send any L3-RSRP reports. During the duration of T1, the time alignment timer is running on and pTAG and sTAG, and the TA is maintained on both PCell and PSCell. At the end of T1, the test equipment sends a MAC CE message for activation of the PUCCH SCell and DL SCell simultaneously on the SCG.

The point in time at which the MAC message is received at the UE antenna connector is denoted as slot #n1, defines the start of time period T2. The UE shall be able to report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and valid CSI on SpCell for the activated PUCCH SCell at latest in slot n1+ Tactivate_total_PUCCH_SCell and report L1-RSRP by by slot n1+ Tactivation_time_multiple_scells and valid CSI on SpCell for the activated DL SCell at latest in slot n1+ Tactivate_total_other_SCell, as defined in clause 8.3.13.

For Cell3 activation, the UE shall start reporting CSI in PCell in slot n1+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell4 activation, the UE shall start reporting CSI in PCell in slot n1+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n1+1+THARQNR slot length to n1+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

Time period T3 starts when a MAC message for deactivation of PUCCH SCell and DL SCell, sent from the test equipment to the UE in a slot # denoted n2, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n2+THARQ+3msNR slot length, as defined in clause 8.3.15, and the starting point of any SpCell interruption due to the deactivation shall occur in the slot n2+1+THARQNR slot length to n2+1+THARQ+3msNR slot length, as defined in clause 8.3.15.

Time period T4 starts when the Cell3 and Cell4 are deactivated. In the time period T4, UE do not monitor SCC1 and SCC2 (i.e., Cell3 and Cell4). During T4, PSCell do not receive any new TA command from TE and the time alignment timer expires. At the end of T4, UE do not have a valid TA for sTAG and the test equipment sends a MAC message for activation of the PUCCH SCell and DL SCell simultaneously on the SCG.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n3 defines the start of time period T5. By the end of time period T5, UE shall be able to report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and acquire new TA command by sending RACH to PUCCH SCell and UE shall be able to report valid CSI on the activated PUCCH SCell at latest in slot n3+ Tactivate_total_PUCCH_SCell and report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and valid CSI report to activated SpCell for the activated DL SCell at latest in slot n3+ Tactivate_total_other_SCell, as defined in clause 8.3.13.

For Cell3 activation, the UE shall start reporting CSI in PCell in slot n3+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the PUCCH SCell activation has been completed. For Cell4 activation, the UE shall start reporting CSI in PCell in slot n3+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the DL SCell activation has been completed.

Any PCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n3+1+THARQNR slot length to n3+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

Time period T6 starts when a MAC message for deactivation of PUCCH SCell and DL SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot n4+THARQ+3msNR slot length, as defined in clause 8.3.15, and the starting point of any SpCell interruption due to the deactivation shall occur in the slot n4+1+THARQNR slot length to n4+1+THARQ+3msNR slot length, as defined in clause 8.3.15.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in SpCell during activation and deactivation of PUCCH SCell and DL SCell, respectively.

The test equipment verifies the PUCCH SCell activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received. The test equipment verifies the DL SCell activation time by counting the slots from the time when the DL SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the PUCCH SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for PUCCH SCell is discontinued. The test equipment verifies the DL SCell deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for DL SCell is discontinued.

Table A.4.5.3.X4.1-1: unknown FR1 PUCCH SCell and DL SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.3.X4.1-2: General test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three NR radio channel (1, 2, 3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. As specified in clause A.3.7.2.1 |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated PUCCH SCell |  | Cell 3 | Configured deactivated PUCCH secondary cell on NR RF channel number 3 |
| Configured deactivated DL SCell |  | Cell 4 | Configured deactivated DL secondary cell on NR RF channel number 4 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | For both cell 2 and cell 3 |
| Cell3 / Cell 4 timing offset to Cell 2 | s | 0 |  |
| Time alignment error between cell2 and cell1; cell3 and cell1; cell3 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 6 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| T4 | s | 6 | During this time the PSCell shall be known and the SCell configured and detected. |
| T5 | s | 1 | During this time the UE shall activate the PUCCH SCell and DL SCell. |
| T6 | s | 1 | During this time the UE shall deactivate the PUCCH SCell and DL SCell. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.4.5.3.X1.1-3: Cell specific test parameters for unknown FR1 PUCCH SCell and DL SCell activation case

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 and Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1/T4 | T2/T5 | T3/T6 | T1/T4 | T2/T5 | T3/T6 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | 5 |  |  | 5 (on cell 2) |  |  |
|  | Config 3 |  | 10 |  |  | 10 (on cell 2) |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | 2 |  |  | 2 (on cell 2) |  |  |
|  | Config 3 |  | 4 |  |  | 4 (on cell 2) |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. Note 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 cl. 5.2.2.5. |  |  |  |  |  |  |  |  |

##### A.4.5.3.X4.2	Test Requirements

If UE receives a MAC CE command to activate PUCCH SCell, in slot #n1, if the TA is valid, the UE shall be able to report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and report valid CSI on SpCell for the activated PUCCH SCell at latest in slot n1+ Tactivate_total_PUCCH_SCell and report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and report valid CSI on SpCell for the activated DL SCell at latest in slot n1+ Tactivate_total_other_SCell, as defined in clause 8.3.13.

Any PCell or PSCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n1+1+THARQNR slot length to n1+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

When UE receives deactivation MAC CE at n2 slot, the UE shall stop sending CSI reports for both PUCCH SCell and DL SCell no later than slot n2+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any SpCell interruption due to the deactivation shall occur in the slot n2+1+THARQNR slot length to n2+1+THARQ+3msNR slot length, as defined in clause 8.3.15.

When UE receive MAC CE command to activate PUCCH SCell, in slot #n3, if the TA is invalid, the UE shall be able to report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and report valid CSI on SpCell for the activated PUCCH SCell at latest in slot n3+ Tactivate_total_PUCCH_SCell and report L1-RSRP by slot n1+ Tactivation_time_multiple_scells and report valid CSI on SpCell for the activated DL SCell at latest in slot n3+ Tactivate_total_other_SCell, as defined in clause 8.3.13.

Any PCell or PSCell interruption due to activation of PUCCH SCell or DL SCell shall occur in the slot n3+1+THARQNR slot length to n3+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.13, where Ninterruption is the interruption length given in clause 8.2.2.2.7.

When UE receives deactivation MAC CE at n4 slot, the UE shall stop sending CSI reports for both PUCCH SCell and DL SCell no later than slot n4+THARQ+3msNR slot length, as defined in clause 8.3. The starting point of any SpCell interruption due to the deactivation shall occur in the slot n4+1+THARQNR slot length to n4+1+THARQ+3msNR slot length, as defined in clause 8.3.15.

### <End of Change 1>