3GPP TSG-RAN WG4 Meeting #104-e	R4-2213985

# Electronic meeting, August 15 – 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0010 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.176-1 with eIAB Rel-17 updates to test tolerance for timing error between IAB-DU and IAB-MT. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Perf |  |  |  |  |  | Date: |  |  | 10-08-2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This is CR to TS 38.176-1 Rel-17 that introduces updates to test tolerance for timing error between IAB-DU and IAB-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition clause 6.6.5 Timing error between IAB-DU and IAB-MT in table 4.1.2.2-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 IAB conformance test specification will not include test tolerance for timing error between IAB-DU and IAB-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.176-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in part 1 of the present document, i.e. to all conducted tests defined for FR1. The frequency ranges FR1 and FR2 are defined in clause 5.1 of TS 38.174 [2].

The minimum requirements are given in TS 38.174 [2] and the references therein. Test Tolerances for the conducted test requirements explicitly stated in the present document are given in Annex C of the present document.

Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in Annex C.

### 4.1.2	Acceptable uncertainty of Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the Test System is specified below for each test defined explicitly in the present specification, where appropriate. The maximum acceptable uncertainty of the Test System for test requirements included by reference is defined in the respective referred test specification.

For IAB type 1-H when a requirement is applied per TAB connector then the test uncertainty is applied to the measured value. When a requirement is applied for a group of TAB connectors then the test uncertainty is applied to sum of the measured power on each TAB connector in the group.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For RF tests, it should be noted that the uncertainties in clause 4.1.2 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 IAB output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.3.1 IAB-DU Output power dynamics | ± 0.4 dB |  |
| 6.3.2 IAB-MT Output power dynamics | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |
| 6.4.1 Transmit OFF power | ±2.0 dB, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.4.2 Transmitter transient period | N/A |  |
| 6.5.2.1 IAB-DU Frequency error | ± 12 Hz |  |
| 6.5.2.2 IAB-MT Frequency error | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz |  |
| 6.5.3 EVM | ± 1% |  |
| 6.5.4 Time alignment error | ± 25 ns |  |
| 6.6.5 Timing error between IAB-DU and IAB-MT | [± 25 ns] |  |
| 6.6.2 Occupied bandwidth | 10 MHz IAB Channel BW: ±100 kHz 15 MHz, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz IAB Channel BW: ±300 kHz 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz IAB Channel BW: ±600 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR/ CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) CACLR BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB CACLR absolute power ±2.0 dB, f ≤ 3 GHz CACLR absolute power ±2.5 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.6.4 Operating band unwanted emissions | ±1.5 dB, f ≤ 3 GHz ±1.8 dB, 3 GHz < f ≤ 6 GHz (Note) |  |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 19 GHz: ±4.0 dB 19 GHz < f ≤ 26 GHz: ±4.5 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Additional spurious emission requirements | ±2.0 dB for > -60 dBm, f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 4.2 GHz ±3.0 dB, 4.2 GHz < f ≤ 6 GHz ±3.0 dB for ≤ -60 dBm, f ≤ 3 GHz ±3.5 dB, 3 GHz < f ≤ 4.2 GHz ±4.0 dB, 4.2 GHz < f ≤ 6 GHz |  |
| 6.6.5.2.3 Transmitter spurious emissions, Co-location | ±3.0 dB |  |
| 6.7 Transmitter intermodulation (interferer requirements) This tolerance applies to the stimulus and not the measurements defined in 6.6.3, 6.6.4 and 6.6.5 | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests in 6.6.3 (ACLR), 6.6.4 (OBUE) and 6.6.5 (spurious emissions) which have to be carried out in the presence of the interferer. ±1.0 dB | The uncertainty of interferer has double the effect on the result due to the frequency offset |
| NOTE: Test system uncertainty values for 4.2 GHz < f ≤ 6 GHz apply for IAB operates in licensed spectrum only. |  |  |