3GPP TSG-RAN4 Meeting #104e	R4-2213989

# Electronic Meeting, 15th Aug – 26th Aug, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR: Introduce test setup and FRC for CRS-IM without NWA for FDD scenario 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the big CR R4-2210660, the test setup for CRS-IM without NWA for FDD scenario 2 is TBA. The FRC for cases with inter-RAT measurement enabled are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the corresponding test setup and related FRC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test setup and FRC are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1,A.3.2.2,5.2.2.1.x,5.2.3.1.x,5.2.2.2.x,5.2.3.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AGC		Automatic Gain Control

CA	Carrier Aggregation

CC	Component Carrier

CCE	Control Channel Element

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CW	Codeword

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRI	CSI-RS Resource Indicator

CRS	Cell-specific Reference Signal

CRS-IM	CRS-Interference Mitigation

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DMRS	Demodulation Reference Signal

DPS	Dynamic Point Selection

EPRE	Energy Per Resource Element

EN-DC	E-UTRA-NR Dual Connectivity

FR	Frequency Range

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite System

HARQ	Hybrid Automatic Repeat Request

HST 	High Speed Train

HST-SFN 	High Speed Train Single Frequency Network

LI	Layer Indicator

MAC	Medium Access Control

MCS	Modulation and Coding Scheme

MIB	Master Information Block

NR	New Radio

NSA	Non-Standalone Operation Mode

OCC	Orthogonal Cover Code

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PBCH	Physical Broadcast Channel

Pcell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PMI	Precoding Matrix Indicator

PRB	Physical Resource Block

PRG	Physical resource block group

PSBCH	Physical Sidelink Broadcast Channel

PSCCH	Physical Sidelink Control Channel

PSFCH	Physical Sidelink Feedback Channel

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PTRS	Phase Tracking Reference Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-location

RB	Resource Block

RBG	Resource Block Group

RE	Resource Element

REG	Resource Element Group

RI	Rank Indicator

RRC	Radio Resource Control

SA	Standalone operation mode

SCI	Sidelink Control Information

SCS	Subcarrier Spacing

SINR	Signal-to-Interference-and-Noise Ratio

SL	Sidelink

SLSS	Sidelink Synchronization Signal

SNR	Signal-to-Noise Ratio

SS	Synchronization Signal

SSB	Synchronization Signal Block

SSS	Secondary Synchronization Signal

TCI	Transmission Configuration Indicator

TDM	Time division multiplexing

TRxP	Transmission and Reception Point

TTI	Transmission Time Interval

UL	Uplink

V2X	Vehicle to Everything

VRB	Virtual Resource Block

5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

5.2.2.1.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.2.1.x-4 and Table 5.2.2.1.x-5, with the addition of test parameters in Table 5.2.2.1.x-2 and 5.2.2.1.x-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.x-1.

Table 5.2.2.1.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.2.1.x-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
|  |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports [0,1,2,3] | Antenna ports [0,1,2,3] |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.1.x-4.

Table 5.2.2.1.x-4: Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.1.x-6 with following test procedure:

The network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations in Table 5.2.2.1.x-5. PDSCH is not scheduled and throughput is not countered within [4.34s] after the beginning of test.  PDSCH is not scheduled in the measurement gaps.

Table 5.2.2.1.x-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | [7] |
| Measurement gap timeing advance | ms | 0 |

Table 5.2.2.1.x-6: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x2, ULA Low | 70 | TBD |

5.2.3.1.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.3.1.x-4 and Table 5.2.3.1.x-5, with the addition of test parameters in Table 5.2.3.1.x-2 and 5.2.3.1.x-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.x-1.

Table 5.2.2.1.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.2.1.x-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
|  |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports [0,1,2,3] | Antenna ports [0,1,2,3] |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.1.x-4:

Table 5.2.3.1.x-4: Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.1.x-6 with following test procedure:

The network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations in Table 5.2.3.1.x-5. PDSCH is not scheduled and throughput is not countered within [4.34s] after the beginning of test.  PDSCH is not scheduled in the measurement gaps.

Table 5.2.3.1.x-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | [7] |
| Measurement gap timeing advance | ms | 0 |

Table 5.2.3.1.x-6: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | TBD |

##### 5.2.2.2.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.2.2.x-4 and Table 5.2.2.2.x-5, with the addition of test parameters in Table 5.2.2.2.x-2 for the serving cell and Table 5.2.2.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.x-1.

Table 5.2.2.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.2.2.x-2: Tests parameters for  serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.x-4 with following test procedure:

[TBA]

Table 5.2.2.2.x-4: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.x-5:

Table 5.2.2.2.x-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-4.1 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

##### 5.2.3.2.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.3.2.x-4 and Table 5.2.3.2.x-5, with the addition of test parameters in Table 5.2.3.2.x-2 for the serving cell and Table 5.2.3.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.x-1.

Table 5.2.3.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.3.2.x-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.x-4 with following test procedure:

[TBA]

Table 5.2.3.2.x-4: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.x-5:

Table 5.2.3.2.x-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-4.1 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

##### 5.2.2.2.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.2.2.x-4 and Table 5.2.2.2.x-5, with the addition of test parameters in Table 5.2.2.2.x-2 for the serving cell and Table 5.2.2.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.x-1.

Table 5.2.2.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.2.2.x-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Time offset to the serving cell |  | us | 1.5 | -0.5 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Refer to [X.Y.Z] Note 2: The channel for the LTE interference cells and the serving cell are independent. Note 3: No MBSFN is configured on LTE carrier. Note 4: Network-based CRS interference mitigation is disabled on LTE carrier. Note 5: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.x-5 with following test procedure:

The network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations in Table 5.2.2.2.x-4. PDSCH is not scheduled and throughput is not countered within [4.34s] after the beginning of test.  PDSCH is not scheduled in the measurement gaps.

Table 5.2.2.2.x-4: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | [0] |
| Measurement gap timeing advance | ms | 0 |

Table 5.2.2.2.x-5: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.2.2.x-5:

Table 5.2.2.2.x-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | TBA |

##### 5.2.3.2.x	Minimum requirements for PDSCH with inter cell CRS interference

The performance requirements are specified in Table 5.2.3.2.x-4 and Table 5.2.3.2.x-5, with the addition of test parameters in Table 5.2.3.2.x-2 for the serving cell and Table 5.2.3.2.x-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.x-1.

Table 5.2.3.2.x-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.3.2.x-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.x-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.x | As specified in clause B.x |
| Time offset to the serving cell |  | us | 1.5 | -0.5 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Refer to [X.Y.Z] Note 2: The channel for the LTE interference cells and the serving cell are independent. Note 3: No MBSFN is configured on LTE carrier. Note 4: Network-based CRS interference mitigation is disabled on LTE carrier. Note 5: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame |  |  |  |  |

The requirements for UE capable of performing CRS-IM without the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.x-4 with following test procedure:

The network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations in Table 5.2.3.2.x-4. PDSCH is not scheduled and throughput is not countered within [4.34s] after the beginning of test.  PDSCH is not scheduled in the measurement gaps.

Table 5.2.3.2.x-4: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | [0] |
| Measurement gap timeing advance | ms | 0 |

Table 5.2.3.2.x-4: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

The requirements for UE capable of performing CRS-IM with the assistance of network signaling on LTE channel bandwidth are specified in Table 5.2.3.2.x-5:

Table 5.2.3.2.x-5 Minimum performance for Rank 1 with the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBA | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | TBA |

A.3.2.1	FDD

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.1.1-7: PDSCH Reference Channel for FDD LTE-NR coexistence scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-7.1 FDD | R.PDSCH.1-7.2 FDD | R.PDSCH.1-7.3 FDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 9 | 11 | 9 |  |  |
| Allocated slots per 2 frames | Slots | 16 | 16 | 16 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 13 |  |  |
| Modulation |  | QPSK | QPSK | 16QAM |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 18 | 18 | 12 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 2472 | 3240 | 8456 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 16 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | CBs | N/A | N/A | N/A |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | CBs | 1 | 1 | 2 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 11 | Bits | 7760 | 10256 | 16224 |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {1,…, 9, 12, …, 19} | Bits | 8384 | 10880 | 17472 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 1.978 | 2.592 | 6.764 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

Table A.3.2.1.1-x: PDSCH Reference Channel for FDD CRS interference mitigation for NR scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-17.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 16 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 13064 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | CBs | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {0,…,19} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,10,15 | Bits | N/A |  |  |  |  |
| For Slots i = 11 | Bits | 22880 |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} for i from {1,…, 9, 12, …, 19} | Bits | 24128 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 10.4512 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell |  |  |  |  |  |  |

Table A.3.2.1.1-x: PDSCH Reference Channel for FDD CRS interference mitigation for NR scenario for inter-RAT measurement enabled

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-17.2 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 4 frames | Slots | 26 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,7,8,9,10,11,12,15,20,25,30,35 | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} and i≠7,8,9,11,12 | Bits | 13064 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,7,8,9,10,11,12,15,20,25,30,35 | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} and i≠7,8,9,11,12 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,7,8,9,10,11,12,15,20,25,30,35 | CBs | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} and i≠7,8,9,11,12 | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = 0,5,7,8,9,10,11,12,15,20,25,30,35 | Bits | N/A |  |  |  |  |
| For Slots i = 31 |  | 22880 |  |  |  |  |
| For Slots i, if mod(i, 5) = {1,2,3,4} and i≠7,8,9,11,12,31 | Bits | 24128 |  |  |  |  |
| Max. Throughput averaged over 4 frames | Mbps | 8.492 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 4 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell Note 4: No user data is scheduled on slots used for measurement |  |  |  |  |  |  |

A.3.2.2	TDD

A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.2.1-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.15-1 and LTE-NR coexistence scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 TDD | R.PDSCH.1-1.2 TDD | R.PDSCH.1-1.3 TDD |  |  |
| Channel bandwidth | MHz | 10 | 10 | 20 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Allocated resource blocks | PRBs | 52 | 52 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 9 | 11 | 9 |  |  |
| Allocated slots per 2 frames |  | 7 | 7 | 7 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 4 | 4 | 13 |  |  |
| Modulation |  | QPSK | QPSK | 16QAM |  |  |
| Target Coding Rate |  | 0.30 | 0.30 | 0.48 |  |  |
| Number of MIMO layers |  | 1 | 1 | 11 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 12 | 12 | 12 |  |  |
| Overhead for TBS determination |  | 18 | 18 | 18 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 2472 | 3240 | 15880 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 16 | 16 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A | N/A | N/A |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 1 | 1 | 2 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 10, 11 | Bits | 7760 | 10256 | 32672 |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 8384 | 10880 | 33920 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 0.865 | 1.134 | 5.558 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

Table A.3.2.2.1-x: PDSCH Reference Channel for TDD CRS interference mitigation for NR scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 7 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 26632 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 4 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 47936 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 49184 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.3212 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell |  |  |  |  |  |  |

Table A.3.2.2.1-x: PDSCH Reference Channel for TDD CRS interference mitigation for NR scenario for inter-RAT measurement enabled

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-4.2 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,20} for i from {0,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 4 frames |  | 16 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,20} for i from {0,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,20} for i from {0,…,39} | Bits | 26632 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,20} for i from {0,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,20} for i from {0,…,39} | CBs | 4 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {2,3,4,8,9} and i = {0,1,5,20} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=10,11,30,31 | Bits | 47936 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,5,6,7} and i ≠ {0,1,5,10,11,20,30,31} for i from {0,…,39} | Bits | 49184 |  |  |  |  |
| Max. Throughput averaged over 4frames | Mbps | 10.65 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell Note 4: No user data is scheduled on slots used for measurement |  |  |  |  |  |  |

A.3.2.2.2 Reference measurement channels for SCS 30 kHz  FR1

Table A.3.2.2.2-19 : PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1  with overlapping spectrum for LTE and NR

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-19.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 51 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = {0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,…,39} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 23 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19} for i from {0,…,39} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = {0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,…,39} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19,} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = {0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,…,39} | Bits | 12808 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19,} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) ={0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,…,39} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19} for i from {0,…,39} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = {0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,…,39} | CBs | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 20) = {4,5,7,8,9,17,18,19} for i from {0,…,39} | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 25704 |  |  |  |  |
| For Slot i, if mod(i, 20) = {0,1,2,3,6,10,11,12,13,14,15,16} for i from {1,..,9,12,...39} | Bits | 26928 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 14.7292 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 40 ms. Note 2: Slot i is slot index per 2 frames. Note 3: No PDSCH data scheduling on slots with LTE PBCH/PSS/SSS. |  |  |  |  |  |  |

Table A.3.2.2.2-20 : PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1  with overlapping spectrum for LTE and NR for inter-RAT measurement enabled

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-4.3 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |
| Allocated resource blocks | PRBs | 51 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 |  | 4 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4} and i ≠ {0,1,2,3,4,5,40,41} for i from {0,…,79} |  | 12 |  |  |  |  |
| Allocated slots per 4 frames |  |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 |  | 6 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4} and i ≠ {0,1,2,3,4,5,40,41} for i from {0,…,79} |  | 12 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 |  | 8456 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4} and i ≠ {0,1,2,3,4,5,40,41} for i from {0,…,79} | Bits | 12808 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,7} and i ≠ {0,1,2,3,4,5,40,41} for i from {0,…,79} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,7} and i ≠ {0,1,2,3,4,5,40,41} for i from {0,…,79} | CBs | 4 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {5,6,8,9 } and i = {0,1,2,3,4,40,41} for i from {0,…,79} | Bits | N/A |  |  |  |  |
| For Slot i=20,21,60,61 | Bits | 25704 |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 |  | 17808 |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,7} and i ≠ {0,1,2,3,4,5,20,21,40,41,60,61} for i from {0,…,79} | Bits | 26928 |  |  |  |  |
| Max. Throughput averaged over 4frames | Mbps | 14.7292 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell Note 4: No user data is scheduled on slots used for measurement |  |  |  |  |  |  |

## B.x	Interference model for PDSCH requirements with LTE-NR spectrum sharing

This clause provides interference modelling for each explicitly modelled LTE interfering cell. Each interfering cell involved in PDSCH performance requirements for LTE-NR spectrum sharing is characterized by its associated interferer to noise ratio (INR) value as defined in [B.Y.1].

In each subframe, each interfering cell shall transmit 16QAM randomly modulated data over the entire PDSCH region and over the full transmission bandwidth according to the probabilities of occurrence. Transmitted physical channels shall include PSS, SSS and PBCH. Probabilities of occurrence of LTE PDSCH in each subframe are as specified in requirements.

For each subframe, a transmission rank shall be randomly determined independently from interfering cells. Probabilities of occurrence of each possible transmission rank are as specified in requirements.

For each subframe, a precoding matrix for the number of layers  associated to the selected rank shall be selected randomly from Table 6.3.4.2.3-1 of TS 36.211 [15]. Note that codebook index 0 shall be excluded from random precoder selection when the number of layers is .

Precoding for spatial multiplexing with CRS for the number of antenna ports shall be applied to 16QAM randomly modulated layer symbols, as specified in subclause 6.3.4.2.1 of TS 36.211 [15] with the selected precoding matrices for each subframe.

For unallocated REs in the control region, precoding for transmit diversity for the number of antenna ports in the simulation scenario shall be applied to QPSK randomly modulated layer symbols, as specified in subclause 6.3.4.3 of TS 36.211 [15]. The EPRE ratio for these REs shall be as defined for PDCCH in Annex C.3.2 of TS 36.101 [4].