3GPP TSG-RAN WG4 Meeting #104-e	R4-2214005

# Electronic Meeting, August 15 - 26, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	PC2 Back-off Measurements for NS_05

Agenda Item:	5.1.2	High power UE (power class 2) for NR FDD band [NR_PC2_UE_FDD]

Document for:	Discussion

# Introduction

This contribution presents power amplifier (PA) back-off measurements for NS_05 for power class 2 (PC2) operation.

# Discussion

The following assumptions are used for filter rejection:

- Filter rejection at B34 Rx: 11dB filter rejection.

NS_05 measurement data are presented to verify proposed A-MPR specified in Table 6.2.3.4-12 / Table 1 below.

Table 1: PC2 NS_05 agreed A-MPR (from Table 6.2.3.4-12)

The NS_05 is defined to ensure protection of PHS services with requirements reproduced from Table 6.5.3.3.4-1 into Table 2.

Table 2: Additional requirements for "NS_05" (from Table 6.5.3.3.4-1)

## Power Amplifier Calibration

- PA calibration points: Due to the large back-off agreed for PC2 FDD operation ( 13dB), we use two PA voltage supply levels (Vcc1, Vcc2) using the waveform 20 MHz channel bandwidth (CBW), SCS 15 kHz, QPSK, DFT-S-OFDM, Lcrb=100 RB at Rbstart= 0:

  - Vcc1 to meet -31dBc ACLR at 1 dB PC2 MPR;

  - Vcc2 to meet -37dBc ACLR at 11 dB PC2 MPR.

- Post PA losses: 4dB;

- Power Class 2 (PC2) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- Counter-IM3 (C-IM3): -60dBc;

- Counter-IM5 (C-IM5): -70dBc; and

- we adopt notation xRy to refer to waveform of Lcrb=”x” RBs positioned at RBstart=”y”.

## NS_05 PHS band Protection Results

Table 1 below summarizes the measurement results obtained for 20MHz CBW, Fc=1930MHz to meet band 34 protection requirements.

Table 2: Measured PC2 back-off levels for PHS band protection/NS_05 at 20MHz CBW, carrier Fc = 1930MHz.

| RB length | RB start | Back-off [dB] |
| --- | --- | --- |
| 001 | 000 | 8.5 |
| 002 | 000 | 8 |
| 003 | 000 | 7.3 |
| 005 | 000 | 6.8 |
| 001 | 027 | 0 |
| 001 | 028 | 0 |
| 001 | 029 | 0 |
| 001 | 030 | 0 |
| 001 | 105 | 9.1 |
| 050 | 000 | 12.1 |
| 075 | 000 | 12 |
| 100 | 000 | 12 |

The highest observed raw back-off level is approximately 12dB for PC2 operation for RB allocations 50R0,75R0 and 100R0, this back-off being driven by the Vcc2, i.e., the lowest PA calibration voltage. The agreed A-MPR therefore provides 1dB margin with regard to these measurements. We summarize in Figure 1 simulations the PC2 NS_05 results from [1],[2],[3]. We note that for these allocations, the simulated results spread across a wide range: 13dB to 14dB in [1]; 6.5 to 7dB in [2] and 11 to 12.5 dB in [3].

Figure 1: NS_05 Simulation results from [1] (left), [2] (center), [3] (right).

Observation 1:

The highest measured raw back-off is 12dB for region A1 RB allocations, i.e., 1 dB margin from the agreed [13] dB A-MPR in table 6.2.3.4-12.

At last meeting RAN4#103e, it was proposed to increase the PC2 A-MPR to [15] dB based on APT-like PA PC2 measurements [4]. Some companies objected and noted that this would allow a PC2 UE to have more than 3dB A-MPR difference compared to a PC3 UE, meaning that the PC2 UE would be allowed to have worst performance than a PC3 UE. We also share the opinion that 3dB is the maximum difference in A-MPR that can be accepted between a PC2 UE and a PC3 UE. Considering band n1 PC3 UEs are legacy devices, we cannot see how the PC3 A-MPR for NS_05 should be revisited at this stage.

Observation 2:

With [13] dB A-MPR for A1 region, the difference in A-MPR between the band n1 PC2 UE and the PC3 UE is 3dB (10dB A-MPR is specified in Table 6.2.3.4-2). The benefits of PC2 operation for FDD band n1 would be lost if more than 3dB PC2-PC3 A-MPR difference were allowed.

Based on observations 1 and 2, we propose to remove brackets for PC2 A-MPR in Table 6.2.3.4-12.

Proposal: Remove brackets in Table 6.2.3.4-12 for PC2 NS_05 A-MPR.

# Conclusions

In this contribution, we present our measurements for QPSK waveforms for n1 PC2 FDD operation for NS_05/PHS band protection.

Observation 1:

The highest measured raw back-off is 12dB for region A1 RB allocations, i.e., 1 dB margin from the agreed [13] dB A-MPR in table 6.2.3.4-12.

Observation 2:

With [13] dB A-MPR for A1 region, the difference in A-MPR between the band n1 PC2 UE and the PC3 UE is 3dB (10dB A-MPR is specified in Table 6.2.3.4-2). The benefits of PC2 operation for FDD band n1 would be lost if more than 3dB PC2-PC3 A-MPR difference were allowed.

Based on observations 1 and 2, we propose to remove brackets for PC2 A-MPR in Table 6.2.3.4-12.

Proposal: Remove brackets in Table 6.2.3.4-12 for PC2 NS_05 A-MPR.

# References

[1]	R4- 2201834, On A-MPR requirements for PC2 FDD bands, 3GPP TSG-RAN WG4 Meeting #101-ebis, Huawei, HiSilicon

[2]	R4-2200444, HPUE A-MPR Proposal for NS_05, 3GPP RAN WG4 Meeting #101bis-e, Apple Inc.

[3] R4-2204221, n1 PC2 FDD NS_05 AMPR, 3GPP TSG-RAN WG4 Meeting #102e, Qualcomm Incorporated.

[4] R4- 2209181, Consideration on practical PA implementation for NS_05 A-MPR, 3GPP TSG-RAN WG4 Meeting #103-e, Huawei, HiSilicon