3GPP TSG-RAN4 Meeting #104-e	R4-2214039

Aug 2022

Title:	DC location reporting for different features

Source:	Qualcomm Incorporated

Agenda item:	9.4.3

Release:	Rel-17

Document for:	Approval

# 1.	Introduction

An LS [28] that completes the RAN4 work for this feature was sent in Ran4#103e. RAN4 needs to wait until RAN2 completes the work and then RAN4 can updated its specifications.

This paper discusses applicability of the DC location signalling methods and what impact it should have to the RAN4 specifications.

# 2. 	Discussion

The reporting methods except for Rel-15 are optional with capabilities but not all cases can be supported with all reporting methods. Below we present a table which specified options can be supported with which signalling methods:

| Feature/Reporting method |  | R15 | R16 | R17 |
| --- | --- | --- | --- | --- |
| Single CC |  | Yes | Yes | Yes |
| DL CA, single UL CC | UL DC on UL CC | Yes | Yes | Yes |
|  | UL DC on DL CC | No | No | Yes |
| Contiguous UL CA up to 2 UL CCs | Single LO on UL CC | Yes, but one DC per UL CC | Yes | Yes |
|  | Single LO outside UL CC | No | No | Yes |
|  | Dual LO on UL CC | Yes, one DC per UL CC | Yes | Yes |
|  | Dual LO on DL CC (N/A in RAN4 specs) | Yes | No | Yes |
| Contiguous UL CA > 2 UL CCs | Single LO, all cases | Yes, one DC per UL CC | No | Yes |
| Non-contiguous UL CA | Single LO on UL CC | Yes, but one per CC | Yes | Yes |
|  | Single LO outside UL CC | No | No | Yes |
|  | Dual LO on UL CC | Yes, one per CC | No | Yes |
|  | Dual LO, at least one outside UL CC | No | No | Yes |

We invite comments on the table and perhaps an approved document in the absence of TR for this WI is appropriate to summarise applicable cases.

RAN4 can update the RAN4 specifications indicating with what method UE can declare the carrier leakage frequency to the corresponding specification clauses. While RAN4 waits for RAN2 to conclude the specification, RAN4 can discuss if all possible methods will be added or should some flexibility be limited. For example, it might not make sense to allow UE to report FR2 UL CA with R16 method since then it would apply only for up to 2 CC and only 2 UL CC deployment is not very common but up to 8 CCs is more frequently used. Also for FR1 NC UL CA, single LO case can not be supported with R16 method so RAN4 could discuss if RAN4 should enable R16 for NC UL CA at all since R16 can not support all variations.

Observation 1: RAN4 should discuss for which feature variants which method for DC location signalling is applicable

An other discussion point for RAN4 is that will exceptions because of carrier leakage continue to be allowed even if UE does not signal the location of that carrier leakage. For example in the TS 38.101-1 Table 6.4A.2.1.2-2 states that:

“The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs”

This was originally copied from LTE and despite Rel-16 WI enabled a method for signalling the location of the LO for the dual LO and single LO cases, the specification even after close of Rel-17 allows exceptions regardless UE signals the location or not.

Observation 2: RAN4 specification allows exceptions for carrier leakage even if UE decides not to inform the location of the DC with the signalling methods

On our view, this should be changed, and UE should be mandated to use the signalling method if it needs the exception.

Proposal: RAN4 specifications will be changed to allow exceptions for carrier leakage and IQ image only if UE declares support for an appropriate method for signalling the DC location

CR can be prepared in next meeting with the help of the table 1 or its revision. WF on this issue in this meeting is encouraged.

This does not make any signalling method mandatory since UE can choose to not declare the location but in that case the carrier leakage and IQ image requirements become more stringent for that UE

# Conclusion

Observation 1: RAN4 should discuss for which feature variants which method for DC location signalling is applicable

Observation 2: RAN4 specification allows exceptions for carrier leakage even if UE decides not to inform the location of the DC with the signalling methods

Proposal: RAN4 specifications will be changed to allow exceptions for carrier leakage and IQ image only if UE declares support for an appropriate method for signalling the DC location

# References

[1]	R2-2102208, “Uplink Tx DC location reporting for two carrier uplink CA”, Apple Inc, 3GPP TSG RAN WG2 Meeting #113e, Electronic meeting, 25th Jan – 5th Feb, 2020

[2] RP-202893, “Moderator's summary of email discussion [90E][21][DC_location_reporting]”, Huawei, 3GPP TSG-RAN WG Meeting #90 Electronic, Online, 7 – 11 December 2020

[3]	R2-2102207, “Uplink Tx DC location reporting for two carrier uplink CA”, Apple Inc, 3GPP TSG RAN WG2 Meeting #113e, Electronic meeting, 25th Jan – 5th Feb, 2020

[4]	RP-201746, “On DC location reporting for intra-band UL CA”, Huawei, HiSilicon, 3GPP TSG-RAN meeting #89-e Sep 14th –  18th, 2020, Electronic meeting

[5]	R2-2102209, “Reply LS on DC location reporting for intra-band UL CA”, RAN2, 3GPP TSG RAN WG2 Meeting #113e, Electronic meeting, 25th Jan – 5th Feb, 2020

[6]	RP-210377, “DC location for > 2CC UL CA”, Qualcomm, RAN#91-e, Online, March 2021

[7]	RP-210914, “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, 3GPP TSG RAN meeting #91e, Electronic Meeting, March 16-26, 2021

[8]	R4-2105395, “WF on DC location parameters	“, Qualcomm, RAN4#98-Bis-e

[9]	R4-2107858, “WF on default DC location and offset signalling”, Nokia, RAN4#99e

[10]	R4-2114754, “WF on DC-location”, Oppo, 3GPP TSG-RAN WG4 Meeting#100e, E-meeting, 16th – 27th Aug, 2021

[11]	R4-2114948, “WF on PC2 intra-band UL NC CA and contiguous CA with 2Tx architecture”, Skyworks,  3GPP TSG-RAN WG4 Meeting # 100e, Electronic Meeting, Aug. 16-27, 2021

[12] R4-2119492, “CR to remove LO exceptions”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[13]	R4-2119964, “WF on DC-Location”, Vivo, RAN4#101e, Nov 2021

[14]	R4-2119965, “LS on DC location for larger than 2CC”, Qualcomm, RAN4#101e, Nov 2021

[15]	R4-2118883, “R17 FR2 DC reporting”, OPPO, RAN4#101-e, Nov 2021

[16]	R4-2203517 (R2-2201978), “Reply LS on DC location for >2CC”, RAN2, (Qualcomm), 3GPP TSG RAN WG2#116bis-e, e-Meeting, 17th - 25th January, 2022

[17]	R4-2202346, “WF on DC-Location”,  vivo, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, Jan. 17th – 25th, 2022

[18]	R4-2200333, “Further details and optimizations on DC location”, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, Jan. 17th – 25th, 2022

[19]	R4-2200944, “Discussion on DC location”, vivo, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, Jan. 17th – 25th, 2022

[20] 	R4-2201273, “R17 FR2 DC reporting”, Oppo, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, Jan. 17th – 25th, 2022

[21]	R4-2201959, “Further study on DC location reporting”, Huawei, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, Jan. 17th – 25th, 2022

[22]	R4-2205883, ‘Two DC location and RAN2 LS discussion’, 3GPP TSG-RAN4 Meeting #102-e, Feb 2022

[23]	RP-220968, “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”,   Nokia, 3GPP TSG RAN Meeting #95e, Electronic Meeting, Mar 17-23, 2022

[24]	RP-220469, “Motivation for 2CC and more and DL CA DC location”, Qualcomm, 3GPP TSG RAN Meeting #95e, Electronic Meeting, Mar 17-23, 2022

[25]	R4-2206602, “Reply LS on DC location for >2CC”, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 102-e, Electronic Meeting, 21 Feb - 3 Mar, 2022

[26]	R4-2206601, “WF on DC location”, vivo, 3GPP TSG-RAN WG4 Meeting # 102-e, Electronic Meeting, 21 Feb - 3 Mar, 2022

[27]	RP-220970, “Rel-17 Work Item Exception for Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, 3GPP TSG RAN meeting #95e, Electronic Meeting, Mar 17-23, 2022

[28]	R4-2210782, “LS on DC location for intra-band CA”, vivo, RAN4#103e, Online, May 2022