3GPP TSG-RAN WG4 Meeting #104-e	R4-2214040

Aug 2022

Title:	DMRS bundling pcmax issue and CA applicability

Source:	Qualcomm Incorporated

Agenda item:	9.16.1.1

Release:	Rel-17

Work Item:	NR_cov_enh

Responsible WG:	RAN1

Document for:	Approval

# 1.   Introduction

RAN4#103e approved a WF [35] on FR1 CA with the agreement:

Issue 1-2-2: Phase continuity requirements for FR1 inter-band CA with one UL CC

GTW Agreement:

- DMRS bundling requirements apply to FR1 DL CA with one uplink CC configured in Rel-17

Proposed further clarification: DMRS bundling requirements apply to FR1 DL CA with one uplink CC configured in Rel-17, meaning no switching to transmit SRS on another carrier.

In the next meeting, proponents are encouraged to bring the CR to implement the above agreement and clarification.

RAN4#103e also sent an LS to RAN1 with the following text:

It has been raised during RAN4 discussions that in TS 38.214 v17.1.0 section 6.1.7 it states that “The UE shall maintain power consistency and phase continuity within an actual TDW…”.

RAN4 would therefore appreciate feedback from RAN1 on whether the text in TS 38.214 section 6.1.7 prevents the UE from modifying its Tx power when necessary, i.e. prevents the UE from following TS 38.213 clause 7.1.1 and 7.2.1, or prevents the UE from adapting P-MPR, during transmission of a DMRS bundle

In this paper we discuss further the CA operation with DMRS bundling.

# 2. 	Discussion

## 2.1	The DL CA and implementation of the agreement in 1-2-2

Agreement states that DMRS bundling requirements applies for any CA case as long as there is only one uplink CC configured. Since section 6.4.2.5 requirements apply for in band, the need is just to clarify that in case of DL CA, the requirements apply for the one UL CC in case there are DL CCs configured and activated.  We provide CR [37] to execute this agreement.

## 2.2	UL CA with DMRS bundling

The question in the LS as referred in the introduction has an impact on the CA operation. If UE is required to maintain power for the duration of the bundle, and there is a transmission scheduled during the bundle, then UE may need to lower power during the bundle. UE therefore needs to have knowledge of the scheduling for both bands PUSCH preparation time before the transmission of the bundle. Our proposal in [38] needs one more option D),

Proposal 1: Define inter-band UL CA requirements for DMRS bundling.

Ran4 can further restrict or discuss cases as follows:

- Only one band can support DMRS bundling at a time

- In case two bands are supporting DMRS bundling, the bundles are not overlapping in time

- In case bundles are overlapping in time their time windows are aligned

- Only one band is scheduled for transmissions at the same time

Option D can be implemented for multiple features for inter-band UL CA with:

- Scheduling restriction so that only one band is scheduled for UL at a time

- TX switching SUL option

- SUL

The first option puts the burden of the scheduling to the network and UE behaviour is not defined when simultaneous transmissions occur. It also should be noted that there can not be simultaneous transmissions during actual TDW on either band but more about the actual TDW in the next section.

Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover interband UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands

## 2.3	Pcmax reference time

An other issue emerging from ran1 specification 38.214 is the Pcmax reference time. For single UL CC, the timing of the Pcmax is written as follows:

This means UE needs to set the Pcmax for each physical channel length, e.g. for 2 symbol PUCCH and 8 symbol PUSCH in a same slot needs to have different Pcmax_L but if the channel is split with a hop, the Pcmax_L is according to the lower one.

The physical channel is defined in clause 6 of 38.211 as PUSCH, PUCCH and PRACH and DMRS, PT-RS and SRS are designated as physical signals. SRS can be scheduled outside physical channel so in detail, where we do not need to go further in this context, RAN4 Pcmax should then use “physical channel or signal length”.

RAN1 38.213 power control clause 7.1 uses the “transmission occasion” for physical channels and signals as more generic term for individual transmissions:

Using “transmission occasion” in RAN4 may not be best choice since in RAN4, transmissions are considered from one physical transmitter that may transmit multiple channels and even multiple CCs at a time.

For DMRS bundling yet an other term for transmission length is introduced in RAN1. TS 38.214 clause 6.1.4 states that “The UE shall maintain power consistency and phase continuity within an actual TDW”. If UE now changes the Pcmax for each physical channel, it may end up changing the power since the Pcmax changes and the UE behaviour now depends if it is implemented according to TS 38.214 or TS 38.101-1/-2.

Observation: TS 38.214 and TS 38.101-1 are not aligned for the power control timing parts.

To be able to use the “actual TDW” in RAN4, we must have common understanding on it. Ran1 specification is not super clear about it but we must understand the “nominal TDW” first. In TS 38.214 it is stated that

The nominal TDW is either given by PUCCH/PUSCH-TimeDomainWindowLength or computed as MIN(maxDMRS-BundlingDuration, M) where M duration of the repetitions. Further the start of the nominal TDW is the first PUSCH/PUCCH slot and end is the last PUSCH/PUCCH computed from the parameters.

Within one nominal TDW there can be multiple actual TDW’s in case semi static or scheduled UL signal interrupts nominal TDW. Picture below shows this:

Figure 1. Nominal TDW vs actual TDW

This is more relevant in the field operation and in RAN4 requirement and RAN5 test conditions we can control the environment such way that nominal TDW and actual TDW are the same. In conclusion, for Pcmax reference time, RAN4 can specify “actual TDW”.

Proposal 2: Define Pcmax reference time as “Actual TDW” for DMRS bundling.

## 2.4 	CA capability aspects for DMRS bundling

With the discussion in section 2.2 and proposal 1, and allready agreed FR1 + FR2 CA, the capability aspects for DMRS bundling for CA should be revisited.

RAN4 earlier agreed the capability should be per band and not per UE. If the capability for example for maximum duration should be per band per band combination is under discussion.

UE may have a different frequency plan for a band depending if it is configured for CA or not. For example, the well know band 8 + 3 example, where 2nd harmonic of band 8 TX will land on RX of band 3. In this case, UE may want to employ different fundamental VCO frequency for the TX of band 8 when it receives also band 3. In this case, the other PLL may have different characteristics in terms of frequency stability leading to different capability in terms of maximum duration of phase continuity.

Also, the ability to transmit and receive simultaneously in certain band combinations may depend on DMRS bundling, or the other way around, in order to support simultaneous tx and rx, the support for DMRS bundling may need to be changed or dropped depending on if CA is configured or not.

Proposal 3: DMRS bundling capabilities are per band per band combination

# Conclusion

We made following observations:

Observation: TS 38.214 and TS 38.101-1 are not aligned for the power control timing parts.

Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover interband UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands

Proposal 2: Define Pcmax reference time as “Actual TDW” for DMRS bundling.

Proposal 3: DMRS bundling capabilities are per band per band combination

# References

[1]	R4-2100014, ”LS on PUCCH and PUSCH repetition”, RAN1, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[2]	R4-2103393, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[3]	R4-2103363, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, 3GPP TSG-RAN WG4 Meeting #98-e, 24 Jan – 5 Feb 2021

[4]	R4-2105417, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#98-Bis-e

[5]	R4-2105418, “Way forward on phase continuity and power consistency for PUCCH and PUSCH repetition” Huawei, RAN4#98-Bis-e

[6]	R4-2107611, “Reply LS on PUCCH and PUSCH repetition”, RAN1, RAN4#99e

[7]	R4-2107880, “Reply LS on PUCCH and PUSCH repetition”, RAN4 (Qualcomm), RAN4#99e

[8]	R4-2107881, “WF on phase continuity and power consistency for PUCCH and PUSCH repetition”, Huawei, RAN4#99e

[9]	R4-2111194, “Reply LS to RAN1 latest question on phase discontinuity”, Ericsson, RAN4#99e

[10]	R4-2111706, “LS on joint channel estimation for PUSCH and PUCCH”, RAN1, RAN4#100-e, Online, Aug 2021

[11] 	R4-2114991, “Reply LS on PUCCH and PUSCH transmissions”, Qualcomm, RAN4#100-e, Online, Aug 2021

[12]	R4-2114992, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, HiSilicon, RAN4#100-e, Online, Aug 2021

[13]	R4-2117006, “Reply LS on PUCCH and PUSCH repetition”. RAN1, RAN4#101e, Nov 2021

[14]	R4-2120002, ”Reply LS on PUCCH and PUSCH transmissions”, RAN4 (Qualcomm Incorporated), 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[15]	R4-2120003, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[16]	R4-2119494, “UE requirement for phase continuity”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #101-e, Nov 2021

[17] R4-2200339, “draft CR for EVM based requirements”, Qualcomm, RAN4#101Bis-e, Jan, 2022

[18]	R4-2119495, “Simulation results and criterias for setting requriements for UE”, Qualcomm Incorporated, RAN4#101e, Nov 2021

[19]	R4-2202418, “WF on phase continuity and power consistency for PUCCH and PUSCH transmissions”, Huawei, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[20]	R4-2202368, “Reply LS on Maximum duration for DMRS bundling” RAN4(Qualcomm), 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[21]	R4-2200020, “Updated RAN4 RF work plan for NR coverage enhancements WI”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 101-bis-e, Electronic Meeting, January 17-25, 2022

[22]	R4-2200338, “Discussion on UE requirement for JCE”, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #101Bis-e, Jan 2022

[23]	R4-2206537, “Reply LS on Maximum duration for DMRS bundling”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[24]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[25]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[26]	R4-2206593, “WF on issues for maintenance of NR coverage enhancements”, Ericsson. 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[27]	R4-2206580, “Reply LS on Length of Maximum duration”, China Telecom, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[28]	R4-2206541, “CR on measurment for DMRS bundling in TS 38.101-1”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[29]	R4-2206542, “CR on measurment for DMRS bundling in TS 38.101-2”, Ericsson, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[30]	R4-2206538, “CR on UE RF requirements for DMRS bundling in TS 38.101-1”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[31]	R4-2206539, “CR on UE RF requirements for DMRS bundling in TS 38.101-2”, Huawei, 3GPP TSG-RAN WG4 Meeting #102-e, Feb 2022

[32]	RP-220931, “Status report for WI NR coverage enhancements; rapporteur: China Telecom”, RAN1, RAN#95e, March 2022

[33]	R4-2207659, “CR 38.101-1 DMRS for CA”, Qualcomm, RAN4#103e, May 2022

[34]	R4-2207658, Discussion on additional requirements for DMRS bundling, Qualcomm, RAN4#103e, May 2022

[35]	R4-2210548, “WF on DMRS bundling for CA, SUL and FR2-2”, China Telecom, 3GPP TSG-RAN WG4 Meeting # 103-e, Electronic Meeting, May 09 - May 20, 2022

[36]	R4-2211225, “Reply LS to RAN1/RAN2 on DMRS bundling”, MediaTek, 3GPP TSG-RAN WG4 Meeting #103-e, Electronic Meeting, May 09 - May 20, 2022

[37]	R4-22xxx, “CR DMRS bundling for DL CA”, Qualcomm, RAN4#104e, Aug 2022

[38]	R4-2210215, “Handling of UL CA for DMRS bundling”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #103-e, Electronic Meeting, May 09 - May 20, 2022

# Annex: LS to Ran1 and Ran2 about the capabilities

3GPP TSG-RAN WG4 Meeting # 104-e 		R4-220xx

Online Meeting, Aug 2022

Title:	Reply LS on Maximum duration for DMRS bundling

Response to:	R1-2200773, (R4-2203504) “LS on DMRS bundling for PUSCH and PUCCH”, RAN1

Release:	Rel-17

Work Item:	NR_cov_enh

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact person:	Ville Vintola

vvintola@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

Ran4 has further discussed the CA with DMRS bundling and came to the conclusion that due to UE implementation issues, the DMRS bundling capability and maximum duration capability should per band and per band combination

# 2	Actions

To RAN1 and RAN2

ACTION: RAN4 respectfully asks RAN1 and RAN2 to take the above information in to consideration in their work.

# 3	Dates of next TSG RAN WG 4 meetings

3GPP RAN 4 RAN4#104-bis-e	2022-10-10	2022-10-19	Electronic meeting

3GPP RAN 4 RAN4#105		2022 November		Canada