3GPP TSG-RAN WG4 Meeting#104-e	R4-2214050

Online, 15th Aug – 26th Aug, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation results for CRS Interference Mitigation in NR

Agenda item:	9.10.1.3

Document for:	Discussion

- Introduction

In RAN4#103-e, all the simulation assumptions were finalized. In this paper, we provide simulation results for all agreed tests.

- Simulation Results

We provide the simulation results comparing the gains with 2 and 4 CRS ports below.

| Case Number | Duplex Mode and SCS | Scenario | PDSCH Start Symbol and Symbol Length | Overhead for TBS calculation | MIMO | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 10MHz, 15kHz SCS | Scenario 1 | S = 3, L = 9 | 12 | 2T2R Low | 10.34 | 12.34 |
| 2 |  |  |  |  | 2T4R Low | 6.22 | 8.22 |
| 3 |  | Scenario 2 | S = 2, L = 12 | 0 | 4T2R Low | 10.48 | 12.48 |
| 4 |  |  |  |  | 4T4R Low | 6.51 | 8.51 |
| 5 | TDD 20MHz, 15kHz SCS | Scenario 1 | S = 3, L = 9 | 18 | 4T2R Low | 9.78 | 11.78 |
| 6 |  |  |  |  | 4T4R Low | 6.3 | 8.3 |
| 7 |  | Scenario 2 | S = 2, L = 12 | 0 | 4T2R Low | 11.04 | 13.04 |
| 8 |  |  |  |  | 4T4R Low | 7.41 | 9.41 |
| 9 | TDD 20MHz, 30kHz SCS | Scenario 2 | S = 2, L = 12 | 0 | 4T2R Low | 10.14 | 12.14 |
| 10 |  |  |  |  | 4T4R Low | 6.76 | 8.76 |

- Conclusions

This paper provides simulation results for defining requirements for techniques to handle CRS interference in RAN4.