3GPP TSG RAN WG4 Meeting #104-e	R4-2214060

Electronic Meeting, 15-26 August 2022

Agenda Item:	9.11.6.1

Source: 	Ericsson

Title:	On SSB_RP/CSI-RS_RP conditions for RRM requirements with satellite access

Document for:	Discussion

- Introduction

The RRM core and performance requirements for satellite access (i.e. NTN) are applicable under certain side conditions in terms of minimum SSB_RP and minimum CSI-RS_RP and Es/Iot values.

In this paper we analyze and propose the applicable side conditions for RRM requirements in TS 38.133.

# Analysis of conditions for satellite access bands

In annex B of TS 38.133, the conditions for RRM requirement applicability in terms of minimum SSB_RP and minimum CSI-RS_RP for the two satellite access bands (n255 and n256) need to be specified for the following measurements:

- RRM measurements in RRC idle/inactive state e.g. measurements for servng cell evaluation and cell reselection to intra-frequency or inter-frequency cells.

- RRM measurements in RRC connected state e.g. intra-frequency and inter-frequency measurements for handovers.

- L1-RSRP measurements e.g. for link recovery procedures.

- RRC connection release with rediection e.g. for RRC connection mobility control.

As analyzed in another contribution in [1] that the UE REFSENS of the satellite access bands (SABs) n255 and n256 is the same as that of the terrestrial band n1.

Furthermore, the SSB Es/Iot and CSI-RS Es/Iot side conditions for corresponding measurement requirements are also the same for the UE served by the satellite access network and terrestrial network. Therefore, the conditions (minimum SSB_RP and minimum CSI-RS_RP) for satellite bands n255 and n256 are the same as defined for band n1 for corresponding measurements. These means the minimum SSB_RP and minimum CSI-RS_RP defined for band n1 can be reused for the SABs n255 and n256. The minimum SSB_RP and minimum CSI-RS_RP for the SABs will be specified in annex B of TS 38.133 but in a separate clause dedicated to the satellite access.

# Summary

The following are the observations and proposals related to the conditions for satellite access bands for applicability of RRM requirements in TS 38.133:

- Observation 1: Conditions are defined for bands in terms of minimum SSB_RP and minimum CSI-RS_RP at corresponding SSB Es/Iot and CSI-RS Es/Iot respectively for different SCSs.

- Observation 2: SSB Es/Iot and CSI-RS Es/Iot side conditions for the same type of measurement requirements are also the same for satellite access and terrestrial network.

- Proposal #1: The conditions (minimum SSB_RP and minimum CSI-RS_RP) for satellite bands n255 and n256 are the same as defined for band n1 for corresponding measurements.

- Proposal #2: The conditions for satellite bands n255 and n256 are defined in new sections in annex B of TS 38.133.

A Rel-17 draft CR to introduce the conditions for SABs in annex B of TS 38.133 is provided in [2].

- References

- R4-2214058, “On band grouping for RRM requirements with satellite access”, Ericsson

- R4-2214061, “Conditions for RRM requirements for satellite access band in TS 38.133”, Ericsson