3GPP TSG RAN WG4 Meeting #104-e	R4-2214065

Electronic Meeting, 15-26 August 2022

Agenda Item:	9.19.2.2.2

Source: 	Ericsson

Title:	On PRP and PRS Ês/Iot conditions for NR PRS-based measurements

Document for:	Discussion

- Introduction

The core and performance requirements for PRS measurements are applicable under certain side conditions in terms of minimum PRS_RP and PRS Es/Iot values for different SCSs applicable in FR1 and FR2.

In this paper we analyze and propose the applicable side conditions for requirements for PRS based measurements introduced in Rel-17 in TS 38.133.

# Analysis of conditions for PRS based measurements

In Rel-17 the following new type of PRS measurements or PRS based measurement requirements have been specified whose conditions need to be defined in the annex B of TS 38.133

- PRS-RSRPP measurements (without latenacy reduction) and

- Following PRS measurements when they are performed with reduced number of samples namely:

  - RSTD for reference and neighbour cell PRS resources,

  - PRS-RSRP,

  - PRS-RSRPP and

  - UE Rx-Tx time difference measurements

In annex B of TS 38.133, the conditions for the PRS requirement for the above PRS based measurements in terms of minimum PRS_RP need to be specified.

The existing conditions for the PRS requirement are defined in annex B.2.14 since Rel-16.

For PRS-RSRPP measurements (without latency reduction), the side conditions are the same as defined for PRS-RSRP. Therefore, the PRS-RSRPP can be made applicable to the conditions for which the PRS-RSRP applies in the existing FR1 and FR2 tables in Annex B.2.14.

PRS measurements when they are performed with reduced number of samples the side conditions were agreed in [1]. It was also agreed that the same conditions are applicable for the PRS measurements with reduced number of samples in RRC_INACTIVE and RRC_CONNECTED states [2]. In summary according to the agreement [1]:

- PRS Es/Iot for RSTD measurement on reference and neighbor cells’ PRS resources are ≥ -3 dB and ≥ -6 dB respectively.

- PRS Es/Iot for PRS-RSRP, PRS-RSRPP and UE Rx-Tx time difference measurements are ≥ -6 dB respectively

As general principle for PRS based measurements, the minimum PRS_RP depends on the frequency band and the FR; but is the same regardless of the PRS Es/Iot. Therefore, the existing minimum PRS_RP can be reused also for PRS measurement with reduced number of samples. However, the applicable PRS Es/Iot need to be specified in the annex B. In our view this can be done in the existing tables of annex B.2.14.

# Summary

The following are the observations and proposals related to the conditions which are applicability for requirements for PRS based measurements introduced in Rel-17 in TS 38.133:

- Observation 1: Conditions are defined for each applicable band in terms of minimum PRS_RP at corresponding PRS Es/Iot for different SCSs ion FR1 and FR2.

- Observation 2: PRS Es/Iot for PRS-RSRPP without latency reduction is the same as defined for PRS-RSRP since Rel-16.

- Observation 3: Side conditions for PRS measurements with reduced number of samples are:

  - PRS Es/Iot for RSTD measurement on reference and neighbor cells’ PRS resources are ≥ -3 dB and ≥ -6 dB respectively.

  - PRS Es/Iot for PRS-RSRP, PRS-RSRPP and UE Rx-Tx time difference measurements are ≥ -6 dB respectively

- Observation 3: In general, the minimum PRS_RP for PRS measurement depends on the frequency band and FR but it is the same regardless of the PRS Es/Iot.

- Proposal #1: The existing conditions defined for PRS-RSRP in annex B.2.14 are made applicable for PRS-RSRPP measurement (without latency reduction).

- Proposal #2: The applicable PRS Es/Iot values for PRS measurements performed with reduced number of samples are introduced in the existing tables of Annex B.2.14.

A Rel-17 draft CR to introduce the conditions for PRS measurements in annex B.2.14 of TS 38.133 is provided in [3].

- References

- R4-2210600, “WF on NR Positioning Enhancements (Part 1)”, Ericsson.

- R4-2210602, “WF on NR Positioning Enhancements (Part 2)”, CATT.

- R4-2214066, “Set 1-2: PRP and PRS Ês/Iot conditions for NR PRS-based measurements”, Ericsson.