3GPP TSG-RAN WG4 Meeting #104-e 												         R4-2214090

Electronic Meeting, August 15 - 26, 2022

Agenda item:			9.16.1, 9.16.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [104-e][112] NR_cov_enh_maintenance

Document for:	Information

Introduction

This email thread discusses the UE RF aspects for Rel-17 NR coverage enhancements WI, including the following topics:

- Topic #1: Phase continuity requirement for DMRS bundling

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Invite comments on the recommended WF under each issue in section 1.2.

  - Invite comments on the CRs in section 1.3.2.

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| China Telecom | Shan YANG | yangshan@chinatelecom.cn |
| Apple | Anatoliy Ioffe | aioffe@apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Phase continuity requirement for DMRS bundling

1.1 Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211622 | China Telecom | Updated summary of RF agreements for NR coverage enhancements WI |
| R4-2211623 | China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR1 |
| R4-2211624 | China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR2 |
| R4-2213247 | Ericsson | Title: On DMRS bundling for CA_DC support - RAN4 specs should be sufficient to support DMRS bundling requirements for FR1+FR2 CA, FR1+FR2 DC, and DC with FR2 NR applied per CC when two carriers are configured; no RAN1 spec changes are needed. - For issue 1-2-1, there could be a carrier switching delay / OFF period on the NR carrier configured for DMRS bundling for some band combinations with 3 carriers during which UE phase coherence may not be guaranteed. - It is not expected for a UE that switches a carrier to comply with phase coherence within the switching period if the switch-to carrier is the NR carrier configured with DMRS bundling. |
| R4-2213248 (Not available) | Ericsson | CR on DMRS bundling support for CA in FR1 |
| R4-2213249 | Ericsson | CR on DMRS bundling support for CA in FR1+FR2 |
| R4-2213375 | Ericsson | CR on DMRS bundling support for CA in FR1 |
| R4-2213738 | Huawei, HiSilicon, China Telecom, CMCC | CR for TS 38.101-1: clarification on DMRS bundling RF requirements for SUL |
| R4-2214040 | Qualcomm Incorporated | Title: DMRS bundling pcmax issue and CA applicability Observation: TS 38.214 and TS 38.101-1 are not aligned for the power control timing parts. Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover interband UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands Proposal 2: Define Pcmax reference time as “Actual TDW” for DMRS bundling. Proposal 3: DMRS bundling capabilities are per band per band combination |
| R4-2214041 | Qualcomm Incorporated, China Telecom | CR 38.101-1 DMRS DL CA |

1.2 Open issues summary

1.2.1 FR1 inter-band CA and SUL with DMRS bundling

1.2.1.1	Issue 1-1: FR1 inter-band CA and SUL with DMRS bundling

- Background: RAN4 LS to RAN1 in R4-2211225

RAN4 discussed whether applying DMRS bundle to FR1 inter-band UL CA would have any RAN1 spec impacts, and would appreciate RAN1 feedback before making further decision:

Considering DL CA with “additional” UL carrier configured with SRS only (i.e. no PUCCH/PUSCH configured) with the following conditions: - For carrier switching back and forth between UL carrier and SRS carrier, if the switching happens within the DMRS bundling duration, then the phase continuity is not maintained by the UE. Considering FR1 inter-band UL CA with DMRS bundling with following conditions: - UE shall only have ongoing transmissions on a single uplink carrier at the same time. If overlapping transmissions of PUSCH, PUCCH, and/or SRS are erroneously scheduled/configured by the gNB on more than one carrier, then the phase continuity of DMRS bundling will be broken. - Only configuration of a single TAG is supported. - If there is any carrier switching back and forth between two carriers and the switching happens within the DMRS bundling duration, then the phase continuity is not maintained by the UE. - Can only one band can be configured with DMRS bundling at a time?

RAN4 also discussed whether applying DMRS bundle to SUL would have any RAN1 spec impacts, and would appreciate RAN1 feedback before making further decision:

Considering SUL with DMRS bundling with following conditions: - Can only one band can be configured with DMRS bundling at a time? - If there is any carrier switching back and forth between SUL and NUL carriers and the switching happens within the bundling duration, then the phase continuity is not maintained by the UE.

- Proposals

  - Proposal 1: Ran4 to define DMRS bundling requirement applicability to cover FR1 inter-band UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands (Qualcomm)

  - Proposal 2: Ran4 to define DMRS bundling requirement applicability to cover SUL band (Huawei)

- Moderator’s Recommendation

  - From RAN4 perspective, it is feasible to define DMRS bundling requirement applicability to cover the following 3 scenarios:

    - FR1 inter-band UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands

    - SUL band

    - DL CA with “additional” UL carrier configured with SRS only

  - RAN4 CRs on DMRS bundling requirements applicability for the above 3 scenarios can be agreed ONLY if:

    - it is also confirmed as feasible from RAN1 perspective, and,

    - based on the conditions stated in the RAN4 LS to RAN1 in R4-2211225 as well as other conditions (if any) introduced in RAN1

Discussion in GTW:

Apple: our view is quite clear that we do not consider UL CA and SUL. They are not the scope of physical layer design. It is premature. We do see the benefit for UL-CA scenario. This work is definitely worthy to see in Rel-18. Strongly urge proponents to propose them for Rel-18.

Ericsson: it seems like inter-band UL CA has already been agreed in the latest spec. There are something that need be clarified here. For the detailed analysis, we need wait for RAN1.

ZTE: for inter-band UL CA, it should be supported. The consistence can be maintained in one carrier.

Huawei: in last meeting, we agreed that we should wait for RAN1 decision.

China Telecom: firstly, we support the scenarios here. The use of SUL is to extend the coverage. SUL+DMRS bundling would be beneficial. We would like to clarify RAN1 meeting starts next week. We are OK to postpone the discussion. Regarding Ericsson discussion on the UL-CA, actually the CR for UL-CA has been agreed in the merged final.

Mediatek: we should wait for RAN1. Last meeting we agree the DL CA with one uplink configured. We should wait.

Qualcomm: to Apple, how feasible to propose them for Rel-18. Can Apple comment? Is there any activity in RAN1?

Apple: In Rel-17 RAN1 will discuss the LS we sent. From our side, we think the physical layer design cannot be adapted to UL-CA.

- Recommended WF

  - Consensus was not reached in GTW. For any additional comments & response, please add in the table below:

| Company | Comments |
| --- | --- |
| Qualcomm | Technically, we, ran1 or ran4, have no problems as long the two bands are not scheduled to transmit at the same time. Outside technical reasons, hard to counter the comments. |
| China Telecom | We agree that it is technically feasible for the 3 scenarios in RAN4 and also in RAN1 (based on the tdocs submitted to RAN1). |
| Apple | None of these scenarios are feasible in Rel-17, in our understanding. The physical layer does not support these modes, and we will hear RAN1 response after they conclude the discussions. We should also be cognizant of UE design cycles: with Rel-17 core part concluded, design decisions are being made based on what has been concluded; the introduction of these new features so late in the TEI phase can lead to unstable UE implementations. We suggest to focus the proponents’ energies on preparing these scenarios as a proposal for Rel-18 work scope. At least from our perspective, we would be supportive of developing this feature further in Rel-18. |
| MediaTek | Given the fact that this discussion on including UL CA was only triggered in RAN4 after the WI was declared 100% complete by the rapporteur, after previously telling RAN1 (just before the WI was declared 100% complete) that we had not discussed UL CA support in the Rel-17 timeframe, then we believe that it would make sense to wait for RAN1/2 response in order to avoid more confusion about RAN4 status in other WGs. Regarding technical feasibility we suggest to wait until we see RAN1/2 status. |
| Ericsson | We agree with Moderator recommendation. This means that RAN4 should wait RAN1 reply on the previous RAN4 LS and it will be good to have response from RAN1 early next week (RAN1 has f2f meeting). |

1.2.1.2	Issue 1-1A: RAN4 specfication for FR1 CA DMRS bundling

- Background:

The CR in R4-2207659 (content shown below) was marked as agreed in the RAN4 #103e final EOM report and already implemented in v17.6.0 of TS 38.101-1.

#### 6.4A.2.3 Transmit modulation quality for inter-band CA For inter-band carrier aggregation with one uplink carrier assigned to one NR band, the transmit modulation quality requirements in subclause 6.4.2 apply including phase continuity requirements for DMRS bundling [IE name]. For inter-band carrier aggregation with two contiguous carriers assigned to one NR band, the transmit modulation quality requirements in subclause 6.4A.2.1 apply for those carriers. For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmit modulation quality requirements in subclause 6.4A.2.2 apply for those carriers. For inter-band carrier aggregation with uplink assigned to two NR bands, the transmit modulation quality requirements shall apply on each component carrier as defined in clause 6.4.2 with all component carriers active: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured. For DMRS bundling [ IE name], requirements for phase continuity in clause 6.4.2.5 apply for PCC when SCC has no UL allocation for the duration of the bundle on PCC. For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per operating band), the transmit modulation quality requirements specified in subclause 6.4.2 apply for the NR band supporting one component carrier, and for the NR band supporting two contiguous component carriers the requirements specified in subclause 6.4A.2.1 apply.

- Recommended WF

  - Based on Chairman guidance, moderator would like to ask whether it is ok to agree a new CR to remove the changes in 7659 in the first week?

| Company | Comments |
| --- | --- |
| China Telecom | We have uploaded the CR draft in the inbox, and the formal version will be submitted before 17:00 UTC Thursday (round 1 commenting deadline). https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_104-e/Inbox/Drafts/%5B104-e%5D%5B112%5D%20NR_cov_enh_maintenance/Revised%20CRs/draft%20R4-22xxxxx_FR1%20CA%20for%20DMRS%20bundling%20(correction).docx The v1 version of the CR has been uploaded, with editorial correction on the cover sheet. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_104-e/Inbox/Drafts/%5B104-e%5D%5B112%5D%20NR_cov_enh_maintenance/Revised%20CRs/draft%20R4-22xxxxx_FR1%20CA%20for%20DMRS%20bundling%20(correction)_v1.docx |
| Apple | The draft CR looks fine |

1.2.2 Pcmax reference time

1.2.2.1 Issue 1-2: Pcmax reference time

- Background:

  - In TS 38.214 v17.1.0 section 6.1.7 it states that “The UE shall maintain power consistency and phase continuity within an actual TDW…”.

- Proposals

  - Option 1: Define Pcmax reference time as “Actual TDW” for DMRS bundling. (QC)

    - QC: TS 38.214 and TS 38.101-1 are not aligned for the power control timing parts.

Discussion in GTW:

Huawei: we would like to better understand the intention for the change. It is clearly defined in RAN1 and RAN4 during actual TDW as long as the feature is activated UE needs to maintain the phase continuity. No need to clarify.

Apple: 38.213 has already defined what transmission occasion is.

Mediatek: last meeting we asked the question about the ambiguity. We should wait for RAN1 response and discussion.

Qualcomm: To Huawei, we do not agree on the actual spec. Physical channel length last for a number of OFDM symbols. If 38.214 is not aligned with TS38.101-1, 214 says that UE needs to maintain in the whole bundling. To Apple, why is there LS to ask the alignment if transmission occasion is well defined? Maybe the 38.101-1 is misaligned with UE behaviour. To MTK, we do not think RAN1 will discuss. We can wait for the next meeting.

Ericsson: our view is that in the test we only test UE capability which is the maximum time for UE to keep to consistent. That is not specified from power perspective. We do not see the need to update the Pcmax. If the actual TDW really needs be updated, we need further discuss how we should reflect this.

Qualcomm: It just means removing the whole Pcmax section.

- Recommended WF

  - Consensus was not reached in GTW. For any additional comments & response, please add in the table below:

| Company | Comments |
| --- | --- |
| Qualcomm: | So this table is wrong in our view: And this allows UE to e.g. check P-MPR for every physical channel and for example drop a transmission mid bundle. Of course, if we ignore pcmax, then all is good. And to Apple, transmission occation in 38.213 is: is defined by a slot index 𝑛𝑠,𝑓 𝜇 within a frame with system frame number 𝑆𝐹𝑁, a first symbol 𝑆 within the slot, and a number of consecutive symbols L. L little hard to find but it is withing the symbols in one slot. |
| Nokia | We don’t see the necessity to clarify here. The condition is defined using phase, no need to add power reference here. |
| Apple | When RAN4 specified P-MPR, coexistence with technologies outside of 3GPP and compliance with regulation are cited as the reasons. In both cases the timelines for the UE tracking its output power with respect to these P-MPR criteria can be different than 3GPP symbols. In our understanding, it is up to UE implementation to apply P-MPR when necessary and to also update Pcmax as specified in the 3GPP specification. When RAN4 defined the requirements on DMRS bundling, the side conditions captured the constraint that there is no change in UE Tx level. However, in our understanding, this does not imply that the UE, when configured for DMRS bundling, is pre-determined to not apply P-MPR when the relevant conditions call for it. This is why we believe that the recent RAN1 changes to their specification were made without consideration for this aspect. Thus, we prefer to allow RAN1 to conclude their discussion of our LS from last meeting before we made any further agreements in RAN4. |
| MediaTek | We would prefer to wait for feedback from RAN1 on whether the 38.213 or 38.214 text is actually taking precedence, as in our understanding the UE is not required to maintain power consistency (as agreed in RAN4 in the past where ensured that the test case did not verify this by not changing DL power level). |

1.2.3 CA capability for DMRS bundling

1.2.3.1 Issue 1-3: CA capability for DMRS bundling

- Background: Updated RAN1 UE feature list in LS R4-2211513/R1-2205609

  - Per band granularity was agreed for FG30-4

  - The granularities for FG30-4a/b/…./h are still in []

| Features | Index | Feature group | Components | Type |
| --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | Per band |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [Per UE] |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [Per UE] |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [Per UE] |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [Per UE] |

- Proposals

  - Option 1: DMRS bundling capabilities are per band per band combination. (QC)

Discussion in GTW:

Huawei: granularity should be decided by RAN1. From 30-4x, they are RAN1 capability.

ZTE: tend to agree with Huawei. Some capability comes from RAN1. If we have the agreement in RAN4, we can send them to RAN1. We see the intention to leave more flexibility for UE to implement.

Qualcomm: we recognize the capability is initiated by RAN1. If RAN4 identifies some difficulty to implement, RAN4 can give the feedback. There is implementation and challenge.

Apple: question to Qualcomm to clarify the motivation. Is the intention to enable DL CA or UL&DL CA?

China Telecom: to Qualcomm, is the proposal for 30-4 or 30-4a/b/c? We cannot reach any agreement for 30-4a/b/c. When RAN1 starts their work, they can take our input. To Apple, this proposal can be applied to DL CA or DL&UL CA.

Mediatek: we need be careful not to confuse RAN1. We should not send LS based on the LS sent in the last meeting. We should wait for RAN1 decision.

Qualcomm: LS does not say RAN4 has concluded. It is open item. To CTC, our view, 30-4 and 30-4a/b/d should be per band per band combination. To Apple it is for future release.

Samsung: we agree with Huawei and Mediatek.

- Recommended WF

  - Consensus was not reached in GTW. For any additional comments & response, please add in the table below:

| Company | Comments |
| --- | --- |
| Apple | We should wait for RAN1 to reach a decision on this topic |
| MediaTek | We would also prefer to wait for RAN1/3 feedback. |

1.2.4 Clarification on the phase coherence exemption when switching period happened

1.2.4.1 Issue 1-4: Clarification on the phase coherence exemption when switching period happened

- Proposals

  - Option 1: It is not expected for a UE that switches a carrier to comply with phase coherence within the switching period if the switch-to carrier is the NR carrier configured with DMRS bundling. (E///)

    - E///: There could be a carrier switching delay / OFF period on the NR carrier configured for DMRS bundling for some band combinations with 3 carriers during which UE phase coherence may not be guaranteed.

- Recommended WF

  - Encourage comments

| Company | Comments |
| --- | --- |
| Qualcomm | Issue seems to be releated to the UL CA. Agree with the observation. Also phase continuity is lost on the other carrier in case of switching period is configured to the other band. |
| China Telecom | We have two questions: 1) Is the target scenario UL CA with 3 uplink carriers/bands? 2) For the carrier not located with Tx switching period, the phase continuity can also not be maintained by switching back and forth. |
| Apple | It is not clear to us how carrier switching is applicable to the single-carrier requirement we have defined. Does the physical layer support DMRS bundling + carrier switching? Do we need to check with RAN1? |
| MediaTek | We have only agreed that DMRS bundling is “configured” on a single UL carrier at a time in the case of FR1+FR2 CA. Therefore there would be no UL switching based on current RAN4 agreements. |
| Ericsson | Reply TO CT: - Is the target scenario UL CA with 3 uplink carriers/bands? For inter-band NR CA between FR1 and FR2, Inter-band EN-DC including FR2 and Inter-band NE-DC including FR2, e.g CA_n1A-n3A-n257A, the UL configuration is among CA_n1A-n3A, CA_n1A-n257A and CA_n3A-n257A; for EN-DC, uplink EN-DC could be DC_1A_n3A-n257A2. In LS las meeting, RAN4 agrees “FR1+FR2 UL CA, FR1+FR2 DC, and EN-DC with NR on FR2. DMRS bundling configuration is limited to one uplink NR carrier in total on all FRs at a time.” So we discuss the cases where there may be 2 or 3 uplink carrier configuration but still meeting the RAN4 agreed limitation that DMRS bunding only limited to one uplink NR carrier on all FRs at a time. - For the carrier not located with Tx switching period, the phase continuity can also not be maintained by switching back and forth. We are discussing at the start/end of the DMRS bundling time window where there is switching period. If the swiching happens in the middle of the DMRS bundling, it is not discussed and we think we need wait RAN1 reply. Switch period. Reply To Apple: The switching period is defined in RAN4 specification and thus we think it is RAN4 discussion on whether such period should or should not apply to DMRS bundling, based on last meeting RAN4 agreements. Reply To MTK: this is not our understanding, only one uplink carrier at a time is the limitation, for example, network can still configure two uplink carrier but only schedule one uplink carrier at a time. |

1.3 Companies views’ collection for 1st round

1.3.1 Open issues

Provided under each issue in section 1.2

1.3.2 CR comments collection

| tdoc number | Title/Content | Comment collection |
| --- | --- | --- |
| R4-2211622, China Telecom | Updated summary of RF agreements for NR coverage enhancements WI | Moderator’s note: The summary is for information purpose. |
| R4-2211623, China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR1 | Summary of change: 1) Added the UL waveform for phase continuity test. 2) Added the TDD pattern for 15kHz SCS. 3) Updated the capability name according to the latest RAN2 spec. |
|  |  | Rohde & Schwarz: There are more updates required to the new RMC in the Annex. Current TDD RMCs reuse the pattern from Refsense for which the pattern is DDDSU for 15kHz. According to the CR coversheet the RMC shall use 7D1S2U, so a new RMC pattern definition is required. Also new table A.2.1-2 specifies to use slots 8,9,18,19 for 15 kHz UL, but for 15kHz there are only 10 slots in a frame. |
| R4-2211624, China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR2 | Summary of change: 1) Added the UL waveform for phase continuity test. 2) Updated the capability name according to the latest RAN2 spec. |
| R4-2213248, Ericsson (Not available) | CR on DMRS bundling support for CA in FR1 |  |
| R4-2213249, Ericsson | CR on DMRS bundling support for CA in FR1 + FR2 (38.101-3 CR) | Summary of change: Inroduce clearification on the phase coherance exemption when switching period happened within the swtiching-to uplink carrier where DRMS bundling is configured. |
|  |  | Apple: Issue 1-4 should be resolved first |
| R4-2213375, Ericsson | CR on DMRS bundling support for CA in FR1 (38.101-1 CR) | Summary of change: Inroduce clearification on the phase coherance exemption when switching period happened within the uplink carrier where DRMS bundling is configured. |
|  |  | Apple: Issue 1-4 should be resolved first |
|  |  | MediaTek: See comments to issue 1-4. |
| R4-2213738, Huawei, HiSilicon, China Telecom, CMCC | CR for TS 38.101-1: clarification on DMRS bundling RF requirements for SUL | Summary of change: Clarify the application of the phase continuity requirements for DMRS bundling for PUSCH and PUCCH to SUL band. |
|  |  | China Telecom: we support the CR |
|  |  | Apple: Issue 1-1 should be resolved first |
| R4-2214041, Qualcomm, China Telecom | CR 38.101-1 DMRS DL CA | Summary of change: Clarify the DMRS bundling requirements for DL CA based on the RAN4 agreement in RAN4 #103e. |
|  |  | China Telecom: we support the CR |
|  |  | Apple: we agree that this CR correctly implements the agreement from RAN4 #103 |
|  |  | See proposed updated text in Drafts folder, Draft Revision_R4-2214041_MTK. Original text is slightly contradictory and may give the impression that it is when a single UL carrier is configured for DMRS bundling. So would not like to go around in circles on that again. |

1.4 Summary for 1st round

1.4.1 Open issues

Issue 1-1: FR1 inter-band CA and SUL with DMRS bundling

Summary of round 1 discussion:

- Moderator’s Recommendation in round 1:

  - From RAN4 perspective, it is feasible to define DMRS bundling requirement applicability to cover the following 3 scenarios:

    - FR1 inter-band UL CA with the restriction that UE is not schedule to transmit simultaneously on two bands

    - SUL band

    - DL CA with “additional” UL carrier configured with SRS only

  - RAN4 CRs on DMRS bundling requirements applicability for the above 3 scenarios can be agreed ONLY if:

    - it is also confirmed as feasible from RAN1 perspective, and,

    - based on the conditions stated in the RAN4 LS to RAN1 in R4-2211225 as well as other conditions (if any) introduced in RAN1

- Feedback on the technical feasibility from RAN4 perspective:

  - Feasible (QC, CTC, E///)

  - Not feasible (Apple)

    - Apple: We should also be cognizant of UE design cycles:  with Rel-17 core part concluded, design decisions are being made based on what has been concluded

  - Wait for RAN1/2 response (MTK, E///)

Recommendation for round 2:

  - Encourage companies to focus on the technical feasibility from RAN4 perspective

Issue 1-1A: RAN4 specification for FR1 CA DMRS bundling

Tentative agreement:

  - The CR on Correction of DMRS bundling requirements for FR1 CA in R4-2214210 is agreeable.

Issue 1-2: Pcmax reference time

Summary of round 1 discussion:

  - Option 1: Define Pcmax reference time as “Actual TDW” for DMRS bundling. (QC)

    - Nokia: don’t see the necessity to clarify here.

    - Apple: It is up to UE implementation to apply P-MPR when necessary and to also update Pcmax as specified in the 3GPP specification. When RAN4 specified P-MPR, coexistence with technologies outside of 3GPP and compliance with regulation are cited as the reasons.

    - MTK: Wait for RAN1 feedback, and in our understanding the UE is not required to maintain power consistency.

Recommendation for round 2:

  - In moderator’s understanding, the Pcmax is determined in RAN4 spec, and can be discussed in RAN4. Meanwhile, there is technical comment from Apple on the proposed option 1.

  - Further discussion on the RAN4 technical aspect is encouraged.

Issue 1-3: CA capability for DMRS bundling

Summary of round 1 discussion:

  - Option 1: DMRS bundling capabilities (FG30-4 and 30-4a/b/d) are per band per band combination. (QC)

  - Option 2: Wait for RAN1 to reach a decision

Recommendation for round 2:

  - In moderator’s understanding, the granularity for FG30-4 has been agreed in RAN1. FG 30-4a/b/d (for which the granularities are not concluded yet) are related to the baseband functionality discussed in RAN1. It would be better to have the discussion in RAN1.

Issue 1-4: Clarification on the phase coherence exemption when switching period happened

Summary of round 1 discussion:

  - Option 1: It is not expected for a UE that switches a carrier to comply with phase coherence within the switching period if the switch-to carrier is the NR carrier configured with DMRS bundling. (E///, QC)

  - CTC, Apple, MTK raised questions on the target scenario for option 1.

Recommendation for round 2:

  - Further discuss/clarify based on the CRs.

1.4.2 CRs

Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised”

| tdoc number | Title/Content | CRs/TPs Status update recommendation |
| --- | --- | --- |
| R4-2211622, China Telecom | Updated summary of RF agreements for NR coverage enhancements WI | No comment was raised. To be Noted. |
| R4-2211623, China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR1 | To be revised, based on R&S comments. |
| R4-2211624, China Telecom | CR: Maintenance of phase continuity requirements for DMRS bundling in FR2 | Agreeable |
| R4-2213249, Ericsson | CR on DMRS bundling support for CA in FR1 + FR2 (38.101-3 CR) | To be revised (A revision tdoc number is requested in case any update is needed) |
| R4-2213375, Ericsson | CR on DMRS bundling support for CA in FR1 (38.101-1 CR) | To be revised (A revision tdoc number is requested in case any update is needed) |
| R4-2213738, Huawei, HiSilicon, China Telecom, CMCC | CR for TS 38.101-1: clarification on DMRS bundling RF requirements for SUL | Return to |
| R4-2214041, Qualcomm, China Telecom | CR 38.101-1 DMRS DL CA | To be revised (A revision tdoc number is requested in case any update is needed) |

1.5 Discussion on 2nd round

# 2. Recommendations for Tdocs

## 2.1 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on DMRS bundling | China Telecom | Used for round 2 discussion on Issue 1-1 and 1-2. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2211622 |  | Updated summary of RF agreements for NR coverage enhancements WI | China Telecom | Noted |  |
| R4-2211623 |  | CR: Maintenance of phase continuity requirements for DMRS bundling in FR1 | China Telecom | Revised | To capture R&S comments |
| R4-2211624 |  | CR: Maintenance of phase continuity requirements for DMRS bundling in FR2 | China Telecom | Agreeable |  |
| R4-2213247 |  | On DMRS bundling for CA_DC support | Ericsson | Noted |  |
| R4-2213248 |  | CR on DMRS bundling support for CA in FR1 | Ericsson | Withdrawn |  |
| R4-2213249 |  | CR on DMRS bundling support for CA in FR1_FR2 | Ericsson | Revised | A revision tdoc number is requested in case any update is needed |
| R4-2213375 |  | CR on DMRS bundling support for CA in FR1 | Ericsson | Revised | A revision tdoc number is requested in case any update is needed |
| R4-2213738 |  | CR for TS 38.101-1: clarification on DMRS bundling RF requirements for SUL | Huawei, HiSilicon, China Telecom, CMCC | Return to |  |
| R4-2214040 |  | DMRS bundling pcmax issue and CA applicability | Qualcomm Incorporated | Noted |  |
| R4-2214041 |  | CR 38.101-1 DMRS DL CA | Qualcomm Incorporated, China Telecom | Revised | A revision tdoc number is requested in case any update is needed |
| R4-2214210 |  | CR: Correction of DMRS bundling requirements for FR1 CA | China Telecom | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2.1 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents