3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214112

Electronic Meeting, Aug. 2022

Agenda item:			11.8.1, 11.8.2

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [104-e][134] FR2_multiRx_UERF

Document for:	Information

# Introduction

This document captures the first round of discussion for this meeting on the how to set requirements for NR FR2 multi-Rx chain DL reception.

Contact Information

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | Sumant Iyer | sumanti@qti.qualcomm.com |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| vivo | Sanjun Feng | fengsanjun@vivo.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |
| Sony | Kun Zhao | kun.1.zhao@sony.com |
| LG Electronics | Yoonoh Yang | yoonoh.yang@lge.com |
| Huawei | Xiang Gao | gaoxiang74@huawei.com |
| Xiaomi | Juan Zhang | zhangjuan8@xiaomi.com |
| Samsung | Bozhi Li | bozhi.li@samsung.com |

# Topic #1: 11.8.1 Work Plan

## Companies’ contributions summary

| T-doc number | T-doc name | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2211814 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 must consider in this work item how to specify the DL spherical coverage requirement of the FR2-1 multi-Rx chain UE to ensure satisfactory real-life performance of the FR2-1 multi-Rx chain UE for two AoAs simultaneous reception in terms of both spherical coverage percentage and sensitivity. |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated, vivo | (Moderator summary: Work plan proposed, can it be agreed) |
| R4-2213053 | Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception | vivo, Qualcomm | (Moderator summary: Work plan proposed, can it be agreed) |

## Open issues summary

### Work plan for UE RF Proposal (R4-2212333)

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Discussion on system assumptions, UE assumptions | Agree on system assumptions |
| RAN4#104-Bis | Oct. 2022 | UE hardware assumptions | Agree on UE assumptions |
| RAN4#105 | Nov. 2022 | Requirement concept |  |
| RAN4#106 | Feb. 2023 | Requirement concept and Simulation discussion | Agree on details pertaining to simulation effort |
| RAN4#106-Bis | Apr. 2023 | Simulation results discussion | Draft CR? |
| RAN4#107 | May 2023 | Requirement Parameter details | Draft CR |
| RAN4#108 | Aug. 2023 | Requirement Parameter details | Feature CR |

Suggested WF on 1.2.1: Agree with proposal, or provide refinements below:

| Company | Comments |
| --- | --- |
| Nokia | OK with suggested WF. |
| Apple | In our understanding, requirement concept means what constitutes an appropriate requirement in RAN4 and what the requirement should look like. We believe requirement concept is important and may take quite some time to converge. Therefore, we propose to start earlier with the discussion and add requirement concept to RAN4#104-Bis in parallel with UE hardware assumption. |
| Sony | Fine with the proposal. |

### Work plan for UE RRM Core part (R4-2213053)

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#104 | Aug. 2022 | Study and initial discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of at least some of the requirements Approve the RRM work plan |
| RAN4#104-Bis | Oct. 2022 | Continue discussion on the feasibility/necessity of specifying RRM core requirements for the following procedures/measurements, and corresponding core requirements - L1-RSRP measurement delay - L3 measurement delay (both cell detection delay and measurement period can be considered) - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) | Conclusion on the study/feasibility/necessity of all the requirements Agreements on assumptions of receive timing difference between different directions Initial agreements on the confirmed RRM requirements |
| RAN4#105 | Nov. 2022 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106 | Feb. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM core requirements for the following procedures/measurements, if feasibility/necessity is agreed - L1-RSRP measurement delay - L3 measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI | Further agreements on the confirmed RRM requirements Work split on the Draft CRs |
| RAN4#107 | May 2023 | Continue discussion on the remaining issues for the RRM core requirements | Initial discussion on the Draft CRs Further agreements on the confirmed RRM requirements |
| RAN4#108 | Aug. 2023 | Continue discussion on the remaining issues for the RRM core requirements | Conclude all the RRM requirements Endorse all the Draft CRs for core part of the WI |

Suggested WF on 1.2.2: Agree with proposal, or provide refinements below:

| Company | Comments |
| --- | --- |
| Nokia | OK with suggested WF. |
| LG Electronics | It’s better to be discussed in [233] thread. |

### Work plan for UE RRM performance part (R4-2213053)

| RAN 4 meeting | Date | Discussion | Work progress goal |
| --- | --- | --- | --- |
| RAN4#106 | Feb. 2023 | Initial discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Initial discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused |  |
| RAN4#106-Bis | Apr. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused Initial discussion on list of test cases |  |
| RAN4#107 | May 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Initial agreement on list of test cases |
| RAN4#108 | Aug. 2023 | Continue discussion on the RRM Performance requirements and test cases corresponding to the new core requirements Continue discussion on RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused | Work split on the Conclude the list of test cases Work split on the Draft CRs for performance requirements and test cases |
| RAN4#108-Bis | Oct. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Initial discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#109 | Nov. 2023 | Continue discussion on the remaining issues for performance requirements and test cases Continue discussion on the Draft CRs for performance requirements and test cases |  |
| RAN4#110 | Feb. 2024 | Continue discussion on the remaining issues for performance requirements and test cases | Conclude all the performance requirements and test cases Endorse all the Draft CRs for the performance part of the WI |

Suggested WF on 1.2.3: Agree with proposal, or provide refinements below:

| Company | Comments |
| --- | --- |
| Nokia | OK with suggested WF. |
| LG Electronics | It’s better to be discussed in [233] thread. |

## Companies views’ collection for 1st round

### Open issues

See previous subsection 1.2

### CRs/TPs comments collection

N/A

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Description | Status summary |
| --- | --- | --- |
| 1.2.1 | UE RF workplan | Tentative agreements: none after 1st round Candidate options: none Recommendations for 2nd round: Continue discussion in WF |
| 1.2.2 | UE RRM Core workplan | Discussion moved to [233] |
| 1.2.3 | UE RRM performance workplan | Discussion moved to [233] |

### CRs/TPs

N/A

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #2: 11.8.2 	UE RF

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | T-doc name | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2211815 | Enhanced spherical coverage requirements for multi-rx chains UEs | Nokia, Nokia Shanghai Bell | Observation 1: With broad UE beams, the spherical coverage of multi-rx chains UEs varies by at least +1.8 dB and up to +2.4 dB at the 50-th %-ile compared to single-rx chain UEs Observation 2: With narrow UE beams, the spherical coverage of multi-rx chains UEs varies by at least +1.9 dB and up to +2.8 dB at the 50-th %-ile compared to single-rx chain UEs. Proposal 1: The scope of an RX chain architecture includes possible implementations and UE capabilities as listed below: - Multiple Antenna panel - Multiple Antenna panel + AGC - Multiple Antenna panel + AGC + front-end, FE (time and frequency sync) Proposal 2: Clarify that the scope includes UE RF architectures where 4 layers may be supported by a single panel or by two or more panels in order to progress on the discussion on the separation between the AoA. Observation 3: When a single panel supports 4 layers, even though the AoAs may have a small separation the simultaneous reception can be supported. When a single panel only supports 2 layers (and 2 panels are needed to simultaneously operate for 4 layer reception), the separation between the AoA needs to be such that they are directed towards two different UE panels. |
| R4-2211882 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple | Observation 1: For the multi-panel case with simultaneous reception, the panels may not have the same performance, where physical limitations and constraints, such as thermal noise effects, routing losses, and other design constraints, may restrict the scanning range and gain of each panel. Observation 2: The scenario where a single panel is used to receive two AoAs should not be considered in this WI. Proposal 1:The new RF requirements should be specified with the aim to minimize RF impact on legacy UEs/RF designs. Proposal 2: The new RF requirements should be defined in an implementation-agnostic manner and the associated testing should be specified in such a way that a UE is not required to disclose its implementation details such as the number of panels or how each panel performs. Proposal 3: The range of two AoAs, which concerns the level of EIS of two AoAs and how much 2 AoAs can cover over the whole sphere, should be investigated. Proposal 4: Open issues should be further investigated. |
| R4-2211978 | On multi-RX chain | Xiaomi | Will be treated in [233] |
| R4-2211993 | Requirements for FR2 simultaneous DL reception from different directions | Samsung | Observation 1: polarization diversity is needed in FR2 receiver RF requirements. The MIMO mode performance with maximum layer can be covered by demodulation requirements Observation 2: spatial diversity mode could not characterize the new feature of the enhanced UE in receiver RF requirements. So the spatial multiplex mode instead of spatial diversity mode needs to be configured. Proposal 1: RAN4 to specify new dual AoA RF requirements for UE supporting simultaneousReceptionDiffTypeD-r16 with rank 2 configuration based on the assumption of “polarization diversity + spatial multiplex”. Observation 3: For new spherical coverage requirements based on dual AoAs, it is not implementable to measure all the permutation of direction pairs. Proposal 2: dual AoA spherical coverage is divided into two separate 3D scan per Reference Signal. Fix the AoA of one RS and perform 3D EIS scan for another RS; and then vice versa. For the direction of RS which is not being tested, its direction plays the role of anchor. Proposal 3: the N% value of each power class should not be changed. RAN4 further discuss the metric for dual AoAs spherical coverage with the 50% value for PC3, and requirement relaxation is expected compared with legacy spherical coverage requirement. Observation 4: spatial MIMO performance will be degraded when the two AoAs are close to each other. Proposal 4: a minimum angle can be considered to address the “different direction” condition in RF requirements for simultaneous DL reception from different directions |
| R4-2212185 | Discussion on UE RF requirements for simultaneous DL reception with up to 4 layer MIMO | LG Electronics | Proposal 1: Consider at least the activated Back to Back 2 panels to define RF requirements for enhanced FR2-1 UEs. Proposal 2: Consider 1 layer as a baseline for enhanced FR2-1 UE RF Rx requirements. Proposal 3: Consider diversity gain of ‘non-zero’ for REFSENS of enhanced FR2-1 UEs. Proposal 4: For EIS spherical coverage, consider CDF based on all combinations of two different Rx directions. |
| R4-2212332 | System assumptions for multi-Rx chain DL reception | Qualcomm Incorporated | Proposal 1: For this feature, the system assumes that the UE connects to 2 TRPs and can support polarization MIMO in DL with each. Proposal 2: The UE uses Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16 to convey to the network what QCL-D source signal it can support for the multi-chain Rx feature. Proposal 3: For this feature, if the network can provide a CSI-RS-based QCL-D source signal, it configures the 2-port type of the same from each TRP. Proposal 4: The RF requirement on the UE assumes the following sequence of events: 1. starting condition: basic connection between the UE and a single TRP 2. network configures UE for joint reporting with other TRPs visible to the UE 3. network requests CSF based on RS pairs reported by the UE with ‘joint reporting’ 4. network configures second active TCI state based on CSF 5. network continues to provide resources to refine the UE beams for a 2-active TCI state condition 6. (UE is ready to be evaluated) Proposal 5: The single DCI scheme is adopted as a baseline for setting the UE RF requirement. |
| R4-2212593 | Discussion on simultaneous multi-Rx chain DL reception | Xiaomi | Proposal 1: Each panel of the simultaneous multi-Rx chain reception should meet the legacy spherical coverage requirement for reception from a single direction. Proposal 2: RAN4 considers specifying the EIS total spherical coverage requirement Z dBm for simultaneous multi-Rx chain reception @ N%-tile (N% = 50% for PC3), how to improve Z based on Y dBm is FFS. |
| R4-2212806 | General analysis on Multi-Rx requirements | vivo | Observation 1: A typical UE implementation has to active the multi-panels for Rx beamforming for different QCL type D RS. Observation 2: Use one layer per AoA for RF test was proposed. Observation 3: Multi-panel design already used from Rel-15 with one panel active at a time, and unequal performances between panels are normal. Observation 4: There is no panel specific requirements or configuration in the core and testing spec from Rel-15. Observation 5: The design of 2AoA directions selection for multi-RX would have significant impact on the requirements and the feasibility. Observation 6: Testing all 2AoA combinations is not possible, and some selection/simplification is needed. Observation 7: 2AoA directions Option 1 (Full set AoA1 + samples of AoA2 for each AoA1) is unduly complex than the verification can support. Observation 8: 2AoA directions option 2 (One Fixed AoA1 (e.g. Peak) + Full set AoA2) have a number of merits and maybe the most simplified configuration. Observation 9: 2AoA directions Option 2a (Multiple AoA1 + Full set AoA2) can improve the 2AoA selection coverage, but the test efforts would also increase proportionally. Observations 10: Small angle differences between different AoA would cause extra interference. It is still not clear whether and how to consider it. Observation 11: With the 2 EIS values for each AoA pair, performance metric can be either combined together or treat them separately. Observation 12: How to treat EIS for 2AoA? How to treat EIS for 2AoA?; Pros; Cons. Combined; Better reflect overall performance.; Not easy to do mapping with spherical coverage or reuse legacy requirements or concept.. Separate; May easier to do mapping with spherical coverage or reuse legacy requirements or concept.; More difficult to reflect overall performance.. Proposal 1: The concept of panel should not be explicitly used in core requirements and test configurations. Proposal 2: Discuss the design of 2AoA configuration together with the requirements. Proposal 3: For 2AoA directions, option 2(One Fixed AoA1 (e.g. Peak) + Full set AoA2) is proposed to be baseline, and option 3(Multiple AoA1 + Full set AoA2) can also be considered. Whether and how further exceptions, such as the cases of small angle difference, would be considered also need discussion. Proposal 4: Two EIS values can be obtained for a combination of two AoAs, by means of fixing the power of one AoA and do measurements of another one. Proposal 5: Further study how to define performance requirements for 2AoA case, and consider the case with interference during the process. |
| R4-2213568 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson | Observation 1: the spherical coverage of simultaneous reception depends not only on the absolute direction of each AoA but also on the offset between the two AoAs. Observation 2: There are two possible way of defining the spherical coverage of simultaneous reception from different directions: 1) The range of solid angle that the device can receive the the DL reception from any two different directions within it. 2) the spherical coverage of the second Rx chain while the first Rx chain is already in connected mode. Observation 3: It is possible to verify the UE spherical coverage of simultaneous reception from two different directions by having one AoA fixed at beam peak direction and sweeping the other AoA. While how to ensure the AoA align with beam peak direction while UE is rotating need to be further studied. Observation 4: It is also possible to verify the UE spherical coverage of simultaneous reception from two different directions by having a fixed offset between the two AoAs but swept both probes simoutanously. The value of the offset and the number of offset values need to be further studied for each power class. Observation 5: There is some similarity between the CDF of two AoAs reception and single AoA spherical coverage. However, further study is needed to investigate how to obtain the performance of two AoAs spherical coverage from the RF test. Proposal 1: RAN4 shall first clarify the scope of the spherical coverage requirements for devices with simultaneous reception from different directions. Proposal 2: RAN4 shall focus on the scenario of single-layer reception for each DL direction with dual TCI configuration for the spherical coverage requirements for devices with simultaneous reception from different directions. Proposal 3: RAN4 shall first agree on the definition of spherical coverage with simultaneous reception from different directions in the core requirement. |
| R4-2213737 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon | Observation 1: The assumption for current RAN4 RF requirements is single activated panel, but there is no specific limitation of the UE implementation, e.g. total panel number and layout. Observation 2: The correlation between the following two factors is very strong: • relative angular offset between 2 AoAs • definition of the spherical coverage requirement and the test method design Proposal 1: All existing implementation shall be carefully considered when the general RF requirements will be defined for simultaneous DL reception with two different QCL Type-D RSs on single component carrier. Proposal 2: For simultaneous DL reception with two different QCL Type-D RSs on single component carrier, the legacy spherical coverage requirement for reception from a single direction shall be kept. • Demod requirements and test cases for 4 layer MIMO could be further considered. Proposal 3: Send an LS to RAN1 for asking more background info at least about: • All necessary SLS assumptions to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on. • The valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS. Proposal 4: Taking the current 50%-tile EIS spherical coverage requirement (for PC3) for single band as the baseline, the new spherical coverage requirement shall be further discussed at least considering the following two factors: • angular offset between two AoAs • reception power imbalance of the DL signals between two AoAs Proposal 5: Simulation is needed to discuss whether and how new spherical coverage requirement can be specified. Proposal 6: The simplification of the new test method for 2 AoAs shall be further considered. |

## Open issues summary

### LS to RAN1 on system assumptions (R4-2213737)

Proposal: Send an LS to RAN1 for asking more background info at least about:

- All necessary SLS assumptions to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on.

- The valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS.

| Company | Comments |
| --- | --- |
| Nokia | As shown in R4-2213737 with 3 cases for discussing the Peak EIS and the spherical coverage, it is obvious that the optimum case for peak EIS and highest spherical coverage is just one of many, many cases so the LS may serve for clarification from RAN1, though it risks delaying the work of RAN4 for this release. Rel-16 like M-TRP PDSCH scheme, discussion is already closed in RAN1. |
| Qualcomm | We are open to this idea if we can generate a specific list of questions that are relevant to the objectives of the WI. The proposal above however seeks deployment details from RAN1, RAN4 is probably a more relevant source of that information. |
| vivo | It is our understanding that RAN1 may not have information that pursued here, and it seems not that necessary for sending LS. |
| Apple | We would like to have some discussion in RAN4 first to have a better idea of the requirement concept, i.e., what constitutes an appropriate requirement in RAN4 and what the requirement should look like. Once there is some common understanding, we can discuss what info is missing in RAN4 in order to define the requirement and should be supplied by other WGs. At this moment, the range of angular offset between 2 AoAs may be one piece of such info, depending on the discussion of the requirement concept. |
| Sony | We understand that those questions are for RAN4 to better understand what the actual deployment scenario is like for mTRP. However, We believe RAN4 can figure those questions out by ourselves based on the input from network vendors and operators, and we don’t see the necessity to send LS to RAN1. |
| LG Electronics | At this time, LS seems not be necessary. After discussing in RAN4 first, if there are some issues to be asked, then we can go LS. |
| Verizon | For 3 cases, RAN4 should be a more relevant information source. Some more discussions may be needed |
| Huawei | As commented by Nokia, the m-TRP WI has been discussed by RAN1 from Rel-16. It is understandable that performance gain would be derived with some conditions, and it is expected that some scenarios are preferred based on SLS simulation performed by RAN1. Though the deployment scenario can be discussed by RAN4 as well, however, we are not sure the observations by RAN4 could be exactly aligned with the study by RAN1 as some analysis methodology used by different WGs are different. We don't want to head into the discussion with blind directions, and put a lot of energy to consider some scenarios which could possibly be ruled out or filtered by RAN1. Additionally, for instance, AoA angular offset is directly related to the RF requirement derivation, test method design (and possibility on reducing the test burden). Since this feature has been fully discussed in RAN1, we think they must have reasonable precondition/assumption on the AoA angular offset, so that obvious gain could be observed and evaluated. We believe RAN4 has many precedents that enough inputs are provided before further discussion on RF requirements. In conclusion, we think this LS should be sent out as soon as possible, and RAN4 could get feedback in time to facilitate more efficient discussion on RF requirements. The detailed content in the LS could be further discussed. |
| Xiaomi | We think RAN4 should first discuss the main application scenarios and related requirement concept, then decide whether need send LS to RAN1 for more information. |
| Samsung | RAN1 simulation assumption may not be very helpful for our RAN4 work on these aspects. LS could be sent when there is concrete questions when necessary rather than high level questions. e.g., angle separation perspective is a concrete question if RAN4 consider this issue needs RAN1’s input. Firstly RAN4 should struggle to work it out within the group. |
| Nokia | We do not support sending LS to RAN1 for SLS assumptions in this meeting, more RAN4 discussion should be done first. |
| Ericsson | No need to send an LS to RAN1 at this stage. |

### DL split between TRPs for 4L (R4-2211993, similar view in R4-2212332)

Proposal: (4L DL supported by) UE when it is scheduled multi-panel simultaneous reception from different QCL Type-D RS based on the assumption of “polarization diversity + spatial multiplex”.  (i.e., rank 2 from each of two directions)

| Company | Comments |
| --- | --- |
| Nokia | We support this proposal with the clarification of rank 2 from two directions are received simultaneously. |
| Qualcomm | Support |
| vivo | Support this proposal. |
| Apple | We believe to support 4 layer MIMO in DL, we need to assume “polarization multiplex + spatial multiplex”. However, for the RF requirement, we agree that “polarization diversity + spatial multiplex” is reasonable. |
| Sony | It is not clear to us the relation between this issue and issue 2.2.6. If this proposal intends to define a general scope of the whole WI, then we are fine with it, and the 4L DL can possibly be tested with a demoudulation test. However, for RF spherical coverage requirement, we think it is sufficient to verify with a single layer per direction. In addition, we share similar understanding as Apple that the “polarization multiplexing+spatial multiplexing” is 4L DL. For “polarization diversity + spatial multiplexing”, it is rank 1 per direction in our understanding. |
| LG Electronics | Topic 2-2-2 is only for 4 Layer. In our understanding, ‘polarization diversity + spatial multiplex’ can support ‘2 Layer’. So, it is not aligned with this Topic. According to WID, up to 4 Layer should be supported, such as, 1, 2 and 4 layer. As Sony mentioned, we need to discuss it with issue 2.2.6 together. FYI, our comments in issue 2.2.6 are captured. [2.2.6 : LGE’ comment] So far, UE RF requirements were specified based on 1 layer. It is for only single AoA. For two AoAs, there can be different cases as follows. - Case 1 : 1 layer reception for two DL directions (total 1 layer reception). - Case 2 : 1 layer reception for each DL direction (total 2 layer reception) - Case 3 : 2 layers reception for each DL direction (total 4 layer reception) Which case is baseline needs to be decided firstly. Preference is Case 1. 2 layer and 4 layer performance can be verified in demodulation part. |
| Huawei | We can accept the proposal in general. |
| Xiaomi | Similar understanding with Apple, to support 4L MIMO in DL, the assumption of “polarization multiplex + spatial multiplex” is needed, and it can be verified by demod. For RF requirements, I think both of “polarization diversity + spatial multiplex” and “polarization diversity + spatial diversity” for RF requirements are OK. Considering spatial diversity from multi-TRP will have the highly demand for synchronization and collaboration between different TRP, assuming “polarization diversity + spatial multiplex” is better, how to verify it from the RF requirements need further discuss. |
| Samsung | Support the proposal. The understanding is rank 1 per direction, in total it is rank 2 for both directions. That is the Case 2 in LGE comments above. To avoid misunderstanding, the wording in brackets maybe need to be reworded: (i.e., rank 2 from each of two directions) (i.e., rank 1 for each direction and in total rank 2) For the Case 1 in LGE comments, the problem is there is no much performance difference compared with R15 which was based on a panel switching mechanism, and such framework could not guarantee UE can satisfy 2 and/or 4 layers reception. e.g., a UE with one very good panel and another very bad panel, it could still pass the Case 1 requirements but it actually does not support 2AoA reception with good performance for both AoA. |
| Ericsson | The RF requirements like spherical coverage can be based on single-layer transmission per direction for a large test coverage (UE capability). |

### DCI scheme used by network for 4L DL (R4-2212332)

Proposal: The single DCI scheme is adopted as a baseline for setting the UE RF requirement.

| Company | Comments |
| --- | --- |
| Nokia | In the WID, it is clear that “The case of single TCI is handled as a second priority”. Multi-TCI should be prioritized. Single DCI may unnecessarily restrict the deployment scenarios therefore multi-DCI should be baseline, while we acknowledge that single DCI should not be scoped out. |
| Qualcomm | Multi-DCI treats the DL transmissions from each TRP as an independent transmission. This means the network can use FDM rather than SDM. The MCS adaptation is independent also. Because of the general nature of multi-DCI, the UE cannot presume 4x4 demod is feasible. This in turn means that L1/L2 will always be an interferer to L3/L4 and vice versa. So the envelope of AoA pairs where the UE can sustain DL from 2 TCI states will be greatly reduced for high MCS. To Nokia: would you clarify why single TCI is relevant here, given that we are already focusing on 2-TCI state configuration? |
| vivo | Support using single DCI can server as a baseline configuration. Single TCI does not have to be considered. |
| Apple | We support using single DCI with multi-TCI. Multi-DCI involves difference cases such as completely overlapping resources, completely non-overlapping resources, or partially overlapping resources between the two PDSCH. From RAN4 RF requirement’s standpoint, such complexity is unnecessary. |
| Sony | We support this proposal. Single DCI and dual TCI can be adopted as baseline scheme. |
| LG Electronics | Support this proposal for only RF requirement setting. Because, we think that RRM specification can be impacted depending on single DCI or dual DCI. |
| Huawei | We think single-DCI can be the baseline. Meanwhile multi-DCI doesn’t need to be precluded. |
| Xiaomi | Support this proposal, use single DCI as the baseline. |
| Samsung | We support the proposal. Single DCI is appropriate for RF requirements |
| Nokia | It is clear that 2-TCI is the focus. The discussion is on single DCI or dual DCI as a baseline. Single DCI risks down-scoping the WI (the WID is not only limited to MIMO therefore non-collocated TRPs should be handled as well) and dual DCI is already supported in Rel-17. For 4L DL MIMO, we can agree to single DCI assumption. For other scenarios, dual DCI shall still be discussed. |

### Principle for determining UE requirements (R4-2211814, R4-2211882, R4-2213737)

Options:

- RAN4 must consider in this work item how to specify the DL spherical coverage requirement of the FR2-1 multi-Rx chain UE to ensure satisfactory real-life performance of the FR2-1 multi-Rx chain UE for two AoAs simultaneous reception in terms of both spherical coverage percentage and sensitivity.

- The new RF requirements should be specified with the aim to minimize RF impact on legacy UEs/RF designs.

- All existing implementation shall be carefully considered when the general RF requirements will be defined for simultaneous DL reception with two different QCL Type-D RSs on single component carrier.

| Company | Comments |
| --- | --- |
| Nokia | We support option 1 to “ensure real-life performance” with the wording used in option 3: “carefully considered”. Regarding option 2, minimizing impact on legacy UEs should not limit the achievable performance of Rel-18 4-layer UEs. |
| Qualcomm | These proposals are subjective, but not sure how they would contribute to meaningful boundary conditions. Proposal 1 sounds reasonable, but can proponents define what specific parameters we should include in our analysis Proposal 2: Can proponents clarify how ‘minimize RF impact’ should be quantified? Proposal 3: Can proponents clarify how the proposal can be reworded to streamline work? |
| vivo | All these options are reasonable, and it seems the final solution would be a somehow compromise. However, as Qualcomm commented, it is still not clear how to really transform those guidelines into specific parameters. |
| Apple | On Proposal 2, as discussed in our paper, UEs may have different architectures to support beamforming capabilities. Therefore, from UE hardware’s standpoint these legacy UEs should be also considered for the requirement definition. By “minimize RF impact”, we mean the requirement to be defined in RAN4 should aim to accommodate those implementations to the maximum extent possible. We also agree the requirement needs to ensure real-life performance of UEs supporting this feature, as mentioned in Proposal 1. We also support proposal 3 that all existing implementations should be carefully considered or duly considered. |
| Sony | We don’t see the need to make an agreement here. Those are general principles that RAN4 should always follow when defining any requirements, e.g., the RAN4 requirement shall ensure the device's real-life performance and be implementation agonistic. |
| LG Electronics | Support option 1. The wording ‘ensure real-life performance’ needs to be updated by proper wording or can be removed. For option 2, it is not clear that ‘RF impact’ means ‘legacy RF requirement’ or not. According to the current WID, the following is already included. - The legacy spherical coverage requirement for reception from a single direction will be kept |
| Verizon | This work should align on the work item for how to specify the DL spherical coverage. Also, we agree that other two options are important too for this work, |
| Huawei | In general all options are not conflict with each other. We support to capture the option 2 and option 3 as working assumptions of this WI since we think they are the principle for RF requirements discussion. Specifically, for existing implementations, multi-panels were already assumed in defining the RF requirements in previous release studies, though simultaneous reception is not supposed, but it doesn’t mean that these UEs cannot support simultaneous multi-panel reception from implementation perspective. The study in Rel-18 should not be considered targeting for brand-new UE implementations. |
| Samsung | These proposals as high level principles are reasonable, however they not very implementable. For agreement, more detailed aspects are needed. |

### UE architecture assumption

Clarify that the scope includes UE RF architectures where 4 layers may be supported by a single panel or by two or more panels in order to progress on the discussion on the separation between the AoA (R4-2211815)

Proposals (choose one of the options below):

- The concept of panel should not be explicitly used in core requirements and test configurations. (R4-2212806, similar view in R4-2211882)

- Consider at least the activated Back to Back 2 panels to define RF requirements for enhanced FR2-1 UEs. (R4-212185, similar view in R4-2211815)

- Different UE implementations/architectures should be accommodated. (R4-2211882)

- Others.

| Company | Comments |
| --- | --- |
| Nokia | We support 3. As highlighted in proposal 2 of [R4-2211882] there should be no limitation in the aspects of how the 4-layer operation is supported in the antenna design, as long as the scheme is followed. We find it limiting to assume back 2 back panels for testing, but rather that we acknowledge that the choice of antenna implementation and supporting RF architecture will impact the performance relative to the test scenario, and that needs to be considered in the architectural assumptions. |
| Qualcomm | Option 1 |
| vivo | Option 1, this is a necessary for accommodation of various implementations, which is also aligned with option 2. For option2, it might be used in the analysis as a starting point, but final requirements may still consider other possible architectures. |
| Apple | We support proposal 1 and proposal 3. |
| Sony | As a general principle, we support option 1. From the RAN1 aspect, the panel is a virtual concept without constraining specific physical implementation. Therefore, we should not use the panel concept in the RAN4 spec to avoid potential conflict. However, when deriving the actual requirement, we may need to consider different UE implementations as suggested in option 3, but this is just business as usual in RAN4. |
| LG Electronics | Support Option 1 for specification of core requirements. To align the requirements, we need to assume baseline of panel placement. For it, preference is Option 2. |
| Huawei | We support Option 1 and Option 3. |
| Xiaomi | From the Spec perspective, the requirements should be implementation agnostic. But to align the requirement, the assumption of Antenna module/set number and location are needed, like the definition for R15 spherical coverage. |
| Samsung | Support option 1 in core requirements and specification. When discussing UE assumption, detailed panel implementation can be considered including Option 2 and 3. |

### Scope definition for UE RF requirements (R4-2213568)

Proposal: (for UE RF) RAN4 shall focus on the scenario of single-layer reception for each DL direction with dual TCI configuration for the spherical coverage requirements for devices with simultaneous reception from different directions

| Company | Comments |
| --- | --- |
| Nokia | This proposal would only support 2 layers DL MIMO where the scope of the WID is 4 layers DL MIMO. Therefore, we need at least dual-layer reception for each DL direction with dual TCI configuration. |
| Qualcomm | We support the proposal. Legacy UE RF requirements do not include one on rank 2 DL from single AoA, the proposal is an extension of that concept. 4L performance verification can be left to demod. |
| vivo | Support this proposal that for RF requirement, and this is also aligned with what RF has concentrated. 4 layer operation can be tested in demod. |
| Apple | We support the proposal. |
| Sony | We support this proposal as a proponent. High order DL (e.g., Rank 3 or 4) can be verified in demodulation tests. For RF, it is most important to examine the UE coverage on two RF chains/panels, and it is enough to test with one layer per direction. |
| LG Electronics | So far, UE RF requirements were specified based on 1 layer. It is for only single AoA. For two AoAs, there can be different cases as follows. - Case 1 : 1 layer reception for two DL directions (total 1 layer reception). - Case 2 : 1 layer reception for each DL direction (total 2 layer reception) - Case 3 : 2 layers reception for each DL direction (total 4 layer reception) Which case is baseline needs to be decided firstly. Preference is Case 1. 2 layer and 4 layer performance can be verified in demodulation part. |
| Verizon | We support this proposal. |
| Huawei | We support this proposal since it is proposed for RF requirement definition. |
| Xiaomi | Support it just for RF requirement, I think the meaning is the same with the assumption of “polarization diversity + spatial multiplex” in 2.2.2. |
| Samsung | This issue is similar as issue 2-2-2. We support the proposal |
| Nokia | We are ok with this proposal for UE RF requirements. |

### How the network/TE configures the UE for mTRP connection (R4-2212332)

The RF requirement on the UE assumes the following sequence of events:

- starting condition: basic connection between the UE and a single TRP

- network configures UE for joint reporting with other TRPs visible to the UE

- network requests CSF based on RS pairs reported by the UE with ‘joint reporting’

- network configures second active TCI state based on CSF

- network continues to provide resources to refine the UE beams for a 2-active TCI state condition

- (UE is ready to be evaluated).

| Company | Comments |
| --- | --- |
| Nokia | Detailed test procedure can be discussed after core/test requirements are decided. |
| Qualcomm | We are open to refining the list further. The assumptions above are important because it has to be demonstrated that there is a way for the UE to refine its simultaneous 2TCI-state beams prior to verification of performance. This assumption will be needed to specify requirements. Without this type of understanding, the UE can only promise to use rough beams towards each TCI-state, with accompanying loss of spatial filtering. |
| vivo | Still not quite clear on the need and how detailed info would need to be specified. |
| Apple | In general, the sequence of events looks OK. Perhaps a comment on step 5. After step 4, the UE is likely engaged in receiving two AoAs already and BMRS (such as CSI-SSB) is being continuously transmitted. In step 5, does “provide resources” means BMRS resources? Also, how to use such resources is up to UE. Based on the clarification from QC, re-wording of step 5 is needed. |
| Sony | We appreciate the clarification of the procedure, which helps to understand how the actual mTRP configuration works. But do we need any specific agreement here? in our understanding, those procedure does not need to be specified in RAN4 spec. |
| LG Electronics | Same view with Nokia. |
| Huawei | We would like to ask about whether this procedure is proposed for test or requirement definition? If it is targeting for the test, we think it is better to be postpone before clear conclusion for core requirements could be formed. If not, we think such procedure is highly related to RAN1 previous study on mTRP, and RAN4 shall ask for RAN1 inputs in the LS. Because for instance the feasible deployment scenarios for gNB to configure mTRP operation has already been identified by RAN1, and RAN1 could further clarify other details like what we have been explained for Issue 2.2.1.In this way, we think it is necessary to ask RAN1 inputs on the procedure. Regarding the procedure itself, we also would like to better understand the purpose of “refine” in step 5. It is more like UE implementation rather than fundamental procedure. |
| Xiaomi | Prefer to discuss it later |
| Samsung | Thanks for the information provided and we also have the same question as Sony whether these procedures need to be specified in core RF requirements. |
| Ericsson | The test procedure should be decided later |

### How to determine candidate AoA pairs for setting the UE RF requirement

Multiple contributions have detailed considerations on the subject. For this discussion the following shorthand is used: AoA’N’ is AoA of RS ‘N’ (R4-2211993). For the direction of RS which is not being tested, its direction plays the role of anchor (R4-2211993).

Proposals (choose one of the options below):

- One Fixed AoA1 (e.g. Peak) + Full set AoA2. (R4-2212806, similar view in R4-2211993)

- Multiple AoA1 + Full set AoA2 (R4-2212806)

- Pending on the feedback from RAN1. (related to Issue 2.2.1)

- Fixed offset between the two AoAs, both probes swept simultaneously (R4-2213568)

| Company | Comments |
| --- | --- |
| Nokia | Support option 3, as discussed in issue 2.2.1 above. |
| Qualcomm | Option 2 seems the best compromise, and we note that option 1 is a subset of option 2. We are ok to develop option 4 further, like having multiple ‘fixed offsets’, but a single offset may not work for all future UEs. |
| vivo | Option1 or Option 2. For option 3, it is doubtful how RAN1 feedback may help. For option 4, fixed offset is problematic since no one typical value is reasonable. If multiple offsets considered, the test burden with both swept can still be unduly high. |
| Apple | We would like to have more discussions on the requirement concept before settling down on one or two options. |
| Sony | Obviously, we will not test all the possible AoA combinations, and thus, we have to select one of the options from 1,2, or 4. We are open to all solutions but we think we shall first study the feasibility of each test method. One concern on having one AoA fixed (option 1 and 2) is how we can ensure the AoA is fixed to UE orientation if the UE is rotating during the test. To our understanding, this is not always possible for all kinds of test equipment. For option 4, it is possible to verify under two offset values for PC3, to mimic wide and small angel separation of TRPs, which only requires two sweepings. We think the test time of such a scheme would be similar at least comparing to option 2. |
| LG Electronics | Even though not testing all the possible AoA pairs, core requirements need to be defined based on all the possible AoA pairs. And, CDF of the AoA pairs can be different depending on Case 1, 2 and 3 in our comment of issue 2.2.6. We would like to add ‘proposal 5’ based on the R4-2212185. Proposal 5 : Full set AoA1 + Full set AoA2 (R4-2212185) |
| Huawei | Option 3. We don’t think that all possible AoA pairs can provide necessary performance gain for multi-panel reception scenarios, especially for 4 layer MIMO. UE implementation should not be accommodated to those unrealistic scenarios with no gain for supporting the feature. Our preference is to wait for some inputs from RAN1. |
| Samsung | We support option 1. Option 1 already brings double test efforts compared with legacy spherical coverage test. If going with option 2, the test efforts is not affordable, and actually is not so necessary. About option 4, we think there are some issues. For the Back to Back 2 panel implementations, the test could not be done with fixed offset between 2AoAs. Moreover, How to set the direction of the fixed offset between the two AoAs (vertical, horizontal, or other directions) will cause different relative position between UE and the two AoAs. |
| Rohde & Schwarz | We need to consider also the testability aspect in the requirement definition otherwise the requirement may be untestable. Option 4 could be a reasonable approach for testing, when measurement on both directions at the same time is required and can be realized in e.g. the RRM 2AoA system today. Here also different fixed offsets would be possible. For sequential measurements as e.g. outlined by Samsung (find AoA1, fix and transfer to link/anchor, scan AoA2), Option 1 or Option 2 could be feasible as well. This may however require a specific test function that allows fixing one of the beams (similar to current UBF). Additionally, more thinking is required how to actually measure the Refsens value when receiving multiple signal. Currently, both polarizations are measured sequentially and then summed, but it is not sure that this will work for multiRx since the UE shall receive two signals from two different directions. |
| Ericsson | It is not obvious how to test the proposed options 1-3 assuming a test with a rotating DUT and two probes – measure spherical coverage with different separations between the probes? Option 4 may be feasible. |
| Lenovo | We agree with LG and support of Proposal 5. Requirement should be defined in terms of all possible AOA pairs even if all pairs cannot be tested. If requirement is not defined in terms of all possible pairs of directions, then some orientation of the device relative to the TRP’s must be assumed when defining requirements. |

### Discussion on how to treat test condition with small separation between AoAs

Multiple contributions have detailed considerations on the subject. There are no clear proposals, so below is intended as open-ended discussion to determine common views if any. Some considerations to trigger discussion (add others as needed):

- What conditions define ‘AoA’ separation as too small?

- Would the ‘too-small’ criterion be a function of DL MCS?

- (others)

| Company | Comments |
| --- | --- |
| Nokia | Detailed test condition can be discussed after core/test requirements are decided. |
| Qualcomm | In our view common understanding of UE behavior in these conditions is necessary for requirement specification. It cannot wait until requirements are decided. In our view, too small could be tied to a throughput degradation of some kind. This in turn depends on whether the UE can perform 4x4 demod or if L1/L2 and L3/L4 are mutual interferers. |
| vivo | Basically “small” AoA is not that likely to be used for spatial multiplexing, and do have some inconsistency with the tentative configurations. However, for RF test, it is still not clear whether “small” AoA should be precluded, as this may introduce a lot more discussion. Maybe accept some performance degradation is also a possible way forward. |
| Apple | When the two AoAs are too close, it is expected either the UE cannot support two AoAs at all, i.e., PDSCH becomes undetectable because of large interference (the signals from the two AoAs are interference to each other because of the lack of spatial separation), or there is significant SNR degradation leading to lower MCS. We can further discuss this. |
| Sony | This depends on the outcome of 2.2.8. If we select option 4 (fixed offset between two probes) in 2.2.8, we can avoid such an issue by choosing a proper offset. However, if we go with the option that one AoA is fixed, we wonder if this must be treated? Even for single EIS spherical coverage, there are blind areas where the UE has very low performances, which in our view, is no different from the issue we are talking about here. |
| LG Electronics | According to WID, layer 1, layer 2 and layer 4 are included. So, it is possible to receive the signals with small AoA with one panel for some case. The case should be considered. |
| Huawei | If this is for the test, we suggest to postpone the discussion before we can have some conclusion on core requirement. As the proposal itself, we think that is quite relevant to the AoA angular offset lower threshold for qualified simultaneous reception performance by 2 panels. |
| Samsung | First of all, the WID says to define requirements for “different direction”, when the two direction are two close, spatial isolation is not guaranteed and we consider it is not the expected scenario for this WI. For RF requirements, it also related with measurement grid. Spherical coverage measurement grid in constant step size is 15deg, but in polar region the solid angle is much smaller than 15deg. In case of constant density grid, the solid angle between two adjacent test points is almost the same. At least we need to make sure in spherical coverage test, the angle separation between 2AoA will not impact the performance. |
| Rohde & Schwarz | Are there any baseline assumptions for the angular separation of the TRPs in the real network? Since the concept of multiRx relies on spatial separation, there should be some minimum spatial separation which is required. So defining requirements below that limit makes no sense. Also we need to take into account that the requirement needs to be testable. |
| Lenovo | It should be possible to receive two AOA’s with one panel. Otherwise it is not clear how 50% can be met. Consider two panels each covering orthogonal hemispheres. If it is not possible to receive two AOA’s from one panel, then receiving from two AOA’s is only possible if one AOA is in the first hemisphere covered by the first panel and the second AOA is in the second hemisphere covered by the second panel. The probability of this occurring is ½ -- at least this is our understanding. |

## Companies views’ collection for 1st round

### Open issues

### CRs/TPs comments collection

N/A

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Topic | Description | Status summary |
| --- | --- | --- |
| 2.2.1 | LS to RAN1 on system assumptions | 9 against, 1 for sending LS Some views in the ‘against’ camp were: - need a specific list of questions - wait for requirement concept to take shape - let RAN4 take a crack at the problem first The ‘for’ camp view is: - not sure the observations by RAN4 could be exactly aligned with the study by RAN1 Recommendations for 2nd round: Discussion can pause for the rest of the meeting. Further discussion can be resumed after developing requirement concept |
| 2.2.2 | DL split between TRPs for 4L (general deployment assumption, not aimed at UE RF assumption) | Potential WF: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE. Recommendations for 2nd round: Confirm above in WF |
| 2.2.3 | DCI scheme used by network for 4L DL | Potential WF: For UE RF requirements when configured with 2 active TCI states, single DCI scheme is assumed for the network. Recommendations for 2nd round: Confirm above in WF |
| 2.2.4 | Principle for determining UE requirements | General sentiment is that the principles were agreeable and are ok to be followed subjectively. Majority view was that they were ‘not implementable’ as agreements that narrow scope of work. Recommendations for 2nd round: Discuss if wording refinement can yield actionable agreement. |
| 2.2.5 | UE architecture assumption | Potential WF: The concept of panel should not be explicitly used in core requirements and test configurations Recommendations for 2nd round: Confirm above in WF There was also scattered support for ‘Different UE implementations/architectures should be accommodated’ and ‘Consider at least the activated Back to Back 2 panels to define RF requirements Recommendations for 2nd round: Discuss if wording refinement can yield actionable agreement. |
| 2.2.6 | Scope definition for UE RF requirements | One company preferred 1L reception from 2 AoA as UE RF, while majority prefer 1L per AoA as an extension of legacy requirements. Another company preferred RF requirements for 4L DL Potential WF along majority view: UE RF requirements for this feature shall be based on single-layer reception for each DL direction with dual TCI configuration. Recommendations for 2nd round: Align company views |
| 2.2.7 | How the network/TE configures the UE for mTRP connection | General sentiment was there was nothing to capture as actionable agreement. Recommendations for 2nd round: Discuss if wording refinement can yield actionable agreement. |
| 2.2.8 | How to determine candidate AoA pairs for setting the UE RF requirement | There was no majority view, and further discussion is needed Recommendations for 2nd round: Continue discussion in WF, discussion will be recorded here if no agreement |
| 2.2.9 | How to treat test condition with small separation between AoAs | There was good discussion Recommendations for 2nd round: Discussion can pause for the rest of the meeting. Further discussion can resume while developing requirement concept |

### CRs/TPs

N/A

## Discussion on 2nd round (if applicable)

(WF assigned to Sony, discussion will be transferred here after close of window)

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on multi-Rx chain DL reception | Sony | (request new Tdoc) |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2211815 | Enhanced spherical coverage requirements for multi-rx chains UEs | Nokia, Nokia Shanghai Bell | noted |  |
| R4-2211882 | RF requirement for NR FR2 multi-Rx chain DL reception | Apple | noted |  |
| R4-2211978 | On multi-RX chain | Xiaomi | noted |  |
| R4-2211993 | Requirements for FR2 simultaneous DL reception from different directions | Samsung | noted |  |
| R4-2212185 | Discussion on UE RF requirements for simultaneous DL reception with up to 4 layer MIMO | LG Electronics | noted |  |
| R4-2212332 | System assumptions for multi-Rx chain DL reception | Qualcomm Incorporated | noted |  |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated | noted |  |
| R4-2212593 | Discussion on simultaneous multi-Rx chain DL reception | Xiaomi | noted |  |
| R4-2212806 | General analysis on Multi-Rx requirements | vivo | noted |  |
| R4-2213568 | Views on multi-Rx chain DL reception in FR2 | Sony, Ericsson | noted |  |
| R4-2213737 | On UE RF requirement for FR2 multi-Rx chain DL reception | Huawei, HiSilicon | noted |  |
| R4-2211814 | Proposal on spherical coverage requirements for FR2-1 multi-Rx chain DL reception | Nokia, Nokia Shanghai Bell | noted |  |
| R4-2212333 | Work plan for FR2 UE multi-Rx chain DL reception | Qualcomm Incorporated, vivo | noted |  |
| R4-2213053 | Work plan for RRM requirement for NR FR2 multi-Rx chain DL reception | vivo, Qualcomm | Treated in [233] |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-211xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-211xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-211xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

# Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)