3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214139

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.14.7

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [104-e][219] NR_ext_to_71GHz_RRM_2

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round

  - Test case structure

  - Test configurations

  - Test cases

  - RRM measurement performance

  - CCA aspects

  - Draft CRs

  - Test-cases and Work Split

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Ming Li | Ming.l.li@ericsson.com |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Nokia | Erika Almeida | Erika.almeida@nokia.com |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211690 | CATT | Proposal 1: When considering how to model an unavailable SSB/SMTC group in the test cases for NR extension to 71 GHz, the first concern should be whether to verify the beam sweeping behavior of the UE. Proposal 2: If beam sweeping behavior of the UE is to be verified, an unavailable SSB/SMTC group can be modelled as that there is one SSB index directing towards the UE not transmitted by TE in the available SSB/SMTC occasion when UE Rx beam direct towards the TE. Proposal 3: For test cases where no beam sweeping behavior of the UE to be verified, an unavailable SSB/SMTC group can be modelled as none of SSB index directing towards the UE is transmitted by TE in N consecutive SSB/SMTC occasions. |
| R4-2212910 | Nokia | Observation 1: The beam scaling sweeping factor in FR2-2 was agreed to be 12, which is different from the one considered in FR2-1. Proposal 1: Update the test case list agreed in RAN4 #103, considering that all test cases are also defined for 120 kHz SCS. Proposal 2: An unavailable SSB/SMTC group can be modelled as that there is one SSB not transmitted by TE in N consecutive SSB/SMTC occasions Observation 2: The accuracy requirements defined for RSSI measurements in NR-U Rel-16 apply onlty to 15 kHz and 30 kHz SCS. Observation 3: New RSSI accuracy requirements are needed for 120 kHz, 480 kHz and 960 kHz SCS. Proposal 3: RAN4 to define accuracy requirements for RSSI measurements with 120 kHz, 480 kHz and 960 kHz. |
| R4-2212961 | Huawei | Proposal 1: For licensed operation, create dedicated test cases for FR2-2 in existing sections (A.7). Proposal 2: Following SCS/BW configurations to be used in test cases for FR2-2 Proposal 3: Discuss whether UE is required to be tested using configurations for 480/960 kHz when the corresponding SCS is supported. Proposal 4: Exceeding Lmax shall be avoided in all test cases. Proposal 5: For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions |
| R4-2213026 | Vivo | Observation 1: For FR2-2, when the SCS is 960kHz, deriveSSB-IndexFromCell may be not enabled. For intra-frequency measurement, the SSB index detection may be needed. Proposal 2: The following two test cases for intra-frequency measurement for FR2-2 should be added compared with FR2-1: 1. SA event triggered reporting test with SSB time index detection without gap under non-DRX 2. SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX Proposal 3: The test configurations for inter-frequency measurement for FR2-2+FR2-2 should be defined as follows: Table 4: Supported test configurations for inter-frequency measurement for FR2-2+FR2-2 Configuration; Description. 1; 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode. Proposal 4: The test configurations for inter-frequency measurement for FR1+FR2-2 should be defined as follows: Table 6: SA event triggered reporting tests without SSB index reading for FR1-FR2-2 Config; Description of serving cell; Description of target cell. 1; NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode. 2; NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. 3; NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. Note: The UE is only required to be tested in one of the supported test configurations. Proposal 5: The test configurations for UE transmit timing for FR2-2 should be defined as follows: Table 7: Supported test configurations for FR2-2 Pcell Configuration; Description. 1; NR TDD, SSB SCS 960 kHz, data SCS 960 kHz, BW 400 MHz. |
| R4-2213349 | Ericsson | Proposal 1: Define SSB configurations for FR2-2 comprising 120KHz, 480KHz and 960KHz which in Clause RRM test configurations are after SSB configurations for FR1 and FR2. Proposal 2: Use 400MHz as bandwidth for test cases for FR2-2, which can be compliant with 120KHz, 480KHz and 960KHz. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Test case structure

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1-1: Test case structure

- Proposal 1 (Huawei): For licensed operation, create dedicated test cases for FR2-2 in existing sections (A.7)

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| MTK | Fine with Proposal 1 in general. |
| Ericsson | A question is if all FR2-2 cases are under A.7, existing FR2 cases shall be replaced by FR2-1 or any structure update can differentiate FR2-1 and FR2-2 under A.7? |
| Huawei | Support proposal 1. To Ericsson: It is a good question. A.7 is for FR2 (both FR2-1 and FR2-2), the newly added test cases is with the tile “…. FR2-2”. For legacy test cases, it is only considered for FR2-1. One possible approach may be adding generic statement that test cases apply to FR2-2 only with explicit statement in each test case. If we have all FR2-2 test cases in dedicated sections, we think we face the same problem on how to treat the legacy test cases (e.g. A.7) as mentioned by Ericsson. |
| Qualcomm | We are okay with proposal 1 in general but in order to be future proof (e.g. FR2-1 + FR2-2 are defined in future releases), another possible way could be to modify the current tests to also include FR2-2 configurations. This may be a lot of work and may all test cases may need to be checked carefully. |
| Apple | Proposal 1 is OK. |
| vivo | Agree with Proposal 1. For the question of Ericsson, we suggest that the existing FR2 cases shall be replaced by FR2-2. Then we can define the new dedicated test cases for FR2-2. |
| CATT | Proposal 1 is ok for us. It is suggested to modify ‘FR2’ to ‘FR2-1’ and add new dedicated test cases for FR2-2. If the existing test case is applicable to both FR2-1 and FR2-2, it suggested to use a statement to indicate that the test case in FR2-1 is also apply to FR2-2. |

### Sub-topic 1-2: Test configurations

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: Test configurations

- Proposal 1-1 (Huawei): Following SCS/BW configurations to be used in test cases for FR2-2

  - 120 kHz, 100 MHz bandwidth

  - 480 kHz, 400 MHz bandwidth

  - 960 kHz, 400 MHz bandwidth

- Proposal 1-2 (Ericsson): Following SCS/BW configurations to be used in test cases for FR2-2

  - 120 kHz, 400 MHz bandwidth

  - 480 kHz, 400 MHz bandwidth

  - 960 kHz, 400 MHz bandwidth

- Proposal 2 (vivo): The test configurations for intra-frequency measurement for FR2-2 should be defined as follows:

| Configuration | Description |
| --- | --- |
| 1 | 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| 2 | 960 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

- Proposal 3 (vivo): The test configurations for inter-frequency measurement for FR2-2+FR2-2 should be defined as follows:

| Configuration | Description |
| --- | --- |
| 1 | 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |

- Proposal 4 (vivo): The test configurations for inter-frequency measurement for FR1+FR2-2 should be defined as follows (SA event triggered reporting tests without SSB index reading):

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 480 kHz SSB SCS, 400 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

- Proposal 5 (vivo): The test configurations for UE transmit timing for FR2-2 should be defined as follows

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 960 kHz, data SCS 960 kHz, BW 400 MHz |

- Recommended WF

  - Discuss the proposals individually

  - Discuss whether UE is required to be tested on only one of the supported test configurations (wherever applicable)

| Company | Comments |
| --- | --- |
| Ericsson | On Proposal 1, If 120KHz shall be verified in FR2-2, we suggest keep 400MHz as common BW for all SCSs. We agree on Proposal 4. |
| Huawei | We think both test cases shall cover 120/480/960 kHz not only 480/960 kHz. Besides, as proposed in our paper, 480/960kHz are optionally supported by UE, and if UE is only required to be tested in one of configurations (120/480/960), it means 480/960 may never be tested. |
| Nokia | Proposal 1-1: given that 100 MHz is used in 120 kHz test cases in FR2-1 we agree to this proposal. Proposal 2, Proposal 3 and Proposal 4: Do not agree. Given that the RX beam sweeping factor was changed in the last meeting for FR2-2, we support that the test cases are also defined to 120 kHz SCS.Proposal 5: Do not agree. 960 kHz subcarrier spacing is optional. We need the timing test case to cover all the SCSs, especially considering that the requirements are different. |
| Qualcomm | Support proposal 1-1: We prefer to keep 100MHz BW for 120kHz SCS. Since this configuration is already tested for FR2-1, we only need to consider additional configurations for optional SCSs in FR2-2 where a UE may support either one or both of them. We can define configurations for both of optional SCS, but UE may be required to test only in one configuration. So we are fine with proposal 2, 3 and 4. We cannot agree with proposal 5. A UE may support 480kHz SCS but not 960kHz SCS. In that case, UE should be tested for 480kHz SCS and not for 960kHz SCS |
| Apple | Proposal 1-1 is OK. Note there will be RF tests for 400MHz CBW. |
| vivo | We agree with Proposal 1-1. For Proposal 2,3,4,5, our initial motivation is that for FR2-1, the UE is only required to be tested in one of the supported test configurations. Our initial motivation is that in order to reduce the number of test cases, FR2-2 can follow the same rule with FR2-1. The UE is only required to pass the test cases where the requirement is the strictest. However, we also agree with other companies that for FR2-2, 480kHz and 960kHz is optionally supported for UE. So It is necessary to define all test cases for different types of UE. |
| CATT | We have the same view with Nokia, we support proposal 1-1 and don’t agree with proposal 2, proposal 3, proposal 4 and proposal 5. |

### Sub-topic 1-3: Test cases

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: Test cases for intra-frequency measurement

- Proposal 1 (vivo): The following two test cases for intra-frequency measurement for FR2-2 should be added compared with FR2-1:

  - SA event triggered reporting test with SSB time index detection without gap under non-DRX

  - SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | Agree on Proposal 1. |
| Nokia | We support the proposal. SSB time index was defined for FR2-2 so it should be tested. Can we update the proposal to clarify that this test is also needed in CCA? |
| Qualcomm | Fine with proposal 1 |
| vivo | Support Proposal 1. For FR2-1, deriveSSB-IndexFromCell is always enabled, i.e., neighbour cell and target cell has the same SSB index. Therefore, when defining the test cases, there is no need to consider the SSB time index detection. However, for FR2-2, when the SCS is 960kHz, deriveSSB-IndexFromCell may be not enabled. In this case, it is necessary to consider whether the SSB time index detection is needed. So we suggest to add the above two test cases for intra-frequency measurement for FR2-2. |

Issue 1-3-2: Test cases without CCA

- Proposal 1 (Nokia): Update the test case list agreed in RAN4 #103, considering that all test cases are also defined for 120 kHz SCS in FR2-2 with beam scaling sweeping factor of 12

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| MTK | Agree with Proposal 1. |
| Ericsson | Agree on Proposal 1. |
| Huawei | Agree with proposal 1. |
| Qualcomm | Fine with proposal 1. |
| Apple | Proposal 1 is OK. |
| vivo | Agree with Proposal 1. |
| CATT | OK with proposal 1. |

### Sub-topic 1-4: RRM measurement performance

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4-1: RSSI measurement accuracy requirements

- Proposal 1 (Nokia): RAN4 to define accuracy requirements for RSSI measurements with 120 kHz, 480 kHz and 960 kHz.

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| Ericsson | Agree on Proposal 1 |
| Qualcomm | Fine with proposal 1, but UE may be tested only for one configuration |

### Sub-topic 1-5: CCA aspects

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-5-1: Exceeding Lmax in test cases

- Proposal 1 (Huawei): Exceeding Lmax shall be avoided in all test cases

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| MTK | Fine with Proposal 1 in general. |
| Ericsson | We prefer still keeping Lmax in scope. |
| Huawei | If the behavior of exceeding Lmax is to be verified in the test cases. TE has to guarantee that none of SSB is available for Lmax groups. It means we may have to consider different models for each test cases. |
| Nokia | We agree with the proposal. Similar approach as used in NRU can be applied to FR2-2. |
| Qualcomm | We are fine with proposal 1 |
| vivo | We are fine with proposal 1 |

Issue 1-5-2: UL LBT model

- Proposal 1 (Nokia): The same UL LBT model from NR-U in FR-1 can be applied to FR2-2 with CCA

- Recommended WF

  - Discuss the proposal

| Company | Comments |
| --- | --- |
| MTK | Fine with Proposal 1 in general. |
| Ericsson | Generally OK with Proposal 1. |
| Huawei | Fine with proposal 1. |
| Qualcomm | Okay with the proposal |
| vivo | We are OK with Proposal 1. |
| CATT | Ok with Proposal 1. |

Issue 1-5-3: Modelling unavailable SSB/SMTC

- Proposal 1a (CATT): When considering how to model an unavailable SSB/SMTC group in the test cases for NR extension to 71 GHz, the first concern should be whether to verify the beam sweeping behaviour of the UE

- Proposal 1b (CATT): If beam sweeping behaviour of the UE is to be verified, an unavailable SSB/SMTC group can be modelled as that there is one SSB index directing towards the UE not transmitted by TE in the available SSB/SMTC occasion when UE Rx beam direct towards the TE.

- Proposal 1c (CATT): For test cases where no beam sweeping behaviour of the UE to be verified, an unavailable SSB/SMTC group can be modelled as none of SSB index directing towards the UE is transmitted by TE in N consecutive SSB/SMTC occasions

- Proposal 2 (Nokia, Huawei, Ericsson): For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions

- Proposal 2a (Ericsson): Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index

- Recommended WF

  - Discuss the proposals

| Company | Comments |
| --- | --- |
| Ericsson | We support Proposal 2, but we suggest the ‘one SSB’ can be shifted per N consecutive SSB/SMTC occasions as Proposal 2a. |
| Huawei | Support option 2 We support proposal 2. For proposal 1, the beam sweeping is up to UE implementation and TE cannot determine which SSB is the one that directing towards UE. More typically, there will be only one AoA, and TE will only transmit 1/2 SSB in each burst, which seems not the targeting scenarios for proposal 1.The problem is to decide how to define the test requirements (how long the delay should be in each test.) For option 2a, we have similar suggestions that the unavailable SSB index in each group can be randomly chosen. |
| Nokia | We agree to proposal 2 and the proposal 2a. |
| Qualcomm | We don’t think UE beam sweeping needs to be tested as it’s implementation dependent. We are fine with proposal 2. |
| Apple | We are OK with option 2. |
| vivo | We prefer Proposal 2. It seems that the scenario mentioned in Proposal 2 may be more accorded with the real deployment. |
| CATT | We can compromise to Proposal 2 and agree that UE beam sweeping is implementation dependent. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Test case structure GTW agreements: - For operation in FR2-2 bands without CCA, create dedicated test cases for FR2-2 in existing sections (A.7) - FFS on how to modify the existing test cases for FR2-1. Candidate options: - Option 1: Add generic statement that test cases apply to FR2-1 only with explicit statement in each test case - Option 2: Replace “FR2” in existing FR2 test cases by FR2-1 Recommendations for 2nd round: Discuss how to modify the test cases for FR2-1. |
| Sub-topic #1-2 | Issue 1-2-1: Test configurations Tentative agreements: - Following SCS/BW configurations to be used in test cases for FR2-2 - FFS: - Option 1: 120 kHz, 100 MHz bandwidth, - Option 2: 120 kHz, 400 MHz bandwidth, - 480 kHz, 400 MHz bandwidth - 960 kHz, 400 MHz bandwidth Candidate options: - Option 1: 120 kHz, 100 MHz bandwidth - Option 2: 120 kHz, 400 MHz bandwidth Recommendations for 2nd round: Further discuss the options for 120kHz SCS |
| Sub-topic #1-3 | Issue 1-3-1: Test cases for intra-frequency measurement Tentative agreements: - The following two test cases for intra-frequency measurement for FR2-2 should be added compared with FR2-1: - SA event triggered reporting test with SSB time index detection without gap under non-DRX - SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX Candidate options: Recommendations for 2nd round: No further discussion needed Issue 1-3-2: Test cases without CCA Tentative agreements: - Update the test case list agreed in RAN4 #103, considering that all test cases are also defined for 120 kHz SCS in FR2-2 with beam scaling sweeping factor of 12 Candidate options: Recommendations for 2nd round: No further discussion needed |
| Sub-topic #1-4 | Issue 1-4-1: RSSI measurement accuracy requirements GTW agreements: - RAN4 to define accuracy requirements for RSSI measurements with 120 kHz, 480 kHz and 960 kHz - FFS whether UE should pass one test or all the tests Candidate options: Recommendations for 2nd round: Further discuss whether UE should pass one test or all the tests |
| Sub-topic #1-5 | Issue 1-5-1: Exceeding Lmax in test cases Tentative agreements: - Exceeding Lmax shall be avoided in all test cases Candidate options: Recommendations for 2nd round: Consider agreeing to the tentative agreement Issue 1-5-2: UL LBT model Tentative agreements: - The same UL LBT model from NR-U in FR-1 can be applied to FR2-2 with CCA Candidate options: Recommendations for 2nd round: No further discussion needed Issue 1-5-3: Modelling unavailable SSB/SMTC GTW agreements: - For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions - FFS: Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index Candidate options: - Option 1: Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index - Option 2: No need to shift the SSB index - Option 3: FFS Recommendations for 2nd round: Continue discussion on the candidate options |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: Draft CRs for test cases

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212962 | Huawei | Draft CR on test cases of HO for FR2-2 |
| R4-2212963 | Huawei | Draft CR on test cases of timing requirements for FR2-2 |
| R4-2212964 | Huawei | Draft CR on test cases of BWP switch for FR2-2 |
| R4-2213027 | Vivo | Draft CR on introduction of SSB configurations for FR2-2 |
| R4-2213028 | Vivo | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 |
| R4-2213347 | Ericsson | Draft CR on Test Cases for cell re-selection for extending NR operation to 71GHz |
| R4-2213348 | Ericsson | Draft CR on Test Cases for SCell activation to 71GHz |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212962 Draft CR on test cases of HO for FR2-2 | Nokia: This CR depends on open issues. We believe that there is an error in the configuration: Table A.7.3.1.X1.2-1: Intra-frequency handover from FR2-2 to FR2-2 test configurations Configuration; Description. 1; NR TDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz. 2; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. 3; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. Configurations 2 and 3 are the same. We believe that it should be 960 kHz in configuration 3. This is replicated in other tests in the draft CR. |
|  | Company B |
| R4-2212963 Draft CR on test cases of timing requirements for FR2-2 | Nokia: this CR depends on open issues. We believe there is an error on the configuration: Table A.7.4.1.x1.1-1: Supported test configurations for FR2 PCell Configuration; Description. 1; NR TDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz. 2; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. 3; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. Configurations 2 and 3 are the same. We believe that it should be 960 kHz in configuration 3. This is replicated in other tests in the draft CR. |
|  | Company B |
| R4-2212964 Draft CR on test cases of BWP switch for FR2-2 | Nokia: Test case doesn’t include 960 kHz SCS configuration. Table A.7.5.6.1.X1.1-1: DL BWP switch supported test configurations Configuration; Description. 1; NR TDD, SSB SCS 120 kHz, data SCS 120 kHz, BW 100 MHz. 2; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. 3; NR TDD, SSB SCS 480 kHz, data SCS 480 kHz, BW 400 MHz. Configurations 2 and 3 are the same. We believe that it should be 960 kHz in configuration 3. This is replicated in other tests in the draft CR |
| R4-2213027 Draft CR on introduction of SSB configurations for FR2-2 |  |
| R4-2213028 Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | Nokia: Configurations do not include 120 kHz SCS configuration. Since we changed the beam sweeping scaling factor for FR2-2, 120 kHz SCS needs to be tested as well. |
| R4-2213347 Draft CR on Test Cases for cell re-selection for for extending NR operation to 71GHz | Nokia: Configuration for 120 kHz is using 400 MHz, agreement on the CR should depend on the discussion on test configurations. |
| R4-2213348 Draft CR on Test Cases for SCell activation to 71GHz | Nokia: 400 MHz should be used for 480/960 kHz SCS. Table A.14.X.3.1.1-1: Supported test configurations for FR2-2 SCell activation case Configuration; Description. 1; NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode. 2; NR 480 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode. 3; NR 960 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Topic #3: Test cases & Work Split

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions

### Sub-topic 3-1: Test cases for RRM performance requirements without CCA

Sub-topic description: The table below lists the initial set of test cases. Companies are encouraged to volunteer for test cases and provide comments, if any, in the box below the table.

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS | Company |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  |  | inter-frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | yes | Ericsson |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Huawei |
|  |  | inter-frequency, unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Huawei |
|  | FR1->FR2-2 | unknown target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Huawei |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |  |
|  |  | inter frequency | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  |  | intra frequency without serving cell timing | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes | Nokia |
|  |  | Non-contention based | Reuse FR2-1 | Yes | Nokia |
| RRC Connection Release with Redirection | FR2-2->FR2-2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |  |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes | Huawei |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes | Huawei |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |  |
|  |  | One FR2-2 cell | Reuse FR2-1 | Low priority |  |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes | Huawei |
|  | SCell dormancy switch | FR2-2->FR2-2 | Reuse FR2-1 | Low priority | Huawei |
|  |  | FR1->FR2-2 | Reuse FR2-1 | Low priority |  |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes | Huawei |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Ericsson |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Ericsson |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Ericsson |
|  |  | In-sync, in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  |  | Out-of-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  |  | In-sync, in DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Ericsson |
|  | Scheduling restrictions |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Nokia |
|  |  | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Nokia |
|  |  | CSI-RS-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority |  |
|  |  | CSI-RS -based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
|  |  | inter-band in non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | Ericsson |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | Ericsson |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | No |  |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240 kHz SSB, so new test is needed for 120 kHz SCS | Yes |  |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes | Nokia |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority | Nokia |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Reuse FR2-1 | Yes |  |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | vivo |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Low priority | vivo |
|  |  | under DRX | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority | vivo |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority | vivo |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority | vivo |
|  |  | PCell in FR2-2, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
|  |  | PCell in FR1, DRX is not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes | vivo |
|  |  | PCell in FR1, DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority | vivo |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |  |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  | CSI-RS based | DRX not used | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  |  | DRX is used | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |  |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |  |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |  |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  |  | Inter frequency FR2-2 serving and target cell | Can be reused from FR2-1 if RX beam sweeping is not changed | Low Priority |  |
|  | L1-RSRP | SSB based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |
|  |  | CSI-RS based | Can be reused from FR2-1 if RX beam sweeping is not changed | Yes |  |

| Company | Comments |
| --- | --- |
| XXX |  |

### Sub-topic 3-2: Test cases for RRM performance requirements without CCA

Sub-topic description: The table below lists the initial set of test cases. Companies are encouraged to volunteer for test cases and provide comments, if any, in the box below the table.

| Group of requirements | Test cases | Sub-test | Needed / Not needed | Company |
| --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120 kHz and 480 kHz |  |
|  |  | inter-frequency | Yes, at least with 120 kHz and 480 kHz |  |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes |  |
|  |  | inter-frequency, unknown target cell | Low priority |  |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes |  |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | intra frequency | Low priority |  |
|  |  | inter frequency | Yes |  |
|  |  | intra frequency without serving cell timing | Yes |  |
| Random access | 4-step RACH | Contention based | Yes |  |
|  |  | Non-contention based | Yes |  |
| RRC Connection Release with Redirection | FR2-2->FR2-2 (CCA) |  | Low priority |  |
| Timing | UE Transmit Timing |  | Yes |  |
|  | Timing advance adjustment accuracy |  | Yes |  |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2 ->FR2-2 (CCA) | Yes, but this test case can be same as the one defined for operation without CCA |  |
|  |  | FR1->FR2-2 (CCA) | Low priority |  |
|  |  | One FR2-2 (CCA) cell | Low priority |  |
|  | RRC-based active BWP switch |  | Yes, but this test case can be same as the one defined for operation without CCA |  |
|  | UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA | FR2-2 (CCA) | Yes |  |
|  |  | FR1->FR2-2 | Not needed |  |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes |  |
|  |  | In-sync, in non-DRX mode | Low priority |  |
|  |  | Out-of-sync, in DRX mode | Low priority |  |
|  |  | In-sync, in DRX mode | Yes |  |
|  | Scheduling restrictions |  | Yes, but this test case can be same as the one defined for operation without CCA |  |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes |  |
|  |  | SSB-based in non-DRX mode | Low priority |  |
|  |  | CSI-RS-based in non-DRX mode | Low priority |  |
|  |  | CSI-RS -based in non-DRX mode | Yes |  |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes, but this test case can be same as the one defined for operation without CCA |  |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes |  |
|  |  | inter-band in non-DRX | Low priority |  |
|  | FR1+FR2 (CCA) inter-band with target SCell in FR2 |  | Yes |  |
| PSCell addition and release delay | Known PSCell |  | No |  |
|  | Unknown PSCell |  | Yes |  |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes |  |
|  | RRC based | PCell in FR2-2 (CCA) | Low priority |  |
| Interruptions | during measurements on deactivated NR SCC in FR2-2 |  | Yes |  |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes |  |
|  |  | under DRX | Low priority |  |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Low priority |  |
|  |  | under DRX | Yes |  |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes |  |
|  |  | PCell in FR2-2, DRX is used | Low Priority |  |
|  |  | PCell in FR1, DRX is not used | Low Priority |  |
|  |  | PCell in FR1, DRX is used | Yes |  |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Low Priority |  |
|  |  | PCell in FR2-2, DRX is used | Yes |  |
|  |  | PCell in FR1, DRX is not used | Yes |  |
|  |  | PCell in FR1, DRX is used | Low Priority |  |
| L1-RSRP measurement for beam reporting | SSB based | DRX not used | Low Priority |  |
|  |  | DRX is used | Yes |  |
| Accuracy for measurements | SS-RSRP | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR1 serving and FR2-2 target cell | covered by the requirements without CCA |  |
|  | SS-RSRQ | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  | SS-SINR | Intra-frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  |  | Inter frequency FR2-2 serving and target cell | covered by the requirements without CCA |  |
|  | L1-RSRP | SSB based | covered by the requirements without CCA |  |
|  |  | CSI-RS based | covered by the requirements without CCA |  |
|  | RSSI | Intra-frequency | Needed in all SCS |  |
|  |  | Inter-frequency | Needed in all SCS |  |

| Company | Comments |
| --- | --- |
| XXX |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR extension to 71 GHz RRM performance requirements | Qualcomm | WF to capture the agreements and open issues |
|  | Test cases and work split for NR extension to 71 GHz RRM performance requirements | Qualcomm | Captures the test cases and work split |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212962 |  | Draft CR on test cases of HO for FR2-2 | Huawei | Revise |  |
| R4-2212963 |  | Draft CR on test cases of timing requirements for FR2-2 | Huawei | Revise |  |
| R4-2212964 |  | Draft CR on test cases of BWP switch for FR2-2 | Huawei | Revise |  |
| R4-2213027 |  | Draft CR on introduction of SSB configurations for FR2-2 | Vivo | Agreeable |  |
| R4-2213028 |  | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | Vivo | Revise |  |
| R4-2213347 |  | Draft CR on Test Cases for cell re-selection for extending NR operation to 71GHz | Ericsson | Revise |  |
| R4-2213348 |  | Draft CR on Test Cases for SCell activation to 71GHz | Ericsson | Revise |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents