3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214151

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.24.4, 9.24.5

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104-e][231] NB_IOTenh4_LTE_eMTC6_RRM

Document for:	Information

# Introduction

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - Discuss the CR on maintenance of core requirements.

  - Discuss the performance requirements and provide comments on CRs for test cases.

- 2nd round:

  - Finalize CR for core requirements maintenance .

  - Finalize CR for performance requirements and test cases.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Zhongyi Shen | shenzhongyi3@huawei.com |
| Qualcomm, Inc. | Carlos Cabrera Mercader | ccmercad@qti.qualcomm.com |
| Ericsson | Santhan Thangarasa | Santhan.thangarasa@ericsson.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Core requirements mentainance

Only one CR submitted under 9.24.5. Companies please provide comments to CR direction in 1.1.1.

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212965 (Huawei, HiSilicon) | Qualcomm: OK |
|  | Ericsson: OK |
|  | Nokia: CR is agreeable. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

# Topic #2: Performance requirements and test cases

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212966 | Huawei, HiSilicon | Observation 1: Inter-frequency neighbour cell measurement is similar as the RRC Re-establishment procedure where the carrier selection can be up to UE implementation. Observation 2: In inter-frequency RRC Re-establishment test cases, the requirements are verified provided that UE is indicated with the target frequency carrier information (via SIB in RAN5 spec). Proposal 1: Define inter-frequency neighbor measurement test cases, and state in the test cases that the target carrier frequency information is provided to the UE to ensure that UE has the context of the target carrier frequency. Proposal 2: Update test cases list as follows: - TC1:HD-FDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA/in-band/guard-band - TC2: TDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA//in-band/guard-band - TC3:HD-FDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band - TC4:TDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band - UE is only required to be tested in one operation mode. |

## Open issues summary

### Sub-topic 2-1 Test cases for neighbour cell measurement for NB-IoT

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT

- Proposals

  - Option 1: Define inter-frequency neighbour measurement test cases, and state in the test cases that the target carrier frequency information is provided to the UE to ensure that UE has the context of the target carrier frequency.

- Recommended WF

  - Discuss above option.

Comments collection

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT |
| Huawei | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT We support option 1 to adopt similar approach in RRC Re-establishment test cases as follows: ### A.6.1.23 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage #### A.6.1.23.1 Test Purpose and Environment The purpose is to verify that the NB-IoT TDD inter-frequency RRC re-establishment delay is within the specified limits. These tests will verify the requirements for Cat-NB1 UE in clause 6.5. The test parameters are given in table A.6.1.23.1-1 and table A.6.1.23.1-2 below. nCell1 and nCell2 are NB-IoT cells on different frequency carriers. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be indicated with the carrier frequency of nCell 2 to ensure that the UE has the context of the carrier frequency of nCell 2.. We think the situation is same as RRC Re-establishment where NW cannot configure on which carrier UE performs RRC Re-establishment. But the test case is to verify that UE perform the procedure on the targeting carrier first. In RAN5, it is reflected as the carrier information is provided in SIB5. |
| Qualcomm | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT We would support option 1 if the test cases are defined with serving anchor carrier. |
| Huawei2 | To QC: We can further clarified in the test that the serving carrier is anchor carrier. |
| Ericsson | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT The proposed way of defining the test case by Huawei (option 1) looks reasonable and acceptable. |
| Nokia | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT We are fine with option 1 and the limitation to anchor carrier. |

Issue 2-2: Test cases list

- Proposals

  - Option 1:

  - TC1:HD-FDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA/in-band/guard-band

  - TC2: TDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA//in-band/guard-band

  - TC3:HD-FDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band

  - TC4:TDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band

  - UE is only required to be tested in one operation mode.

- Recommended WF

  - TC1 and TC2 are agreed in RAN4#103e. Companies please provide comments on whether to define TC3 and TC4 based on issue 2-1.

Comments collection

| Company | Comments |
| --- | --- |
| Huawei | Issue 2-2: Test cases list Agree on TC list in option 1. |
| Qualcomm | We would support adding TC3 and TC4 if they defined with serving anchor carrier. |
| Ericsson | Issue 2-2: Test cases list Option 1 is fine. |
| Nokia | Issue 2-2: Test cases list We are fine with option 1. Please clarify in option 1 for the operation mode that one is selected out of SA, in-band, guard-band. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212967 DraftCR on test cases for HD-FDD intra-frequency neighbour cell measurement of NB-IoT R17 | Qualcomm: - Editorial: Fix time units in Table A.8.1.x1.1-1. Ms -> ms. - The assumption is that the UE will start measuring neighbor cells during T3, correct? Is that guaranteed by the test configuration? We didn’t see details of the measurement triggering configuration in the CR. Maybe we missed it? - Table A.8.1.x1.1-2, nCell1 Es/Noc = Es/Iot during T3 (nCell2 is active)? nCell1 power shouldn’t change between T2 and T3, correct? - Do we need three sub-tests? Is there an applicability rule? |
|  | Huawei2: Thanks for QC’s comments: - The typo will be corrected in revised version. - The assumption is that UE will start measurement beween T2 –T3 and shall finish cell measurement before end of T3. The NRSRP of serving cell is configured as -101 dBm which will fulfill the threshold for intra-f measurement (s-MeasureIntra) considering the measurement accuracy. T2 is the time for UE serving cell measurement that UE shall identify that s-MeasureIntra is fullfiled. T3 is the time for neighbor cell measurement. Yes, the configurations of s-MeasureIntra is missing. It will be added in revised version. - Yes, nCell 1 Es/Noc during T3 shall be corrected as -3 dB. The NRSRP level is same between T2 and T3, but the SINR is different as nCell2 is activated. - According to the agreement in last meeting, UE only needs to be tested in one mode. We can add the applicability rule in the revised version. |
|  | Nokia: Editorial change in 3 clauses: “The purpose is to verify that the NB-IoT intra-frequency neighbour cell measurement requirement in clause 8.14.6.3.” => “… is met.” We agree to Qualcomm’s comment related to the applicability rule. On which basis the operation mode is to be selected for testing, e.g. if several operation modes are supported? |
| R4-2212968 DraftCR on test cases for TDD intra-frequency neighbour cell measurement of NB-IoT R17 | Qualcomm: Similar comments as for R4-2212967 |
|  | Nokia: Same comments as for R4-2212967. |
| R4-2212969 DraftCR on test cases for HD-FDD inter-frequency neighbour cell measurement of NB-IoT R17 | Qualcomm: - Pending agreement on issue 2-1. - The assumption is that the UE will start measuring neighbor cells during T3, correct? Is that guaranteed by the test configuration? We didn’t see details of the measurement triggering configuration in the CR. Maybe we missed it? - For inter-freq test cases, will the UE perform both intra-freq and inter-freq neighbor cell measurements? Is T3 long enough? - Why are the SINR levels different in the sub-tests? - Do we need three sub-tests? Is there an applicability rule? |
|  | Huawei: - We can clarify in the test that the serving carrier is anchor carrier. - Yes, the configurations of s-MeasureInter is missing. It will be added in revised version. - After double checking inter-f RRC Re-establishment test cases, we agree you that intra-f and inter-f measurement time should be considered in T3. - The value in A.8.1.x2/x3 are not correct which needs to be corrected. - According to the agreement in last meeting, UE only needs to be tested in one mode. We can add the applicability rule in the revised version. |
|  | Nokia: reference to section 8.14.6.3 is wrong, should be 8.14.6.4. We agree with Qualcomm to clarify the application rule. |
| R4-2212970 DraftCR on test cases for TDD inter-frequency neighbour cell measurement of NB-IoT R17 | Qualcomm: Similar comments as for R4-2212969 |
|  | Nokia: same comments as for R4-2212969. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT 4 companies commented in first round. Option 1 is agreeable to all in general. 2 companies commented to limit the serving cell on anchor carrier. Tentative agreements: Define inter-frequency neighbour measurement test cases with the serving cell in anchor carrier, and state in the test cases that the target carrier frequency information is provided to the UE to ensure that UE has the context of the target carrier frequency. Candidate options: NA Recommendations for 2nd round: Check whether tentative agreement. |
|  | Issue 2-2: Test cases list 4 companies commented in 1st round. Option 1 is agreeable to all. 1 company commented to clarify that the operation mode is selected out of SA, in-band, guard-band. Tentative agreements: Update test cases list as: - TC1:HD-FDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA/in-band/guard-band - TC2: TDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA//in-band/guard-band - TC3:HD-FDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band - TC4:TDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band UE is only required to be tested in one operation mode out of SA, in-band, guard-band. Candidate options: NA Recommendations for 2nd round: Check whether tentative agreement |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on RRM requirements for NB_IOTenh4_LTE_eMTC6 | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212965 |  | CR on maintenance of neighbour cell measurement for NB-IoT | Huawei, HiSilicon | Agreeable |  |
| R4-2213565 |  | Big CR on performance requirements for NB_IOTenh4_LTE_eMTC6 | Huawei, HiSilicon | Return to |  |
| R4-2212967 |  | DraftCR on test cases for HD-FDD intra-frequency neighbour cell measurement of NB-IoT R17 | Huawei, HiSilicon | Revised | Capture comments in 1st round |
| R4-2212968 |  | DraftCR on test cases for TDD intra-frequency neighbour cell measurement of NB-IoT R17 | Huawei, HiSilicon | Revised | Capture comments in 1st round |
| R4-2212969 |  | DraftCR on test cases for HD-FDD inter-frequency neighbour cell measurement of NB-IoT R17 | Huawei, HiSilicon | Revised | Capture comments in 1st round |
| R4-2212970 |  | DraftCR on test cases for TDD inter-frequency neighbour cell measurement of NB-IoT R17 | Huawei, HiSilicon | Revised | Capture comments in 1st round |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents