3GPP TSG-RAN WG4 Meeting # 104-e												         R4-2214241

Electronic Meeting, 15– 26 August 2022

Agenda item:			11.2.4

Source:	Moderator (CATT)

Title:	Email discussion summary for [104-e] [130] FS_NR_700800900

Document for:	Information

Introduction

This email discussion is to discuss enhancement for 700800900MHz band combinations. RAN4#104e meeting is the 1st meeting to discuss this WI.

The targets of the two rounds are as following,

- 1st round:

  - Discuss the issues listed in the summary and try to have some tentative agreements.

- 2nd round:

  - Approve the WF.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| OPPO | Jinqiang Xing | xingjinqiang@oppo.com |
| Skyworks | Dominique Brunel | dominique.brunel@skyworksinc.com |
| Murata | Pushp Trikha | ptrikha@psemi.com |
| vivo | Shuai Zhou | Shuai.zhou@vivo.com |
| Qualcomm | Antti Immonen | aimmonen@qti.qualcomm.com |
| Xiaomi | Yuan Gao | Gaoyuan23@xiaomi.com |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| Apple | James Wang | fucheng_wang@apple.com |
| China Unicom | Basaier Jialade | basejld@chinaunicom.cn |
| Huawei | Peng (Henry) Zhang | zhangpeng169@huawei.com |
| Samsung | Yuanyuan(Tina) Zhang | Tina55.zhang@samcung.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: TP skeleton and work plan

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211713 | CATT | Proposal: Approve the following work plan for the SI FS_NR_700800900_combo_enh. RAN4#104-e: - Identify the critical issues for the band combinations. - Discuss the skeleton of the TR. RAN4#104b-e: - Discuss the potential solutions for the identified issues for each band combination. - Approve the TR skeleton. RAN4#105: - Further discuss the solutions. - Approve some TPs. RAN4#106: - Approve the conclusions for each band combination of the SI. - Approve all of the TPs and the first version of TR. |
| R4-2211714 | CATT | A draft TR skeleton is provided in Annex to collect comments. |

Open issues summary and company views collection

Sub-topic 1-1

Issue 1-1: Work plan

- Proposals

  - Proposal in R4-2211713

RAN4#104-e:

- Identify the critical issues for the band combinations.

- Discuss the skeleton of the TR.

RAN4#104b-e:

- Discuss the potential solutions for the identified issues for each band combination.

- Approve the TR skeleton.

RAN4#105:

- Further discuss the solutions.

- Approve some TPs.

RAN4#106:

- Approve the conclusions for each band combination of the SI.

  - Approve all of the TPs and the first version of TR.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |

Sub-topic 1-2

Issue 1-2: TR skeleton

- Proposals

  - The draft TR skeleton R4-2211714

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| XXX |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Issue 1-1: Work plan | No comments in the 1st round. Moderator suggests including it in the WF. No discussion in 2nd round. |
| Issue 1-2: TR skeleton | No comments in the 1st round. Moderator suggests using R4-2211714 as a starting point to draft the TR skeleton in next meeting. |

Discussion on 2nd round (if applicable)

Continue discuss in the WF. No comments in 2nd round.

Topic #2: CA_n5-n8

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211927 | China Unicom | Observation 1: Even though there is partial overlapping between n5 DL and n8 UL, operators may not own the overlapped frequency. Proposal 1: Consider Table 1 as an example frequency combination for investigation and feasibility study of 800/900MHz enhancement (CA_n5-n8). Table 1 Proposed frequency for 800/900MHz feasibility study UL; DL. Frequency 1 (800MHz); 824MHz - 835MHz; 869MHz - 880MHz. Frequency 2 (900MHz); 904MHz - 915MHz; 949MHz - 960MHz. |
| R4-2212015 | Samsung | Observation 1: For CA_n5-n8, analysis (MSD due to IMD and cross band isolation) could be firstly made with the assumption: - Spectrum restriction on n5 and n8 - Reuse the conventional L+L architecture with existing commercial n5/n8 duplexer/ DRx filter for evaluation |
| R4-2212356 | Apple | Observation 4: The main issue for CA_n5-n8 is the frequency overlap between n5 DL and n8 UL inasmuch there would be no feasible filter to isolate n5 DL and n8 UL. Observation 5: Frequency range restriction for CA_n5-n8 to potentially enable filter design to isolate n5 DL and n8 UL would not be very practical as the range reduction could be substantial and the quadplexer cannot be used for single-band full-range operation. Observation 6: For CA_n5-n8, n5 DL REFSEN impact due to n8 UL needs to be addressed. Observation 7: One possibility to enable CA_n5-n8 operation in a smartphone form factor is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL in conjunction with a triplexer implementation. Observation 8: The feasibility of a single triplexer for CA_n5-n8 needs to be studied. Observation 9: The impact on n5 out-of-band blocking requirement due to extended filter bandwidth in triplexer implementation for CA_n5-n8 needs to be addressed. |
| R4-2212609 | Xiaomi | Observation 4: Either 3-antenna or 2-antenna architecture can be considered for CA_n5A-n8A and CA_n5A-n28A. It is expected to have some relaxation on RF requirements for smart phone. Observation 5: The frequency range of n5 and n8 can be restricted to ensure adequate frequency separation. For example, the UL band of n8 can be restricted to 895MHz to 915MHz and the DL band of n5 can be restricted to 869MHz to 889MHz. As a result, both bands have a total of 20MHz frequency span and n8 transmission and n5 reception have 6MHz separation. |
| R4-2212610 | Xiaomi | Proposal 3: For CA_n5A-n8A with two UL transmission, it is observed that IMD3 fall into band n5 reception even with spectrum restriction. The MSD due to IMD and cross band isolation should be evaluated and some relaxation is expected for small phone. |
| R4-2212698 | China Telecom | Observation: CA Configuration; Uplink configuration; Observation. Case 1; CA_n5-n8; n5; Focus on the filter ability to separate the DL n5 and n8 when these two bands are on simultaneous reception. Case 2; CA_n5-n8; n8; From our side, the restriction of (869-880MHz) for Band n5 DL and (904-915MHz) for Band n8 UL could be a starting point.. Case 3; CA_n5-n8; CA_n5-n8; This is the bottleneck case that needs to be considered jointly with fall back case.. Proposal 1: When discussing the filter applicability and frequency restriction, UL_CA_n5-n8 is the bottleneck case that needs to be considered jointly with fall back cases. Proposal 2: The restriction of (869-880MHz) for Band n5 DL and (904-915MHz) for Band n8 UL could be a starting point. |
| R4-2212713 | ZTE Corporation | Proposal 2. 2DL/2UL CA_n5-n28, CA_n8-n20, CA_n8-n28 could be treated as normal LB-LB NR CA combination, where non-block approval approach is used in Rel-17. Proposal 3. To support 2DL/2UL n5-n8 NR CA, frequency restriction should be studied to avoid the overlapping. |
| R4-2212798 | vivo | Proposal 1: Clarify the frequency assumptions for CA_n5-n8 before decide the RF front-end for this band combination. |
| R4-2212799 | vivo | Proposal 1: The following RF requirements framework can still apply for 700/800/900 MHz NR inter-band CA: - Operating bands for CA - Channel bandwidths per operating band for CA - Co-existence studies - ∆TIB and ∆RIB values - REFSENS requirements |
| R4-2213160 | Huawei, HiSilicon | Observation 1: it’s very important to consider the following metrics -6dB impedance bandwidth, antenna gain, radiation efficiency and ground clearance for smartphone antenna. Proposal 1: it’s feasible to implement a smartphone antenna which can cover low band frequency range 703~960MHz. Proposal 2: it’s feasible to implement multiple (more than two) low-band antennas for smartphone. |
| R4-2213162 | Huawei, HiSilicon | Observation 1: it’s feasible to implement the fallback mode DL_n5-n8_UL_n5 by reusing the existing UE RF components. Proposal 1: the performance values of RF front end components can be assumed to further evaluate Tib, Rib and whether MSD should be specified for DL_n5-n8_UL_n5 as below. Attenuation and isolation parameter; Value. (Band n5 Tx to Ant) Attenuation at DL band n8 frequency range; 40dB. (Band n8 filter Ant to Rx)Attenuation at UL band n5 frequency range; 40dB. Antenna ISO; 10dB. RF front end loss; 4dB. Observation 2: the spectrum allocation of band n5 and band n8 in a certain region or country isn’t overlapping between UL and DL frequency range. |
| R4-2213195 | Qualcomm Finland RFFE Oy | Observation 1: The assumption in all studied is 2RX for each of the bands Observation 2: There are several LB-LB-(LB) CA RF FE architecture alternatives to be analyzed, whose feasibility then depends a lot on the LB antenna interface configuration Observation 3: The feasible number of LB antennas in smartphone must be investigated thoroughly, and special attention must be paid on feasible number of UL capable LB antennas Observation 4: Method of managing intermodulation products could be related to overall RF FE architecture and/or to more thoroughly considered RF parameters resulting in smaller MSD Observation 6: CA_n5-n8 has an overlap between n8 UL and n5 DL which is a very specific case. Different alternatives on how to address the overlap should be discussed during the study, recognizing it impacts to all aspects in this combination |
| R4-2213314 | OPPO | Observation 1: n8 Tx will interfere n5 Rx due to overlapping frequency, spectrum isolation is necessary. Observation 2: Current proponent operators doesn’t hold the whole spectrum of band n5 and n8. Observation 3: With restriction on spectrum in n5(825 - 835 UL and 870 - 880 DL) and n8 (905 - 915 UL and 950 - 960 DL), the IMD interference can be avoided. Proposal 1: Take n5 (825 - 835 UL and 870 - 880 DL) and n8 (905 - 915 UL and 950 - 960 DL) as starting point for the band combination of n5+n8 with referring to the operator’s spectrum status. Observation 4: It is quite challenging to use one antenna to cover the n5 and n8 whole spectrum, and separate antennas for n5 and n8 are needed. Proposal 2: Use separate antennas for n5 and n8 in n5+n8 CA as reference architecture. |
| R4-2213754 | Skyworks Solutions Inc. | Proposal for CA_n5-n8: - Only the China restricted spectrum is studied: band n5 DL 825-835MHz and band 8 UL 890-915MHz - Maximum UL and DL CBW per band must be defined - Use of standard n5 and n8 duplexer is unlikely and even with additional antennas the tight coupling between LB antennas will not isolate the filters sufficiently: the study should include dedicated filter design with all its limitations for wide support Proposal on PA/Antenna Architecture: - Baseline architecture should be agreed for 2nd PA being on Main, diversity or third antenna for all cases and a generic approach is targeted for two LB on one side and three LB on the other side - Coupling between the two or three antennas should be agreed for the 700MHz to 960MHz range in a smartphone context to assess the different cases |

Open issues summary and company views collection

Sub-topic 2-1: Spectrum restriction asssumption

Issue 2-1: Spectrum restriction assumption for the analysis

- Proposals

  - Option 1: Consider Table 1 as an example frequency combination for investigation and feasibility study of 800/900MHz enhancement (CA_n5-n8) (China Unicom, R4-2211927).

Table 1 Proposed frequency for 800/900MHz feasibility study

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

  - Option 2: The restriction of (869-880MHz) for Band n5 DL and (904-915MHz) for Band n8 UL could be a starting point. (China Telecom, R4-2212698)

  - Option 3: The frequency range of n5 and n8 can be restricted to ensure adequate frequency separation. For example, the UL band of n8 can be restricted to 895MHz to 915MHz and the DL band of n5 can be restricted to 869MHz to 889MHz. As a result, both bands have a total of 20MHz frequency span and n8 transmission and n5 reception have 6MHz separation. (Xiaomi, R4-2212609)

  - Option 4: Take n5 (825 - 835 UL and 870 - 880 DL) and n8 (905 - 915 UL and 950 - 960 DL)  (R4-2213314	OPPO)

  - Option 5: Only the China restricted spectrum is studied: band n5 DL 825-835MHz and band 8 UL 890-915MHz. (R4-2213754	, Skyworks Solutions Inc.)

- Recommended WF

  - Option 1

| Company | Comments |
| --- | --- |
| OPPO | Option 1/2/4 are similar. If use 869-880 for n5DL and 904-915 for n8UL, the total spectrum is 16MHz, is there intention to use 11MHz irregular CBW? Or maybe we can round to 10MHz which is a regular CBW defined in RAN4. |
| Skyworks | In our understanding there is other band 8 spectrum use in Chine by other operators, RAN4 is not supposed to define operator specific solutions. Can the entire spectrum used in China in band 8 be further clarified? |
| Murata | Option 1: In the feasibility study, when the frequency range should restricted for 38.101-1 requirement. For this restriction, there is no IMD3 product landing in band 8. |
| China Telecom | Prefer option1/2. Option 1 and 2 proposed by China Unicom and China Telecom are pointing to the same frequency range. For option 4, we are ok to define the MSD based on maximum CBW as 10MHz as mentioned by option 4 but the requirements shall apply under 11MHz band frequency range. For option 3/5, there is another 15MHz for Band n8 in China, but by now, we don’t see the possibility that the entire Band n8 will be assigned to single operator in China. |
| ZTE | Option 1 and 2 are the same and we are fine. Also, CA BCS should be defined where max. CBW is included. |
| Vivo | After we agreed the spectrum restriction, how do we consider the RF components for band n5 and n8 when considering the RFFE? - Full Range n5/n8 components （Duplex， filter）for partial frequency usage Partial Range n5/n8 components |
| Qualcomm | Spectrum restrictions for this China-specific are likely done. Frequency range restriction does not mean that specific filters were automatically needed. Even if restrictions are agreed we still need to discuss what does that mean in practice; Are restriction-specific RF filters needed (what if yes), or are time domain restrictions (band n8 TX/band n5 RX) feasible, etc. Two questions on the proposed frequency ranges. We understand the proposed frequencies are according to proponent operators spectrum holding in proposal 1 -Are these ranges future-proof, i.e. no changes/extension of ranges is expected? -Do we assume 10MHz NR channel position in both bands is fixed to certain position, or are they assumed to be anywhere within 11MHz according to channel raster? |
| Xiaomi | Option 1/2/4 are OK for us. The frequency restriction can be considered based on operator request but whether maximum 10MHz channel bandwidth is future-proof can be further clarified. |
| Nokia | Whichever option is selected, full range support for each of the single bands (full range) shall be mandatory. Otherwise, UE cannot indicate the respective bands support capability. Hence, new bands and new band combinations with the new bands are needed if so. |
| Apple | In our view, the 3GPP should avoid defining operator specific requirements which would force UE to incorporate an operator specific implementation in order to support the feature. As we commented in our contribution: “Frequency range restriction for CA_n5-n8 to potentially enable filter design to isolate n5 DL and n8 UL would not be very practical as the range reduction could be substantial and the quadplexer cannot be used for single-band full-range operation.” As n5 and n8 are used in many different regions/countries, the UE implementation needs to consider sharing of multiplexer for carrier aggregation with single-band operation to minimize RF front-end complexity and be cost effective. |
| China Unicom | We prefer option 1/2 which points to the same frequency range. |
| Huawei | I think we can start to work with operators’ input about specific frequency restriction. I understand Skyworks’ proposal. Once a new component need to be developed, it’s better to maximum its use to prosper industry and decrease the costs. If one component can cover demands from as many operators as possible, it beneficial the IMT industry. Besides, we welcome operators to show their demands as much as possible, so that RAN4 can conclude the most optimal decision for IMT industry. |
| Skyworks | With the understanding that there is other spectrum used in China for n8 it means that the current restricted spectrum is operator specific. Is this what RAN/RAN4 has agreed to do? At least for us it means that the band 8 filter must support this additional 15MHz. |

Sub-topic 2-2: UE RF analysis assumption

Issue 2-2: UE RF architecture assumption

- Proposals

  - Option 1: Reuse the conventional L+L architecture with existing commercial n5/n8 duplexer/ DRx filter for evaluation (Samsung, R4-2212015)

  - Option 2: The feasibility of a single triplexer for CA_n5-n8 needs to be studied (Apple, R4-2212356)

  - Option 3: Either 3-antenna or 2-antenna architecture can be considered for CA_n5A-n8A and CA_n5A-n28A. It is expected to have some relaxation on RF requirements for smart phone. (Xiaomi, R4-2212609)

  - Option 4: Use separate antennas for n5 and n8 in n5+n8 CA as reference architecture. (R4-2213314, OPPO)

  - Option 5: Use of standard n5 and n8 duplexer is unlikely and even with additional antennas the tight coupling between LB antennas will not isolate the filters sufficiently: the study should include dedicated filter design with all its limitations for wide support. (R4-2213754, Skyworks)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | There are 2 antennas, 3 antennas, and 4 antennas architecture proposed in this meeting as Option 3 and Option 4. Not quite clear how to chose one from them, usually the more separate antennas the better performance but also higher complexity. In our view, the 3 antennas and four antennas probably are ok, but for 2 antennas it means each antenna needs to cover the whole spectrum of n5 and n8 which is too challenging for UE. Therefore, prefer 3 antennas and 4 antennas architecture. |
| Skyworks | As discussed in our contribution we should focus on 2 antennas at least for the two band solutions, for 3 band we have strong reservations in being able to define a pentaplexer solution if Tx/Rx isolations of 50dB should be maintained for all bands. We think that even for 2 antenna case, it needs to be discussed if only one antenna is UL or if 2UL can be on separate antennas. For overlapping bands, unless significant MSD is accepted we do not see that the standard duplexer can provide good performance. Unless relatively good performance is delivered there is little hope that the benefit of using 2UL and 2/3DL will be realized in terms of throughput. |
| Murata | In a realistic readily available case, both bands could be on separate antennas and diversity would require a 5+8 diplexer on the 3rd antenna. Again, this is wasting space. Utilizing 2 antennas with special triplexers would imply some sacrifice in performance on one of the RX ports that maybe acceptable compromise for not wasting space. This could be studied further |
| China Telecom | Prefer 2 antennas or 3 antennas UE architecture. For 2 antennas case, the dedicated filter such as triplexer is intending to be tailored based on the whole frequency assignment in China rather than operators request, as commented by Skyworks in Issue 2-1. That will much more depend on the frequency restriction or request more stringent filter ability. For 3 antennas case, based on the standard components and operators’ request, to study the applicable requirements. We think which is more flexible and feasible to the frequency restriction as we proposed. |
| ZTE | 2 antennas (1 for ULs and 1 for DLs) and 3 antennas (2 for Uls and 1 for DLs) could be andidate. For 2 antenna case, dedicated filter may need to be studied. |
| Vivo | We prefer Option 3 as starting point. For Option 1, we are not sure what the conventional L+L architecture refer to. For 4 antenna proposal, our concern is that normal smartphone type UE may not have plenty room for these extra low band antennas. Does OPPO consider foldable screen type smartphone UE for 4 antennas? |
| Qualcomm | As this is feasibility study, multiple architectural options can be studied. This includes e.g multiplexers, 2 and 3 antenna solutions, normal band 5 and band 8 duplexers, duplexers dedicated for restricted frequency ranges. In our view, companies can provide analysis on architectures they want to be captured in TR. Based on comments, there are several architectures for which proponents can provide analysis. |
| Xiaomi | 2 antenna (1 for Uls and 1 for DLs) or 3 antenna (2 for Uls and 1 for DLs) architecture would be acceptable tradeoff between implementation and performance. We also would like to show concern on 4 antenna for two low band combination due to the limited space of smartphone. Logically more antennas will be needed for three low band combination and it would be possible to use 4 antenna for three low band combination providing that smartphone is capable of accommodating. |
| Nokia | To Apple, triplexer that Apple assumes is full range support of n5 UL, n5 DL+n8 UL(with overlap) and n8 DL, correct? And as filter, the middle frequency range shall accompany the performance to tolerate transmission, right? We mean that required condition to be met as filter is different between Tx and Rx., e.g., Tx needs to have more tolerance against temperature etc. |
| Apple | One possibility to enable CA_n5-n8 operation in a smartphone form factor (2 LB antenna solution) is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL in conjunction with a triplexer implementation. 3 or more LB antenna would be relatively challenging for smartphone form factor. The radiative performance for the 3rd LB antenna could be highly compromised. Since this a study item, we also think it is reasonable to evaluate different options. Certain architecture down-selection may be needed to keep the workload manageable. |
| Huawei | As many Low band + low band combinations have been introduced into the specification, it’s beneficial to discuss the specific implementation in RAN4 and reach some common understanding since we haven’t widely discussed this point. As this is a SI, we can list all the possible implementation and trade off the pros and cons for each other. And in the end we can choose one implementation as baseline to develop the RF requirements in WI phase. |
| Samsung | We are fine to evaluate different options in SI. But we have strong concern on 4Low antennas for smart phone although we understand the intention is to avoid wide bandwidth antenna tuning. The total antenna for 5G smart phone is already up to 10, 3L antennas is quite challenging, let alone 4L. |

Issue 2-3: CBW assumption

- Proposals

  - Maximum UL and DL CBW per band must be defined (R4-2213754, Skyworks Solutions Inc.)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Ok with proposal, and in Issue 2-1 the max 11MHz is proposed there, and max UL/DL CBW could be 10MHz for n5 and n8. |
| Skyworks | As requested the operators need to provide the CBW per band for all the band combinations to understand the valid MSD to be studied. |
| ZTE | Proposal is fine. It relays on the operator’s request and the frequency restriction discussed in issue 2-1. For n5-n8, with the consideration of the operator’s view in issue 2-1, n5: {5,10}MHz, n8{5, 10}MHz could be used as BCS0. |
| Vivo | We are OK with the proposal. |
| Xiaomi | We are OK with the proposal. With the frequency restriction the maximum bandwidth should be 10MHz. |
| Huawei | OK with the proposal. Interested companies’ inputs are welcome. |

Issue 2-4: RF parameters assumption

- Proposals

  - Option 1: the performance values of RF front end components can be assumed to further evaluate Tib, Rib and whether MSD should be specified for DL_n5-n8_UL_n5 as below. (R4-2213160,	Huawei)

| Attenuation and isolation parameter | Value |
| --- | --- |
| (Band n5 Tx to Ant) Attenuation at DL band n8 frequency range | 40dB |
| (Band n8 filter Ant to Rx)Attenuation at UL band n5 frequency range | 40dB |
| Antenna ISO | 10dB |
| RF front end loss | 4dB |

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | - For n5 attenuation @n8 Rx (950-960), the proposed 40dB might be too optimistic, one of the typical n5 duplexer figure can be seen below, and it shows the attenuation is around 30dB. Therefore, our preference is this value is no larger than 30dB. - N8 attenuation at n5Tx is ok with 40dB - Ok with antenna ISO = 10dB - Ok with RFFE IL = 4dB. |
| Skyworks | We are not ready to discuss filter performance yet until we have a clear picture of the spectrum arrangement and RFFE architecture. It will be different if standard or dedicated filters are used and depending on how they are multiplexed at the antenna. Antenna isolation of 10dB is used for band above 1GHz, at LB it needs to be reassessed. The coupling/loading of the filters via the antennas in their overlapped frequency must be considered. |
| ZTE | Similar view with Skyworks. We should discuss the RF architecture based on the spectrum arrangement first. |
| Vivo | We can discuss this later after we agreed on spectrum restriction and RFFE. |
| Qualcomm | More analysis and discussion is needed on this topic. The numbers proposed in option 1 are quite conventional, however we should not restrict the study by agreeing the RF parameter assumptions too early as they may unnecessary limit some RF architecture options. |
| Xiaomi | We can come back to this issue after we have a clear picture of RF front end architecture. This is the first meeting and seems premature to make decision. |
| Apple | Feasibility study should start at the architecture level before we dive into the detailed analysis on certain RF requirement such as MSD. If the intention is to have an estimation on the potential REFSENS degradation to help decide whether the assumed architecture is feasible or not, then we are fine with that. |
| Huawei | We are open to further discuss the filter performance later. |

Sub-topic 2-3: Feasiblity study

Issue 2-5: Feasibility issues need to be analysed

- Proposals

  - Proposal 1: For CA_n5-n8, n5 DL REFSEN impact due to n8 UL needs to be addressed.(Apple, R4-2212356)

  - Proposal 2: The impact on n5 out-of-band blocking requirement due to extended filter bandwidth in triplexer implementation for CA_n5-n8 needs to be addressed. (Apple, R4-2212356)

  - Proposal 3: The MSD due to IMD and cross band isolation should be evaluated for CA_n5A-n8A with two UL transmission. (Xiaomi, R4-2212610)

- Recommended WF

  - The MSD due to IMD and cross band isolation.

  - The impact on n5 out-of-band blocking requirement.

| Company | Comments |
| --- | --- |
| OPPO | The IMD impact need to be decided based on the spectrum restriction in sub-topic 2-1, if go with n5 (825 – 835 UL and 870 – 880 DL) and n8 (905 – 915 UL and 950 – 960 DL) probably no IMD exists. The n5 out of band blocking also depends on the architecture used in the conclusion, can be further discussed after that. |
| Skyworks | Beyond IMDs that will depend on exact spectrum restrictions, there cross band MSD between band 8 UL and band 5 DL must be evaluated based on how these are multiplexed, a quadplexer does not work with the overlapped region. If standard duplexer are used, a third antenna would be needed. |
| ZTE | By using the frequency arrangement (i.e. option 1/2) in issue 2-1, there were no IMD issue. For cross band isolation MSD, it should be studied, but it is not related to 2UL, only for 1UL. |
| Vivo | Shared the same view with OPPO. |
| Qualcomm | These are fine to be studied, in addition any other issue arising from RF architecture discussions can be studied as RF architecture and feasibility go hand in hand |
| Xiaomi | No IMD issue is observed if to use the frequency restriction but the cross band isolation should be evaluated based on UE architecture. |
| Apple | The companies’ comments above are all well taken. As commented above, down-selecting certain potential architectures can help us focus on the feasibility analysis. |
| Huawei | Theses RF requirements can be listed as potential spec impacts. |
| Skyworks | We maintain that for cross band we need to evaluate the cumulated effect of 2UL. On DL can be between the two UL at the worst position |

Sub-topic 2-4: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 2-6: Other observations and proposals

- Proposal in R4-2212799	(vivo): The following RF requirements framework can still apply for 700/800/900 MHz NR inter-band CA:

  - Operating bands for CA

  - Channel bandwidths per operating band for CA

  - Co-existence studies

  - ∆TIB and ∆RIB values

  - REFSENS requirements

| Company | Comments |
| --- | --- |
| ZTE | Anyway, they are the RF requirements framework for inter-band UL CA. BTW, CA MOP for 2UL CA is missing. Also, it may not proper to use REFSEN requirements here, using MSD or REFSEN exceptions would be better. |
| Vivo | These requirements are from the framework for 2UL bands with 2/3 DL bands. We think they still can be applied for low band CA. |
| Apple | The RF requirements framework should be the same as with other FR1 CA combinations. |

- Observation and proposals in R4-2213160	 (Huawei, HiSilicon)

  - Observation 1: it’s very important to consider the following metrics -6dB impedance bandwidth, antenna gain, radiation efficiency and ground clearance for smartphone antenna.

  - Proposal 1: it’s feasible to implement a smartphone antenna which can cover low band frequency range 703~960MHz.

  - Proposal 2: it’s feasible to implement multiple (more than two) low-band antennas for smartphone.

Moderator: The observation is for the whole SI.

| Company | Comments |
| --- | --- |
| OPPO | - For proposal 1: For clarification, when saying it is feasible to implement a smartphone antenna which can cover low band frequency range 703~960MHz, does it mean the antenna can works simultaneously in the whole frequency range or it is applying antenna tuner and only cover part of the antenna at a time? - For proposal 2: Agree. |
| Skyworks | We do not agree that wideband antennas feasibility can be judged based on research papers without considering the entire phone design (size, thickness, number of other antennas…todays smartphones LB antennas use tuners for impedance matching and radiation, this means that they are not wideband, especially when this LB antenna has to support 612-960MHz |
| ZTE | We think size is one of the key factors to limit the implementation of more than 1(or 2) LB antennas. In the other words, if the size is enough (i.e. the smartphone is design as a larger form factor like phablet), then it seems it might feasible anyway. |
| Vivo | Shared the same concern with Skyworks, we may not jump to the conclusion to say that it is feasible to implement one antenna cover 703~960M. |
| Qualcomm | It is very good to get insights from UE vendors using practical designs as examples. Going forward, more input from more vendors would be beneficial in order discuss the feasibility in detail |
| Xiaomi | For proposal 1, in our view it would be very challenging for a low band wideband antenna. For proposal 2, we agree. It is feasible to implement 3 antenna in smartphone based on the existing product. |
| Apple | Thanks to Huawei for providing the valuable research references for LB antenna design and analysis. This can be a good start on the feasibility study from theoretical design perspective. Going forward we need to consider all practical limiting factors in a smartphone design, especially with the complexity of RF front-end to support various band combinations, it would be challenging to maintain good ground clearance for antenna design. As this is just the start of the feasibility study for these band combinations, it is too early to conclude the two proposals in this meeting. |
| Huawei | Proposal 1: As many companies comment, we are open to further discuss it. More inputs from UE vendors about this wideband antenna topic are welcome. These research papers were published from 2011 up to now in IEEE transaction and letter almost more than ten years. I don’t think we can simply conclude these papers can’t be referred by 3GPP. Some research results were published based on the cooperation between UE vendors and research institute. That means there is chance for UE vendors to leverage the results in the real commercial smartphone product considering the system optimization. Not only the real smartphone size, thickness, other antennas for Mid-high band, the dimension of antenna and mobile phone, but also -6dB impedance bandwidth, antenna gain, radiation efficiency, materials of substrate PCB and ground clearance were considered. I think these papers can prove the feasibility of wide band clearly. Proposal 2: can be agreeable. |
| Samsung | Thanks Huawei for providing valuable theoretical search on the feasibility study of multiple antennas. While as many companies mentioned above, practical implementation should also be considered. At least we have strong concern on 4L antennas for smart phone. |

- Proposal in R4-2213754 (Skyworks Solutions Inc.)

Proposal on PA/Antenna Architecture:

  - Baseline architecture should be agreed for 2nd PA being on Main, diversity or third antenna for all cases and a generic approach is targeted for two LB on one side and three LB on the other side

  - Coupling between the two or three antennas should be agreed for the 700MHz to 960MHz range in a smartphone context to assess the different cases

Moderator: The observation is for the whole SI.

| Company | Comments |
| --- | --- |
| OPPO | Generally ok with the proposal, and the architecture need to be agreed first then decide the coupling. |
| Skyworks | We need to agree a complete RFFE architecture that supports a main and diversity path for all bands, but in some cases, it may mean a third antenna or having UL on both antennas…the architecture may be different for the different scenarios but in the end a phone supporting these will still have to support other low bands |
| Murata | Are we saying that 10dB coupling no longer applies between LB-LB for UE. We need clarification why 10dB isolation cannot be applied like in FWA. |
| ZTE | It seems it is not only n5-n8/n5-n28, but also for n8-n20-n28(below), we may need to discuss it separately. For the antenna isolation, does it means traditional 10dB is not longer applicable any more? It seems 10dB is usually still used for some L-L combinations in Rel-16/17. |
| Qualcomm | A clarifying question to Skyworks: On PA architecture, does the proposal mean 2nd PA can be connected to either 2nd or 3rd antenna? On antenna coupling, At some point of time we need to agree the coupling value(s), but before doing that more input from companies is needed. It might also be that a certain coupling value assumption makes certain architecture feasible while different coupling assumption makes an another architecture feasible. |
| Apple | The proposal looks reasonable. |
| Huawei | For the isolation between two low band antennas, we can’t simply to reverse the previous agreement without concrete judgment and results. I tend to agree Skyworks that an analysis with a complete low band RFFE architecture is helpful for this SI and beneficial for all the IMT industry. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Spectrum restriction assumption for the analysis | According to the comments in 1st round, it seems option 1 received wide support. So moderator suggests to double check if it can be agreed. Tentative agreements: The following spectrum restriction can be used as the assumption for further CA_n5-n8 UE RF analysis: UL; DL. Frequency 1 (800MHz); 824MHz - 835MHz; 869MHz - 880MHz. Frequency 2 (900MHz); 904MHz - 915MHz; 949MHz - 960MHz. Candidate options: Recommendations for 2nd round: Double check if the tentative agreements can be agreed. |
| Issue 2-2: UE RF architecture assumption | Based on the comments in 1st round, 2, 3, and 4 antennas are mentioned and supported. 2 and 3 antenna architecture received wide support. So moderator suggest to concentrate 2 and 3 antenna architecture for future study. Tentative agreements: The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8. 2 antenna, 3 antenna Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |
| Issue 2-3: CBW assumption | In the 1st round discussion, companies support the idea that CBW assumption should be fixed. There was proposal from the comments that the following can be the BCS0. n5: {5,10}MHz, n8{5, 10}MHz Moderator suggests double check the above proposal in 2nd round. Tentative agreements: Candidate options: Recommendations for 2nd round: Check if the following can be BCS0: n5: {5,10}MHz, n8{5, 10}MHz |
| Issue 2-4: RF parameters assumption | Most of the companies commented that RF parameters need to wait the conclusion of UE RF architecture and CBW. Moderator suggests continuing the discussion when architecture and CBW are decided. Tentative agreements: Candidate options: Recommendations for 2nd round: No discussion in this meeting. |
| Issue 2-5: Feasibility issues need to be analysed | It seems the recommended WF is ok for the companies. There were comments that if spectrum restriction can be agreed, then IMD may not exist. But at this stage, moderator tend to list all of the possible issues for further check in future meetings. Tentative agreements: The following issues will be analyzed in the feasibility study. - The MSD due to IMD and cross band isolation. - The impact on n5 out-of-band blocking requirement. Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |
| Issue 2-6: Other observations and proposals | For the RF requirement framework, companies have common understanding that the followings apply, - Operating bands for CA - Channel bandwidths per operating band for CA - Co-existence studies - ∆TIB and ∆RIB values - REFSENS requirements (MSD) For For the following two topics, observation and proposals in R4-2213160 (Huawei, HiSilicon) Proposal in R4-2213754 (Skyworks Solutions Inc.) Companies discussed and commented. From moderator perspective, these issues are related to the UE RF architecture, so they can be the background inputs for the UE architecture discussion. It seems no need to conclude anything for these two topics. Tentative agreements: The following RF requirement framework applies to CA_n5-n8, CA_n5-n28 and CA_n8-n20-28. - Operating bands for CA - Channel bandwidths per operating band for CA - Co-existence studies - ∆TIB and ∆RIB values - REFSENS requirements (MSD) Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Continue discuss in the WF, the comments in the WF are copied below.

Spectrum restriction assumption for the analysis

Agreement:

- UE should support the full range of spectrum for single band operations on both n5 and n8

- Use the following frequency ranges for further discussion for spectrum restriction to support uplink CA_n5-n8.

|  | UL | DL |
| --- | --- | --- |
| Frequency 1 (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency 2 (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

Comments from companies:

Apple: Despite the above frequency restriction may be applicable to a specific operator, it should not be used as a baseline for front-end filter implementation feasibility study as the filter design should cover the full band ranges in order to support the single-band operation in different regions.

ZTE: Dedicated filter may need to be studied.

OPPO: Our understanding on the above restriction means that RAN4 only support this frequency ranges in this band combination. And UE need to consider how to implement such band combination. But meanwhile, we share similar view as Apple, the RF component study should be common for future cases.

Samsung: Share similar view with Apple.

China Unicom: As there is frequency overlapping between n5 DL and n8 UL, spectrum restriction would be needed for the study of CA feasibility. This table is proposed as example bands as part of the study.

China Telecom: In our understanding, the feasibility study shall be based on the standard filter which cover the full band ranges, but can be better adapted for the CA_n5-n8 with the frequency restriction, also to see if the requirements are acceptable or additional requirements need to be defined.

Qualcomm: In our view making spectrum restrictions give RAN4 still two alternatives for further studies in upcoming meetings; to assume full n5/n8 RF filters (baseline) or to assume restriction-specific RF filters. This was also captured in SI description notes 1 and 2. The implications of these two alternatives are of course completely different in magnitude.

Huawei: We can start to work based on the frequency restriction as listed above. In addition, we are open to hear other operators’ inputs about the frequency restriction to enlarge the eco-system as soon as possible. To Qualcomm, the baseline implementation has been specified in this SI, but I don’t think the frequency restriction-specific RF filters are excluded.

Skyworks: we are fine to assume that filters should cover the full band but then it implies for a 2 antenna case that there is non simultaneous Tx/Rx for n8UL with n5DL at least for one of the DL path (ie n5DL could operate as 1Rx while transmission in n5 with the related MSD for the proposed range. I suggest that both 2 and 3 antenna architecture are studied, in the case of a third antenna there may still be options for an optimized filter.

Vivo: What is the impact of this frequency restriction on RF architecture? Whether Full Range RF components or Partial Range RF components can be used as baseline?

Nokia: The restriction isn’t clear. At least devices like duplexer etc shall support full frequency ranges if it indicates n5 and n8.

Skyworks: the frequency restriction is fine for the study but since there are other regions than China that have band 5+ 8 or 26+8 or 18/19+8 we must make sure that the study can also provide insight for those cases.

UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n8.

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

Comments from companies:

ZTE: Question for clairfication, does the ‘2 antenna, 3 antenna’ means the total antenna number which includes not only primary Tx/Rx antenna(s), but also diveristy Rx antenna(s)?

OPPO: If only consider this n5+n8 band combination is ok, but if consider UE that supports also other band combinations like n8+n20+n28, then 4 antennas architecture (include main and diversity antennas) should also be considered here. This needs to be clarified.

Xiaomi: It is feasible to study 2 antenna and 3 antenna as a starting point. To ZTE, in our understanding 2 antenna and 3 antenna architecture here include both primary antenna and diversity antenna.

China Telecom: We support to use 2 antenna and 3 antenna as the starting point to study.

Qualcomm: OK to study 2 and 3 antennas. Let’s make it clear that the number of antennas here means total number of LB antennas including diversity.

Huawei: OK to assume 2 and 3 antennas architecture for further analysis for CA_n5-n8.

Skyworks: The number of antennas should be understood as the total number of antennas to support Main UL/DL and diversity DL for all bands. Like for any combination it should be implementable with or without additional combinations. Since we are looking at minimum requirement this should be based on up to 3 antennas. If UE has a 4th antenna it will provide further freedom for the implementation to exceed the minimum requirement but 4 antenna cannot be mandated for CA_n5-n8.

Vivo: 2-antenna, 3antenna is not that clear since RF architecture is still not clear. Maybe we can give an example block diagram for 2-antenna and 3-antenna.

CATT: We have the same understanding with Skyworks. Respond to OPPO comments: we prefer decouple the three band combinations because

CBW assumption

The following is the assumption of BCS0:

n5: {5,10}MHz, n8{5, 10}MHz

Comments from companies:

OPPO: For clarification, there are 11 MHz in n5 and n8, is there restriction on the 10MHz location, for example in the middle?

China Telecom: For now, we are not quite sure the location for 10MHz, may be more general study is needed independent the location within the 11MHz. This can be further discussed.

Huawei: it should be based on operators’ input.

Skyworks: If the the location is not known we suggest that the 10MHZ is placed at the worst position for the study (ie n5DL at the top and n8UL at the bottom).

Vivo: We share the same view with Huawei.

RF parameters assumption

Continue the RF parameters discussion when the UE RF architecture and CBW are agreed.

Feasibility issues need to be analysed

The following issues will be analyzed in the feasibility study.

  - The MSD due to IMD and cross band isolation.

  - The impact on n5 out-of-band blocking requirement.

Comments from companies:

Apple: The impact on n5 out-of-band blocking requirement is based on the assumption of a triplexer implementation where the middle filter range covers both n5 DL and n8 UL.

OPPO: Can be further studied.

Xiaomi: OK with the recommended WF.

China Telecom: support the WF.

Qualcomm: OK with WF, but OBB study does not need to be restricted to triplexer assumption only

Skyworks: For the triplexer options with merged n5DL and n8UL, non-simultaneous n8UL and n5DL should be assumed. Can it be clarified what OOBB impact is expected in this case? If simultaneous n8UL and n5DL then the blocking impact could be added to the MSD aspect too.

Other observations and proposals

The following RF requirement framework applies to CA_n5-n8, CA_n5-n28 and CA_n8-n20-28.

  - Operating bands for CA

  - Channel bandwidths per operating band for CA

  - UE co-existence requirement

  - ∆TIB and ∆RIB values

  - REFSENS requirements (MSD)

  - Maximum output power

Comments from companies:

ZTE: The listed RF requirements are the classic RF requirements frameworks for inter-band NR CA including 1UL and 2UL. Except the listed ones, ‘UE maximum output power for Inter-band CA’ should be added.

BTW, the ‘Co-existence studies’ is not RF requirement, it should be replaced by ‘Spurious emissions for UE co-existence for Inter-band CA’.

Vivo: Co-existence studies here means harmonics and intermodulation interference study.

CATT: Thanks for the comments. My understanding is that the framework is used in the CA basket WI. The maximum output power and UE co-existence requirement are specific for 2UL. They’re included. Please check.

ZTE2：There is no need to includes co-existence studies. The title for this issue is for ‘RF requirements framework’, as we commented above, ‘Co-existence studies’ is not RF requirement, it aims to harmonics and intermodulation interference study (as vivo said), corresponding to REFSEN requirements (MSD) RF requirements. Note that ‘Co-existence studies and UE co-existence requirement’ are not the same. All in all, ‘Co-existence studies’ should be deleted.

UL configuration

The CBW of UL configuration CA_n5-n8 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

Comments from companies:

Skyworks, thank you for adding this but we also need to agree on exact number of RB of the UL. We suggest that the same RB allocation than single band REFSENS UL configuration is used and placed at the worst position

CATT: Thanks for the comment. It’s included.

Topic #3: CA_n5-n28

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212015 | Samsung | Observation 2: For CA_n5-n28, conventional L+L architecture with existing commercial duplexer/DRx filter could be considered as feasible architecture, further discuss whether to analysis and develop quadplexer. |
| R4-2212356 | Apple | Observation 10: For CA_n5-n28, the low-band antenna design needs to cover the entire spectrum range of 191 MHz simultaneously which is equivalent to a 24% bandwidth ratio that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised. Observation 11: The feasibility of a single quadplexer for CA_n5-n28 needs to be studied. Observation 12: For CA_n5-n28, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed. |
| R4-2212609 | Xiaomi | Observation 4: Either 3-antenna or 2-antenna architecture can be considered for CA_n5A-n8A and CA_n5A-n28A. It is expected to have some relaxation on RF requirements for smart phone. |
| R4-2212610 | Xiaomi | Proposal 4: For CA_n5A-n28A with two UL transmission, the Delta T and Delta R for CA_n5A-n28A with one UL configuration captured in TR 38.717-02-01 can be reused since no additional insertion loss is introduced. The MSD due to cross band isolation should be further evaluated and some relaxation is expected for smart phone. |
| R4-2212713 | ZTE Corporation | Proposal 2. 2DL/2UL CA_n5-n28, CA_n8-n20, CA_n8-n28 could be treated as normal LB-LB NR CA combination, where non-block approval approach is used in Rel-17. Moderator: It’s discussed in sub-topic 2-4. |
| R4-2212799 | vivo | Observation 1: For CA_n5-n28, with 2UL bands transmission, there is no potential interference for DL n5 and n28. |
| R4-2213195 | Qualcomm Finland RFFE Oy | Observation 1: The assumption in all studied is 2RX for each of the bands Observation 2: There are several LB-LB-(LB) CA RF FE architecture alternatives to be analyzed, whose feasibility then depends a lot on the LB antenna interface configuration Observation 3: The feasible number of LB antennas in smartphone must be investigated thoroughly, and special attention must be paid on feasible number of UL capable LB antennas Observation 4: Method of managing intermodulation products could be related to overall RF FE architecture and/or to more thoroughly considered RF parameters resulting in smaller MSD Observation 7: CA_n5A-n28A has no IMD’s |
| R4-2213314 | OPPO | Observation 5: Current 38.101-1 already include n5+n28 with partial spectrum in some requirements but seems not complete in other requirements. And the existing requirement like delta Tib and MSD for cross band isolation need to be updated for the whole n28 band spectrum case. Proposal 3: Use separate antennas for n5 and n28 in n5+n28 CA as reference architecture. |
| R4-2213754 | Skyworks Solutions Inc. | Proposal for CA_n5-n28: - n28F spectrum is supported but architectures based on both n28F duplexer and n28A+B dual duplexer approach are studied - Maximum UL and DL CBW per band must be defined - As a starting point a two-antenna tri-plexing with UL on both antennas is preferred |

Open issues summary and company views collection

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: UE RF analysis assumption

Issue 3-1: UE RF architecture assumption

- Proposals

  - Observation 2: For CA_n5-n28, conventional L+L architecture with existing commercial duplexer/DRx filter could be considered as feasible architecture, further discuss whether to analysis and develop quadplexer. (R4-2212015, Samsung)

  - The feasibility of a single quadplexer for CA_n5-n28 needs to be studied. (R4-2212356	Apple)

  - Either 3-antenna or 2-antenna architecture can be considered for CA_n5A-n8A and CA_n5A-n28A. It is expected to have some relaxation on RF requirements for smart phone. (R4-2212609	 Xiaomi)

  - Use separate antennas for n5 and n28 in n5+n28 CA as reference architecture. (R4-2213314	 OPPO)

  - n28F spectrum is supported but architectures based on both n28F duplexer and n28A+B dual duplexer approach are studied. (R4-2213754, Skyworks Solutions Inc.)

  - As a starting point a two-antenna tri-plexing with UL on both antennas is preferred. (R4-2213754, Skyworks Solutions Inc.)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Prefer to use separate antenna for n5 and n28 considering the large frequency ranges between them. And the concurrent antenna BW in low band in our view should not be larger than 100MHz considering the antenna efficiencies. |
| Skyworks | We do not understand the comment of reusing existing duplexers for 2DL/2UL scenario, does this assume 3 antennas? We think that 2 antenna solutions is preferable as a starting point and only use a third antenna when no other solutions, note in that case that this third antenna may not have the same performance (borrowed?). We are OK to look at quadplexer on main plus duplexer on diversity (2UL on main) or triplexer on both antennas (1UL/antenna) |
| ZTE | RAN4 had already completed some L-L band combination such as n5-n12/n14, not sure what is different here for n5-n28? |
| Vivo | We prefer the third bullet. |
| Qualcomm | As this is feasibility study, multiple architectural options can be studied. This includes e.g duplexers, multiplexers, 2 and 3 antenna solutions. In our view, companies can provide analysis on architectures they want to be captured in TR. Based on comments, there are several architectures for which proponents can provide analysis. |
| Xiaomi | Either 2 antenna or 3 antenna solution can be considered. Based on the current implementation, separate antenna would be preferred even in smartphone but more aggressive solution can be expected by considering a feasibility study. |
| Nokia | If single Quadplexer is considered, it means pass-bandwidth of n28 is 45 MHz. Then, handling of NS_17 as during single band operation needs to be taken into account. Currently no A-MPR for NS_17 assuming that n28 has dual duplexer which provides some isolation toward the additional spurious emission, if my memory is correct. |
| Apple | We prefer to start with 2 antenna solutions, either two PAs on the main antenna or one PA on main antenna and one PA on diversity antenna. |
| Huawei | We share the similar view with Qualcomm. We don’t need to exclude any architectures which companies are interested in. In this SI phase, we can list all the possible options and trade off the pros and cons. To ZTE, due to the block approval procedure and dramatic amount of band combinations, RAN4 can’t discuss the RF architecture and details completely. That’s why this WI is beneficial. At least, we don’t know the assumed RF architecture and detailed implementation for CA_n5-n12. |
| Samsung | Share similar view with Skyworks and Apple, 2 antennas as starting point (2UL in one antenna or separate antennas) would be good. Response to Skyworks’s question, reusing existing duplexers/DRx for 2DL/2UL scenario means dual on switch+ duplxer+DRx filter in each TRx antenna which essentially is a triplexer, or dual on switch + 2duplxer in TxPRx antenna which essentially is qualplexer. |
| Skyworks | To ZTE, there is a huge difference between n5-n12 and n5-14 and n5-n28: n28 band is wide and still uses dual duplexers as being difficult in itself. n28DL is much closer to n5 UL than n12/n14 DL. Overall the antenna multiplexing is much more complex |

Issue 3-2: CBW assumption

- Proposals

  - Maximum UL and DL CBW per band must be defined (R4-2213754, Skyworks Solutions Inc.)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| skyworks | Any relaxation in UL and DL BW can help reduce the MSD and filter constraints, it is important that the CA BCS in known |
| ZTE | Similar view with skyworks. In the SID, CA BCS are all missing for the targeted band combination. Without the supported channel bandwidth, the impact on MSD it unclear. |
| Huawei | In current spec, BCS0 has been specified with maximum 20 MHz in UL band n5. |

Sub-topic 3-2: Feasiblity study

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-3: Feasibility issues need to be analysed

- Proposals

  - Proposal 1: Observation 12: For CA_n5-n28, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed. (R4-2212356	Apple)

  - Proposal 2: The MSD due to cross band isolation should be further evaluated and some relaxation is expected for smart phone (R4-2212610	Xiaomi)

- Recommended WF

  - The MSD due to cross band isolation

| Company | Comments |
| --- | --- |
| OPPO | Ok with Proposal 1 and 2. |
| Skyworks | Once UL and DL CBW are known we can look into cross-band MSD. |
| ZTE | Like some normal L-L combination, cross band isolation MSD needs to be checked, and also it is related to the supported max. BW. Again, there were several L-L combination have been completed, not sure what is the special for n5-n28? |
| Qualcomm | The WF recommendation fine to be studied, in addition any other issue arising from RF architecture discussions can be studied as RF architecture and feasibility go hand in hand |
| Xiaomi | OK with both proposals. |
| Apple | We support both proposals. |
| Huawei | Interested companies can provide MSD analysis for this band combination. |
| Samsung | Is it already specified? n5; n28; 834; 20; 15; 20 (RBstart=0); 800.5; 5; [17.5]; ACLR2. |

Sub-topic 3-3: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 3-4: Other observations and proposals

Observation 1: Tib and Rib for 1UL CA_n5A-n28A can be reused by 2UL CA_n5A-n28A (R4-2212610	Xiaomi)

| Company | Comments |
| --- | --- |
| OPPO | Tib and Rib can be discussed further after the architecture is decided. |
| Skyworks | We agree with OPPO |
| ZTE | We do not understand the question here. For any 2ULCA combination, 1UL specific requirements should be defined first, like Tib and Rib/ cross band isolation MSD. For 2UL CA, there is no need to check Tib and Rib requirements, only focus on IMD issue if any, which means no matter 2UL is supported or not, Tib/Rib should be checked first. |
| Qualcomm | More analysis is needed, as different architectures may show different values |
| Xiaomi | We can come back until the architecture is stable. |
| Apple | We need to wait for the architecture assumption before evaluating the relaxations. |
| Huawei | This can be further discussed in WI phase. |
| Samsung | Have same question as ZTE, Tib /Rib/harmonic/harmonic mixing/ cross band isolation are specified when the 2DL 1UL is introduced, according to current band combination workflow, no need to specify and revisit these values for 2DL 2UL. Why companies want to revisit these values specifically for this combo? Are you asking for new band combination workflow? |

Observation 2: No IMD for 2UL CA n5-n28	 (R4-2212799, vivo; R4-2213195, Qualcomm Finland RFFE Oy)

| Company | Comments |
| --- | --- |
| OPPO | Agree. |

Proposal: 2DL/2UL CA_n5-n28 could be treated as normal LB-LB NR CA combination, where non-block approval approach is used in Rel-17. (R4-2212713	ZTE)

| Company | Comments |
| --- | --- |
| Skyworks | This is an option, but only after the SI is completed anyhow, ie the BCS is known so given we have this SI focusing on the issue, I do not see the benefit of moving to non-block approval yet. |
| ZTE | As mentioned above, there were several L-L combination (such as such as n5-n12/n14, also n18-n28) have been completed, not sure what is the special for n5-n28? Our intention here is this combination can be treated like some other LB-LB NR CA combinations in the past(non-block approval). But of course, BCS should be known first. This is SID, which means n5-n28 will not be introduced in the spec in the end, and aslo it cannot be discussed in R18 non-block approval agenda item. |
| Qualcomm | In our view, it would be better to handle all combinations which are part of the SI under this SI. |
| Xiaomi | As mentioned by ZTE, if operator has prompt request for implementation, it seems a reasonable way to move this to non-block approval. Otherwise, it is OK to handle in this SI. |
| Huawei | In dedicated SI/WI, more RF details can be widely discussed. No need to come back to basket WI for these low + low band combinations. |
| Skyworks | The work in “not for block approval” is related to basket WI and results in TPs CRs. This is an SI and no spec change is applicable in this case. Once the SI phase is finished, we can move on |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: UE RF architecture assumption | Companies provide many good comments for this issue. 2 and 3 antenna architecture seem can be the assumption for future study. Tentative agreements: The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28 2 antenna, 3 antenna Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |
| Issue 3-2: CBW assumption | Companies support the idea that CBW assumption should be agreed before further study. However, there’s not many CBW proposals yet. Tentative agreements: Candidate options: Recommendations for 2nd round: Continue to discuss and seek inputs from proponents of this CA. |
| Issue 3-3: Feasibility issues need to be analysed | Most of the companies agree MSD due to cross band isolation should be analyzed. Tentative agreements: The following issues can be analyzed in the feasibility study. - The MSD due to cross band isolation Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |
| Issue 3-4: Other observations and proposals | The following observation seems ok for all of the companies. Others need more time to further check. Tentative agreements: No IMD for 2UL CA n5-n28 Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Continue discuss in the WF, the comments in the WF are copied below.

UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n5-n28

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

Comments from companies:

ZTE: Similar question as above, does the ‘2 antenna, 3 antenna’ means the total antenna number which includes not only primary Tx/Rx antenna(s), but also diveristy Rx antenna(s)?

OPPO: If only consider this n5+n28 band combination is ok, but if consider UE that supports also other band combinations like n8+n20+n28, then 4 antennas architecture (include main and diversity antennas) should also be considered here. This needs to be clarified.

Xiaomi: Ok with 2 antenna and 3 antenna that include primary antenna and diversity antenna.

Qualcomm: OK to study 2 and 3 antennas. Let’s make it clear that the number of antennas here means total number of LB antennas including diversity.

Huawei: OK to assume 2 and 3 antennas architecture for further analysis for CA_n5-n28.

Skyworks, thank you for adding this but we also need to agree on exact number of RB of the UL. We suggest that the same RB allocation than single band REFSENS UL configuration is used and placed at the worst position

Vivo: One suggestion: give an example RF architecture block diagram to illustrate 2 antenna, 3 antenna solution.

CATT: The clarification is added. Please check.

CBW assumption

n5: 5, 10, 15, 20

n28: 5, 10, 15, 20, 25, 30

Comments from companies:

OPPO: For clarification, is there CBW location restriction in this band combination?

Huawei: To OPPO, I don’t think we have CBW location restriction in this band combination.

Skyworks: If all n5 and n28 CBW are to be supported in the BCS:

- n5UL is 21MHz away from n28DL => ACLR2 interference for 15 and 20MHz UL ~ IMD5 issue

- n5DL is 121MHz away from n28UL => ACLR4 interference for 30MHz UL ~ IMD9 issue should be OK

- n28 dual duplexer should be studied.

Vivo: No need to restrict CBW location.

CATT: It is suggested to use the current 1U/2D BCS0 as the assumption:

| CA_n5A-n28A | - | n5 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

In our understanding, there’s no CBW location restriction. If there’re any difficulties identified, it can be discussed later.

Feasibility issues need to be analysed

The following issues can be analyzed in the feasibility study.

  - The MSD due to cross band isolation

Comments from companies:

ZTE: The supported CBW should be defined.

Skyworks: at least it needs to be confirmed if n5 UL includes 15 and 20MHz.=> worst case cross band MSD

Other observations and proposals

The following proposal is agreed: No IMD for 2UL CA n5-n28

UL configuration

The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

Comments from companies:

Skyworks, thank you for adding this but we also need to agree on exact number of RB of the UL. We suggest that the same RB allocation than single band REFSENS UL configuration is used and placed at the worst position

CATT: Thanks for the comment. It’s included.

Topic #4: CA_n8-n20-n28

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212015 | Samsung | Proposal 1: DL CA_n8A-n28A with UL CA_n8A-n28A could be defined in the Basket WI (not for block approval), reuse the architecture of DC_n8A-n28A, no IMD issue Proposal 2: DL CA_n8A-n20A with UL CA_n8A-n20A could be defined in the Basket WI (not for block approval), architecture and MSD (due to IMD) values of DC_n8A-n20A could be reused Observation 3: DC_8A-20A_n28A with UL DC_8A_n28A and DC_20A_n28A has been defined as targeted to FWA form factor in Rel-17. Observation 4: For DL CA_n8-n20-n28 with UL CA_n8-n20, CA_n8-n28 and CA_n20-n28, the possible architecture are switches+ duplexer+ triplexer+ DRX filters+3 antennas, or penta-plexer+2 antennas, architecture of DC_8-20_n28 could be reused, but the MSD due to UL CA_n8-n20 which falls into n28DL needs to be evaluated. Observation 5: Supporting DL CA_n8-n20-n28 with UL CA_n8-n20, CA_n8-n28, CA_n20-n28 for smart phone is quite challenging, considering the significant MSD, the poor performance of wideband antennas and the space limitation. |
| R4-2212356 | Apple | Observation 1: For CA_n8-n20-n28, the low-band antenna design needs to cover the entire spectrum range of 257 MHz simultaneously which is equivalent to a 31% bandwidth ratio that would far exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination is expected to be compromised. Observation 2: For CA_n8-n20-n28, the feasibility on low-band pentaplexer (single or dual) needs to be studied. Observation 3: For CA_n8-n20-n28, the REFSENS impact due to self-band isolation, cross-band isolation, 2UL IMD as well as the impact on n20 and n28 out-of-band blocking requirement due to the extended DL filter bandwidth with pentaplexer implementation need to be addressed. |
| R4-2212609 | Xiaomi | Observation 1: Two Tx/Rx capable low-band main antennas are used for CA_8A-20A-28A and no limitation are introduced for UE type. 1-antenna architecture (without considering diversity antenna) have also been investigated but the high insertion loss of pentaplexer of hexplexer is unacceptable. Observation 2: Two Tx/Rx capable low band main antennas plus one Rx capable diversity antenna are used for DC_8A-20A_n28A. 2-antenna architecture (1 Tx/Rx main antenna, 1 Rx diversity antenna) with a pentaplexer was discussed and companies identified numerous challenges such as high insertion loss, Tx-Rx isolation, low band wideband antenna, etc. Observation 3: The problems of 2-antenna architecture identified in Rel-17 still remain and even worse for smart phone. The hardware implementation such as pentaplexer is impractical and premature at current stage. For smart phone form factor, 3-antenna architecture can be considered for L-L-L band combination and some relaxation on RF requirements, e.g. additional MSD due to cross band isolation, is expected. |
| R4-2212610 | Xiaomi | Proposal 1: If RAN4 agree on the RF architecture specified for FWA UE to be used for smart phone form factor, the RF requirements of Delta T and Delta R can be reused for CA_n8A-n20A-n28A and the MSD for n8 for DC_8-20_n28 due to IMD issue can be considered as a starting point. The MSD for n28 due to IMD issue should be further studied. Some relaxation on MSD for two UL transmission, i.e. additional MSD due to cross band isolation can be expected for smart phone. Proposal 2: For CA_n8A-n20A-n28A, the frequency of band n28 should be restricted to 703-733MHz for the uplink and to 758-788MHz for the downlink. |
| R4-2212713 | ZTE Corporation | Proposal 1. For 3DL/2UL CA_n8-n20-n28, three LB antennas implementation or two LB antennas implementation might feasible for smartphone with large form factor in case of the antenna performance is acceptable. Proposal 2. 2DL/2UL CA_n5-n28, CA_n8-n20, CA_n8-n28 could be treated as normal LB-LB NR CA combination, where non-block approval approach is used in Rel-17. |
| R4-2212798 | vivo | Proposal 2: Two-antenna solution as the starting point for CA_n8-n20-n28. |
| R4-2212799 | vivo | Proposal 1: The following RF requirements framework can still apply for 700/800/900 MHz NR inter-band CA: - Operating bands for CA - Channel bandwidths per operating band for CA - Co-existence studies - ∆TIB and ∆RIB values - REFSENS requirements Observation 2: For CA_n8-n20-n28 with UL CA_n8-n20 configuration, IMD3 of band n8 and n20 would fall into n8 DL and n20 DL. Observation 3: For CA_n8-n20-n28 with UL CA_n8-n28 configuration, IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination. Observation 4: For CA_n8-n20-n28 with UL CA_n20-n28 configuration, IMD3 of band n20 and n28 would fall into n8 DL. |
| R4-2213161 | Huawei, HiSilicon | Proposal 1: it’s feasible to implement the CA band combination CA_n8-n20-n28 and the corresponding fallback modes according to the proposed RF architecture. Proposal 2: RF requirements for the CA band combination CA_n8-n20-n28 and the corresponding fallback modes can be further studied based on the summary in table 1. |
| R4-2213195 | Qualcomm Finland RFFE Oy | Observation 1: The assumption in all studied is 2RX for each of the bands Observation 2: There are several LB-LB-(LB) CA RF FE architecture alternatives to be analyzed, whose feasibility then depends a lot on the LB antenna interface configuration Observation 3: The feasible number of LB antennas in smartphone must be investigated thoroughly, and special attention must be paid on feasible number of UL capable LB antennas Observation 4: Method of managing intermodulation products could be related to overall RF FE architecture and/or to more thoroughly considered RF parameters resulting in smaller MSD Observation 5: CA_n8A-n20A has both IMD3 and IMD5 for both DL bands. CA_n8A-n28A has no IMD’s |
| R4-2213314 | OPPO | Observation 6: It is unclear in the WID whether there is spectrum restriction in n8+n20+n28 especially for n20 and n28. Observation 7: According to current 38101-1, when n20+n28 is subset of higher order band combinations, then n28 is restricted to 703-733 MHz for UL and 758-788 MHz for DL. Proposal 4: Clarify that n28 is restricted to 703-733 MHz for UL and 758-788 MHz for DL in n8+n20+n28 band combination. Observation 8: There is chance to use one antenna to cover the n20 whole spectrum and n28DL spectrum, or n28 whole spectrum and n20DL spectrum via antenna tuning though difficult. Proposal 5: Four antenna architecture as shown in figure 4 for n8+n20+n28 can be used for further performance analysis which can cover any UL CA configurations within n8+n20+n28. |
| R4-2213754 | Skyworks Solutions Inc. | Proposal for CA_n8-n20-n28: - Only n28A spectrum is supported based on n28A UL+(n28A+n20) DL + n20 UL tri-plexing as a starting point - Maximum UL and DL CBW per band must be defined Proposal on PA/Antenna Architecture: - Baseline architecture should be agreed for 2nd PA being on Main, diversity or third antenna for all cases and a generic approach is targeted for two LB on one side and three LB on the other side - Coupling between the two or three antennas should be agreed for the 700MHz to 960MHz range in a smartphone context to assess the different cases |

Open issues summary and company views collection

Sub-topic 4-1: Spectrum restriction asssumption

Issue 4-1: Spectrum restriction assumption for the analysis

- Proposals

  - Option 1: For CA_n8A-n20A-n28A, n28 should be restricted to 703-733MHz for the uplink and to 758-788MHz for the downlink. (R4-2212610	Xiaomi)

  - Option 2: Only n28A spectrum is supported based on n28A UL+(n28A+n20) DL + n20 UL tri-plexing as a starting point (R4-2213754 Skyworks)

- Recommended WF

  - Only n28A spectrum is supported

| Company | Comments |
| --- | --- |
| OPPO | Ok with Option 1, i.e. n28 should be restricted to 703-733MHz for the uplink and to 758-788MHz for the downlink. For clarification if understand correctly n28A is the low part of n28, i.e. 703-733MHz for the uplink, is this “n28A” defined in specification? If not probably clearly written the frequency range is clearer. |
| Skyworks | n28A is the same as 703-733MHz for the uplink and to 758-788MHz for the downlink. But we agree that using the frequency range is less ambiguous. |
| ZTE | We think it is pending on the BCS requested by operator. If operator want to use high frequency range of n28B, what should we do? Here it is not clear to use n28A spectrum here although my understanding is that it means the low frequency range, i.e. (UL):703MHz~733MHz/(DL)758MHz~788MHz. We may need to try to avoid to use oral term here. n28A in the specification means the single carrier in the n28 (i.e. class A), not related to the specfic frequency range. |
| vivo | Whether to restrict band n28 depends on the operator’s request on this band combination. Maybe more clarification is needed for the operator in charge of this band combination. |
| Qualcomm | Supporting n28A (703-733/758-788) only is ok as it was the case for CA_n20A-n28A earlier. |
| Xiaomi | For clarification, what the proposal 1 implies is the low frequency range of n28 can be used. However, this can be further confirmed based on operator demand. |
| Nokia | Needs clarification. Only n28A is supported during CA while the entire n28(n28A and n28B) is supported during single band operation by the same UE. Is that the common understnaidng? |
| Apple | Proponent’s clarification is needed on n28 frequency range. |
| Huawei | Since band n20 and band n28 can be deployed together in some countries of region 1. The frequency range for band n28 is clear enough, i.e. (UL):703MHz~733MHz/(DL)758MHz~788MHz. |

Sub-topic 4-2: UE RF analysis assumption

Issue 4-2: UE RF architecture assumption

- Proposals

  - Option 1: The feasibility on low-band pentaplexer (single or dual) needs to be studied. (R4-2212356, Apple)

  - Option 2: 3-antenna architecture can be considered and some relaxation on RF requirements, e.g. additional MSD due to cross band isolation, is expected. (R4-2212609, Xiaomi)

  - Option 3: Only n28A spectrum is supported based on n28A UL+(n28A+n20) DL + n20 UL tri-plexing as a starting point

  - Option 4: For 3DL/2UL CA_n8-n20-n28, three LB antennas implementation or two LB antennas implementation might feasible for smartphone with large form factor in case of the antenna performance is acceptable. R4-2212713	ZTE Corporation

  - Option 5: Two-antenna solution as the starting point for CA_n8-n20-n28. R4-2212798	vivo

  - Option 6: RF architectures in R4-2213161 (Huawei)

  - Option 7: Four antenna architecture as shown in figure 4 for n8+n20+n28 can be used for further performance analysis which can cover any UL CA configurations within n8+n20+n28. (R4-2213314, OPPO)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | For Option 1, ok to study, but using one antenna to cover the whole three band is too challenging for antenna design. For Option 2, the 3 antenna architecture seems using one antenna to cover the n8+n20+n28 Rx is still too challenging due to the quite large concurrent bandwidth. For Option 3, ok. For Option 4, same comment as Option 2. For Option 5, same comment as Option 2. And one clarification question, in the paper it shows the main antenna design with one cover n20+n28 and the other cover n8, how about the diversity antenna, is it using one to cover all or others? For Option 6, probably the three antennas architecture is more suitable than two antennas architecture from antenna performance perspective. For Option 7, four antenna architecture is preferred considering the antenna efficiency and also the implementation complexity. |
| Skyworks | Feasibility of pentaplexer is challenging and it should not be expected that good Tx-Rx isolations can be easily obtained with the small duplex gaps and the small inter band gap. Other options with UL on the second antenna should also be explored |
| Murata | Option 1 has no advantage. More insertion loss and poor VSWR for a good percentage of the frequency range on a primary antenna. Option 2/4 3 antenna solution will have some degradation on 2 out of the 3 antennas, but some antenna performance can be gained at the expense of diversity gain. Option 7 will have space disadvantage and this can only be considered for FWA devices. |
| ZTE | Different challenges for different options. In terms of architectures proposed by the companies, it seems there were two grouped architectures: 1. 1Tx/Rx antenna+ 1Rx antenna: i.e. 1LB antenna with pentaplexer implemtation 2. nTX/Rx antennas + mRx antennas: (n>1, m≥1): i.e. multiple LB antennas. It may need to know how many LB antennas can be supported for 3DL/2UL CA implementation in a smart phone? 3? 4? 5? 6? |
| vivo | For clarification of Option 5, two-antenna means two TRx antenna. With Diversity Rx, we are considering on DRx antenna for covering the whole band combination range. I think we share the same view with Option 2. |
| Qualcomm | As this is feasibility study, multiple architectural options can be studied. This includes e.g duplexers, multiplexers, 2 and 3 antenna solutions. Using pentaplexer and/or 4 antenna solution for LB’s is something new at least in the context of 3GPP so they should be further discussed. In our view, companies can provide analysis on architectures they want to be captured in TR. Based on comments, there are several architectures for which proponents can provide analysis. |
| Xiaomi | We prefer to consider 3 antenna as a starting point and also accept 4 antenna (Option 7) even if 3 antenna is used in FWA UE. Actually 4 antenna with more antenna isolation would alleviate the challenge of wideband antenna design but the space issue should be carefully evaluated especially for UE vendors. Option 1 is too challenging for antenna design but we are happy to study the feasibility of pentaplexer. |
| Nokia | We need clarification on issue 4-1 first. |
| Apple | We prefer to start with 2 antenna solutions. It is understandable that pentaplexer design would be challenging. But this is also part of the feasibility study. |
| Huawei | We share the similar view with Qualcomm. We don’t need to exclude any architectures which companies are interested in at this stage. In this SI phase, we can list all the possible options and trade off the pros and cons. To ZTE, based on the feedback from UE vendors, 3 and 4 low band antennas can be achieved. |

Issue 4-3: CBW assumption

- Proposals

  - Maximum UL and DL CBW per band must be defined (R4-2213754, Skyworks Solutions Inc.)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Skyworks | Without this we cannot study crossband MSD and we cannot also check if some relaxed filter parameter is feasible |
| ZTE | Same as above. |
| Huawei | I think the general assumption can be started as below. n8: 5, 10, 15, 20 n20: 5, 10, 15, 20 n28: 5, 10, 15, 20, 25, 30 |

Sub-topic 4-3: Feasiblity study

Issue 4-4: Feasibility issues need to be analysed

- Proposals

  - Proposal 1: the MSD due to UL CA_n8-n20 which falls into n28DL needs to be evaluated. (R4-2212015, Samsung)

  - Proposal 2: For CA_n8-n20-n28, the REFSENS impact due to self-band isolation, cross-band isolation, 2UL IMD as well as the impact on n20 and n28 out-of-band blocking requirement due to the extended DL filter bandwidth with pentaplexer implementation need to be addressed. (R4-2212356, Apple)

  - Observation 3: For CA_n8-n20-n28 with UL CA_n8-n28 configuration, IMD3 of band n8 and n28 would not interfere with the DL reception for this CA band combination. (R4-2212799	vivo)

  - Observation 4: For CA_n8-n20-n28 with UL CA_n20-n28 configuration, IMD3 of band n20 and n28 would fall into n8 DL. (R4-2212799	vivo)

  - Observation 5: CA_n8A-n20A has both IMD3 and IMD5 for both DL bands. CA_n8A-n28A has no IMD’s (R4-2213195	Qualcomm)

- Recommended WF

  - UL CA_n8-n20 (IMD3 and IMD5), UL CA_n8-n28 (IMD3), UL CA_n20-n28 (IMD3), CA_n8A-n28A (TBD, different views from vivo and QC).

  - The impact on n20 and n28 out-of-band blocking requirement

| Company | Comments |
| --- | --- |
| OPPO | Ok with WF. |
| Skyworks | IMDs are for the largest channel BW, for such a 3band case we assume that no operator can have max BW in all the bands, again it is critical we use a reasonable BCS. |
| ZTE | Similar view with Skyworks. In addition, we think it should be discuss RF architecture first before we do the MSD evulation. Also, for 3DL/2UL CA_n8-n20-n28, the fallbacks of 2DL/2UL of CA_n8-n20 and CA_n8-n28 have not completed yet. Moreover, 2DL/2UL n20-n28 have already been defined in the 38.101-1, are we going to re-visit it? |
| Qualcomm | While we fully recognize that LB-LB-LB must be analysed, it was agreed in SI description that LB-LB fallbacks are to be studied first. Hence RAN4 should first concentrate on LB-LB cases if we follow the SI description. |
| Xiaomi | OK with the recommended WF. |
| Apple | Ok with the recommended WF. |
| Huawei | I think most of the requirements related to MSD due to IMD have been studied in corresponding ENDC band combination. Anyway, we are open to further discuss this. |
| Samsung | CA_n20-n28 with 2UL have already been defined with n28 limited to 703-733MHz for UL and 758-788MHz for DL, configurations as below. CA_n20A-n28A; CA_n20A-n28A; n20; 5, 10, 15, 20; 0. n28; 5, 10, 15, 20. n20; 5, 10, 15, 20; 1. n28; 5, 10, 15, 20, 30. |

Sub-topic 4-4: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 4-5: Other observations and proposals

Handling of 2UL/2DL CA_n8A-n28A and CA_n8A-n20A (R4-2212015, Samsung)

2DL/2UL CA_n8-n20, CA_n8-n28 could be treated as normal LB-LB NR CA combination, where non-block approval approach is used in Rel-17 (R4-2212713 ZTE and R4-2212015 Samsung)

| Company | Comments |
| --- | --- |
| Skyworks | As already commented this can only be after the SI is complete and on the basis of a known BCS |
| ZTE | Same as above. |
| Qualcomm | In our view, it would be better to handle all combinations which are part of the SI under this SI. |
| Huawei | I share the similar view with Qualcomm. As we have established this SI, there is no need to discuss them in another basket WI. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1: Spectrum restriction assumption for the analysis | Some companies support to restrict n28, some companies comment clarification from proponents are needed. Tentative agreements: Candidate options: Recommendations for 2nd round: Further discuss to check if the following restriction for n28 can be assumed in this study. UL: 703MHz~733MHz DL: 758MHz~788MHz. |
| Issue 4-2: UE RF architecture assumption | According to the comments in the first round, company would like to keep all of possibilities in current stage. So moderator suggests keeping 2, 3 and 4 antenna architecture. Tentative agreements: The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n8-n20-n28. 2 antenna, 3 antenna, 4 antenna. Candidate options: Recommendations for 2nd round: Further check if the tentative agreements can be agreed. |
| Issue 4-3: CBW assumption | Companies support the question but not many inputs from companies. Huawei proposed the following as starting point, n8: 5, 10, 15, 20 n20: 5, 10, 15, 20 n28: 5, 10, 15, 20, 25, 30 Moderator suggests companies can double check if the above proposal is ok. Tentative agreements: Candidate options: Recommendations for 2nd round: Further check if the following CBW assumption can be used in future study. n8: 5, 10, 15, 20 n20: 5, 10, 15, 20 n28: 5, 10, 15, 20, 25, 30 |
| Issue 4-4: Feasibility issues need to be analysed | Companies had common understanding on this issue. But moderator found that for CA_n8A-n28A, moderator didn’t read correctly for the companies’ proposals. Both vivo and QC proposed no IMD. So moderator encourages companies to double check the following recommended WF in 2nd round. - UL CA_n8-n20 (IMD3 and IMD5), UL CA_n8-n28 (IMD3), UL CA_n20-n28 (IMD3), CA_n8A-n28A (TBD, different views from vivo and QC). - The impact on n20 and n28 out-of-band blocking requirement Tentative agreements: Candidate options: Recommendations for 2nd round: Further check if the followings can be agreed. The following issues will be analyzed in the feasibility study. - UL CA_n8-n20 (IMD3 and IMD5), UL CA_n8-n28 (IMD3), UL CA_n20-n28 (IMD3). - The impact on n20 and n28 out-of-band blocking requirement |
| Issue 4-5: Other observations and proposals | For handling of 2UL/2DL in normal basket WI, companies think they should first studied in this SI then decide where to discuss the requirement. Moderator thinks there’s no need to conclude this before the SI is closed. Tentative agreements: Candidate options: Recommendations for 2nd round: No discussion in 2nd round. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Continue discuss in the WF, the comments in the WF are copied below.

Spectrum restriction assumption for the analysis

- UE should support the full range of spectrum for single band operations on n8, n20 and n28

- Use the follow frequency ranges for further discussion for n28 spectrum restriction to support CA_n8-n20-n28.

UL: 703MHz~733MHz

DL: 758MHz~788MHz

Comments from companies:

Nokia: The assumption should make sure that the restriction only applies to the status that a UE is configured with CA_n8-n20-n28 while when the UE is in Non-CA state, the restriction doesn’t apply. FFS for which fallback configuration(s) requires the restriction.

Xiaomi: This assumption can be applied to CA case like DC specified in Rel-17.

Samsung: We think this restriction is for CA_n8-n20-n28.

QC: Ok with WF. As pointed out by Nokia, the n28 restriction is only for CA_n20-n28 and it’s supersets e.g. CA_n8-n20-n28.

Skyworks: the only n20 and n28 related already specified case is with proposed restriction and is critical for the architecture asumption. Non-CA state is unclear does it mean that this UE should support full n28 in single band? It it is the case we may need to duplicate/add other duplexers for n8 and n28. Note anyhow that 38.101-1 say this “For a higher order band combination of which CA_n20-n28 is a subset, the frequency range in band n28 is restricted for the higher order band combination to 703-733 MHz for the UL and 758-788 MHz for the DL.”

Vivo: Whether to restrict band n28 depends on operator’s request.

CATT: Suggest to use the same wording with n5-n8.

TIM: The spectrum range for n28 reported in the WF is ok for us.

VDF: The spectrum range for n28 reported in the WF is also ok for us.

Orange: OK also with the spectrum range for n28 reported in the WF

UE RF architecture assumption

The following UE RF architectures can be assumed in the future meetings’ analysis for CA_n8-n20-n28.

2 antenna, 3 antenna, 4 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands.

Comments from companies:

ZTE: Similar question as above: does the ‘2 antenna, 3 antenna, 4 antnna’ means the total antenna number which includes not only primary Tx/Rx antenna(s), but also diveristy Rx antenna(s)?

OPPO: For the three band combinations in this SI, do we need to consider a UE support all of them or studied separately but in the end UE in the market has to support all of them. This might need to be clarified in the SI. For 4 antennas architecture include main and diversity antennas.

Xiaomi: We are OK to study 4 antenna architecture for 3 low band BC. In our understanding the antenna number here means the sum of primary antenna and diversity antenna.

Qualcomm: OK to study at least 2 and 3 antennas. We are ok include 4 antenna studies as well, however practically thinking going in 3GPP from 2LB antenna to max 4 for smartphone form factor sounds a stretch. Let’s make it clear that the number of antennas here means total number of LB antennas including diversity.

Skyworks: The number of antennas should be understood as the total number of antennas to support Main UL/DL and diversity DL for all bands. In this case it is OK have up to 4 antennas.

Vivo: We need clearer picture than 2 antenna, 3 antenna, 4 antenna. Maybe an example diagram can be given for each solution.

CATT: Clarification is added.

CBW assumption

The following CBW assumption can be used in future study

n8: 5, 10, 15

n20: 5, 10, 15, 20

n28: 5, 10, 15, 20

Comments from companies:

Skyworks: is it really true that we need to consider all CBW and that an operator will max CBW in all 3 bands? then should we consider the simultaneous  interference of 2 UL for the cross band cases?

Note that CA_n20-n28 has a BCS0 with only up to 20MHz in n28.

CATT: Thanks the information. The folloiwngs are the fall back mode CA in latest spec. I suggest keeping the current proposal and wait the inputs from operators.

| CA_n8A-n20A | - | n8 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n8A-n28A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n20A-n28A | CA_n20A-n28A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n20 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |

TIM: We are interested in the 10 MHz CBW for all the 3 bands (i.e. n8, n20 and n28).

VDF: We are interested in up to 15 MHz CBW for n8 and n28, and up to 20 MHz for n20.

Orange: We are interested in 10 MHz CBW for n8 and up to 20 MHz for n20 and n28.

Feasibility issues need to be analysed

The following issues will be analyzed in the feasibility study.

  - UL CA_n8-n20 (IMD3 and IMD5), UL CA_n20-n28 (IMD3).

  - The impact on n20 and n28 out-of-band blocking requirement

Comments from companies:

Apple: The impact on n20 and n28 out-of-band blocking requirement is based on the assumption of a pentaplexer implementation where one of bandpass filter covers both n20 and n28 DL ranges.

ZTE: n20-n28 inter-band UL CA have already existed in the spec.

Samsung: Agree with ZTE, as we commented in first round, ULCA_n20-n28 has already been specified with n28 limited to 703-733MHz (UL) and 758-788MHz (DL). If 2.4.1 confirm the frequency restriction, no need to redefine.

Qualcomm: OK with WF, but OBB study does not need to be restricted to pentaplexer assumption only

CATT: Thanks for the comments. If companies are confident to remove UL CA_n20-n28 (IMD3), it can be removed. For the OOB, it also can be removed if companies think there’s no problem. I suggest keeping it until companies are confident.

UL configuration

UL configuration of CA_n8-n20, CA_n8-n28, CA_n20-n28 are supported for CA_n8-n20-n28. The CBW assumption for UL and DL are the same. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

Comments from companies:

Skyworks, thank you for adding this but we also need to agree on exact number of RB of the UL. We suggest that the same RB allocation than single band REFSENS UL configuration is used and placed at the worst position

CATT: Thanks for the comment. It’s included.

Skyworks: it is not included so I’ve added it.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2214445 | WF on study on FS_NR_700800900 | CATT |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2211713 |  | Discussion of the work plan for enhancement for 700/800/900MHz band combinations | CATT | Noted |  |
| R4-2211714 |  | Discussion of the TR skeleton for TR 38.872 | CATT | Noted |  |
| R4-2211927 |  | Considerations on 800/900MHz band combination | China Unicom | Noted |  |
| R4-2212015 |  | On 700800900 | Samsung | Noted |  |
| R4-2212356 |  | On 700/800/900MHz band combinations for NR | Apple | Noted |  |
| R4-2212609 |  | Discussion on feasibility study for 700800900MHz | Xiaomi | Noted |  |
| R4-2212610 |  | Discussion on requirement impact for 700800900MHz | Xiaomi | Noted |  |
| R4-2212698 |  | Discussion on band combination for 800MHz and 900MHz | China Telecom | Noted |  |
| R4-2212713 |  | Discussion on 700_800_900MHz | ZTE Corporation | Noted |  |
| R4-2212798 |  | Considerations on feasibility aspects for 700/800/900MHz CA band combinations | vivo | Noted |  |
| R4-2212799 |  | Impacts on RF requirements for 700800900MHz CA band combinations | vivo | Noted |  |
| R4-2213160 |  | Discussion on feasibility of low band wideband antenna | Huawei, HiSilicon | Noted |  |
| R4-2213161 |  | Discussion on RAN4 requirement's impacts for CA_n8-n20-n28 | Huawei, HiSilicon | Noted |  |
| R4-2213162 |  | Discussion on RAN4 requirement's impacts for CA_n5-n8 | Huawei, HiSilicon | Noted |  |
| R4-2213195 |  | Considerations on 700-800-900 NR CA | Qualcomm Finland RFFE Oy | Noted |  |
| R4-2213314 |  | R18 Discussion on 700+800+900MHz UE architecture | OPPO | Noted |  |
| R4-2213754 |  | Input on feasibility of LB-LB-LB band combinations | Skyworks Solutions Inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214445 |  | WF on study on FS_NR_700800900 | CATT | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents