3GPP TSG-RAN4 Meeting #104-e 																R4-2214267

E-meeting, 15th Aug. – 26th Aug., 2022

Agenda item:			9.11.6

Source:	Moderator (Xiaomi)

Title:	Email discussion summary for [104-e][215] NR_NTN_solutions_RRM_2

Document for:	Information

Introduction

The scope of this email discussion contains NTN RRM performance requirements (9.11.6). All the submitted TDocs in this agenda were reviewed and the relevant observations and proposals are included in this email discussion.

The timeline for 1st and 2nd round email discussions can be referred in TDoc of “RAN4#104-e E-meeting Arrangements and Guidelines”

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest to them

- Ensure that their comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Qualcomm | CH Park | chparkqc@qti.qualcomm.com |
| Xiaomi | Xuhua Tao | taoxuhua@xiaomi.com |
| Ericsson | Ming Li | ming.l.li@ericsson.com |
| Apple | Jie Cui | Jie_cui@apple.com |
| Huawei | Li Zhang | zhangli164@huawei.com |
| LGE | Jin Woong Park | jinwoong.park@lge.com |
| MediaTek | Hsuanli Lin | Hsuanli.Lin@mediatek.com |
| CMCC | Shiyuan Wang | wangshiyuan@chinamobile.com |
| CATT | Yanze Fu | fuyanze@catt.cn |
| Nokia, Nokia Shanghai Bell | Anthony Lo | Anthony.Lo@nokia.com |
| THALES | Dorin Panaitopol |  |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Performance requirements for NTN RMM

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211638 | CATT | Proposal 1: The performance requirement for frequency error may not be discussed in RRM session. Proposal 2: The margin for propagator model error should not be introduced when evaluating the accuracy performance. It can be considered in RAN5 test tolerance. Proposal 3: The option 2 should be adopted for test coverage in term of scenarios for NTN RRM test cases, i.e. • The RRM test should consider both LEO and GEO scenario, which can be determined by case-by-case basis. • One neighbour satellite is modelled in the tests besides the serving satellite. Proposal 4: These notes it not needed due to satellite UE should supporting both GSO and NGSO. The test case can be defined based on NGSO for the GSO-based test cases can be skipped. Some GSO-based test cases should be used for verifying UE support GSO and for verifying performance more accuracy. Proposal 5: It is agreed that NTA,common = F3+F4 and NTA,UE-specific = S3 + S4 for reference timing for UE transmit timing. Proposal 6: In test for UE transmit timing error, the reference propagator model need not considered. Proposal 7: NTA,common is configured in the test cases by broadcasted parameters of TACommon, TACommonDrift and TACommonDriftVariation based on satellite ephemeris and position assumption of the uplink time synchronization reference point, i.e. prefer option 1A. |
| R4-2211959 | Xiaomi | Proposal 1: 0.5dB is relaxed based on existing SS-RSRP accuracy requirements for NTN measurement. Proposal 2: In test of UE timing requirements, the following updates should be included based on the existing test requirement: – The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN. – The clarification of NTA,common and NTA,UE-specific are needed, which are: ◦ The NTA,UE-specific and NTA,common are ideal value, no estimation or calculation error will be included. ◦ Reference timing for NTA,UE-specific and NTA,common is the slot when UL transmission is supposed to arrive at the target satellite based on true satellite position. |
| R4-2211960 | Xiaomi | CR on measurement accuracy requirement for NTN |
| R4-2211961 | Xiaomi | Test case list for NTN RRM requirements |
| R4-2211962 | Xiaomi | CR on UE conditions for measurement performance requirements for NTN |
| R4-2212182 | Qualcomm Incorporated | Numerology and Duplex mode Proposal 1: Test configurations for TDD and numerology higher than 15kHz are not considered. Serving and Neighbor Satellite configurations Proposal 2: RAN4 to define the following NTN specific configurations for GEO and LEO at 600km altitude in a common section, which can be referred to from NTN RRM test cases: • Common configurations for Serving and Neighbor satellites, e.g. o SSC.1 and SSC.2 are Serving Satellite Configurations for GSO and NGSO, respectively. o NSC.1 and NSC.2 are Neighbor Satellite Configurations for GSO and NGSO, respectively. o For NGSO, a deployment of quasi-earth fixed cell is assumed. o For each set of configurations, NTN specific parameters in System information shall be included. o UE specific NTN parameters, if needed, are separately defined. • Cells belonging to the same satellite can have different parameters for, e.g. o Koffset, Kmac, beam footprint information, cell service time, etc. • FFS on exact values and parameters o The values should be derived based on a realistic satellite constellation with respect to UE position • In order to alleviate the impact of uncertainty on UE uplink transmission timing due to quantized feeder link delay information, the feeder link delay can be set to zero. Proposal 3: In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position. • For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model. • The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized. • The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models. Test applicability rules Proposal 4: RAN4 to define the following test applicability rule in a common section that applies to NTN RRM test cases. • For all test case: o Satellite information (included in System information) shall remain the same during the test, e.g. Validity timer should be larger enough. o UE GNSS information shall remain the same during the test, i.e. UE mobility is not considered unless an exceptional case is identified, e.g. distance-based conditional handover. o The overall overhead ratio due to scheduling restriction caused by all configured SMTCs (e.g. scheduling restriction overhead of all SMTCs in one periodicity / SMTC periodicity) shall be less than or equal to 75%. • For neighbor cell measurement test cases: o Ephemeris o Epoch time o Feeder link propagation delay o Validity timers o Downlink polarization information • For handover test cases: o Ephemeris o Epoch time o Feeder link propagation delay o Validity timer o Downlink and uplink polarization o Koffset o Kmac • If UE capable of both GSO and NGSO passes a test case for NGSO, the corresponding test case for GSO is skipped. NTN specific Timelines (HARQ, MAC) Proposal 5: For those test cases that verify UE performance in terms of latency and interruption based on UE UL transmission, the following NTN specific timeline shall be correctly reflected: • Koffset (to ensure the causality between DL/UL scheduling and UL transmission) • Kmac (to ensure a synchronous MAC update timeline between DL and UL at SRP, as well as to postpone the corresponding timer and monitoring window) |
| R4-2212898 | Nokia, Nokia Shanghai Bell | Proposal 1: Conventional handover should be used as baseline when evaluating the handover performance in NTN. Proposal 2: RAN4 should decide to keep the measurement accuracy at least as accurate as in terrestrial networks and should consider tightening the requirements further. |
| R4-2212194 | LG Electronics Inc. | Proposal 1: For evaluating measurement accuracy, introduce margin for propagator model error. - Proposal 2: To verify time based cell reselection measurement, the signal level of active cells should be larger than Srxlev or Squal threshold in the test design. - Proposal 3: To verify reporting of propagation delay difference between serving and neighbor cells, the intra-frequency measurement accuracy test should consider updating SMTC configuration according to the reported propagation delay difference. |
| R4-2212399 | MediaTek inc. | Proposal 1: Clarify the test cases for GSO are also applicable for GEO. Proposal 2: Clarify the test cases for NGSO are applicable for LEO and MEO but not GEO. Proposal 3: At least one TC under each requirement group is defined for NGSO/LEO. Proposal 4: For cell reselection to FR1 intra/inter-frequency NR cell, TCs are defined for both GSO and NGSO. Proposal 5: For RLM/BFD, select either non-DRX or DRX of each sub-test to test. Proposal 6: For inter-frequency measurement TC, two fixed Doppler shifts are defined, one for the serving cell and one for the neighbouring cell. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled. Two overlapped SMTCs are configured. Proposal 7: For inter-frequency measurement, introduce TC for the case of one MG. |
| R4-2212882 | Nokia, nokia Shanghai Bell | Proposal 1: To include, in the list of test configurations, reference scenarios for emulation of common delay parameters for NGO and NGSO. Proposal 2: To include, in the list of test configurations, reference scenarios for emulation of ephemeris information for NGO and NGSO. |
| R4-2213354 | Ericsson | Proposal 1: RRM test cases shall consider GEO and LEO both simultaneously, otherwise tests on UE which is capable to support GEO or LEO, especially to UE supporting GEO only, are incomplete. But applicability rules for UE supporting both LEO and GEO can be defined. Proposal 2: Test cases shall take multi-SMTC and multi-satellite tests into account. |
| R4-2213523 | Huawei, HiSilicon | Proposal 1: RAN4 to discuss the assumption on the time and frequency error and the side condition related to ephemeris information for the accuracy requirements. Inputs from the satellite system vendors are appreciated. Proposal 2: RAN4 to define orbit parameters for the virtual satellites as RMC in 38.133, e.g. in the format of ephemeris information. Other parameters, if required, can be also defined separately. Proposal 3: For RRM tests, the transmission timing, frequency and power of the TE should be aligned with the RMC for the virtual satellite. Proposal 4: RAN4 not to define the following TCs for NTN. - Random access - PL-RS switching - Intra-frequency measurement with gap - Inter-frequency measurement without gap Proposal 5: As a generic principle, RAN4 to define TCs for both GEO and LEO. Further discuss if the TCs for GEO and LEO should be captured in the same or separate clauses. Proposal 6: UE supporting both GEO and LEO is only required to pass the test for LEO. |
| R4-2213524 | Huawei, HiSilicon | CR on measurement accuracy requirements for NTN |
| R4-2214060 | Ericsson | - Observation 1: Conditions are defined for bands in terms of minimum SSB_RP and minimum CSI-RS_RP at corresponding SSB Es/Iot and CSI-RS Es/Iot respectively for different SCSs. - Observation 2: SSB Es/Iot and CSI-RS Es/Iot side conditions for the same type of measurement requirements are also the same for satellite access and terrestrial network. - Proposal #1: The conditions (minimum SSB_RP and minimum CSI-RS_RP) for satellite bands n255 and n256 are the same as defined for band n1 for corresponding measurements. - Proposal #2: The conditions for satellite bands n255 and n256 are defined in new sections in annex B of TS 38.133. |
| R4-2214061 | Ericsson | The draft CR defines conditions for the applicability of RRM requirements for satellite access bands in 38.133 |
| R4-2211639 | CATT | Test cases for Intra- and inter-frequency HO with known cell for NTN |
| R4-2211640 | CATT | Test cases for Intra- and inter-frequency CHO for NTN |
| R4-2211963 | Xiaomi | CR on test case for cell reselection to FR1 intra-frequency NR cell for satellite access |
| R4-2212298 | CMCC | Proposal 1: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. Proposal 2: For the test requirement, several updates should be included based on current test requirement: - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN. - The clarification of NTA,common and NTA,UE-specific are needed, which are: - The NTA,UE-specific and NTA,common used by test system will not include UE GNSS estimation error and satellite positioning error from UE calculation. |
| R4-2212381 | LG Electronics UK | Proposal 1: The test procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused. Proposal 2: RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition. Proposal 3: Study whether to define the measurement relaxation test case for GEO. |
| R4-2212400 | MediaTek inc. | Introduction of test cases for Inter-frequency measurement delay for satellite access with gap |
| R4-2212401 | MediaTek inc. | Introduction of test cases for Accuracy for SS-RSRQ for satellite access |
| R4-2212883 | Nokia, nokia Shanghai Bell | Proposal 1: Design the test case for intra-frequency and inter frequency cell reslection to evaluate the UE response to “t-service” configuration. Include in the test case both trigger cases: “t-service+10” and “T1+10”. Proposal 2: Discuss whether to include “K_multi” parameter in the tests, and how to evaluate for it. Proposal 3: Include a test case where the UE inter-frequency cell reselection criteria is triggered by the distance threshold. Proposal 4: Clarify if Dmargin is an accuracy requirement or effectively a margin to be added to the distance threshold. If Dmargin is an accuracy requirement, it should be explicitly specified as such. |
| R4-2212884 | Nokia, Nokia Shanghai Bell | Proposal 1: For UE supporting both GSO and NGSO, UE is only required to pass either the NGSO-based test cases or the GSO-based test cases. Proposal 2: For UE supporting only GSO, the NGSO-based test cases can be skipped. |
| R4-2212885 | Nokia, Nokia Shanghai Bell | Proposal 1: RAN4 to discuss the “fully synchronized” criteria for UE testing regarding ephemeris and validity timer acquisition. Proposal 2: RAN4 to discuss the ephemeris validity timer to be used during test procedures, so that the UE timing alignment is maintained during the test. |
| R4-2213352 | Ericsson | draft CR on test cases for Beam Failure Detection and Link Recover for NTN |
| R4-2213353 | Ericsson | draft CR on test cases for L1-RSRP measurement delay for NTN |
| R4-2213356 | Ericsson | Proposal 1: Regarding test cases for Cell reselection to intra- and inter-frequency neighbor cell without time or location trigger, test cases can refer to legacy TN test cases. Proposal 2: Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra defined in A.6.1.1.1.3) for reselection. Proposal 3: Regarding test cases for reselection measurement based on location trigger, the time instant fulling DdistanceThresh is at T3 (defined in A.6.1.1.1.3) to examine whether UE can start measurements after DdistanceThresh. Proposal 4: Define test cases for relaxed measurement requirements on reselection in GEO scenario. |
| R4-2213357 | Ericsson | Proposal 1: Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2. Proposal 2: Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure ). Proposal 3: Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2. Proposal 4: RAN4 shall check the necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario. |
| R4-2213358 | Ericsson | Proposal 1: Regarding test cases for Intra- and inter-frequency HO with known cell, test cases can refer to legacy TN test cases. |
| R4-2213359 | Ericsson | Proposal 1: Add new contents for definition of 2 SMTCs per MO, 2 satellites (i.e. 2 SSBs) per SMTC, 2 MGs in clause A.3 RRM test configuration. |
| R4-2213475 | Huawei, HiSilicon | Proposal 1: It is suggested to define the test principles for NTN UE timing related test cases - The UE supporting NTN features only in GSO scenario need to be tested in GSO scenario - The UE supporting NTN features only in NGSO scenario need to be tested in NGSO scenario - The UE supporting NTN features in both GSO and NGSO scenarios need to be tested in NGSO scenario need to be tested in either GSO or NGSO scenario. Proposal 2: Two types of reference NTN configuration need to be introduced for GSO and NGSO scenarios respectively. - Reference NTN configuration 1 provides the typical NTN configuration in GSO scenario. - Reference NTN configuration 2 provides the typical NTN configuration in NGSO scenario. Proposal 3: RAN4 shall decide which format, format of position and velocity state vector or format of orbital parameters, will be used to provide satellite ephemeris in reference NTN configuration. Proposal 4: For NTN UE timing related tests, GNSS signals need to be simulated for UE positioning and the test parameters for GNSS signals defined for V2X tests can be reused. |
| R4-2213476 | Huawei, HiSilicon | DraftCR on UE transmit timing tests for NTN |
| R4-2213525 | Huawei, HiSilicon | CR on TCs for SSB based RLM for NTN |
| R4-2213526 | Huawei, HiSilicon | CR on TCs for intra-frequency measurement delay for NTN |
| R4-2213527 | Huawei, HiSilicon | CR on TCs for RRC Re-establishment for NTN |
| R4-2213528 | Huawei, HiSilicon | CR on TCs for RSRP accuracy for NTN |
| R4-2212184 | Qualcomm Incorporated | draft CR of BWP switch and CBW change test cases |

Open issues summary and Companies views’ collection for 1st round

Measurement Accuracy requirement for NTN

Issue 1-1: Margin assumption for evaluating measurement accuracy.

- Option 1: (CATT)

  - The margin for propagator model error should not be introduced when evaluating the accuracy performance. It can be considered in RAN5 test tolerance.

- Option 2: (LGE)

  - For evaluating measurement accuracy, introduce margin for propagator model error.

- Option 3: (Huawei)

  - RAN4 to discuss the assumption on the time and frequency error and the side condition related to ephemeris information for the accuracy requirements. Inputs from the satellite system vendors are appreciated.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 2 and Option 3. Here, the source of the additional error/uncertainty is different from what RAN5 typically deals with. RAN5 introduces test tolerance mainly due to inaccuracy of test equipment and so on. Here, in NTN, the uncertainty is resulting from the provided ephemeris information is only true at the tangential coordinates when the information is generated (i.e. at epoch time) apart from the quantization error. The periodicity of the ephemeris information update is up to NW implementation, and the projection error increases proportionally to the actual time gap from the epoch time to actual UL slots. In order for RAN4 to analyze the impact on the required margin, we need at least some input from satellite companies and a reference propagator model. RAN5 can add an additional margin if they identify testability issues, e.g. time-varying propagation delay change, time/frequency error and sampling clock offset cannot be accurately modeled in DL transmission in test box. |
| Xiaomi | Fine with option 2 and option 3, in my understanding, the propagator model error will cause the time and frequency error which will impact the RSRP measurement, thus, this error need to be considered for accuracy evaluation. |
| Ericsson | For sake of simplicity on setting test parameters, we slightly prefer Option1. Finding a reasonable propagator model error may not work. |
| Apple | We are fine with option 1 and option 3. The margin shall be considered in the testing tolerance based on information from Satellite Infra vendors, and where to reflect such tolerance can be either RAN4 or RAN5. |
| Huawei | Support P2 and P3. Besides the propagation error, we would also like to discuss whether the inaccuracy of the ephemeris information needs to be considered. We do agree that the inaccuracy of the ephemeris information has nothing to do with the UE performance, and it is not something UE can help, but the requirements should be defined for typical conditions, i.e. if in the real world the inaccuracy of ephemeris information is unavoidable and can cause large T/F error (thus has large impacts on the accuracy), then the imperfection should be considered in side condition and accuracy. To determine the assumption on the time and frequency error (including both the inaccuracy of the ephemeris information and propagation error), we understand some inputs from the satellite system vendors are needed. |
| LGE | Prefer option 2. In RAN4, the margin needs to be considered due to propagator model error. The test tolerance in RAN5 is for inaccuracy/error/uncertainty of test procedure/model/equipment, but the propagator model error is not caused by such test error. So, we think the error margin can be introduced in RAN4. |
| MTK | Fine with option 2 and option 3 to avoid misunderstanding on the test margin. |
| CMCC | For accuracy requirements, we are fine with Option 2 and Option 3. |
| CATT | For proposal 1, our intention is only for UE transmit timing error performance in our discussion paper. For proposal 2&3 and also the relaxed margin in Issue 1-2, there is no such evidence the additional margin is needed. |
| THALES | Option 3 and Option 2 are fine. In Release-17, the UE should be able to pre-compensate the UL transmission based on ephemeris data and GNSS. The question for testing purposes is: “Is the precision/accuracy fine within the given requirements?” The test should be as much realistic as possible, taking into account realistic Doppler shift and required TA values. In any case, when it comes to UE, different UEs can use different pre-compensation algorithms, and we need to be sure that all UEs will react similarly to the test environment and in real conditions, with respect to given requirements. We also think some reference propagator model is required and should be considered. During previous meeting we proposed Eckstein Hechler (J6). See also below a comparison: Orbit propagation model comparison Maximum errors. Without fitting; Model; Duration of prediction (minute); DP (m); UE specific TA Error (µs); DV (m/s); Doppler error (Hz). Kepler; 3; 261; 1.74; 2.64; 17.60. Eckstein Hechler (J6); 21; 303; 2.02; 0.48; 3.20. 2x2 num; 22,5; 301.4; 2.01; 0.42; 2.80. 6x6 num; 27; 302.5; 2.02; 0.44; 2.93. Maximum errors. Model; Duration of prediction (second); DP (m); UE specific TA Error (µs); DV (m/s); Doppler error (Hz). Without fitting; Kepler; 45; 43.4; 0.29; 0.689; 45.93. Eckstein Hechler (J6); 300; 43.4; 0.29; 0.093; 6.20. 2x2 num; 300; 44.3; 0.30; 0.1; 6.67. 6x6 num; 350; 44.2; 0.29; 0.084; 5.60. With fitting; TLE; 370; 44,1; 0.29; 0.1; 6.67. Eckstein Hechler (J6); 390; 44,2; 0.29; 0.083; 5.53. 2x2 num; 375; 44; 0.29; 0.092; 6.13. 6x6 num; 395; 44,2; 0.29; 0.079; 5.27. Based on simulation results, prediction 45 s ahead for UE pre-compensation is accurate within 0.29 µs for round trip delay error and within 45.93 Hz for Doppler error when Kepler model is used. This is sufficient to pre-compensate even at 120 kHz. |

Issue 1-2: Measurement accuracy.

- Option 1: (Xiaomi)

  - 0.5dB is relaxed based on existing SS-RSRP accuracy requirements for NTN measurement.

- Option 2: (Nokia)

  - RAN4 should decide to keep the measurement accuracy at least as accurate as in terrestrial networks and should consider tightening the requirements further.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Oppose Option 2. A bit in favor of Option 1 for now. Depending on whether and how to adapt DL transmission power in test configurations, there would be an issue too. |
| Xiaomi | Option 1, considering the propagator error, the accuracy requirements need to be relaxed, FFS the exact relaxed value. |
| Ericsson | We understand the motivation of Option 1 and Option 2, but it may rely on outcome of Issue 1-1. Regarding Option2, we assume it is a motivation to improve accuracy and shall be further studied. Maybe it’s difficult to request higher accuracy in Rel-17 now. the HO rate may be improved in next release. |
| Apple | We also think it’s up to the conclusion from issue 1-1, and don’t understand why 0.5dB is selected in option 1. |
| Huawei | Suggest FFS or go with option 1. We agree with E/// that the exact accuracy depends on outcome of issue 1-1. While we can understand the motivation of option 2, we believe it is difficult to tighten the accuracy for NTN. Instead, considering the possible errors discussed in issue 1-1, option 1 is a safer choice. On the other hand, we understand there can be some NW implementation based solutions to improve the mobility performance in NTN. |
| LGE | Based on the conclusion of Issue 1-1, we can further discuss. FFS for relaxed accuracy value. |
| MTK | We can consider either keep or relax the accuracy. |
| CMCC | We share similar view with Apple. This issue can be further discussed based on the output of Issue 1-1. |
| CATT | For proposal 1, we don’t understand the explanation in Xiaomi’s discussion paper. The measurement is for DL. But Te is for uplink transmission. DL measurement accuracy has no relationship with uplink. |
| Nokia | For Option 2, legacy or terrestrial measurement accuracy should be used as a baseline. |
| THALES | Preference for Option 1. For Option 2, it depends how good the measurement accuracy is, in realistic conditions the values may deviate from terrestrial. We do not think we should “consider tightening the requirements further”. TBD |

Test case coverage, design and configuration

Issue 2-1: Test cases for GSO and NGSO.

- Option 1: (Ericsson)

  - RRM test cases shall consider GEO and LEO both simultaneously, otherwise tests on UE which is capable to support GEO or LEO, especially to UE supporting GEO only, are incomplete. But applicability rules for UE supporting both LEO and GEO can be defined.

- Option 2: (Huawei, CMCC, Nokia)

  - RAN4 to define TCs for both GEO and LEO. Further discuss if the TCs for GEO and LEO should be captured in the same or separate clauses.

  - UE supporting both GEO and LEO is only required to pass the test for LEO.

- Option 3: (MTK)

  - Clarify the test cases for GSO are also applicable for GEO.

  - Clarify the test cases for NGSO are applicable for LEO and MEO but not GEO.

  - At least one TC under each requirement group is defined for NGSO/LEO.

- Option 4: (Qualcomm)

  - If UE capable of both GSO and NGSO passes a test case for NGSO, the corresponding test case for GSO is skipped.

- Recommended WF

  - RAN4 to define TCs for both GSO and NGSO scenarios.

  - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - FFS the TCs for GEO and LEO should be captured in the same or separate clauses.

| Company | Comments |
| --- | --- |
| Qualcomm | Support the recommended WF. |
| Xiaomi | Support the recommended WF. |
| Ericsson | Agree recommend WF. TC for GEO and LEO can be captured in the same clauses. |
| Apple | Support the recommended WF. |
| Huawei | Support the recommended WF. TC for GEO and LEO can be captured in the same clauses. |
| LGE | Support the recommended WF. |
| MTK | support the Recommended WF. Slightly prefer to capture GSO/NGSO in the same clause, and the benefit would be sharing the common configuration and keep spec concise. Suggest to align the wording "GSO" in TCs, instead of GEO. |
| CMCC | Support the Recommended WF. |
| CATT | Support the first bullet of Recommended WF. For the second bullet, UE should support both GSO and NSO. There is no such capability and UE type, right? Fine with third bullet of Recommended WF. |
| THALES | Fine with the moderator recommended WF. |

Issue 2-2: Test coverage regarding NTN RRM requirements.

- Option 1: (Huawei)

  - RAN4 not to define the following TCs for NTN.

    - Random access

    - PL-RS switching

    - Intra-frequency measurement with gap

    - Inter-frequency measurement without gap

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Xiaomi | For the UE supporting TN and NTN, those test cases can be ignore. For the UE only supporting NTN, e.g. VSAT UE, those test cases need to be introduced for verifying the corresponding requirements. |
| Ericsson | Since test cases for NTN is a new framework literally, we’re concerned that missing some cases may harm integrity of test coverage. |
| Apple | We are not very convinced about the method on how to rule out certain test cases, e.g., for RACH testing, not only Tx timing is verified for preamble transmission but also the preamble Tx power is also verified; how can it be represented by other NTN test cases? We may need a clearer method for test case selection. |
| Huawei | Support option 1. We can understand the consideration on the test coverage, but we also need to consider the testing efforts for NTN UEs, especially when the test is not for NTN specific requirements. We are open to consider what Xiaomi mentioned, e.g. if a UE passes the tests in option 1 for TN, then it does not need to pass the same tests for NTN. |
| MTK | We can support Option 1 or to define the test applicability rule i.e. if has passed the corresponding TN test, than no need to test again. |
| CMCC | We share similar understanding with Ericsson, test coverage should be guaranteed. |
| CATT | We support option 1 at least in Rel-17. |
| Nokia | Share the same view as Ericsson. Differently from previous cases, the UE is now required to apply TA before RA, which means having a validity timer. UE may need to verify if the UE is complied with these requirements. |
| THALES | At least : - Random access - Intra-frequency measurement with gap should be probably considered. TBD |

Issue 2-3: Test coverage regarding numerology and duplex mode.

- Option 1: (Qualcomm)

  - Test configurations for TDD and numerology higher than 15kHz are not considered.

- Recommended WF

  - Test configurations for TDD and numerology higher than 15kHz are not considered.

| Company | Comments |
| --- | --- |
| Qualcomm | Support the recommended WF. |
| Xiaomi | Support the recommended WF. |
| Ericsson | Agree on recommended WF |
| Apple | Can proponent clarify why 30kHz is precluded? No preference from our side but just try to understand the technical reason. In RF sepc, 30kHz is available for n255. |
| Huawei | Support the recommended WF. |
| MTK | Fine with the Recommended WF. |
| CMCC | Fine with the recommended WF. |
| CATT | Fine with no TDD. Why no TC for 30kHz SCS? We have timing requirement for 30kHz. if 30KHz SCS test configuration is needed, we suggest to use it in timing TC. |
| THALES | Fine with WF. Is obvious that TDD is not considered (see 38.108 for instance). Ok for 15 kHz, but in some situations 30 kHz might be useful and the requirements (e.g. in terms of timing errors for instance) are a bit different. |

Issue 2-4: Test coverage regarding multiple SMTCs.

- Option 1: (Ericsson)

  - Test cases shall take multi-SMTC and multi-satellite tests into account.

  - Add new contents for definition of 2 SMTCs per MO, 2 satellites (i.e. 2 SSBs) per SMTC, 2 MGs in clause A.3 RRM test configuration.

- Option 2: (LGE)

  - To verify reporting of propagation delay difference between serving and neighbor cells, the intra-frequency measurement accuracy test should consider updating SMTC configuration according to the reported propagation delay difference.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Not against Option 1 and Option 2. As for Option 1: - The number of satellites doesn’t have to be more than 2 for test purpose. As for Option 2: - We do not yet think a separate test case for PropDelayDiffReport. Even if the test case is defined, we are not quite convinced if “updating SMTC re-configuration” is really necessary for the test purpose. |
| Xiaomi | Option 1: we are fine to have the following configuration for test purpose: - SMTCs per MO configuration to verify multi-SMTC - 2 satellites per SMTC configuration to verify the UE measurement capability on number of LEO per SMTC - 2 MGs configuration to verify multi-SMTC Option 2: since there is no RRM requirement for propagation delay difference, we donnot think we need to test the updating SMTC configuration. |
| Ericsson | We support Option 1 and Option 2, they’re not controversial. We’re open to the exact numbers of SMTCs, satellites and so on. |
| Apple | Agree with Xiaomi’s comments. |
| Huawei | We are fine with option 1. On option 2, we are not sure if it is necessary to have SMTC reconfiguration during the test. If the intention is to verify the PD estimation, we assume it can be achieved by other tests, e.g. Tx timing. |
| LGE | For option 1, we are fine with Xiaomi’s clarification. For option 2, in intra-frequency measurement, SMTCs for neighbor cells are configured based on UE’s delay difference report when UE in RRC_CONNECTED state performs intra-frequency measurement. We think it is important UE behavior for multiple SMTC configuration, and there will be no separated test to verify reporting delay difference. So, we’d like to consider the SMTCs configuration using UE’s delay difference report should be considered in accuracy test. |
| MTK | Agree Option 1 in general. |
| CMCC | Fine with Xiaomi’s proposal on Option 1. For Option 2, it looks like a combined test for UE and network. We are not sure about what is the test requirement. |
| CATT | Support option 1. For option 2, there is no reporting delay value test. We don’t understand how this test is. |
| Nokia | Option 1 is fine. |
| THALES | Ok for Option 1, |

Issue 2-5: Serving and Neighbour Satellite configurations.

- Option 1: (Qualcomm)

  - RAN4 to define the following NTN specific configurations for GEO and LEO at 600km altitude in a common section, which can be referred to from NTN RRM test cases:

    - Common configurations for Serving and Neighbor satellites, e.g.

      - SSC.1 and SSC.2 are Serving Satellite Configurations for GSO and NGSO, respectively.

      - NSC.1 and NSC.2 are Neighbor Satellite Configurations for GSO and NGSO, respectively.

      - For NGSO, a deployment of quasi-earth fixed cell is assumed.

      - For each set of configurations, NTN specific parameters in System information shall be included.

      - UE specific NTN parameters, if needed, are separately defined.

    - Cells belonging to the same satellite can have different parameters for, e.g.

      - Koffset, Kmac, beam footprint information, cell service time, etc.

    - FFS on exact values and parameters

      - The values should be derived based on a realistic satellite constellation with respect to UE position

    - In order to alleviate the impact of uncertainty on UE uplink transmission timing due to quantized feeder link delay information, the feeder link delay can be set to zero.

- Option 2: (Huawei)

  - Two types of reference NTN configuration need to be introduced for GSO and NGSO scenarios respectively.

    - Reference NTN configuration 1 provides the typical NTN configuration in GSO scenario.

    - Reference NTN configuration 2 provides the typical NTN configuration in NGSO scenario.

  - RAN4 shall decide which format, format of position and velocity state vector or format of orbital parameters, will be used to provide satellite ephemeris in reference NTN configuration.

- Option 3: (Nokia)

  - To include, in the list of test configurations, reference scenarios for emulation of common delay parameters for NGO and NGSO.

  - To include, in the list of test configurations, reference scenarios for emulation of ephemeris information for NGO and NGSO.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Proposals do not seem mutually exclusive. To accommodate all points, maybe we can add the following to Option 1 and call it Option 1a. - Ephemeris information in SSC.1 and SSC.2 is in format of position and velocity state vector - Ephemeris information in NSC.1 and NSC.2 is in format of orbital parameters - For those test cases where UE is required to transmit UL to a cell in a neighbour satellite, ephemeris information of the satellite in format of position and velocity state vector shall be additionally provided. |
| Xiaomi | We can take option 1 as baseline to define the GEO and LEO configuration for RRM tests. |
| Ericsson | As per our understanding, Option 1, 2, 3 are fine with us. |
| Apple | Fine with option 1/2/3 and 1a. |
| Huawei | All the 3 options are aligned, and we are fine to take option 1 as starting point, except that we need more time to check whether to use quasi-earth fixed cell for NGSO tests. On option 1a suggested by QC above, we are not sure if we should use different formats for ephemeris for serving and neighbor satellites – would this be a typical case? |
| MTK | Support Option 1. Regarding the Ephemeris format, slightly prefer to format of position and velocity state vector, which is more straightforward to use. And fine to select some test to use orbital parameters. Besides, the Ephemeris of the target satellite should be provided (as issue 2-6), not only for the TC where UE is required to transmit UL to a cell in a neighbour satellite. |
| CMCC | We are fine to take Option 1 and Option 1a as the starting point. |
| CATT | Fine with option 1 as the starting point. Fine with option 2&3. |
| Nokia | Option 1a is fine if GSO and NGSO are taken into account. |
| THALES | TBD |

Issue 2-6: Test applicability rule for NTN RRM test cases.

- Option 1: (Qualcomm)

  - RAN4 to define the following test applicability rule in a common section that applies to NTN RRM test cases.

    - For all test case:

      - Satellite information (included in System information) shall remain the same during the test, e.g. Validity timer should be larger enough.

      - UE GNSS information shall remain the same during the test, i.e. UE mobility is not considered unless an exceptional case is identified, e.g. distance-based conditional handover.

      - The overall overhead ratio due to scheduling restriction caused by all configured SMTCs (e.g. scheduling restriction overhead of all SMTCs in one periodicity / SMTC periodicity) shall be less than or equal to 75%.

    - For neighbor cell measurement test cases:

      - Ephemeris

      - Epoch time

      - Feeder link propagation delay

      - Validity timers

      - Downlink polarization information

    - For handover test cases:

      - Ephemeris

      - Epoch time

      - Feeder link propagation delay

      - Validity timer

      - Downlink and uplink polarization

      - Koffset

      - Kmac

    - If UE capable of both GSO and NGSO passes a test case for NGSO, the corresponding test case for GSO is skipped.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 1. The second and third bullets are in line with RAN4 agreement made in one of the previous meetings, hence, not arguable. And the same goes for the third sub-bullet of the first main bullet. The first and second sub-bullets of the first main bullet are to alleviate the impact of test uncertainty. |
| Xiaomi | Fine with option1 |
| Ericsson | Generally OK with Option 1 except for the last bullet. |
| Apple | Fine with option 1. |
| Huawei | Fine with option 1 |
| LGE | Generally fine with the option 1. |
| MTK | Fine with Option 1. |
| CMCC | We are fine with Option 1. |
| CATT | Fine with option 1. |
| THALES | Fine with option 1. |

Issue 2-7: RMC for virtual satellite.

- Option 1: (Huawei)

  - RAN4 to define orbit parameters for the virtual satellites as RMC in 38.133, e.g. in the format of ephemeris information. Other parameters, if required, can be also defined separately.

  - For RRM tests, the transmission timing, frequency and power of the TE should be aligned with the RMC for the virtual satellite.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | The Option 1 can be discussed as part of Issue 2-5. |
| Apple | Agree with QC to discuss together in issue 2-5. |
| Huawei | Agree with QC that option 1 is somehow already covered by issue, and no need to have separate discussion. Perhaps the unique point is that the TE transmission should be aligned with the RMC for the virtual satellite, and we assume this should not be controversial. |
| CMCC | We support the unique point which is pointed out by Huawei, TE transmission should be aligned with the RMC for the virtual satellite |

Issue 2-8: Koffset and Kmac configurations.

- Option 1: (Qualcomm)

  - For those test cases that verify UE performance in terms of latency and interruption based on UE UL transmission, the following NTN specific timeline shall be correctly reflected:

    - Koffset (to ensure the causality between DL/UL scheduling and UL transmission)

    - Kmac (to ensure a synchronous MAC update timeline between DL and UL at SRP, as well as to postpone the corresponding timer and monitoring window)

- Recommended WF

  - The Koffset and Kmac should be additional taken into account according to RAN1 spec if they are involved in the test procedure and test requirements.

| Company | Comments |
| --- | --- |
| Qualcomm | UE requirements described in test procedures and defined in the requirement section for each test case shall correctly reflect Option 1 according to RAN1/2 spec. |
| Xiaomi | Support the recommended WF and it can be discussed together with issue 2-5 |
| Apple | Fine with recommended WF. |
| Huawei | Fine with the recommended WF. |
| MTK | Fine with the Recommended WF. |
| CMCC | Fine with the Recommended WF. |
| CATT | Fine with the Recommended WF |
| THALES | Fine with the Recommended WF |

Test case design for Cell reselection

Issue 3-1: General test design for intra/inter-frequency cell reselection.

- Option 1: (MediaTek)

  - For cell reselection to FR1 intra/inter-frequency NR cell, TCs are defined for both GSO and NGSO.

- Option 2: (Nokia)

  - Design the test case for intra-frequency and inter frequency cell reslection to evaluate the UE response to “t-service” configuration. Include in the test case both trigger cases: “t-service+10” and “T1+10”.

  - Discuss whether to include “K_multi” parameter in the tests, and how to evaluate for it.

- Option 3: (LGE)

  - RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition.

  - To verify time based cell reselection measurement (time based), the signal level of active cells should be larger than Srxlev or Squal threshold in the test design.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Xiaomi | Option 1: since this test principle applies to all RRM test cases, there is no need to have additional consensus on cell reselection test case. In general fine with the points in option 2 and option 3 |
| Ericsson | Support Option 1, 2 and 3. K_multi in Option 2 can follow multi-satellite test configurations. |
| Apple | Fine with option 1 and 3. For option 2, is the “Include in the test case both trigger cases: “t-service+10” and “T1+10” meant to verify the UE behavior of initiating cell selection? Since we don’t have requirement for initial cell selection, we think it’s not necessary to have such condition in the test case. |
| Huawei | Option 1 is covered in issue 2-1. On option 2, we assume the TCs are for cell reselection, and we agree that UE triggering reselection measurement based on t-Service should be verified, but “t-service+10” and “T1+10” are for cell selection, and we understand it is not tested in existing test cases for TN. On option 3, the first bullet is covered in issue 2-5, and we are fine with the second bullet. |
| LGE | Fine with option 1 and option 3 to consider test case design. For option 2, we have similar view with Apple, and further clarification is needed. |
| MTK | Fine with Option 1, 3 in general. On Option 2, our understanding is "the UE shall initiate cell selection procedures….within 10 s since time instance T1 provided that ‘t-Service’> T1 ", which doesn't seem a part of a cell reselection test. |
| CMCC | Generally, we are fine with Option 1,2 and 3. For the test setup of time-based cell-reselection, we think further clarification is needed. |
| CATT | Fine with option 1/2/3.. For the proper signal power, since the RF receiver sensitive level is the same as in TN system, we think the similar values can be use. |
| THALES | Fine with options 1 and 3. |

Issue 3-2: Test case for intra/inter-frequency cell reselection without timer or location trigger.

- Option 1: (LGE)

  - The test procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused.

- Option 2: (Ericsson)

  - Regarding test cases for Cell reselection to intra- and inter-frequency neighbor cell without time or location trigger, test cases can refer to legacy TN test cases.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | At least NTN specific information shall be provisioned. |
| Xiaomi | Fine with option 1 and option 2 |
| Ericsson | Agree on Option 1 and 2. |
| Apple | Fine with option 1 and 2. |
| Huawei | Fine with option 1 and option 2 |
| LGE | Support option 1 and 2. |
| MTK | Fine with Option 1 and Option 2. |
| CMCC | Fine with Option 1 and Option 2 |
| CATT | Fine with option 1 and option 2. |
| Nokia | Support both options. |
| THALES | Agree with any of the options. However, which is the difference? |

Issue 3-3: Test case for intra/inter-frequency cell reselection with timer trigger.

- Option 1: (MediaTek)

  - For cell reselection to FR1 intra/inter-frequency NR cell, TCs are defined for both GSO and NGSO.

- Option 2: (Ericsson)

  - Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra  defined in A.6.1.1.1.3) for reselection.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Xiaomi | Option 1: since this test principle applies to all RRM test cases, there is no need to have additional consensus on cell reselection test case. Fine with option 2 |
| Apple | Agree with option 1 and 2. |
| Huawei | Option 1 is covered in issue 2-1. One comment is that t-Service is only applicable for LEO fixed cell, so we assume the test is not applicable for GEO. Option 2 is fine. |
| LGE | Fine with both options. |
| MTK | Agree with Option 1 and Option 2. |
| CMCC | Generally fine with both options. We think the wording can be revised to “time period between the end point of T3 (refer to A.6.1.1.1.3) and Tserivce” is more accurate, since T3 is a time period. |
| CATT | GSO in option 1 is not necessary. Fine with option 2. |
| Nokia | Support both options. |

Issue 3-4: Test case for intra/inter-frequency cell reselection with location trigger.

- Option 1: (Nokia)

  - Include a test case where the UE inter-frequency cell reselection criteria is triggered by the distance threshold.

  - Clarify if Dmargin is an accuracy requirement or effectively a margin to be added to the distance threshold.  If Dmargin is an accuracy requirement, it should be explicitly specified as such.

- Option 2: (Ericsson)

  - Regarding test cases for reselection measurement based on location trigger, the time instant fulling DdistanceThresh is at T3 (defined in A.6.1.1.1.3) to examine whether UE can start measurements after DdistanceThresh.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Apple | For option 1, we think Dmargin is a margin to add to the distance threshold. Fine with option 2, but additional time period shall be considered for UE to response when DdistanceThresh condition is met, i.e., TE changed the distance between UE and serving cell, but UE needs response time to ‘realise’ that change, e.g., GNSS response time and time margin for information delivery from GNSS to NTN module. |
| Huawei | We are fine with both options. On Dmargin, we understand in the test setup, the UE GNSS location should be set to a value such that the distance threshold is exceeded by 50m. |
| LGE | Option 1 Dmargin should not be added to the distance threshold since the threshold is configured by NW. |
| Nokia | Support both options. |

Issue 3-5: Test case for the relaxed intra/inter-frequency cell reselection requirements in GEO scenario.

- Option 1: (LGE)

  - Study whether to define the measurement relaxation test case for GEO.

- Option 2: (Ericsson)

  - Define test cases for relaxed measurement requirements on reselection in GEO scenario.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Xiaomi | Fine with option 2 |
| Ericsson | Relaxation for GEO shall be tested. |
| Apple | Option 1. Can we refer the test case to TN relaxation? |
| Huawei | Support option 1, we do not see big value in testing relaxed measurement requirements for NTN. |
| LGE | We don’t have strong view. But, since legacy measurement relaxation is reused in NTN, it may not need to define measurement relaxation test for GEO. Also, in our understanding, in RAN2 specification, UE measurement relaxation capability for NTN is not defined yet. So, we think relaxed measurement test for GEO can be skipped (or replaced by TN test). |
| CMCC | Support Option 2. |
| CATT | Support option 2. |
| Nokia | Support option 2. |

Test case design for handover

Issue 4-1: Test case for intra/inter-frequency handover with known cell for NTN.

- Option 1: (Ericsson)

  - Regarding test cases for Intra- and inter-frequency HO with known cell, test cases can refer to legacy TN test cases.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | At least NTN specific information shall be provisioned. |
| Xiaomi | Fine with option 1 |
| Apple | Fine with option 1 with QC’s comment. |
| Huawei | Fine with option 1 |
| MTK | Agree with Option 1 with QC’s comment. |
| CMCC | Fine with Option 1 and QC’s comment |
| CATT | Fine with option1. |
| Nokia | Support option 1. |
| THALES | Agree with Qualcomm. |

Issue 4-2: Test case for CHO with time-based or location-based condition.

- Option 1: (Ericsson)

  - Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2.

  - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure ).

  - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2.

  - RAN4 shall check the necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Xiaomi | Fine with option 1 |
| Ericsson | The basic idea of the last bullet is to verify if time/location information really triggers CHO. The details can be studied. |
| Apple | Fine with option 1. |
| Huawei | Fine with the first 3 bullets. Not sure if we need to further consider the last bullet, in TN CHO tests, we did not consider the case where HO condition is not met. |
| CATT | For first bullet, we prefer to reuse current T2 usage. T2 is the start point of signal power condition. RRC message should be sent in T1. Fine with bullet #2 and 3. For the last bullet, prefer not to add such TC. |
| Nokia | Support option 1. |
| THALES | Fine with Option 1. |

Test case design for UE timing requirements

Issue 5-1: Test principle for NTN UE timing related test cases.

- Option 1: (Huawei, Nokia)

  - The UE supporting NTN features only in GSO scenario need to be tested in GSO scenario

  - The UE supporting NTN features only in NGSO scenario need to be tested in NGSO scenario

  - The UE supporting NTN features in both GSO and NGSO scenarios need to be tested in NGSO scenario need to be tested in either GSO or NGSO scenario.

- Option 2: (CMCC)

  - The UE supporting NTN features only in GSO scenario need to be tested in GSO scenario

  - The UE supporting NTN features only in NGSO scenario need to be tested in NGSO scenario

  - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 2. |
| Xiaomi | Option 2, since this test principle applies to all RRM test cases, there is no need to have additional consensus on UE timing test cases. |
| Ericsson | Most of cases can be tested in LEO scenario, unless necessity of GEO test is proved, for UE supporting GEO and LEO both. However for GEO RTT is particular, since very large. This makes it hard to skip GSO, for this case, at least. |
| Apple | Option 2. |
| Huawei | We are fine with option 2 if it is aligned with the test principle for other RRM tests. |
| LGE | Support option 2 |
| MTK | Prefer to Option 2, and good to agree it as the general principle |
| CMCC | We support Option 2. |
| CATT | We want to confirm with companies is there the capability for only supporting GSO or NGSO firstly. Timing Advance adjustment accuracy is better to be tested in GSO. In this test, UE and satellite are not moving. It is more accuracy. For other requirements, They can be tested in NGSO. |

Issue 5-2: GNSS signal for NTN UE timing related test cases.

- Option 1: (Huawei)

  - For NTN UE timing related tests, GNSS signals need to be simulated for UE positioning and the test parameters for GNSS signals defined for V2X tests can be reused.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Which specific GNSS parameters can be borrowed from V2X? UE position information that can be obtained from GNSS fix should take into account parameters of satellite constellation and beam footprint. E.g. elevation angles between satellites and UE should be based on some realistic scenarios. |
| Xiaomi | Fine with option 1, the test parameter for GNSS signal is defined in B.4.1 This clause defines the reference signal power levels of generated salellites for a corresponding GNSS, which will be used in V2X test cases. Table B.4.1-1: GNSS Referenece Signal Power Parameters System; Parameters; Unit; Value. Number of generated satellites per system; -; 6. GPS(1); Reference signal power level for all satellites; dBm; -128.5. Galileo; Reference signal power level for all satellites; dBm; -127. GLONASS; Reference signal power level for all satellites; dBm; -131. BDS; Reference signal power level for all satellites; dBm; -133. NOTE 1: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. NOTE 2: The DUT UE does not need to support all systems. The DUT UE shall support at least one system and will be test for the supported systems.. |
| Apple | Fine with option 1 for signal power assumption, for other parameters, we are wondering if can borrow from 38.171. |
| Huawei | Option 1. The UE positioning error assumed for Te_NTN is developed from Spec TS38.171. As mentioned by Xiaomi, GNSS parameters for V2X is defined in B.4.1, which is also developed from Spec TS38.171. |
| LGE | Fine with option 1 and it might be signal power parameters as captured by Xiaomi. |
| MTK | This issue may also be discussed in RF session, regarding the UE location acquisition For the UE to acquire its location, there could be two ways: - GNSS simulator approach: Use a GNSS simulator, as used in UE Positioning test case. - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates, as used in some existing 3GPP test cases today. However, we believe that the GNSS simulator approach may be cumbersome and complex. It has the following drawbacks: - It combines a demod test case and an RF/RRM test case, so more complexity. - The GNSS signal power levels would need to be decided appropriately by RAN4 to acquire the satellite with sufficient accuracy within a certain time. - Acquisition of the GNSS location would take some time so increase test time. Thus, we suggest to re-use the existing AT command (the 2nd approach) to avoid some penitential issues |
| CATT | Fine with option 1. In V2X, only power is considered. In addition, we think GNSS information should be aligned with ephemeris. |
| Nokia | Option 1 is fine. |

Issue 5-3: Reference time for NTN UE timing related test cases.

- Option 1: (CMCC, Xiaomi)

  - For the test requirement, several updates on reference time should be included based on current test requirement:

    - The formula (NTA + NTA_offset) ×Tc ± Te should be updated to (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN, the parameter Te should be updated to Te_NTN.

    - The clarification of NTA,common and NTA,UE-specific are needed, which are:

      - The NTA,UE-specific and NTA,common used by test system will not include UE GNSS estimation error and satellite positioning error from UE calculation.

- Option 2: (Qualcomm)

  - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.

    - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model.

    - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized.

    - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models.

- Option 3: (CATT)

  - It is agreed that NTA,common = F3+F4 and  NTA,UE-specific = S3 + S4 for reference timing for UE transmit timing.

  - In test for UE transmit timing error, the reference propagator model need not considered.

  - NTA,common is configured in the test cases by broadcasted parameters of TACommon, TACommonDrift and TACommonDriftVariation based on satellite ephemeris and position assumption of the uplink time synchronization reference point, i.e. prefer option 1A.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 2. We share the same view as the first bullet of Option 3. The last bullet of Option 1 is not crystal clear to us. Does that mean NTA,UE-specific and NTA,common will be based on ideal values, i.e. any inaccuracy due to quantized ephemeris and feeder delay information will be part of UE error no matter how long the gap between epoch time and UL transmission is? |
| Xioami | Option 1 |
| Ericsson | Regarding Option1: Agree with T_eNTN update. N_TA_Common is system information, but N_TA_UE specific is important UE characteristic which is fundamental for NTN UE network performance. We need some form of GNSS signal for NTN UE timing related test cases in order to make sure that NTN UE can perform in NTN network. This is the same comment as in Issue 5-2. Regarding Option 2: This is fine for us. Regarding option 3: To avoid RAN4 building its own track and version of reference timing we propose to reuse RAN1 definitions in TS 38.213 section 4.2. As we commented in option 2, we think that a reference propagator could be useful. |
| Apple | Support option 1. |
| Huawei | For option 1, we agree with 1st bullet. For option 2, we agree with “a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.” But the sub-bullets need to be further studied. |
| CMCC | We support Option1. To Ericsson, the NTA,UE-specific we mentioned here is used by test system, it can be derived from real UE position and satellite position, instead of UE GNSS. |
| CATT | Support option 1 and option 3. |
| Nokia | Support Option 1 and 2. |

Test case design for RLM/BFD requirements

Issue 6-1: Test case for RLM/BFD.

- Option 1: (MediaTek)

  - For RLM/BFD, select either non-DRX or DRX of each sub-test to test.

- Option 2: (Nokia)

  - RAN4 to discuss the “fully synchronized” criteria for UE testing regarding ephemeris and validity timer acquisition.

  - RAN4 to discuss the ephemeris validity timer to be used during test procedures, so that the UE timing alignment is maintained during the test.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Ericsson | Among non-DRX and DRX cases, we prefer DRX cases. But we can support testing both cases also. Support Option2 and always mimic ephemeris and validity timer’s validity. |
| Apple | Fine with option 1 and 2. |
| Huawei | Support option 1. On option 2, we assume it is a generic issue and can be discussed in issue 2-6. |
| MTK | Support Option 1 to reduce TCs. Fine with Option 2 and good to agree it as an principle. |
| CMCC | For Option 1, we prefer to test both cases. |
| CATT | Support option 1. |

Test case design for L3 measurement requirements

Issue 7-1: Test case for inter-frequency measurement.

- Option 1: (MediaTek)

  - For inter-frequency measurement TC, two fixed Doppler shifts are defined, one for the serving cell and one for the neighbouring cell. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled. Two overlapped SMTCs are configured.

  - For inter-frequency measurement, introduce TC for the case of one MG.

- Recommended WF

  - Companies are encouraged to provide the views on this issue.

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t fully get how “fixed Doppler shifts” can be achieved. Does this mean “Doppler shift remain the same irrespective of elevation angle change from UE perspective” ? |
| Ericsson | What is timing drifting here? |
| Huawei | We assume the Doppler and timing of the cells will be aligned to the reference parameters such as ephemeris and UE location, so it is more like a generic issue to be discussed in issue 2-5. On the SMTC and MG configuration, we are open to discuss and suggest to use different setups in the different TCs. |
| LGE | Is the meaning of “introduce TC for the case of on MG” that there is no two MG test case for inter-frequency measurement? |
| MTK | @LGE: to intention is to introduce TCs for both single MG and 2 MGs (both FNO, PPO) The Doppler and timing of the cell can be discussed in the reference parameters of Ephemeris, as in Issue 2-6. The proposals assumed Time varying Ephemeris, as the cell timing drift in LEO could be 25 us per sec and the question is whether to model this or not. However, if the Ephemeris remains the same then we don’t need to discuss the time vary Doppler and cell timing. |
| CATT | Fixed Doppler shifts are not needed. The timing drifting either. We think it can be designed by using ephemeris information in the test. Two overlapped SMTC is the same SMTC. We prefer to use non-overlapped SMTCs. |
| THALES | Preference for variable Doppler delay and variable timing drifting in a test environment. We can also separate them or consider them together. |

Test case list for NTN RRM

Issue 8-1: Test case list for NTN RRM.

| Group of requirements | Test cases | Sub-test | Title of test case | Comment (Need or not) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Xiaomi | Ericsson | MTK | Nokia |  |  |  |  |
| Cell reselection for satellite access | Intra-frequency | N.A | Cell reselection to FR1 intra-frequency NR cell | yes | Y, we suggest to add enhanced requirements here | Yes | Need |  |  |  |  |
|  |  | Time based | Time-based cell reselection to FR1 intra-frequency NR cell | yes | Y |  | Need |  |  |  |  |
|  |  |  | Time-based cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] | yes | N |  | Need |  |  |  |  |
|  |  | Location based | Location-based cell reselection to FR1 intra-frequency NR cell | yes | Y |  | Need |  |  |  |  |
|  |  |  | Location-based cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] | yes | N |  | Need |  |  |  |  |
|  | Inter-frequency | N.A | Cell reselection to FR1 inter-frequency NR cell | yes | Y，we suggest to add enhanced requirements here we suggest to add enhanced requirements here | Yes | Need |  |  |  |  |
|  |  | Time based | Time-based cell reselection to FR1 inter-frequency NR cell | yes | Y |  | Need |  |  |  |  |
|  |  |  | Time-based cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] | yes | N |  | Need |  |  |  |  |
|  |  | Location based | Location-based cell reselection to FR1 inter-frequency NR cell | yes | Y |  | Need |  |  |  |  |
|  |  |  | Location-based cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] | yes | N |  | Need |  |  |  |  |
| HO for satellite access | NR SAN HO | Intra-frequency HO | Intra-frequency SAN Handover from FR1 to FR1 | yes | Y | Yes | Need |  |  |  |  |
|  |  | Inter-frequency HO | Inter-frequency SAN Handover from FR1 to FR1 | yes | Y | Yes | Need |  |  |  |  |
|  | NR SAN CHO | Time based CHO | Intra-frequency SAN time-based Conditional Handover from FR1 to FR1 | yes | Y |  | Need |  |  |  |  |
|  |  |  | Inter-frequency SAN time-based Conditional Handover from FR1 to FR1 | yes | Y |  | Need |  |  |  |  |
|  |  | Distance based CHO | Intra-frequency SAN distance-based Conditional Handover from FR1 to FR1 | yes | Y |  | Need |  |  |  |  |
|  |  |  | Inter-frequency SAN distance-based Conditional Handover from FR1 to FR1 | yes | Y |  | Need |  |  |  |  |
| RRC Re-establishment for satellite access | Intra-frequency | N.A | Intra-frequency RRC Re-establishment in FR1 | yes | Y |  | Need |  |  |  |  |
|  | Inter-frequency | N.A | Inter-frequency RRC Re-establishment in FR1 | yes | Y |  | Need |  |  |  |  |
| Random access for satellite access | Contention based | N.A | 4-step RA type contention based random access test in FR1 | yes | Y |  | Need |  |  |  |  |
|  | Non-contention based | N.A | 4-step RA type non-contention based random access test in FR1 | yes | Y |  | Need |  |  |  |  |
| RRC Connection Release with Redirection for satellite access | N.A | N.A | Redirection from NR in FR1 to NR in FR1 | yes | N |  |  |  |  |  |  |
| UE transmit timing for satellite access | UE transmit timing accuracy | N.A | NTN UE transmit timing test | yes | Y | Yes | Need |  |  |  |  |
|  | UE timing advance adjustment accuracy | N.A | NTN UE timing advance adjustment accuracy test | yes | Y | Yes | Need |  |  |  |  |
| RLM for satellite access | SSB based RLM RS | In-sync | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  | Out-of-sync | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  | CSI-RS based RLM RS | In-sync | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  |  | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  | Out-of-sync | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
|  |  |  | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode | yes | Y | Down-select among RLM tests |  |  |  |  |  |
| Beam Failure Detection and Link Recovery for satellite access | SSB-based BFD and LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | yes | Y | Down-select among BFD tests |  |  |  |  |  |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | yes | Y | Down-select among BFD tests |  |  |  |  |  |
|  | CSI-RS-based BFD and LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | yes | Y | Down-select among BFD tests |  |  |  |  |  |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | yes |  | Down-select among BFD tests |  |  |  |  |  |
|  | CSI-RS-based BFD and SSB-based LR | N.A | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode | yes | Y | Down-select among BFD tests |  |  |  |  |  |
|  |  |  | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode |  | Y | Down-select among BFD tests |  |  |  |  |  |
| Active BWP switch delay for satellite access | DCI-based | N.A | NR FR1 DCI-based DL active BWP switch with non-DRX | yes | Y |  |  |  |  |  |  |
|  | RRC-based | N.A | NR FR1 RRC-based DL active BWP switch with non-DRX | yes | Y |  |  |  |  |  |  |
| UE-specific CBW change for satellite access | N.A | N.A | UE specific CBW change on PCell in FR1 in non-DRX | yes | Y |  |  |  |  |  |  |
| Pathloss reference signal switching delay for satellite access | N.A | N.A | MAC-CE based pathloss reference signal switch delay | yes | Y |  |  |  |  |  |  |
| Intra-frequency measurement delay for satellite access | Without gap | N.A | Event triggered reporting tests without gap under non-DRX | yes | Y | Yes |  |  |  |  |  |
|  |  | N.A | Event triggered reporting tests without gap under DRX | yes | Y | Yes |  |  |  |  |  |
|  |  | N.A | Event triggered reporting tests without gap under non-DRX with SSB index reading | yes | Y | Yes |  |  |  |  |  |
|  | With gap | FNO case | Event triggered reporting tests with multiple concurrent gaps under non-DRX | yes | Y | Also need single MG tests |  |  |  |  |  |
|  |  |  | Event triggered reporting tests with multiple concurrent gaps under DRX | yes | Y |  |  |  |  |  |  |
|  |  |  | Event triggered reporting tests with multiple concurrent gaps under non-DRX with SSB index reading | yes | Y |  |  |  |  |  |  |
|  |  | PPO case | Event triggered reporting tests with multiple concurrent gaps under non-DRX | yes | Y |  |  |  |  |  |  |
| Inter-frequency measurement delay for satellite access | Without gap | N.A | Event triggered reporting tests for FR1 without gap under non-DRX | yes | Y |  |  |  |  |  |  |
|  |  |  | Event triggered reporting tests for FR1 without gap under DRX | yes | Y |  |  |  |  |  |  |
|  | With gap | FNO case | Event triggered reporting tests for FR1 without SSB time index detection under non-DRX | yes | Y | Also need single MG tests |  |  |  |  |  |
|  |  |  | Event triggered reporting tests for FR1 without SSB time index detection under DRX | yes | Y |  |  |  |  |  |  |
|  |  |  | Event triggered reporting tests for FR1 with SSB time index detection under non-DRX | yes | Y |  |  |  |  |  |  |
|  |  |  | Event triggered reporting tests for FR1 with SSB time index detection under DRX | yes | Y |  |  |  |  |  |  |
|  |  | PPO case | Event triggered reporting tests for FR1 without SSB time index detection under non-DRX | yes | Y |  |  |  |  |  |  |
| L1-RSRP measurement delay for satellite access | SSB based | N.A | SSB based L1-RSRP measurement when DRX is not used | yes | Y | Yes |  |  |  |  |  |
|  |  |  | SSB based L1-RSRP measurement when DRX is used | yes | Y | Yes |  |  |  |  |  |
|  | CSI-RS based | N.A | CSI-RS based L1-RSRP measurement when DRX is not used | yes | Y |  |  |  |  |  |  |
|  |  |  | CSI-RS based L1-RSRP measurement when DRX is used |  | Y |  |  |  |  |  |  |
| RRM measurement accuracy for satellite access | SS-RSRP | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y | Yes |  |  |  |  |  |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y | Yes |  |  |  |  |  |
|  | SS-RSRQ | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y |  |  |  |  |  |  |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y |  |  |  |  |  |  |
|  | SS-SINR | Intra-frequency | Intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y |  |  |  |  |  |  |
|  |  | Inter-frequency | Inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell | yes | Y |  |  |  |  |  |  |
|  | L1-RSRP | SSB-based | SSB based L1-RSRP measurement | yes | Y | Yes |  |  |  |  |  |
|  |  | CSI-RS based | CSI-RS based L1-RSRP measurement on resource set with repetition off | yes | Y |  |  |  |  |  |  |

| Company | Additional Comments |
| --- | --- |
| MTK | Regarding test with MG, TC with single MG need to be considered, as in the current list FNO/PPO are for 2 MGs, because some NTN UE may not support 2 MGs. |

CRs comments collection for 1st round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2211960 Xiaomi CR on measurement accuracy requirement for NTN | Company A |
|  | Company B |
| R4-2211962 Xiaomi CR on UE conditions for measurement performance requirements for NTN | Company A |
|  | Company B |
| R4-2213524 Huawei CR on measurement accuracy requirements for NTN | Company A |
|  | Company B |
| R4-2214061 Ericsson Conditions for RRM requirements for satellite access band in TS 38.133 | Company A |
|  | Company B |
| R4-2211639 CATT Test cases for Intra- and inter-frequency HO with known cell for NTN | Company A |
|  | Company B |
| R4-2211640 CATT Test cases for Intra- and inter-frequency CHO for NTN | Company A |
|  | Company B |
| R4-2211963 Xiaomi CR on test case for cell reselection to FR1 intra-frequency NR cell for satellite access | Company A |
|  | Company B |
| R4-2212400 MediaTek Introduction of test cases for Inter-frequency measurement delay for satellite access with gap | Company A |
|  | Company B |
| R4-2212401 MediaTek Introduction of test cases for Accuracy for SS-RSRQ for satellite access | Company A |
|  | Company B |
| R4-2213352 Ericsson draft CR on test cases for Beam Failure Detection and Link Recover for NTN | Company A |
|  | Company B |
| R4-2213353 Ericsson draft CR on test cases for L1-RSRP measurement delay for NTN | Company A |
|  | Company B |
| R4-2213476 Huawei DraftCR on UE transmit timing tests for NTN | Ericsson: This is a good CR for test. However we have a comment on 3) in CR. We think that GSO and NGSO can be tested in the same way. This means that also GSO tests are performed as in 1) and 2) in CR with servin satellite ephemeris data higher layer parameters.. |
|  | Huawei: In NGSO scenarios, common TA and serving satellite position generally change with time, which means that downlink timing also changes with time. Therefore, DL timing adjustment depends on the configuration of the common TA and serving satellite position. If common TA and serving satellite position will be configured as a fixed value, we can agree to test NGSO as the same as GSO. |
| R4-2213525 Huawei CR on TCs for SSB based RLM for NTN | Company A |
|  | Company B |
| R4-2213526 Huawei CR on TCs for intra-frequency measurement delay for NTN | Company A |
|  | Company B |
| R4-2213527 Huawei CR on TCs for RRC Re-establishment for NTN | Company A |
|  | Company B |
| R4-2213528 Huawei CR on TCs for RSRP accuracy for NTN | Company A |
|  | Company B |
| R4-2212184 Qualcomm draft CR of BWP switch and CBW change test cases | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Measurement Accuracy requirement for NTN

Issue 1-1: Margin assumption for evaluating measurement accuracy.

|  | Status summary |
| --- | --- |
| Issue 1-1 | Agreements in GTW session: - RAN4 considers propagator model error and timing/frequency error when defining the measurement accuracy. - FFS on the values of propagator model error and timing/frequency error - FFS on the defition of propagator model error Recommendations for 2nd round: - Continue to discuss the FFS part |

Issue 1-2: Measurement accuracy.

|  | Status summary |
| --- | --- |
| Issue 1-2 | This issue is related to the conclusion of issue 1-1, and according to the discussion in GTW session, the common understanding is that no need to have agreement on this issue. Once the propagator model error and time/frequency error in issue 1-1 is concluded, the accuracy requirement can reach agreement. Recommendations for 2nd round: - No need to have further discussion in 2nd round. |

Test case coverage, design and configuration

Issue 2-1: Test cases for GSO and NGSO.

|  | Status summary |
| --- | --- |
| Issue 2-1 | Tentative Agreements: - RAN4 to define TCs for both GSO and NGSO scenarios. - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. - The TCs for GSO and NGSO should be captured in the same clauses. Recommendations for 2nd round: - No further discussion in 2nd round |

Issue 2-2: Test coverage regarding NTN RRM requirements.

|  | Status summary |
| --- | --- |
| Issue 2-2 | Agreements in GTW session: - Define the following TCs for NTN - Random access - PL-RS switching - Intra-frequency measurement with gap - Inter-frequency measurement without gap - And define the applicability rule to avoid the duplication of test for a UE. Recommendations for 2nd round: - Discuss the applicability rule, and the following options are proposed for further discussion: - Option 1: - RAN4 to define test cases for all NTN RRM requirements; - Option 2 - RAN4 to define test cases for NTN specific requirements; - For those NTN requirements that are the same as TN requirements, if a UE passes the test cases for TN requirements, then the UE does not need to pass the test cases for NTN requirements. |

Issue 2-3: Test coverage regarding numerology and duplex mode.

|  | Status summary |
| --- | --- |
| Issue 2-3 | Tentative Agreements: - Test configurations for TDD and numerology higher than 15kHz are not considered. - FFS on whether to define UE timing TC for 30KHz SCS. Recommendations for 2nd round: - Discuss the FFS part. |

Issue 2-4: Test coverage regarding multiple SMTCs.

|  | Status summary |
| --- | --- |
| Issue 2-4 | Agreements in GTW session: - Test cases shall take multi-SMTC and multi-satellite tests into account. - Minimize the test case number. - FFS on the following bullet - “Add new contents for definition of 2 SMTCs per MO, 2 satellites (i.e. 2 SSBs) per SMTC, 2 MGs in clause A.3 RRM test configuration.” - FFS on Option 2. Recommendations for 2nd round: - Discuss the FFS part. |

Issue 2-5: Serving and Neighbour Satellite configurations.

|  | Status summary |
| --- | --- |
| Issue 2-5 | Agreements in GTW session: - Take option 1 as baseline to further discuss how to configure the satellite parameters. Recommendations for 2nd round: - Further discuss how to configure the satellite parameters. |

Issue 2-6: Test applicability rule for NTN RRM test cases.

|  | Status summary |
| --- | --- |
| Issue 2-6 | All the companies are fine with option 1. Tentative Agreements: - RAN4 to define the following test applicability rule in a common section that applies to NTN RRM test cases. - For all test case: - Satellite information (included in System information) shall remain the same during the test, e.g. Validity timer should be larger enough. - UE GNSS information shall remain the same during the test, i.e. UE mobility is not considered unless an exceptional case is identified, e.g. distance-based conditional handover. - The overall overhead ratio due to scheduling restriction caused by all configured SMTCs (e.g. scheduling restriction overhead of all SMTCs in one periodicity / SMTC periodicity) shall be less than or equal to 75%. - For neighbor cell measurement test cases: - Ephemeris - Epoch time - Feeder link propagation delay - Validity timers - Downlink polarization information - For handover test cases: - Ephemeris - Epoch time - Feeder link propagation delay - Validity timer - Downlink and uplink polarization - Koffset - Kmac Recommendations for 2nd round: - No further discussion in the 2nd round. |

Issue 2-7: RMC for virtual satellite.

|  | Status summary |
| --- | --- |
| Issue 2-7 | 4 companies provided the comments, and 3 companies suggest this issue can be discussed together in issue 2-5, no need to have separate discussion. Recommendations for 2nd round: - No further discussion. |

Issue 2-8: Koffset and Kmac configurations.

|  | Status summary |
| --- | --- |
| Issue 2-8 | All the companies agree with the recommended WF. Tentative agreement: - The Koffset and Kmac should be additional taken into account according to RAN1 spec if they are involved in the test procedure and test requirements. Recommendations for 2nd round: - No further discussion. |

Test case design for Cell reselection

Issue 3-1: General test design for intra/inter-frequency cell reselection.

|  | Status summary |
| --- | --- |
| Issue 3-1 | All the companies are fine with option 1 and option 3, option 1 can be covered in issue 2-1, thus, no need to have additional agreement. For option 2, 4 companies has concern on whether ‘t-service’ should be included in the test. Tentative agreement: - RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition. - To verify time based cell reselection measurement (time based), the signal level of active cells should be larger than Srxlev or Squal threshold in the test design. Recommendations for 2nd round: - Further discussion on option 2: - Design the test case for intra-frequency and inter frequency cell reselection to evaluate the UE response to “t-service” configuration. Include in the test case both trigger cases: “t-service+10” and “T1+10”. - Discuss whether to include “K_multi” parameter in the tests, and how to evaluate for it. |

Issue 3-2: Test case for intra/inter-frequency cell reselection without timer or location trigger.

|  | Status summary |
| --- | --- |
| Issue 3-2 | All the companies are fine with option 1 and option 2, Tentative agreement: - For the test case of cell reselection to intra- and inter-frequency neighbour cell without time or location trigger, the test procedure for legacy TN cell reselection to FR1 intra- and inter- frequency with configuration 1 (FDD) can be reused. Recommendations for 2nd round: - No further discussion |

Issue 3-3: Test case for intra/inter-frequency cell reselection with timer trigger.

|  | Status summary |
| --- | --- |
| Issue 3-3 | Option 1 is covered in issue 2-1, thus, no need to have additional agreement. All the companies are fine with option 2, Tentative agreement: - Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between the end of T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra defined in A.6.1.1.1.3) for reselection. Recommendations for 2nd round: - No further discussion |

Issue 3-4: Test case for intra/inter-frequency cell reselection with location trigger.

|  | Status summary |
| --- | --- |
| Issue 3-4 | 3 companies provided the comments on whether the Dmargin is added to the distance threshold and 1 company provided the comment that the response time for UE to response when DdistanceThresh condition is met should be considered, Recommendations for 2nd round: - Continue the discussion on following aspects: - Whether the Dmargin is added to the distance threshold - Whether to consider the response time for UE to response when DdistanceThresh condition is met |

Issue 3-5: Test case for the relaxed intra/inter-frequency cell reselection requirements in GEO scenario.

|  | Status summary |
| --- | --- |
| Issue 3-5 | 5 companies support to define test cases for relaxed cell reselection requirements for GEO scenario and 3 companies support to have further study. Recommendations for 2nd round: - Study whether to define test cases for relaxed cell reselection requirements for GEO scenario. |

Test case design for handover

Issue 4-1: Test case for intra/inter-frequency handover with known cell for NTN.

|  | Status summary |
| --- | --- |
| Issue 4-1 | All companies are fine with option 1 and the NTN specific information shall be provisioned. Tentative agreement: - The test cases of Intra- and inter-frequency HO with known cell can refer to legacy TN test cases. Recommendations for 2nd round: - No further discussion. |

Issue 4-2: Test case for CHO with time-based or location-based condition.

|  | Status summary |
| --- | --- |
| Issue 4-2 | - Option 1: (Ericsson) - Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2. - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure ). - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2. - RAN4 shall check the necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario. 5 companies are fine with option 1, 1 company prefer to reuse the current T2 usage for first bullet and 2 companies have concern on whether to consider the condition where HO condition is not met in test. Tentative agreement: - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure ). - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2. Recommendations for 2nd round: - Have further discussion on first and fourth bullet. |

Test case design for UE timing requirements

Issue 5-1: Test principle for NTN UE timing related test cases.

|  | Status summary |
| --- | --- |
| Issue 5-1 | Majority companies are fine with option 2, 1 company prefer to define TA adjustment accuracy requirements under GSO. Since issue 2-1 is related to test principle for all RRM requirements, thus, no need to have additional agreement on timing test cases. All RRM test cases shall follow the same principle unless the particular case is identified. Recommendations for 2nd round: - No further discussion. |

Issue 5-2: GNSS signal for NTN UE timing related test cases.

|  | Status summary |
| --- | --- |
| Issue 5-2 | - Option 1: (Huawei) - For NTN UE timing related tests, GNSS signals need to be simulated for UE positioning and the test parameters for GNSS signals defined for V2X tests can be reused. 4 companies are fine with option 1 for signal power assumption which is defined in B.4.1, FFS for other parameters. 1 company suggests to re-use the existing AT command. Recommendations for 2nd round: - Have further discussion on GNSS-related parameter configuration. |

Issue 5-3: Reference time for NTN UE timing related test cases.

|  | Status summary |
| --- | --- |
| Issue 5-3 | 5 companies are fine with option 1, 3 companies are fine with option 2 and 2 companies are fine with option 3. In order to move-forward, the following option A which is combined option 1 and option 2 is proposed for further discussion in 2nd round: Option A: - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN - FFS NTA,UE-specific and NTA,common used by test system do not include UE GNSS estimation error and satellite positioning error from UE calculation. - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position. - FFS the following aspects: - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model. - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized. - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models. Recommendations for 2nd round: - Have further discussion on above option. |

Test case design for RLM/BFD requirements

Issue 6-1: Test case for RLM/BFD.

|  | Status summary |
| --- | --- |
| Issue 6-1 | - Option 1: (MediaTek) - For RLM/BFD, select either non-DRX or DRX of each sub-test to test. - Option 2: (Nokia) - RAN4 to discuss the “fully synchronized” criteria for UE testing regarding ephemeris and validity timer acquisition. - RAN4 to discuss the ephemeris validity timer to be used during test procedures, so that the UE timing alignment is maintained during the test. 4 companies are fine with option 1, 2 companies support to define tests for both cases. 3 companies are fine with option 2, and 1 company suggest option 2 can be discussed in issue 2-6. Recommendations for 2nd round: - Have further discussion on option 1 and option 2. |

Test case design for L3 measurement requirements

Issue 7-1: Test case for inter-frequency measurement.

|  | Status summary |
| --- | --- |
| Issue 7-1 | - Option 1: (MediaTek) - For inter-frequency measurement TC, two fixed Doppler shifts are defined, one for the serving cell and one for the neighbouring cell. RAN4 to further discuss whether cell’s timing drifting, corresponding to the fixed Doppler, can be modelled. Two overlapped SMTCs are configured. - For inter-frequency measurement, introduce TC for the case of one MG. 3 companies need some clarification on fix Doppler shift and timing drifting, and the proponent of option 1 think Doppler and timing of the cell can be discussed in the reference parameters of Ephemeris, as in Issue 2-6. Thus, we can continue to discuss the second bullet in the 2nd round. Recommendations for 2nd round: - For inter-frequency measurement, introduce TCs for both single MG and 2 MGs. |

Discussion on 2nd round (if applicable)

Measurement Accuracy requirement for NTN

Issue 1-1: Margin assumption for evaluating measurement accuracy.

Agreements in GTW session:

- RAN4 considers propagator model error and timing/frequency error when defining the measurement accuracy.

  - FFS on the values of propagator model error and timing/frequency error

  - FFS on the definition of propagator model error

Recommendations for 2nd round:

  - Continue to discuss the FFS part

| Company | Comments |
| --- | --- |
| LGE | Further discuss in next meeting with detailed value of margin. |
| Huawei | We can discuss this in next meeting, and inputs from satellite system vendors would be helpful. |

Test case coverage, design and configuration

Issue 2-2: Test coverage regarding NTN RRM requirements.

Agreements in GTW session:

- Define the following TCs for NTN

  - Random access

  - PL-RS switching

  - Intra-frequency measurement with gap

  - Inter-frequency measurement without gap

- And define the applicability rule to avoid the duplication of test for a UE.

Recommendations for 2nd round:

  - Discuss the applicability rule, and the following options are proposed for further discussion:

  - Option 1:

  - RAN4 to define test cases for all NTN RRM requirements;

  - Option 2:

  - RAN4 to define test cases for NTN specific requirements;

  - For those NTN requirements that are the same as TN requirements, if a UE passes the test cases for TN requirements, then the UE does not need to pass the test cases for NTN requirements.

| Company | Comments |
| --- | --- |
| MTK | Support the 2nd bullet under Option 2. Consider the NTN only UE, then the agreed TCs as above may not be able to skip. |
| Xiaomi | we support a new option 3 by considering the NTN only capable UE: - For NTN capable UE, RAN4 to define test cases for all NTN RRM requirements; - For TN/NTN capable UE, the UE shall pass the TCs for NTN specific requirements and does not need to pass the TCs for the requirements are the same as TN requirements. |
| Apple | We are fine with option 3 proposed by Xiaomi and option 2. |
| Ericsson | Support Option 1. Regarding Option 2, the method may cause mess specially to an NTN only UE, e.g. which cases shall follow TN requirements and which cases shall follow NTN requirements. And, reusing TN test case may still need sort of updating of wordings to comply with NTN UE. Option3 is OK, but from definition of test cases perspective, Option 3 is same as Option 1. The Option 3 is about applicability, it’s fine with us. |
| CMCC | We can compromise to Option 3 proposed by Xiaomi. For Option 2, we share similar view with Ericsson. |
| CATT | Support option 1. We are fine with first bullet for option 3. It is same as option 1. For second bullet in option 3, our first question is how to indicate UE supports TN/NTN separate NTN capable UE only. Second, even the test requirements are the same as TN system. The parameter setting should be update for NTN UE. For example, NTN specific parameters such as NW indicator and ephemeris, Bandwidth configuration(only config 1 can be applied). Etc. So it is not the same TC as legacy. In addition, is “NOT” missing in GTW agreement? If I remember correctly, we agree to not define such cases in GREEN. |
| LGE | Fine with Xiaomi’s proposal i.e. option 3 |
| Huawei | We can support the new option 3 from Xiaomi. |

Issue 2-3: Test coverage regarding numerology and duplex mode.

Agreements in 1st round:

  - Test configurations for TDD and numerology higher than 15kHz are not considered.

  - FFS on whether to define UE timing TC for 30KHz SCS.

Recommendations for 2nd round:

  - Discuss the FFS part.

| Company | Comments |
| --- | --- |
| MTK | If 30KHz of timing TC is seemed as necessary, then we suggest to merge it into the same TCs as 15 kHz but with different Test configuration. |
| Xiaomi | Support MTK’s view. |
| Apple | Agree with MTK. |
| Ericsson | Agree with MTK |
| Qualcomm | We do not support FFS part. 30kHz is mostly for TDD. With the large round-trip delay, a larger SCS will just increase payload size in DL/UL grants. |
| CMCC | We are ok with MTK’s proposal |
| CATT | Support MTK’s view. |
| LGE | Agree with MTK. |
| Huawei | We support MTK’s proposal. |

Issue 2-4: Test coverage regarding multiple SMTCs.

Agreements in GTW session:

- Test cases shall take multi-SMTC and multi-satellite tests into account.

  - Minimize the test case number.

- FFS on the following bullet

  - “Add new contents for definition of 2 SMTCs per MO, 2 satellites (i.e. 2 SSBs) per SMTC, 2 MGs in clause A.3 RRM test configuration.”

- FFS on Option 2.

Recommendations for 2nd round:

  - Discuss the FFS part.

| Company | Comments |
| --- | --- |
| MTK | Fine with “Add new contents for definition of 2 SMTCs per MO, 2 satellites (i.e. 2 SSBs) per SMTC, 2 MGs in clause A.3 RRM test configuration.” But it would imply the following configurations. - Config. 1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites - Config. 2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites - Config. 3: 2 SMTC per MO, each SMTC contains 2 SSB/Satellites The Config. 3 may need to be excluded to simply the test and at least excluded for the UE doesn’t support 4 Satellites on the same frequency. |
| Xiaomi | We support the configurations proposed by MTK. And we prefer to define MG configuration in A.3 common configuration for RRM test: Config.1: 1 measurement gap configuration; (reuse the exiting measurement gap configuration) Config.2: 2 measurement gap configurations (FNO case); Config.2: 2 measurement gap configurations (PPO case); |
| Apple | We are fine with Xiaomi and MTK proposals. Regarding MG configuration, we may also need to add one more config for 2 MG fully overlapped case for MGRP=160ms if we can have conclusion in thread #214. Regarding option 2, we think so far we don’t have corresponding requirement and the test case can be postponed. |
| Ericsson | MTK’s suggestion looks fine. MG also shall be tested, Xiaomi’s proposal is fine with us. |
| Qualcomm | Let’s not consider a scenario in which we need more than 2 satellites, which is not really critical in practice considering this is very first NTN release. |
| CMCC | We are fine with MTK’s and Xiaomi’s suggestion. |
| CATT | We are fine with MTK’s and Xiaomi’s suggestion. But we propose not to use all configurations for all tests cases. Just pick some of TC to cover those configurations. |
| LGE | For option 1, fine with MTK’s suggestion. For option 2, we prefer consider option 2. But, for the progress, we fine with majority view. |
| Huawei | We support proposal from MTK and QC that we should not involve more than 2 satellites in the tests, i.e. config 3 in MTK comment. We understand config 1 in MTK comment can be further split into overlapping and non-overlapping cases, so we would have following 3 cases for measurement outside MG. - Case 1a: 2 SMTC per MO which are overlapping, each SMTC contains 1 SSB/Satellites - Case 1b: 2 SMTC per MO which are non-overlapping, each SMTC contains 1 SSB/Satellites - Case 2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites For measurement with MG, the MG can be configured in accordance with SMTC, which is aligned with Xiaomi’s suggestion. However, we assume MG configuration does not need to defined in A.3, but can be defined in each TC as in existing RRM TCs. |

Issue 2-5: Serving and Neighbour Satellite configurations.

Agreements in GTW session:

- Take option 1 as baseline to further discuss how to configure the satellite parameters.

Recommendations for 2nd round:

  - Further discuss how to configure the satellite parameters.

    - Ephemeris format

  - Option 1:

  - PVT information;

  - Option 2:

  - Orbital parameters;

| Company | Comments |
| --- | --- |
| MTK | Regarding the Ephemeris format, slightly prefer to format of position and velocity state vector, which is more straightforward to use. And fine to select some test to use orbital parameters. |
| Xiaomi | Fine with either option |
| Qualcomm | For serving cell or a target cell in HO scenario, it should be based on Option 1. For other neighbor cells for measurement, Option 2 should be a good candidate. RAN4 should consider both to make sure UE can project satellite position based on both types of ephemeris info. |
| CMCC | We think it is hard to say which Option is more common in the network, so we support to use both Option 1 and Option2. QC’s proposal is fine for us. Besides, we think both target cell and eighbor cell using Option 1 or Option 2 should also be verified, we can select some tests to verify such condition. |
| CATT | Prefer option 1 for simplicity. Fine with Qualcomm’s idea, use option 1 in some TCs while use option 2 in some TCs. |
| LGE | Both options are fine to us |
| Huawei | We have one issue with option 1 on “For NGSO, a deployment of quasi-earth fixed cell is assumed”. We understand the earth moving scenario should be also considered at least for some test cases. We are fine with QC’s proposal on the ephemeris format. |

New Issue 2-9: Test coverage regarding DRX and non-DRX.

  - Option 1:

  - Select either non-DRX or DRX of each sub-test to test.

  - Option 2:

  - Define TCs for both DRX and non-DRX

| Company | Comments |
| --- | --- |
| Xiaomi | Option 2 |
| Ericsson | Slightly we can support Option 1 but OK with Option 2. |
| CMCC | Option 2 is preferred. |
| CATT | Option 2. |
| Huawei | We already have the test case list with both DRX and non-DRX TCs, we are not sure if we still need an agreement on this issue. |

Test case design for Cell reselection

Issue 3-1: General test design for intra/inter-frequency cell reselection.

Agreement in 1st round:

  - RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition.

  - To verify time based cell reselection measurement (time based), the signal level of active cells should be larger than Srxlev or Squal threshold in the test design.

Recommendations for 2nd round:

  - Further discussion on the following aspects:

    - Design the test case for intra-frequency and inter frequency cell reselection to evaluate the UE response to “t-service” configuration. Include in the test case both trigger cases: “t-service+10” and “T1+10”.

    - Discuss whether to include “K_multi” parameter in the tests, and how to evaluate for it.

| Company | Comments |
| --- | --- |
| MTK | On the 1st bullet: our understanding is "the UE shall initiate cell selection procedures….within 10 s since time instance T1 provided that ‘t-Service’> T1 ", which doesn't seem a part of a cell reselection test. On the 2nd bullet: If to test K_multi, does it mean TE needs to configure 4 Satellite on one carrier, doesn’t it make the test too complicated? |
| Xiaomi | 1st bullet: we support to follow the legacy way, which “t-service” is not in the part of cell reselection test procedure. 2nd bullet: for UE is capable of measuring more than 1 satellite in one SMTC, we are fine to include “K_multi” parameter in the tests. This can be captured in clause of test requirement. |
| Apple | For 1st bullet, we think this UE behavior is for cell selection and no delay requirement can be applied. So we may not need to test it. For 2nd bullet, if more than 1 LEO satellite is configured in one SMTC and UE can only support 1 Leo in one SMTC, then this factor can be applied. It can be reflected in the final required delay in testing requirement. |
| Ericsson | Regarding the 1st bullet, in legacy requirement, the ‘10s’ isn’t test case. Similarly, we don’t need to test it also. But we’re open to further discussion. Regarding the 2nd bullet, maybe we can use 2 test cases: a) 2 SMTC and one sat. per SMTC, b)1SMTC and 2 sat. per SMTC to verify the coverage, if test on 4 satellites is too complicated. 𝐾𝑚𝑢𝑙𝑡𝑖_𝑆𝑀𝑇𝐶 = ∑𝑁 𝑖=𝑆𝑀𝑇𝐶 1 ,𝑜𝑣𝑒𝑟𝑙𝑎𝑝 ⌈𝑁𝐿𝐸𝑂/ 𝑁𝐿𝐸𝑂 ,𝑠𝑖𝑚𝑢𝑙 ,𝑖 ⌉ |
| Qualcomm | For 1st bullet: okay with not including ‘t-service’ For 2nd bullet: as long as the number of satellite to be modeled is not larger than 2, it is okay with us. |
| CATT | For 1st bullet: agree excluding t-service. For 2nd bullet: we prefer to use 2 SMTC, one sat. per SMTC. |
| Huawei | On first bullet, we assume the TCs are for cell reselection, and we agree that UE triggering reselection measurement based on t-Service should be verified, but “t-service+10” and “T1+10” are for cell selection, and we understand it is not tested in existing test cases for TN. On second bullet, if the number of satellites per carrier is 2, then it should be considered in the test requirements. |

Issue 3-4: Test case for intra/inter-frequency cell reselection with location trigger.

Recommendations for 2nd round:

  - Continue the discussion on following aspects:

    - Whether the Dmargin is added to the distance threshold

    - Whether to consider the response time for UE to response when DdistanceThresh condition is met

| Company | Comments |
| --- | --- |
| Xiaomi | 1st bullet: Dmargin should not be added to the distance threshold, 2nd bullet: open to discuss |
| Apple | For 1st bullet, based on the 1st round discussion, we think this margin can be reflected in the testing configuration rather than threshold, e.g, if the threshold is x meters, then the testing configuration shall be set as x+Dmargin to make sure UE can trigger the distance based measurement. For 2nd bullet, we are open to FFS, and in TS38.171, the periodical reporting interval for GNSS for moving scenario and periodic update can be 2s, we are wondering if this 2s can be reused here. |
| LGE | We have similar view as Xiaomi 1st bullet: It does not need to be added 2nd bullet: Open to discuss |
| Huawei | On first bullet, same view as Apple on how to apply Dmargin in the test. On second bullet, we are open to discuss how to address it |

Issue 3-5: Test case for the relaxed intra/inter-frequency cell reselection requirements in GEO scenario.

Recommendations for 2nd round:

  - Study whether to define test cases for relaxed cell reselection requirements for GEO scenario.

| Company | Comments |
| --- | --- |
| Xiaomi | Fine to define TC for relaxed requirement. |
| Ericsson | We prefer to define test case because legacy test cases have similar stuffs. |
| CMCC | We prefer to define the test cases |
| CATT | Fine to define such TCs. |
| LGE | We don’t have strong view. But, we think relaxed measurement criterion for UE (low Mobility/not cell edge) is not observed clearly since signal power level is almost flat in GEO. So, we think it is not clear to verify this feature. |
| Huawei | Our preference is not since it is less stringent than the non-relaxed reselection test, but we can compromise if majority prefer to have it. |

Test case design for handover

Issue 4-2: Test case for CHO with time-based or location-based condition.

Agreement in 1st round:

  - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure ).

  - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2.

Recommendations for 2nd round:

  - Have further discussion on first and fourth bullet.

  - Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2.

  - RAN4 shall check the necessity of adding test cases in which settings don’t ehavi power based events and time/location based events simultaneously, to examine UE’s ehaviour in this type of scenario.

| Company | Comments |
| --- | --- |
| Xiaomi | 1st bullet: fine with this bullet 2nd bullet: Prefer not to consider such setting. |
| Ericsson | Regarding 1st sub-bullet, we’re OK with it. Regarding 2nd sub-bullet, we’re open. |
| CATT | For 1st bullet, we prefer to follow the legacy CHO test. RRC message is sent at a time earlier than TRRC before the beginning of T2.. For 2nd bullet, prefer not to define. |
| Huawei | On first bullet, fine. On second bullet, prefer not. |

Test case design for UE timing requirements

Issue 5-2: GNSS signal for NTN UE timing related test cases.

  - Option 1: the test parameter for GNSS signal power levels defined in B.4.1 is reused in NTN test cases.

  - Option 2: use AT command approach to acquire UE location

    - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates, as used in some existing 3GPP test cases today.

| Company | Comments |
| --- | --- |
| MTK | Support Option 2, which is simpler in our view. On Option 1, we believe that the GNSS simulator approach may be cumbersome and complex. It has the following drawbacks: - It combines a demod test case and an RF/RRM test case, so more complexity. - The GNSS signal power levels would need to be decided appropriately by RAN4 to acquire the satellite with sufficient accuracy within a certain time. - Acquisition of the GNSS location would take some time so increase test time. |
| Xiaomi | Option 1 |
| CATT | Support option 1 to use same as in V2X. |
| Huawei | Option 1. GNSS positioning error is considered within Te_NTN. |

Issue 5-3: Reference time for NTN UE timing related test cases.

Recommendations for 2nd round:

  - Have further discussion on option A:

    - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN

      - FFS NTA,UE-specific and NTA,common used by test system do not include UE GNSS estimation error and satellite positioning error from UE calculation.

    - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.

      - FFS the following aspects:

        - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model.

        - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized.

        - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models.

| Company | Comments |
| --- | --- |
| Xiaomi | Fine with option A |
| Ericsson | What is option A? It looks like option 1 and option 2 from first round. We agree that NUN UE shall forward its UL tx time within (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN. UE position accuracy is fundamental. Regarding the FFS point: We need some test of UE GNSS estimation error and satellite positioning error from UE calculation. We agree that In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position. We think that this should also me tested in GSO case as well. Regarding FFS: RAN4 defines a reference propagator model. |
| Qualcomm | Okay with Option A |
| CMCC | Fine with Option A as the starting point |
| CATT | Fine with option A. For FFS, we think the first bullet “For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model.” should fine. |
| Huawei | We are fine with the principles in option A. |

Test case design for RLM/BFD requirements

Test case design for L3 measurement requirements

Issue 7-1: Test case for inter-frequency measurement.

- Option 1:

  - For inter-frequency measurement, introduce TCs for both single MG and 2 MGs.

| Company | Comments |
| --- | --- |
| MTK | Support Option 1, which is more fundamental and can be the baseline. |
| Xiaomi | Support option 1 |
| Apple | Fine with option 1. |
| Ericsson | Support Option 1 |
| CMCC | Support Option 1 |
| CATT | Support option 1. |
| LGE | Support option 1 |
| Huawei | Support option 1 |

Test case list for NTN RRM

Issue 8-1: Test case list for NTN RRM.

| Group of requirements |  | TC index | Test case | Section ID | Candidate company | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Cell reselection for satellite access |  | 1-1 | Cell reselection to FR1 intra-frequency NR cell | A.14.1.a1 | Xiaomi |  |
|  |  | 1-2 | Cell reselection to FR1 intra-frequency NR cell for UE configured with [capability for enhanced requirements] | A.14.1.a2 | Xiaomi |  |
|  |  | 1-3 | Time-based cell reselection to FR1 intra-frequency NR cell | A.14.1.a3 | Xiaomi |  |
|  |  | 1-4 | Location-based cell reselection to FR1 intra-frequency NR cell | A.14.1.a4 | Xiaomi |  |
|  |  | 1-5 | Cell reselection to FR1 inter-frequency NR cell | A.14.1.a5 | LGE |  |
|  |  | 1-6 | Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements] | A.14.1.a6 | LGE |  |
|  |  | 1-7 | Time-based cell reselection to FR1 inter-frequency NR cell | A.14.1.a7 | LGE |  |
|  |  | 1-8 | Location-based cell reselection to FR1 inter-frequency NR cell | A.14.1.a8 | LGE |  |
| Handover for satellite access |  | 2-1 | Intra-frequency Handover from FR1 to FR1 | A.14.2.1.b1 | CATT |  |
|  |  | 2-2 | Inter-frequency Handover from FR1 to FR1 | A.14.2.1.b2 | CATT |  |
|  |  | 2-3 | Intra-frequency time-based Conditional Handover from FR1 to FR1 | A.14.2.1.b3 | CATT |  |
|  |  | 2-4 | Inter-frequency time-based Conditional Handover from FR1 to FR1 | A.14.2.1.b4 | CATT |  |
|  |  | 2-5 | Intra-frequency distance-based Conditional Handover from FR1 to FR1 | A.14.2.1.b5 | CATT |  |
|  |  | 2-6 | Inter-frequency distance -based Conditional Handover from FR1 to FR1 | A.14.2.1.b6 | CATT |  |
| RRC Connection Mobility Control | RRC Re-establishment for satellite access | 3-1-1 | Intra-frequency RRC Re-establishment in FR1 | A.14.2.2.1.c1 | Huawei |  |
|  |  | 3-1-2 | Inter-frequency RRC Re-establishment in FR1 | A.14.2.2.1.c2 | Huawei |  |
|  | Random access for satellite access | 3-2-1 | 4-step RA type contention based random access test in FR1 | A.14.2.2.2.d1 | Xiaomi |  |
|  |  | 3-2-2 | 4-step RA type non-contention based random access test in FR1 | A.14.2.2.2.d2 | Xiaomi |  |
|  | RRC Connection Release with Redirection for satellite access | 3-3-1 | Redirection from FR1 to FR1 | A.14.2.2.3.e1 | Xiaomi |  |
| UE timing requirements for satellite access |  | 4-1 | NTN UE transmit timing test | A.14.3.1.f1 | Huawei |  |
|  |  | 4-2 | NTN UE timing advance adjustment accuracy test | A.14.3.2.g1 | CMCC |  |
| RLM requirements for satellite access |  | 5-1 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | A.14.4.1.h1 | Huawei |  |
|  |  | 5-2 | Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | A.14.4.1.h2 | Huawei |  |
|  |  | 5-3 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode | A.14.4.1.h3 | Huawei |  |
|  |  | 5-4 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode | A.14.4.1.h4 | Huawei |  |
|  |  | 5-5 | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode | A.14.4.1.h5 | CMCC |  |
|  |  | 5-6 | Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode | A.14.4.1.h6 | CMCC |  |
|  |  | 5-7 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in non-DRX mode | A.14.4.1.h7 | CMCC |  |
|  |  | 5-8 | Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM RS in DRX mode | A.14.4.1.h8 | CMCC |  |
| Beam Failure Detection and Link Recovery for satellite access |  | 6-1 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode | A.14.4.2.i1 | Ericsson |  |
|  |  | 6-2 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode | A.14.4.2.i2 | Ericsson |  |
|  |  | 6-3 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode | A.14.4.2.i3 | Ericsson |  |
|  |  | 6-4 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode | A.14.4.2.i4 | Ericsson |  |
|  |  | 6-5 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode | A.14.4.2.i5 | Ericsson |  |
|  |  | 6-6 | Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode | A.14.4.2.i6 | Ericsson |  |
| Active BWP switch delay for satellite access |  | 7-1 | NR FR1 DCI-based DL active BWP switch with non-DRX | A.14.4.3.j1 | Qualcomm |  |
|  |  | 7-2 | NR FR1 RRC-based DL active BWP switch with non-DRX | A.14.4.3.j2 | Qualcomm |  |
| UE-specific CBW change for satellite access |  | 8-1 | UE specific CBW change on PCell in FR1 in non-DRX | A.14.4.4.k1 | Qualcomm |  |
| Pathloss reference signal switching delay for satellite access |  | 9-1 | MAC-CE based pathloss reference signal switch | A.14.4.5.m1 | Xiaomi |  |
| Intra-frequency measurement delay for satellite access |  | 10-1 | Event triggered reporting tests without gap under non-DRX | A.14.5.1.n1 | Huawei |  |
|  |  | 10-2 | Event triggered reporting tests without gap under DRX | A.14.5.1.n2 | Huawei |  |
|  |  | 10-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading | A.14.5.1.n3 | Huawei |  |
|  |  | 10-4 | Event triggered reporting tests with measurement gap under non-DRX | A.14.5.1.n4 | OPPO |  |
|  |  | 10-5 | Event triggered reporting tests with measurement gap under DRX | A.14.5.1.n5 | OPPO |  |
|  |  | 10-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading | A.14.5.1.n6 | OPPO |  |
|  |  | 10-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX | A.14.5.1.n7 | OPPO |  |
|  |  | 10-8 | Event triggered reporting tests with FNO concurrent gaps under DRX | A.14.5.1.n8 | OPPO |  |
|  |  | 10-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX | A.14.5.1.n9 | OPPO |  |
| Inter-frequency measurement delay for satellite access |  | 11-1 | Event triggered reporting tests without gap under non-DRX | A.14.5.2.o1 | MTK |  |
|  |  | 11-2 | Event triggered reporting tests without gap under DRX | A.14.5.2.o2 | MTK |  |
|  |  | 11-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading | A.14.5.2.o3 | MTK |  |
|  |  | 11-4 | Event triggered reporting tests with measurement gap under non-DRX | A.14.5.2.o4 | MTK |  |
|  |  | 11-5 | Event triggered reporting tests with measurement gap under DRX | A.14.5.2.o5 | MTK |  |
|  |  | 11-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading | A.14.5.2.o6 | MTK |  |
|  |  | 11-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX | A.14.5.2.o7 | MTK |  |
|  |  | 11-8 | Event triggered reporting tests with FNO concurrent gaps under DRX | A.14.5.2.o8 | MTK |  |
|  |  | 11-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX | A.14.5.2.o9 | MTK |  |
| L1-RSRP measurement delay for satellite access |  | 12-1 | SSB based L1-RSRP measurement when DRX is not used | A.14.5.3.p1 | Ericsson |  |
|  |  | 12-2 | SSB based L1-RSRP measurement when DRX is used | A.14.5.3.p2 | Ericsson |  |
|  |  | 12-3 | CSI-RS based L1-RSRP measurement when DRX is not used | A.14.5.3.p3 | Ericsson |  |
|  |  | 12-4 | CSI-RS based L1-RSRP measurement when DRX is used | A.14.5.3.p4 | Ericsson |  |
| RRM measurement accuracy for satellite access |  | 13-1 | Intra-frequency SS-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.1.q1 | Huawei |  |
|  |  | 13-2 | Inter-frequency SS-RSRP measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.1.q2 | Huawei |  |
|  |  | 13-3 | Intra-frequency SS-RSRQ measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.2.r1 | MTK |  |
|  |  | 13-4 | Inter-frequency SS-RSRQ measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.2.r2 | MTK |  |
|  |  | 13-5 | Intra-frequency SS-SINR measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.3.s1 | Xiaomi |  |
|  |  | 13-6 | Inter-frequency SS-SINR measurement accuracy with FR1 serving cell and FR1 target cell | A.14.6.3.s2 | Xiaomi |  |
|  |  | 13-7 | SSB based L1-RSRP measurement accuracy | A.14.6.4.t1 | Xiaomi |  |
|  |  | 13-8 | CSI-RS based L1-RSRP measurement accuracy on resource set with repetition off | A.14.6.4.t2 | Xiaomi |  |

Recommendations for 2nd round:

  - RAN4 to approve above test case list for NTN RRM.

| Company | Comments |
| --- | --- |
| Xiaomi | Support above test case list. |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
| WF on performance requirements for NTN | Xiaomi |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2211960 | CR on measurement accuracy requirement for NTN | Xiaomi | Merged to R4-2213524 |  |
| R4-2211962 | CR on UE conditions for measurement performance requirements for NTN | Xiaomi | Merged to R4-2214061 |  |
| R4-2213524 | CR on measurement accuracy requirements for NTN | Huawei | Agreeable |  |
| R4-2214061 | Conditions for RRM requirements for satellite access band in TS 38.133 | Ericsson | Agreeable |  |
| R4-2211639 | Test cases for Intra- and inter-frequency HO with known cell for NTN | CATT | Revised |  |
| R4-2211640 | Test cases for Intra- and inter-frequency CHO for NTN | CATT | Revised |  |
| R4-2211963 | CR on test case for cell reselection to FR1 intra-frequency NR cell for satellite access | Xiaomi | Revised |  |
| R4-2212400 | Introduction of test cases for Inter-frequency measurement delay for satellite access with gap | MediaTek | Revised |  |
| R4-2212401 | Introduction of test cases for Accuracy for SS-RSRQ for satellite access | MediaTek | Revised |  |
| R4-2213352 | draft CR on test cases for Beam Failure Detection and Link Recover for NTN | Ericsson | Revised |  |
| R4-2213353 | draft CR on test cases for L1-RSRP measurement delay for NTN | Ericsson | Revised |  |
| R4-2213476 | DraftCR on UE transmit timing tests for NTN | Huawei | Revised |  |
| R4-2213525 | CR on TCs for SSB based RLM for NTN | Huawei | Revised |  |
| R4-2213526 | CR on TCs for intra-frequency measurement delay for NTN | Huawei | Revised |  |
| R4-2213527 | CR on TCs for RRC Re-establishment for NTN | Huawei | Revised |  |
| R4-2213528 | CR on TCs for RSRP accuracy for NTN | Huawei | Revised |  |
| R4-2212184 | draft CR of BWP switch and CBW change test cases | Qualcomm | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Status | Comments |
| --- | --- | --- | --- | --- | --- |
| N.A. | R4-2214473 | WF on performance requirements for NTN | Xiaomi | Approved |  |
| R4-2211639 | R4-2214885 | Test cases for Intra- and inter-frequency HO with known cell for NTN | CATT | Endorse R4-2211639 Withdraw R4-2214885 |  |
| R4-2211640 | R4-2214886 | Test cases for Intra- and inter-frequency CHO for NTN | CATT | Endorsed |  |
| R4-2211960 | N.A. | CR on measurement accuracy requirement for NTN | Xiaomi | Merged to R4-2213524 |  |
| R4-2211962 | N.A. | CR on UE conditions for measurement performance requirements for NTN | Xiaomi | Merged to R4-2214061 |  |
| R4-2211963 | R4-2214570 | CR on test case for cell reselection to FR1 intra-frequency NR cell for satellite access | Xiaomi | Endorsed |  |
| R4-2212184 | R4-2214943 | draft CR of BWP switch and CBW change test cases | Qualcomm | Endorse R4-2212184 Withdraw R4-2214943 |  |
| R4-2212400 | R4-2214954 | Introduction of test cases for Inter-frequency measurement delay for satellite access with gap | MediaTek | Endorsed |  |
| R4-2212401 | R4-2214955 | Introduction of test cases for Accuracy for SS-RSRQ for satellite access | MediaTek | Endorse R4-2212401 Withdraw R4-2214955 |  |
| R4-2213352 | R4-2215048 | draft CR on test cases for Beam Failure Detection and Link Recover for NTN | Ericsson | Endorse R4-2213352 Withdraw R4-2215048 |  |
| R4-2213353 | R4-2215049 | draft CR on test cases for L1-RSRP measurement delay for NTN | Ericsson | Endorse R4-2213353 Withdraw R4-2215049 |  |
| R4-2213476 | R4-2215062 | DraftCR on UE transmit timing tests for NTN | Huawei | Endorse R4-2213476 Withdraw R4-2215062 |  |
| R4-2213524 | N.A. | CR on measurement accuracy requirements for NTN | Huawei | Endorsed |  |
| R4-2213525 | R4-2215066 | CR on TCs for SSB based RLM for NTN | Huawei | Endorse R4-2213525 Withdraw R4-2215066 |  |
| R4-2213526 | R4-2215067 | CR on TCs for intra-frequency measurement delay for NTN | Huawei | Endorse R4-2213526 Withdraw R4-2215067 |  |
| R4-2213527 | R4-2215068 | CR on TCs for RRC Re-establishment for NTN | Huawei | Endorse R4-2213527 Withdraw R4-2215068 |  |
| R4-2213528 | R4-2215069 | CR on TCs for RSRP accuracy for NTN | Huawei | Endorse R4-2213528 Withdraw R4-2215069 |  |
| R4-2214061 | N.A. | Conditions for RRM requirements for satellite access band in TS 38.133 | Ericsson | Endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents