3GPP TSG-RAN WG4 Meeting # 104-e														R4-2214294

Electronic Meeting, August 15 – 26, 2022

Agenda item:			9.5.5.1

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104-e][305] NR_Repeater_RFConformance_Part1

Document for:	Information

# Introduction

This topic deals with the general NR repeater conformance issues such as the test signals, test configurations and declarations. This is the 1st meeting discussing these issues.

List of candidate targets of email discussion for 1st round and 2nd round:

- Test signals

- Test configurations

- Manufacturer declarations

Please note that tdocs R4-2212837 and R4-2212838 are handled in thread [306].

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CATT | Huiping Shan | shanhuiping@catt.cn |
| Nokia | Bartlomiej Golebiowski | Bartlomiej.golebiowski@nokia.com |
| NEC | Tetsu Ikeda | tetsu.ikeda@nec.com |
| Huawei | Michal Szydelko | michal.szydelko@huawei.com |

# Topic #1: Test signals

This topic deals with the stimulus signals for repeater testing and what signals are to be used for UL and DL.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2213714 | ZTE Corporation | Discussion on NR repeater Stimulus signals Proposal 1: to check with ETSI ERM/MSG TFES on FR1 NR based Repeater stimulus signal spectral purity requirements Proposal 2: to check with ETSI ERM/MSG TFES on FR2 NR based Repeater stimulus signal spectral purity requirements |
| R4-2213971 | Nokia, Nokia Shanghai Bell | TP to TS 38.115-1 clause 4.9 RF channels and test models |
| R4-2213972 | Nokia, Nokia Shanghai Bell | TP to TS 38.115-2 clause 4.9 RF channels and test models |

## Open issues summary

This topic deals with the stimulus signals for repeater testing and what signals are to be used for UL and DL.

### Sub-topic 1-1 - Input signal spectral purity

Sub topic addresses the quality/spectral purity of the input signal.

Issue 1-1-1: Input signal spectral purity

- Proposals

  - Option 1: E-UTRA repeater stimulus signal spectral purity requirements

  - Option 2: check with ETSI ERM/MSG TFES on FR1/FR2 NR based repeater stimulus signal spectral purity requirements

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1 - Test signals

| Company | Comments |
| --- | --- |
| CATT | Issue 1-1-1: Input signal spectral purity Could ZTE clarify if there’s any difference between ETSI requirements and the E-UTRA repeater stimulus signal? |
| Ericsson | Our understanding is that there is no spectrum purity specification for NR and no activity currently in ETSI. We would like to check what the expected activity / response would be? |
| Nokia: | For this issue Nokia submitted Tdocs R4-2213976/77 with TPs for stimulus signals to 38.115-1 and 38.115-2 that are discussed in thread 306. In that Tdocs we highlighted that input signal spectral purity needs to be consider for NR repeaters. However, we think that there is no need to check with ETSI ERM/MSG TFES. Current E-UTRA repeater input signal spectral purity was developed in RAN4. Thus, we think similar work can by done by RAN4 for NR repeaters. In submitted Tdocs R4-2213976/77 with TPs for stimulus signals to 38.115-1 and 38.115-2 tables with input signal purity details are in brackets. Companies are encouraged to check details and propose new values if needed. |
| ZTE | Issue 1-1-1: Input signal spectral purity The existing requirement was inherited from ETSI ERM/MSG and this is the same for UTRA and E-UTRA, for FR1 NR, maybe we could reuse the existing one, however for FR2 NR input signals, we think we could not directly reuse FR1 input signal, right? I just checked the history for this requirement, it seems that RAN4 don’t have some technical input for it and just reusing the input from ETSI. Given the current status, not sure how to move forward. |
| Huawei | It is unclear if and how ETSI MSG TFES may help here for the FR2 stimulus signal. Maybe we can further check and continue discussion on the FR2 stimulus signal differentiation, if necessary. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213971 | CATT: For DL, the applicability of the test models needs to be updated according repeater’s requirements. For UL, It seems IAB-MT test models are copied directly to repeater. Some of the name of the wording and some requirements need to be modified, such as transmitter …, receiver …, etc. Some requirement doesn’t exist for IAB-MT, such as ACRR. |
|  | Ericsson: Which tests are intended to be single carrier tests for repeaters? The single carrier text and B, M, T is only needed for such tests. There is a text about “aggregated channel bandwidth”… but there is no channel bandwidth for repeaters. Also the wording around continuous spectrum operation and channel bandwidth with CA needs revision. |
|  | ZTE: similar comments CATT and Ericsson |
|  | NEC: Need more discussion on maximum repeater RF Bandwidth, aggregated repeater channel bandwidth, sub-block bandwidth, etc. Also need more discussion on how single carrier/multi carrier tests can be interpreted in repeater testing. |
| R4-2213972 | CATT: Similar comments with 3971 |
|  | Ericsson: See comments on 3971 |
|  | ZTE: similar comments as before. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Input signal spectral purity | Candidate options: The following two options were discussed during the first round: - Option 1: E-UTRA repeater stimulus signal spectral purity requirements - Option 2: check with ETSI ERM/MSG TFES on FR1/FR2 NR based repeater stimulus signal spectral purity requirements Recommendations for 2nd round: Based on the feedback, the following is proposed by the Moderator: - Proposal 1: No need to check repeater stimulus signal spectral purity requirements with ETSI ERM/MSG TFES, - Proposal 2: For FR1, reuse the repeater stimulus signal spectral purity requirements from E-UTRA repeater specification, as baseline. - Proposal 3: For FR2, further discuss whether the same stimulus signal spectral purity requirements can be used as for FR1. Conclusions to be captured in the WF. NOTE: related TPs discussed in [306]. GTW (Friday 19th) agreements: Proposal 1, 2 & 3 agreed. Further discuss how to deal with FR2 in 2nd round and encourage TE vendors to provide feedback. |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2213971 | TP to TS 38.115-1 clause 4.9 RF channels and test models To be revised. |
| R4-2213972 | TP to TS 38.115-2 clause 4.9 RF channels and test models To be revised. |

## Discussion on 2nd round (if applicable)

### Open issues

Issue 1-2-1 - Input signal spectral purity requirement for FR2

| Company | Comments |
| --- | --- |
|  | Further discuss how to deal with FR2 in 2nd round and encourage TE vendors to provide feedback. |
| Keysight | For FR2, it’s necessary to define first that if this is at RIB (at DUR input) or equipment output. I believe, test signal would be provided through feed antenna then through over-the-air. This makes pathloss reduces noise level but signal power level may not be enough to meet expected level so that power amplifier may be used which changes signal condition (increases noise). Also, numbers defined by dBc (close to edge) may need to be reduced for some amount due to higher in frequency and larger in channel BW. As of now, assuming this would be defined at RIB, define numbers only close to edge where currently defined as dBc in ETSI number for FR1. (also to reduce a bit) Where numbers are dBm are highly up to test setup, so that it’s not easy to set some number, so not to define these. |

# Topic #2: Test configurations

This topic deals with proposal for the test configurations

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2212623 | Ericsson | TP to TS 38.115-1: Test Configurations and Requirement applicability |
| R4-2212624 | Ericsson | TP to TS 38.115-2: Test Configurations and Requirement applicability |
| R4-2213715 | ZTE Corporation | Discussion on NR repeater test configuration Proposal 1: propose to use the nominal repeater channel bandwidth for the definition of NR repeater testing signal; |

## Open issues summary

Test configurations generally deal with how test signasl are applies to utilise the available BW and power allocations.

### Sub-topic 2-1 – test configurations

Issue 2-1-1: Naming of test configurations

- Proposals

  - Option 1: NTCx

  - Option 2: RTCx

- Recommended WF

  - TBA

Issue 2-1-2: Carrier allocation

RAN4 to clarify what to assume for the maximum number of carriers when allocating carriers within the passband

- Proposals

  - Option 1: no limit on the number of carriers

  - Option 2: a nominal maximum number of carriers

- Recommended WF

  - TBA

Issue 2-1-3: BW

- Proposals

  - Propose to use the nominal channel bandwidth for the definition of NR repeater testing signal

- Recommended WF

  - TBA

### Sub-topic 2-2 – others

Issue 2-2-1: Clarification on NTC2 carrier allocation

- Proposals

  - RAN4 clarify NTC2 and CACLR TC

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1 – test configurations

| Company | Comments |
| --- | --- |
| CATT | Issue 2-1-1: Naming of test configurations We support RTCx. Issue 2-1-2: Carrier allocation We slightly prefer to follow BS approach. Issue 2-1-3: BW We’re ok. |
| Ericsson | Issue 2-1-1: No strong opinion; maybe RTC is best to differentiate from BS Issue 2-1-2: It is not possible to use the BS approach since there is no concept of carriers or the declared number of supported carriers. Either a nominal number of carriers is needed or to fill the passband until it is full. We are OK with either option and have a slight preference on no limit for simplicity, although from previous discussions it may be that there is a need to specify a nominal number of carriers in order to prevent that the whole passband is utilized? Issue 2-1-3: OK |
| Nokia | Issue 2-1-1: Naming of test configurations We prefer RTCx to avoid mixing NR and Repeater and other test configurations. Also similar approach was already used for IAB (IABTCx is used in 38.176-1 and 38.176-2 specs). Issue 2-1-2: Carrier allocation Our preference is option 2. Allocation of carriers only at the edges of the passbands in order to increase the PSD close to the edges would create most stringent scenario for test that is beneficial to ensure repeaters will fulfill requirements. Issue 2-1-3: BW OK with proposal. |
| ZTE | Issue 2-1-1: Naming of test configurations We support to use the RTCx to avoid the confusion in future. Issue 2-1-2: Carrier allocation Similar understanding as Ericsson, there is no concept of carrier. The nominal number of carrier is used to fulfill the pass-band until it’s full. Issue 2-1-3: BW OK with proposal. |
| NEC | Issue 2-1-1: RTCx would be better. Issue 2-1-2: We prefer option 2. Option 1 could not be the worst case condition. Issue 2-1-3: Ok with the proposal. |
| Huawei | Issue 2-1-1: RTCx to differentiate from test configs already defined for NR BS Issue 2-1-2: option 2 seems better to model the worst case. Issue 2-1-3: ok with the proposal. |

Sub topic 2-2 –others

| Company | Comments |
| --- | --- |
| CATT | In our understanding, NC spectrum can also be supported in the same passband. So it seems reusing BS approach may be ok. The wording may need some working to include both cases. |
| Ericsson | The figure is not a proposal, it is pointing out what would happen with the current TC; there would not be any carriers close to the CACLR gap between passbands. Probably it would be best to specify that carriers are placed on either side of each gap between passbands. |
| ZTE | We propose to use the BS approach for NRTC3 instead of NRTC2 i think, the detailed approach could be as Ericsson proposed, testing signal follow the Issue 2-1-3 |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212623 | Ericsson: May need some revision depending on the outcome of the discussions above. |
|  | Nokia: As issue 2-1-1 the naming for test configurations should be separate from NR and RTCx should be used to avoid misunderstanding. |
|  | ZTE: please follow the above agreement and further update the CR. |
| R4-2212624 | Ericsson: May need some revision depending on the outcome of the discussions above. |
|  | Nokia: Similar comment as to R4-2212623. |
|  | ZTE: please follow the above agreement and further update the CR. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Naming of test configurations | Candidate options: The following two options were discussed during the first round: - Option 1: NTCx - Option 2: RTCx Recommendations for 2nd round: Based on the feedback, all companies support Option 2. The following is proposed by the Moderator: Proposal 4: Use RTCx naming convention for NR repeater test configurations. GTW (Friday 19th) agreements: Proposal 4 agreed. |
| Issue 2-1-2: Carrier allocation | Candidate options: The following two options were discussed during the first round: - Option 1: no limit on the number of carriers - Option 2: a nominal maximum number of carriers Recommendations for 2nd round: Based on the feedback, it was clarified that it is not possible to directly follow the BS approach, i.e. there is no concept of a maximum number of supported carriers and hence the BS TC method for allocating carriers cannot be directly applied. As Option 2 seems to be slightly preferred by companies, the following is proposed by the Moderator: Proposal 5: further discuss pros and cons and verify whether option 2 is agreeable. GTW (Friday 19th) agreements: - Agreement: “a nominal maximum number of carriers” agreed and further discuss the carrier definition and the details of number - Option 1: 2 times of number of passbands unless passband too small - Other options not precluded |
| Issue 2-1-3: BW | Candidate options: The following option was discussed during the first round: - Option 1: Propose to use the nominal channel bandwidth for the definition of NR repeater testing signal Recommendations for 2nd round: Based on the feedback, all companies support Option 1. The following is proposed by the Moderator: Proposal 6: use the nominal channel bandwidth for the definition of NR repeater testing signal. GTW (Friday 19th) agreements: Proposal 6 agreed |
| Issue 2-2-1: Clarification on NTC2 carrier allocation | Candidate options: The following was discussed during the first round: - RAN4 clarify NTC2 and CACLR TC Recommendations for 2nd round: Based on the feedback, the following is proposed by the Moderator: Proposal 7: Further discuss how to test the non-contiguous spectrum within the band, i.e. more than one passband. GTW (Friday 19th) agreements: For CACLR TC, carriers shall be allocated on top and bottom of each passband. |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212623 | TP to TS 38.115-1: Test Configurations and Requirement applicability To be revised. |
| R4-2212624 | TP to TS 38.115-2: Test Configurations and Requirement applicability To be revised. |

## Discussion on 2nd round (if applicable)

### Open issues

Issue 2-1-2: Carrier allocation

| Company | Comments |
| --- | --- |
|  | Further discuss the carrier definition and the details of number - Option 1: two times of number of passbands unless passband too small - Other options not precluded |

# Topic #3: Manufacturer declarations

This topic area covers other general conformance issues which potentially require declarations including:

- RF channels

- TDD switching

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2211708 | CATT | Discussion of the remaining issues for TDD repeater test Proposal 1: Long group delay should be declared when beginning of the output transient period is later than the beginning of the input transient period. Proposal 2: The beginning of the output signal transient period should refer the output signal or group delay + beginning of the input transient period. Proposal 3: The EVM test should refer output signal’s timing. |
| R4-2212627 | Ericsson | TP to TS 38.115-1: In-band measurements Annex |
| R4-2212628 | Ericsson | TP to TS 38.115-2: In-band measurements Annex |
| R4-2213716 | ZTE Corporation | Discussions on NR repeater test cases Proposal 1: propose to adopt the RF channels in above table for NR repeater conformance testing. |

## Open issues summary

### Sub-topic 3-1 – RF chnanels

Issue 3-1-1: RF test channels

- Proposals

  - To adopt the RF channels in the following table for NR repeater conformance testing

|  | ## RF channel |  |  |
| --- | --- | --- | --- |
|  | Single carrier | Multi-carrier | Multi-carrier, Multi-band |
| ## Repeater output power | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Frequency stability ## Error Vector Magnitude | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Out of band gain | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Unwanted emissions, ## ACLR, spurious emission | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Input intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Output intermodulation | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Adjacent Channel Rejection Ratio (ACRR) | B,M,T | BRFBW, MRFBW, TRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| ## Transmit ON/OFF power | M | MRFBW | BRFBW_T'RFBW, B'RFBW_TRFBW |
| Note: since there is no carrier related definition in repeater, carrier should be placed by nominal channel bandwidth of pass band. |  |  |  |

- Recommended WF

  - TBA

### Sub-topic 3-2 – TDD switching

Issue 3-2-1: TDD Switching

- Proposals

  - Proposal 1: Long group delay should be declared when beginning of the output transient period is later than the beginning of the input transient period.

  - Proposal 2: The beginning of the output signal transient period should refer the output signal or group delay + beginning of the input transient period.

  - Proposal 3: The EVM test should refer output signal’s timing.

- Recommended WF

  - TBA

### Sub-topic 3-3 –Declarations

Issue 3-3-1: Manufacturer declarations for NR FR1

- Proposals

  - Proposal 1: There is no need for declaration for contiguous and non-contiguous spectrum operations.

  - Proposal 2: Note on declarations contiguous or non-contiguous spectrum is removed.

  - Proposal 3: Include length of repeater internal delay to long delay repeater declaration D.15

- Recommended WF

  - TBA

Issue 3-3-2: Manufacturer declarations for NR FR2

- Proposals

  - Proposal 1: As a starting point, change power declarations to be per passband instead of per carrier, but further review the consequences.

  - Proposal 2: Put fractional bandwidth related declarations to square brackets

  - Proposal 3: There is no need for declaration for contiguous and non-contiguous spectrum operations.

  - Proposal 4: Note 7 can be removed from the declaration table.

  - Proposal 5: Include length of repeater internal delay to long delay repeater declaration D.15

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1 - RF channels

| Company | Comments |
| --- | --- |
| Ericsson | Regarding the section about RF channels for single carrier tests, are there any single carrier tests for the repeater? For the multi-carrier and multi-band, how does this relate to the test configuration descriptions in the case of repeater? The section needs to be related to the repeater situation with passbands, possibly even removed if there are no single carrier tests and the TCs are sufficient. |
| Nokia | In general we are fine, some further test effort reduction can be consider for example M channel for spurious emission. |
| ZTE | We are fine with it. From our understanding, single carrier should be still the baseline. For multi-carrier an multi-band test configuration, we need to consider how to place the following repeater testing carrier on it. Table 4.7.2-1: Signal to be used to build FR1 NR repeater TCs Operating Band characteristics; FDL_high – FDL_low. TC signal; BWchannel; min(100MHz, BWpassband). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing. Table 4.7.2.1-2: Signal to be used to build FR2 NR repeater TCs Operating band characteristics; FDL_high – FDL_low ≤ 3250 MHz. TC signal; BWchannel; min(400MHz, BWpassband). characteristics; Subcarrier spacing; Smallest supported subcarrier spacing declared per operating band. |
| NEC | For multi-carrier, we guess it is assumed in the proposal that passbands are filled with carriers. Firstly, we should confirm it is a correct assumption. |

Sub topic 3-2 –TDD switching

| Company | Comments |
| --- | --- |
| CATT | The proposals are from our side. We tried to follow the agreements in previous meetings. But we found very difficult to understand the motivation of referring input signal’s timing. And we also would like to know how to use the declared group delay in the |
| Ericsson | The repeater should be declared as needing the long delay if it is expected that the TDD switching cannot be met (and also the interference is planned to be managed). It is not essential to declare the delay; in fact, the delay in test conditions may in some cases differ from the delay in a deployment. The TDD switching could be referred to the output for long delay repeaters. However the test engineer would need to adjust the EVM calculation to take into account the delay. Alternatively, the delay between input and output timing could be declared and used for the output timing and adjust the EVM. Either could work, but it should be clarified that declared delay for testing may not be the same as deployment. |
| Nokia | The logic of proposal 1 and proposal 2 is unclear. When transient time is measured it is just confirmed that power is low enough / high enough after the transient time has passed. There is need to declare the group delay only if repeater cannot meet the transient time measured based on timing at repeater input. When the declaration is done, the transient time measurement shall be done later in time, shifted by the declared internal delay. For Proposal 3, referring to output timing is valid but this should be obvious: test equipment shall anyway compare the ideal signal to received signal and they need to be time aligned before the comparison can be done. |
| ZTE | We don’t see the big problem since test equipment will always sync with output signals and do the EVM measurement. Or do the measurement with input signal timing +group delay. And it seems 1st option 1 is mor straight forward way. |
| NEC | We do not understand proposal 1 and 2 well. Unless there is no delay, condition in proposal 1 is always met? EVM should be measured on output signal at output signal timing. |

Sub topic 3-3 –Declarations

| Company | Comments |
| --- | --- |
| XXX | Issue 3-3-1: Manufacturer declarations for NR FR1 Issue 3-3-2: Manufacturer declarations for NR FR2 |
| CATT | The two issues are included in [306] summary. They can be removed from this thread. |
| Ericsson | OK to discuss in other thread, for reference our understanding is that the contiguous/non-contiguous spectrum is captured in the passband declarations and not needed separately and that the power declaration should be per passband, since this is how the core requirement is defined. |
| Nokia | Issue 3-3-1: Manufacturer declarations for NR FR1 This is discussed now both in 305 and 306, and discussion in 306 covers also other Tdocs. We prefer to discuss in 306. Issue 3-3-2: Manufacturer declarations for NR FR2 This is discussed now both in 305 and 306, and discussion in 306 covers also other Tdocs. We prefer to discuss in 306. |
| ZTE | Fine to discuss this in another thread. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212627 | CATT: “BS” exists. |
| R4-2212628 | CATT: “BS” exists. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Issue 3-1-1: RF test channels | Candidate options: The following option was discussed during the first round: - Option 1: To adopt the RF channels in the following table for NR repeater conformance testing ## RF channel. Single carrier; Multi-carrier; Multi-carrier, Multi-band. ## Repeater output power; B,M,T; BRFBW, MRFBW, TRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Frequency stability ## Error Vector Magnitude; B,M,T; BRFBW, MRFBW, TRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Out of band gain; B,M,T; BRFBW, MRFBW, TRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Unwanted emissions, ## ACLR, spurious emission; B,M,T; BRFBW, MRFBW, TRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Input intermodulation; M; MRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Output intermodulation; M; MRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Adjacent Channel Rejection Ratio (ACRR); B,M,T; BRFBW, MRFBW, TRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. ## Transmit ON/OFF power; M; MRFBW; BRFBW_T'RFBW, B'RFBW_TRFBW. Note: since there is no carrier related definition in repeater, carrier should be placed by nominal channel bandwidth of pass band.. Recommendations for 2nd round: Based on the feedback, companies seems to be ok to consider the above table as the starting point, with additional simplifications needed, and clarifications requested for multi-carrier and multi-band cases. The following is proposed by the Moderator: Proposal 8: Consider the above table as baseline for RF test channels for single carrier. Further discussion needed to clarify multi-carrier and multi-band cases. GTW (Friday 19th) agreements: Proposal 8 agreed |
| Issue 3-2-1: TDD Switching | Candidate options: The following proposals were discussed during the first round: - Proposal 1: Long group delay should be declared when beginning of the output transient period is later than the beginning of the input transient period. - Proposal 2: The beginning of the output signal transient period should refer the output signal or group delay + beginning of the input transient period. - Proposal 3: The EVM test should refer output signal’s timing. Recommendations for 2nd round: Based on the feedback, there is lack of consensus among companies, with additional clarifications still needed. The following is proposed by the Moderator: Continue the discussion during the second round. |
| Issue 3-3-1: Manufacturer declarations for NR FR1 | Candidate options: Companies preferred to discuss this within [306] with other tdocs. Recommendations for 2nd round: No further discussion. |
| Issue 3-3-2: Manufacturer declarations for NR FR2 | Candidate options: Companies preferred to discuss this within [306] with other tdocs. Recommendations for 2nd round: No further discussion. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212627 | TP to TS 38.115-1: In-band measurements Annex To be revised. |
| R4-2212628 | TP to TS 38.115-2: In-band measurements Annex To be revised. |

## Discussion on 2nd round (if applicable)

### Open issues

Issue 3-2-1: TDD Switching

| Company | Comments |
| --- | --- |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2214367 | WF on [305] NR Repeater_RF Conformance (Part1) agreements | Huawei |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2213714 |  | Discussion on NR repeater Stimulus signals | ZTE Corporation | Noted |  |
| R4-2213971 | R4-2214865 | TP to TS 38.115-1 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2213972 | R4-2214866 | TP to TS 38.115-2 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2212623 | R4-2214785 | TP to TS 38.115-1: Test Configurations and Requirement applicability | Ericsson | Revised |  |
| R4-2212624 | R4-2214786 | TP to TS 38.115-2: Test Configurations and Requirement applicability | Ericsson | Revised |  |
| R4-2213715 |  | Discussion on NR repeater test configuration | ZTE Corporation | Noted |  |
| R4-2211708 |  | Discussion of the remaining issues for TDD repeater test | CATT | Noted |  |
| R4-2212627 | R4-2214789 | TP to TS 38.115-1: In-band measurements Annex | Ericsson | Revised |  |
| R4-2212628 | R4-2214790 | TP to TS 38.115-2: In-band measurements Annex | Ericsson | Revised |  |
| R4-2213716 |  | Discussions on NR repeater test cases | ZTE Corporation | Noted |  |

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214367 |  | WF on [305] NR Repeater_RF Conformance (Part1) agreements | Huawei | Agreeable |  |
| R4-2214865 |  | TP to TS 38.115-1 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2214866 |  | TP to TS 38.115-2 clause 4.9 RF channels and test models | Nokia, Nokia Shanghai Bell | Agreeable |  |
| R4-2214785 |  | TP to TS 38.115-1: Test Configurations and Requirement applicability | Ericsson | Agreeable |  |
| R4-2214786 |  | TP to TS 38.115-2: Test Configurations and Requirement applicability | Ericsson | Agreeable |  |
| R4-2214789 |  | TP to TS 38.115-1: In-band measurements Annex | Ericsson | Agreeable |  |
| R4-2214790 |  | TP to TS 38.115-2: In-band measurements Annex | Ericsson | Agreeable |  |