3GPP TSG-RAN WG4 Meeting # 104-e												R4-2214313

Electronic Meeting, 15 – 26 August 2022

Agenda item:			8.14.9

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [104-e][324] NR_exto71GHz_Demod_Part1

Document for:	Information

# Introduction

This document contains the summary for guiding the discussion on the email thread [104-e][324] NR_exto71GHz_Demod_Part1, which discusses demodulation requirements for extension to 71 GHz, with general issues and issues related to base station demodulation. Issues related to UE demodulation are discussed on [104-e][325] NR_exto71GHz_Demod_Part2.

This summary is based on the contributions submitted to the agenda items:

- 9.14.8	Demodulation and CSI requirements 	[NR_ext_to_71GHz-Perf]

- 9.14.8.1	General 	[NR_ext_to_71GHz-Perf]

- 9.14.8.3	BS demodulation requirements 	 [NR_ext_to_71GHz-Perf]

- 9.14.8.3.1	PUSCH requirements 	[NR_ext_to_71GHz-Perf]

- 9.14.8.3.2	PUCCH requirements 	[NR_ext_to_71GHz-Perf]

- 9.14.8.3.3	PRACH requirements 	[NR_ext_to_71GHz-Perf]

- 9.14.9	Moderator summary and conclusions

The topics for this discussion are organized as follows:

- Topic #1 includes general aspects which apply for both UE and BS demod

- Topic #2 includes BS demodulation aspects only related to PUSCH

- Topic #3 includes BS demodulation aspects only related to PUCCH

- Topic #4 includes BS demodulation aspects only related to PRACH

Please note that some of the discussions in Topic #1 are also repeated in other topics.

It is appreciated that the delegates for this topic put their contact information in the table below:

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia, Nokia Shanghai Bell | Rafael Paiva | Rafael.paiva@nokia.com |
| Anritsu | Osamu Yamashita | Osamu.Yamashita@anritsu.com |
| Ericsson | Nicholas Pu | Nicholas.pu@ericsson.com |
| Rohde & Schwarz | Niels Petrovic | Niels.petrovic@rohde-schwarz.com |
| Samsung | Yunchuan Yang | yc0301.yang@samsung.com |
| Nokia, Nokia Shanghai Bell | Karsten Petersen | Karsten.petersen@nokia-bell-labs.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General aspects

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Issue mapping |
| --- | --- | --- | --- |
| R4-2211540 | Anritsu Corporation | Views on channel model for FR2-2 Observation 1: Due to the limitation of BW (i.e. 400 MHz) with the test equipment, there is a need to merge taps for the delay profiles. Observation 2: Influence of the merge of taps needs to be confirmed if it is acceptable. For example, the RMS delay spread may become 5.46ns for TDLA5 and 10.24ns for TDLA10. Proposal 1: Apply 2.5ns delay resolution at FR2-2. Observation 3: SCS for 480 kHz and 960 kHz are optional in TS 38.101-2. Proposal 2: Channel models for 480 kHz and 960 kHz SCS shall be at least optional. Observation 4: Channel BW 1600 MHz and 2000 MHz need more time to study the feasibility from TE PoV. Observation 5: TDLA10 and TDLD10 channel models are feasible by the test equipment. Observation 6: No strong views on the max doppler frequency from the TE implementation PoV. | 1-2-1 1-2-2 |
| R4-2212105 | Nokia, Nokia Shanghai Bell | On General for ext71GHz Channel model requirements definition Observation 1: 5 ns delay resolution is not enough for modelling channel delay spread of 5 ns and 10 ns. Observation 2: 5 ns delay resolution is used for TDL-A 30 channel model considering 200 MHz CHW. Observation 3: FR2-2 demodulation requirements will imply in test setups using larger sampling frequency in comparison to FR2-1. Proposal 1: RAN4 to specify TDL models for FR2-2 using smaller delay resolution. Observation 4: With 5 ns delay resolution used for TDLA10, not enough taps are used for small delay. Proposal 2: RAN4 to adopt 16 taps with 2 ns tap delay resolution for TDL models with 10 ns delay spread. Proposal 3: RAN4 to adopt the following channel delay profile for TDLA 10: Delay; power. 0; -16.1. 4; 0. 6; -4. 8; -10.2. 16; -18.6. 18; -9.3. 22; -13.7. 24; -17.9. 26; -13.5. 30; -14. 40; -15.4. 44; -18.9. 46; -21. 48; -21.6. 50; -19.3. 96; -25.9. Max Doppler Frequency Observation 5: In our understanding having requirements for also faster UE movement than 3km/h, requirements should be defined for also 10km/h. Proposal 4: RAN4 to define demodulation requirements using 650 Hz Doppler for QPSK and PRACH, PUCCH, PDCCH & PBCH. Doppler of 200kHz to be used for the remaining cases. SCS/CBW Combinations Observation 6: We see it possible portable device will prefer to support 960kHz SCS instead of 480kHz SCS, to avoid the complexity of implementing ICI. Observation 7: Even if it is currently debatable whether all scenarios of 960kHz are testable with legacy TE equipment, it is useful for RAN4 to have good requirement coverage for 960kHz SCS to anticipate the new TEs developed for FR2-2 and to inform operators about expected minimum performance in deployment. Observation 8: There are no physical limitations that make the development of 71GHz TEs with usable SNR ranges at 960 kHz impossible. Proposal 5: RAN4 to define demodulation requirements for 960kHz SCS. Whether to define requirements with 1600 MHz CBW for 480 kHz SCS Observation 9: It will be beneficial to define requirements for higher CBW to give RAN5 the opportunity and incentive to improve the test setup. Proposal 6: RAN4 to define demodulation requirements with 1600 MHz CBW for 480kHz SCS. CBW for 960 kHz SCS Observation 10: It will be beneficial to define requirements for higher CBW to give RAN5 the opportunity and incentive to improve the test setup. Proposal 7: RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS. Whether to update number of PRBs for 960kHz/2000MHz to 156 Observation 11: RF session has agreed maximum number of PRBs of 148 for the combination 960kHz SCS and 2000 MHz CBW. Proposal 8: For UE Demodulation requirements update number of PRBs of 960kHz/2000 MHz to follow RF agreements, i.e., 148 PRBs. UE demod specification Structure Observation 12: We see it to be possible to in-cooperate FR2-2 in the existing structure with minor additions to existing headlines. Observation 13: Agreeing to option 1 or option 2 will introduce new sub sections where potentially the existing subsections 7.2.2.2.1 and 7.2.2.2.2 and 7.2.2.2.3 would have to be introduced. Overall, we see this as breaking the current structure and would introduce new sub-sections. Proposal 9: Do not introduce new subsections in the specification. Update legacy table headings to include FR1-1. Add new table with table heading FR2-2. Change applicability section (“Applicability of requirements for operating bands”) to only required testing of table with FR2-2 in FR2-2 bands. | 1-2-1 1-2-1 1-2-4 1-2-3 1-3-1 1-3-2 1-3-2 Moved to 325 Moved to [325] |
| R4-2212225 | Ericsson | Discussion on general issue on FR2-2 demodulation Observation 1: The PN impact is a bit different for DL and UL cases. Proposal 1: Invite TE vendors to give confirmations on the resolution of tap delay and the number of taps for TDLA10 and TDLD10. Proposal 2: Use 650Hz Doppler shift for FR2-2 NLOS channel model. | 1-2-1 1-2-3 |
| R4-2213803 | Huawei,HiSilicon | Discussion on general issues for FR2-2 demodulation requirements Observation 1: RF part only defined CA scenarios with CA_n41-n263, CA_n77-n263 and CA_n79-n263 but not defined DC scenarios for FR1+FR2-2 Proposal 1: RAN4 to consider following scenarios for FR2-2 requirements definition: - Single carrier (Optional feature) - FR1+FR2-2 CA scenarios (Mandatory feature) Observation 2: From point of performance gain, ICI compensation doesn’t bring performance gain compared to CPE compensation for PDSCH/PUSCH when PN is modelled in RX side for MCS20 and MCS22 with 120 kHz SCS/400MHz CBW. Furthermore, ICI compensation is only applicable for corner conditions: High SNR (larger than 20dB), Large PTRS samples. Observation 3: From point of complexity, typical ICI compensation algorithm brings much matrix operation complexity. Observation 4: From point of testability, ICI compensation only works well on high SNR (Large than 20dB), which can’t be verified considering limited maximum testable SNRBB. Proposal 2: Not consider ICI compensation for FR2-2 performance requirements definition. Proposal 3: Not consider 960kHz for FR2-2 demodulation requirements definition Proposal 4: Reuse the procedure in clause B.2.1 of TS 38.101-4 for simplified TDLA10 and TDLD10 channel generation. Observation 5: Simplification procedure for TDLA channel affect the frequency correlation and performance requirements in FR2-2 band, especially for larger bandwidth. The impact of simplification steps of TDLA channel for 100MHz bandwidth is smaller than that for 400MHz. Observation 6: Simplification procedure for TDLD channel don’t affect the frequency correlation and performance requirements much in FR2-2 band. Proposal 5: Use Table 2-1 and Table 2-2 for simplified TDLA and TDLD channel model respectively. | 1-1-1 1-4-1 1-3-1 1-2-4 1-2-4 |
| R4-2213804 | Huawei,HiSilicon | Discussion on spec structure for FR2-2 demodulation and CSI requirements Proposal 1: Use the same sub-clauses and separate tables for FR2-1 and FR2-2 UE requirements. Proposal 2: Use Table 2.1 for TS 38.101-4 spec structure. Proposal 2: Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of 52.6 GHz~71 GHz”. Proposal 3: Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure. | Move to [325] Move to [325] 1-5-1 1-5-1 |
| R4-2213967 | Qualcomm Incorporated | UE Demodulation Requirements for FR2-2 (up to 71GHz) <Moderator comment> Proposed to be treated in thread [104-e][325] NR_exto71GHz_Demod_Part2 |  |
| R4-2212673 | Nokia, Nokia Shanghai Bell | Draft CR - definition of FR2-2 <Moderator comment> Proposed to be treated in thread [104-e][325] NR_exto71GHz_Demod_Part2 |  |

## Open issues summary

### Sub-topic 1-1 Scenarios for demodulation requirements for FR2-2

Issue 1-1-1: Scenarios for requirements definition

- Proposals

  - Option 1: RAN4 to consider following scenarios for FR2-2 requirements definition:

    - Single carrier (Optional feature)

    - FR1+FR2-2 CA scenarios (Mandatory feature)

  - Recommended WF

    - Please discuss option 1.

### Sub-topic 1-2 Channel model

Issue 1-2-1: Channel model tap delay resolution

- Proposals

  - Proposal 1: Invite TE vendors to give confirmations on the resolution of tap delay and the number of taps for TDLA10 and TDLD10.

  - Proposal 2: Apply 2.5ns delay resolution at FR2-2.

  - Proposal 3: RAN4 to specify TDL models for FR2-2 using smaller delay resolution.

  - Proposal 4: RAN4 to adopt 16 taps with 2 ns tap delay resolution for TDL models with 10 ns delay spread.

- Recommended WF

  - Please discuss among the proposals and list which ones we can agree upon.

  - Proposals are not mutually exclusive

Issue 1-2-2: Optionality of channel models

- Proposals

  - Option 1: Channel models for 480 kHz and 960 kHz SCS shall be at least optional.

- Recommended WF

  - Please discuss option 1

Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz

- Proposals

  - Option 1: Use 650Hz Doppler shift for FR2-2 NLOS channel model.

  - Option 2: RAN4 to define demodulation requirements using 650 Hz Doppler for QPSK and PRACH, PUCCH, PDCCH & PBCH. Doppler of 200Hz to be used for the remaining cases.

- Recommended WF

  - Please discuss Option 1 and Option 2

Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz

- Proposals

  - Option 1: Use the following tables for simplified TDLA and TDLD channel model respectively.

Simplified TDLA10 channel model with different resolution

| Tap # | 2.5ns resolution |  | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] |  |
| 1 | 0 | -17.4 | 0 | -16.1 | Rayleigh |
| 2 | 5 | 0 | 4 | 0 | Rayleigh |
| 3 | 7.5 | -9.5 | 6 | -4 | Rayleigh |
| 4 | 15 | -19.9 | 8 | -10.2 | Rayleigh |
| 5 | 20 | -10.6 | 18 | -8.8 | Rayleigh |
| 6 | 22.5 | -15 | 22 | -13.7 | Rayleigh |
| 7 | 25 | -13.5 | 24 | -17.9 | Rayleigh |
| 8 | 30 | -15.3 | 26 | -13.5 | Rayleigh |
| 9 | 40 | -16.7 | 30 | -14 | Rayleigh |
| 10 | 45 | -18.1 | 40 | -15.4 | Rayleigh |
| 11 | 47.5 | -22.9 | 46 | -15.6 | Rayleigh |
| 12 | 97.5 | -24 | 96 | -23 | Rayleigh |

Simplified TDLD10 channel model with different resolution

| Tap # | 2.5ns resolution |  | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.3 | 0 | -0.3 | LOS |
|  | 0 | -12.4 | 0 | -12.4 | Rayleigh |
| 2 | 5 | -21.1 | 6 | -21.1 | Rayleigh |
| 3 | 12.5 | -22.9 | 14 | -16.8 | Rayleigh |
| 4 | 15 | -18 | 18 | -18.4 | Rayleigh |
| 5 | 17.5 | -18.4 | 26 | -22 | Rayleigh |
| 6 | 25 | -22 | 40 | -27.9 | Rayleigh |
| 7 | 40 | -27.9 | 80 | -23.7 | Rayleigh |
| 8 | 80 | -23.7 | 94 | -24.9 | Rayleigh |
| 9 | 95 | -24.9 | 98 | -30.1 | Rayleigh |
| 10 | 97.5 | -30.1 | 126 | -27.5 | Rayleigh |
| 11 | 125 | -27.5 |  |  | Rayleigh |

  - Option 2: RAN4 to adopt the following channel delay profile for TDLA 10:

| Delay | power |
| --- | --- |
| 0 | -16.1 |
| 4 | 0 |
| 6 | -4 |
| 8 | -10.2 |
| 16 | -18.6 |
| 18 | -9.3 |
| 22 | -13.7 |
| 24 | -17.9 |
| 26 | -13.5 |
| 30 | -14 |
| 40 | -15.4 |
| 44 | -18.9 |
| 46 | -21 |
| 48 | -21.6 |
| 50 | -19.3 |
| 96 | -25.9 |

  - Option 3: Reuse the procedure in clause B.2.1 of TS 38.101-4 for simplified TDLA10 and TDLD10 channel generation.

  - Option 4: For UE Demod requirements in FR2-2, use the TDLD10 Delay Profile from the table below:

TDLD10 (DS = 10 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS Path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 5 | -21.0 | Rayleigh |
| 3 | 15 | -16.7 | Rayleigh |
| 4 | 20 | -18.3 | Rayleigh |
| 5 | 25 | -21.9 | Rayleigh |
| 6 | 40 | -27.8 | Rayleigh |
| 7 | 80 | -23.6 | Rayleigh |
| 8 | 95 | -23.7 | Rayleigh |
| 9 | 125 | -27.7 | Rayleigh |

- Recommended WF

  - Please comment on options 1, 2 and 3

### Sub-topic 1-3 Channel bandwidth and SCS

Issue 1-3-1: SCS for demodulation requirements

- Proposals

  - Option 1: RAN4 to define demodulation requirements for 960kHz SCS.

  - Option 2: Not consider 960kHz for FR2-2 demodulation requirements definition

- Recommended WF

  - Please discuss if Option 1 is agreeable

  - Please notice there are specific issues discussing SCS for PUSCH, PUCCH and PRACH

Issue 1-3-2: Channel bandwidth for demodulation requirements

- Proposals

  - Option 1: RAN4 to define demodulation requirements for 960kHz SCS.

    - RAN4 to define demodulation requirements with 1600 MHz CBW for 480kHz SCS.

    - RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS.

- Recommended WF

  - Please discuss if Option 1 is agreeable. CBW is also discussed in issue 2-1-2

### Sub-topic 1-4 Assumptions of processing

Issue 1-4-1: Use of ICI for demodulation requirements

- Proposals

  - Option 1: Do not consider ICI for BS demodulation requirements

  - Option 2: Not consider ICI compensation for FR2-2 performance requirements definition.

- Recommended WF

  - Please discuss Option 1 and Option 2 and if it has to be considered differently for UE and BS requirements.

Issue 1-4-2: SNR limit

- Proposals

  - Proposal 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget.

- Recommended WF

  - Please discuss Proposal 1.

### Sub-topic 1-5 Spec structure

Issue 1-5-1: Base station structure

- Proposals

  - Proposal 1:. Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of  52.6 GHz~71 GHz”.

  - Proposal 2: Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure

- Proposed positions in TS 38.104 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 11.2.2.1 |
| PUCCH OTA performance requirements for format 0 | 11.3.2.2 |
| PUCCH OTA performance requirements for format 1 | 11.3.2.3 |
| PUCCH OTA performance requirements for format 2 | 11.3.2.4 |
| PUCCH OTA performance requirements for format 3 | 11.3.2.5 |
| PUCCH OTA performance requirements for format 4 | 11.3.2.6 |
| PRACH OTA performance requirements | 11.4.2.2 |

- Proposed positions in TS 38.141-2 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 8.2.1 |
| PUCCH OTA performance requirements for format 0 | 8.3.1 |
| PUCCH OTA performance requirements for format 1 | 8.3.2 |
| PUCCH OTA performance requirements for format 2 | 8.3.3 |
| PUCCH OTA performance requirements for format 3 | 8.3.4 |
| PUCCH OTA performance requirements for format 4 | 8.3.5 |
| PRACH OTA performance requirements | 8.4.1 |

- Recommended WF

  - Please discuss Proposal 1 and Proposal 2.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Anritsu | Sub topic 1-2: Issue 1-2-1: Channel model tap delay resolution It is supposed to be similar discussion with issue 1-2-4. We are fine with either Option 1 with 2.5ns tap resolution in issue 1-2-4 or option 4 in issue 1-2-4. Issue 1-2-2: Optionality of channel models The intention of this proposal is just to keep the consistency between UE TRx spec. Since there is a note in TS38.101-2 clause 5.3.2 that SCS 480 kHz and 960 kHz are optional in Rel-17, demodulation requirement shall also have the same note even if we decide to define the requirements for 480 kHz and 960 kHz. Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz As mentioned in our contribution, our current test equipment has the bandwidth limitation (i.e. 400MHz). Thus, we support either option 1 with 2.5ns resolution or option 4. Issue 1-3-1: SCS for demodulation requirements Similar comment with issue 1-2-2. We are fine to define 480 kHz or 960kHz SCS requirements as far as the they are defined as optional. Issue 1-3-2: Channel bandwidth for demodulation requirements Same as issue 1-3-1. And we need more time to investigate the feasibility of 1600 MHz/ 2000 MHz CBW by TE. |
| Ericsson | Issue 1-1-1: Scenarios for requirements definition We support Option 1 for UE demodulation. Issue 1-2-1: Channel model tap delay resolution We propose TE vendors get confirmation on tap number and resolutions at the first. Basically, smaller resolution is preferred considering the possible performance impact. Issue 1-2-2: Optionality of channel models We don’t think we will use different channel models for 480kHz and 960kHz SCS. Can proposer explain the motivation? Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz We prefer Option 1 to cover outdoor deployment. The 650Hz Doppler is equivalent to 10km/h speed which is the same as FR2-1. Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz Depend on the conclusion of 1-2-1. Issue 1-3-1: SCS for demodulation requirements We support Option 2. 960kHz SCS is optional for UE and is not a typical deployment considering its limited PRB number and PSD. The other issue of 960kHz SCS is its very small CP length, and the corresponding timing would be very tight. It is similar as 60kHz SCS in FR1 and 240kHz SCS in FR2-1, and no requirement was defined for them. The same methodology could be applied for 960kHz SCS. From the test requirement point of view, the PN impact would be also small for other SCSs in UL demodulation tests regarding to the testability. Based on our simulation results, the PN impact is small for at least up to MCS20 cases and the concern on ICI is unnecessary. Issue 1-3-2: Channel bandwidth for demodulation requirements We suggest to sperate the UL and DL discussion for this topic due to different link budget could be. For UL demodulation, we suggest only consider minimum CBW for now until RF have conclusion on the link budget. Issue 1-4-1: Use of ICI for demodulation requirements We suggest to sperate the UL and DL discussion for this topic due to different link budget and test configuration could be. For UL demodulation, we support Option 1. Issue 1-4-2: SNR limit We support Proposal 1. Issue 1-5-1: Base station structure We prefer adding “FR2-1” and “FR2-2” in the requirements table title for distinguishing. Using sperate tables for FR2-2 requirement within the same sub-clause as FR2-1 requirements could be possible, but further case by case check is needed. |
| Rohde & Schwarz | Issue 1-2-1: This point goes more or less hand in hand with Issue 1-2-4. We are ok to keep the same maximum number of taps as for FR2-1. Regarding delay resolution, we propose to keep the 5ns to be used for channel bandwidths up to 200 MHz. For larger channel bandwidths we propose to use 2ns. In the past we used a single delay for all channel bandwidths, but now for FR2-2 with the large spread in bandwidths, using different spreads resolutions is more practical from TE point of view. Ericsson@RS: We don’t quite get the point. Do you want to say “delay resolution” in the last sentence? The delay spread depends on multipath of the deployment scenario not the channel bandwidth in our understanding. @Ericsson: Yes, sorry it is a typo. It should be resolution, not delay spread. Issue 1-2-2: Not really clear on this one. The channel models should not be affected by the SCS Issue 1-2-3: I hope it is meant to be 200 Hz (instead of 200 kHz). Then we are fine with either option. Issue 1-2-4: Option 3 would be our preferred solution, with the delay spread adapted as described for issue 1-2-1. The examples from Option 1 seem to be correct for the 2ns and > 200 MHz bandwidth. |
| Nokia | Issue 1-2-1: Channel model tap delay resolution We support Proposal 4. We think that considering that channel BW is increased to minimum 400 MHz for 480/960 kHz SCS, it would be natural to increase the delay resolution proportionally to the BW. Issue 1-2-2: Optionality of channel models We also don’t understand how the channel models can be optional, considering that the same channel model was also agreed for 120 kHz. Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz We prefer Option 2, with a more strict doppler shift for robust MCS and PRACH/PUCCH, PDCCH, PBCH Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz Our preference is Option 2 for TDLA, We can also agree with Option 3 and Option 1 with 2 ns. We can compromise with Option 1 with 2 ns resolution, however our calculation using clause B.2.1 of TS 38.101-4 results in difference channel delay profile. Can the proponent of Option 1 check the values for 2 ns? 0; -16.1. 4; 0. 6; -4. 8; -10.2. 16; -18.6. 18; -9.3. 22; -13.7. 24; -17.9. 26; -13.5. 30; -14. 42; -13.8. 96; -21.2. Issue 1-3-1: SCS for demodulation requirements We remain with the understanding that companies that want to test 960kHz should have requirements made available. We have repeatedly seen that ICI implementation can be foregone in ICI, which makes it an attractive alternative to 480. I.e., option 1 should be included for those companies that see merit in the complexity tradeoff. To honour the concerns of the contributors that want to prioritise 480kHz, we can compromise to either declaring 960kHz requirements as completely optional (similar to HST). Or alternatively we can have an applicability rule that allows to choose between testing 480 and 960 kHz. Ericsson@Nokia: We don’t quite understand the way of “declaring 960kHz requirements as completely optional (similar to HST).” As our understanding, BS have flexibility to declare which SCS is supported per band, and all declared SCS should be tested. Does Nokia’s proposal mean a BS could declare to support 960kHz without any test? It would be better that Nokia can give more explanation here, thanks. Issue 1-3-2: Channel bandwidth for demodulation requirements We agree with option 1. We prefer testing minimum and maximum BW as a baseline assumption. If RF session decides that a CBW is not testable we can revise the agreement Issue 1-4-1: Use of ICI for demodulation requirements We don’t agree with option 1 and 2. We prefer that ICI is considered in the cases where it shows benefits, i.e. high MCSs and wide bandwidth. Issue 1-4-2: SNR limit We are fine with proposal 1. Issue 1-5-1: Base station structure We are fine in general with Proposal 1 and 2. We just need another clause for PRACH with LRA=1151 and LRA=571 |
| Samsung | Issue 1-1-1: Scenarios for requirements definition For UL, we only consider to define requirement for single carrier, since there is no different processing for CA scenarios. Issue 1-2-1: Channel model tap delay resolution We would like to encourage TE vendors to give confirmations. Compared with FR2-1, 5 ns delay resolution is used. We are ok with 2.5ns Issue 1-2-2: Optionality of channel models In general, we agree 480KHz and 960KHz SCS should be optional, while we are not sure whether it is proper to agree channel model itself is optional or not Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz We support option 2, since we have already agreed to consider 200Hz for TDL-D Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz We support option 3, used the procedure for simplified channel model generation based on agreed delay resolution, not sure whether option1 is generated based on this procedure Issue 1-3-1: SCS for demodulation requirements We support option 2, not to define requirement 960KHz. From initial access perspective, as indicated in WID, 960KHz SCS for the SSB is not supported by UE, and 480 kHz is an optional SSB numerology for initial access for the UE. we would like to prioritize the requirement for UE with supported SCS for both data and initial access. Issue 1-3-2: Channel bandwidth for demodulation requirements We support only to define requirement with 120KHz and 480KHz, with minimum CBW for each SCS, 100MHz for 120KHz SCS , 400MHz for 480KHz SCS Issue 1-4-1: Use of ICI for demodulation requirements We wonder what the difference between option 1 and option 2. In our view, ICI compensation is up to BS implementation Issue 1-4-2: SNR limit We are ok with option 1 Issue 1-5-1: Base station structure We prefer option 1, similar as RF spec for FR2-2, with separate requirement tables and simulation assumption tables Table 9.4.3.3-1: Minimum requirement for BS type 2-O total power dynamic range in FR2-1 SCS; OTA total power dynamic range (dB). (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. 60; 18.1; 21.2; 24.2; N/A. 120; 15.0; 18.1; 21.2; 24.2. Table 9.4.3.3-2: Minimum requirement for BS type 2-O total power dynamic range in FR2-2 |
| Moderator (Nokia) | Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF Issue 3-1: AWGN offset |
| Huawei | Issue 1-1-1: Scenarios for requirements definition It should be discussed in UE performance requirements part. We support option 1 in UE requirements part. Issue 1-2-1: Channel model tap delay resolution We are OK with either 2ns or 2.5ns. We prefer 2ns Issue 1-2-2: Optionality of channel models We don’t understand this issue. Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz We support option 1. Define single Doppler shift for all cases Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz We support option 1. For option 4: Why use 9 taps ? Issue 1-3-1: SCS for demodulation requirements Option 2 Issue 1-3-2: Channel bandwidth for demodulation requirements For 480 kHz SCS, we prefer to only consider 400MHz We don’t support define any cases for 960kHz SCS Issue 1-4-1: Use of ICI for demodulation requirements Based on our simulation results, ICI compensation doesn’t bring obvious gain compared to CPE compensation. We suggest to not consider ICI compensation. Issue 1-4-2: SNR limit OK with Proposal 1. |
| Qualcomm | Issue 1-2-1: Channel model tap delay resolution We are ok with a reduced delay resolution of 2.5ns or 2 ns (better); However, we believe that RAN4 should not use different sampling resolutions for FR2-2 requirements based on CBW, and we should keep the channel same delay profiles for 120kHz/100MHz and 480/400MHz. Considering that we will use lower delay spread with respect to FR2-1, there is no need to use the same delay resolution Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz @Huawei: Option 4 is the outcome of the procedure in clause B.2.1 of TS 38.101-4 for simplified TDLD10 channel generation, based on 5ns sampling resolution. If a different value is agreed according to Issue 1-2-1, Option 4 should be discarded and the procedure repeated with the new tap delay resolution; @Nokia: According to our computations, the values in Option 1 reflect correctly the outcome of the procedure in clause B.2.1 of TS 38.101-4 for TDLA10, and not the table shared in the comments. |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Issue 1-1-1: Scenarios for requirements definition Original proposals: - Option 1: RAN4 to consider following scenarios for FR2-2 requirements definition: - Single carrier (Optional feature) - FR1+FR2-2 CA scenarios (Mandatory feature) Moderator remarks: Companies commenting this issue were not against agreeing on Option 1 for DL requirements. Therefore, the moderator proposes in agreeing on Option 1 for UE demodulation requirements Tentative agreements: - RAN4 to consider following scenarios for FR2-2 UE demodulation requirements definition: - Single carrier - FR1+FR2-2 CA scenarios Candidate options: - none Recommendations for 2nd round: - Confirm the agreement. |
| Sub-topic 1-2 | Issue 1-2-1: Channel model tap delay resolution - Proposals - Proposal 1 (Ericsson): Invite TE vendors to give confirmations on the resolution of tap delay and the number of taps for TDLA10 and TDLD10. - Proposal 2 (Anritsu, Samsung): Apply 2.5ns delay resolution at FR2-2. - Proposal 3: RAN4 to specify TDL models for FR2-2 using smaller delay resolution. - Proposal 4: RAN4 to adopt 16 taps with 2 ns tap delay resolution for TDL models with 10 ns delay spread. Moderator remarks: Diverging view among the companies. Keep discussing. Tentative agreements: Candidate options: - Same as 1st round Recommendations for 2nd round: - Keep discussion Issue 1-2-2: Optionality of channel models Original Proposals: - Option 1: Channel models for 480 kHz and 960 kHz SCS shall be at least optional. Moderator remarks: Diverging views. Not clear for many companies how this optionality would work in practice Tentative agreements: - none Candidate options: - Same as 1st round Recommendations for 2nd round: - Keep discussion Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz Original Proposals: - Option 1 (Ericsson R&S, Huawei): Use 650Hz Doppler shift for FR2-2 NLOS channel model. - Option 2 (R&S, Nokia, Samsung) : RAN4 to define demodulation requirements using 650 Hz Doppler for QPSK and PRACH, PUCCH, PDCCH & PBCH. Doppler of 200Hz to be used for the remaining cases. Moderator remarks: Diverging views in the first round. Tentative agreements: - none Candidate options: - Same as 1st round Recommendations for 2nd round: - Keep discussion Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz Original Proposals: - Option 1: Use the following tables for simplified TDLA and TDLD channel model respectively - Option 2: RAN4 to adopt the following channel delay profile for TDLA 10: - Option 3: Reuse the procedure in clause B.2.1 of TS 38.101-4 for simplified TDLA10 and TDLD10 channel generation. - Option 4: For UE Demod requirements in FR2-2, use the TDLD10 Delay Profile from the table below: Moderator remarks: Diverging views in the first round. Tentative agreements: - none Candidate options: - Same as 1st round Recommendations for 2nd round: - Keep discussion |
| Sub-topic 1-3 | Issue 1-3-1: SCS for demodulation requirements Original Proposals: - Option 1 (Nokia: RAN4 to define demodulation requirements for 960kHz SCS. - Option 2 (Huawei, Samsung, : Not consider 960kHz for FR2-2 demodulation requirements definition Moderator remarks: Diverging views on the first round. Tentative agreements: - none Candidate options: - Option 1: RAN4 to define demodulation requirements for 960kHz SCS. - Option 2: Not consider 960kHz for FR2-2 demodulation requirements definition - Option 3: RAN4 to define demodulation requirements for 960kHz SCS as optional. Recommendations for 2nd round: - Keep discussion Issue 1-3-2: Channel bandwidth for demodulation requirements Original Proposals: - Option 1: RAN4 to define demodulation requirements for 960kHz SCS. - RAN4 to define demodulation requirements with 1600 MHz CBW for 480kHz SCS. - RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS. Moderator remarks: Original options were not clear, more discussion needed. Tentative agreements: Candidate options: - For BS demodulation requirements - Proposal 1: RAN4 to define demodulation requirements with 400 MHz and 1600 MHz CBW for 480kHz SCS. - Proposal 2: RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS. - Proposal 3: RAN4 to define demodulation requirements with 400 MHz CBW for 480kHz SCS. - Proposal 4: RAN4 to define demodulation requirements with 400MHz CBW for 960kHz SCS. - For UE demodulation requirements - Proposal 1: RAN4 to define demodulation requirements with 400 MHz and 1600 MHz CBW for 480kHz SCS. - Proposal 2: RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS. - Proposal 3: RAN4 to define demodulation requirements with 400 MHz CBW for 480kHz SCS. - Proposal 4: RAN4 to define demodulation requirements with 400MHz CBW for 960kHz SCS. Recommendations for 2nd round: - Continue discussion on the second round |
| Sub-topic 1-4 | Issue 1-4-1: Use of ICI for demodulation requirements Original Proposals: - Option 1 (Ericsson): Do not consider ICI for BS demodulation requirements - Option 2 (Huawei): Not consider ICI compensation for FR2-2 performance requirements definition. - Don’t agree (Nokia) Moderator remarks: Diversing views on the 1st round. Tentative agreements - None: Candidate options: - Option 1: Do not consider ICI for BS demodulation requirements - Option 2: Not consider ICI compensation for FR2-2 performance requirements definition. - Option 3: Up to BS implementation Recommendations for 2nd round: - Continue discussion on the second round Issue 1-4-2: SNR limit Original Proposals: - Proposal 1 (Ericsson, Nokia, Samsung, Huawei, ): Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget. Moderator remarks: Companies agree on Proposal 1. In the meanwhile RF has agreed on the GTW indicating feasibility of demodulation requirements. So lets try to analyse if the last sentence is still needed. Tentative agreements: Candidate options: - Option 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget. - Option 2: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Recommendations for 2nd round: - Given the BS RF agreements, can we agree on Option 2? |
| Sub-topic 1-5 | Issue 1-5-1: Base station structure Original Proposals: - Proposal 1 (Huawei, Samsung, : Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of 52.6 GHz~71 GHz”. - Proposal 2 (Huawei, : Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure Moderator remarks: Diverging views Tentative agreements: Candidate options: - Proposal 1: Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of 52.6 GHz~71 GHz” - Proposal 2: Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure - Proposal 3: Use separate clauses for PRACH with LRA=1151 and LRA=571 as in NR-U requirements - Proposal 4: adding “” and “FR2-2” in the requirements table title for distinguishing - Proposal 1: Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of FR2-1” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of FR2-2” Recommendations for 2nd round: - Continue discussion |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

### Sub-topic 1-1 Scenarios for demodulation requirements for FR2-2

Issue 1-1-1: Scenarios for requirements definition

Issue closed, please refer to the 1st round summary for tentative agreement.

### Sub-topic 1-2 Channel model

Issue 1-2-1: Channel model tap delay resolution

- Proposals

  - RAN4 to specify TDLA10 and TDLD10 models for FR2-2 with

    - Option 1: 2 ns delay resolution and 16 taps

    - Option 2: 2 ns delay resolution and 12 taps

    - Option 3: 2.5 ns delay resolution and 16 taps

    - Option 4: 2.5 ns delay resolution and 12 taps

    - Option 5: 5 ns delay resolution and 16 taps

    - Option 6: 5 ns delay resolution and 12 taps

- Recommended WF

  - Please indicate which options are agreeable.

Issue 1-2-2: Optionality of channel models

- Proposals

  - Option 1: Channel models for 480 kHz and 960 kHz SCS shall be at least optional.

- Recommended WF

  - Please discuss option 1. Please refer to the discussion on SCS if the intention of the proposal was to discussion optionality of certain SCS.

Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz

- Proposals

  - Option 1: Use 650Hz Doppler shift for FR2-2 NLOS channel model.

  - Option 2: RAN4 to define demodulation requirements using 650 Hz Doppler for QPSK and PRACH, PUCCH, PDCCH & PBCH. Doppler of 200Hz to be used for the remaining cases.

  - Option 3 (new): Adopt 650 Hz for TDLA and 200 Hz with TDLD.

- Recommended WF

  - Please discuss Option 1, Option 2, and Option 3

Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz

- Proposals

  - Option 1: Use the following tables for simplified TDLA and TDLD channel model respectively.

Simplified TDLA10 channel model with different resolution

| Tap # | 2.5ns resolution |  | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] |  |
| 1 | 0 | -17.4 | 0 | -16.1 | Rayleigh |
| 2 | 5 | 0 | 4 | 0 | Rayleigh |
| 3 | 7.5 | -9.5 | 6 | -4 | Rayleigh |
| 4 | 15 | -19.9 | 8 | -10.2 | Rayleigh |
| 5 | 20 | -10.6 | 18 | -8.8 | Rayleigh |
| 6 | 22.5 | -15 | 22 | -13.7 | Rayleigh |
| 7 | 25 | -13.5 | 24 | -17.9 | Rayleigh |
| 8 | 30 | -15.3 | 26 | -13.5 | Rayleigh |
| 9 | 40 | -16.7 | 30 | -14 | Rayleigh |
| 10 | 45 | -18.1 | 40 | -15.4 | Rayleigh |
| 11 | 47.5 | -22.9 | 46 | -15.6 | Rayleigh |
| 12 | 97.5 | -24 | 96 | -23 | Rayleigh |

Simplified TDLD10 channel model with different resolution

| Tap # | 2.5ns resolution |  | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.3 | 0 | -0.3 | LOS |
|  | 0 | -12.4 | 0 | -12.4 | Rayleigh |
| 2 | 5 | -21.1 | 6 | -21.1 | Rayleigh |
| 3 | 12.5 | -22.9 | 14 | -16.8 | Rayleigh |
| 4 | 15 | -18 | 18 | -18.4 | Rayleigh |
| 5 | 17.5 | -18.4 | 26 | -22 | Rayleigh |
| 6 | 25 | -22 | 40 | -27.9 | Rayleigh |
| 7 | 40 | -27.9 | 80 | -23.7 | Rayleigh |
| 8 | 80 | -23.7 | 94 | -24.9 | Rayleigh |
| 9 | 95 | -24.9 | 98 | -30.1 | Rayleigh |
| 10 | 97.5 | -30.1 | 126 | -27.5 | Rayleigh |
| 11 | 125 | -27.5 |  |  | Rayleigh |

  - Option 2: RAN4 to adopt the following channel delay profile for TDLA 10:

| Delay | power |
| --- | --- |
| 0 | -16.1 |
| 4 | 0 |
| 6 | -4 |
| 8 | -10.2 |
| 16 | -18.6 |
| 18 | -9.3 |
| 22 | -13.7 |
| 24 | -17.9 |
| 26 | -13.5 |
| 30 | -14 |
| 40 | -15.4 |
| 44 | -18.9 |
| 46 | -21 |
| 48 | -21.6 |
| 50 | -19.3 |
| 96 | -25.9 |

  - Option 3: Reuse the procedure in clause B.2.1 of TS 38.101-4 for simplified TDLA10 and TDLD10 channel generation.

  - Option 4: For UE Demod requirements in FR2-2, use the TDLD10 Delay Profile from the table below:

TDLD10 (DS = 10 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS Path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 5 | -21.0 | Rayleigh |
| 3 | 15 | -16.7 | Rayleigh |
| 4 | 20 | -18.3 | Rayleigh |
| 5 | 25 | -21.9 | Rayleigh |
| 6 | 40 | -27.8 | Rayleigh |
| 7 | 80 | -23.6 | Rayleigh |
| 8 | 95 | -23.7 | Rayleigh |
| 9 | 125 | -27.7 | Rayleigh |

- Recommended WF

  - Please comment on options 1, 2 and 3

### Sub-topic 1-3 Channel bandwidth and SCS

Issue 1-3-1: SCS for demodulation requirements

- Proposals

  - Option 1: RAN4 to define demodulation requirements for 120 kHz, 480 kHz and 960kHz SCS.

  - Option 2: Not consider 960kHz for FR2-2 demodulation requirements definition

  - Option 3: RAN4 to define demodulation requirements for 120 kHz, 480 kHz and 960 kHz SCS, where 960kHz SCS as optional.

- Recommended WF

  - Please comment on the options. A new Option 3 was included according to a 1st round discussion suggestion

  - Please notice there are specific issues discussing SCS for PUSCH, PUCCH and PRACH

Issue 1-3-2: Channel bandwidth for demodulation requirements

This issue is now separated into BS and UE demodulation parts.

Issue 1-3-2a: Channel bandwidth for BS demodulation requirements

Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF, Issue 3-1: AWGN offset

- Proposals

- For BS demodulation requirements

  - Proposal 1: RAN4 to define demodulation requirements with 400 MHz and 1600 MHz CBW for 480kHz SCS.

  - Proposal 2: RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS.

  - Proposal 3: RAN4 to define demodulation requirements with 400 MHz CBW for 480kHz SCS.

  - Proposal 4: RAN4 to define demodulation requirements with 400MHz CBW for 960kHz SCS.

- Recommended WF

  - Please discuss the options having the RF agreement in mind.

Issue 1-3-2b: Channel bandwidth for UE demodulation requirements

Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF, Issue 3-1: AWGN offset

- Proposals

- For UE demodulation requirements

  - Proposal 1: RAN4 to define demodulation requirements with 400 MHz and 1600 MHz CBW for 480kHz SCS.

  - Proposal 2: RAN4 to define demodulation requirements with 400MHz and 2000MHz CBW for 960kHz SCS.

  - Proposal 3: RAN4 to define demodulation requirements with 400 MHz CBW for 480kHz SCS.

  - Proposal 4: RAN4 to define demodulation requirements with 400MHz CBW for 960kHz SCS.

  - Proposal 5 (from UE Demod round 1 discussion)

    - Define one set of requirements, which match the currently available legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA).

    - Define one set of requirements, which match the feasibility of 71GHz capable test equipment (480 and 960 kHz with larger CBW).

    - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available.

- Recommended WF

  - Please discuss the options having the RF agreement in mind.

### Sub-topic 1-4 Assumptions of processing

Issue 1-4-1: Use of ICI for demodulation requirements

This issue is now split into BS and UE parts.

Issue 1-4-1a: Use of ICI for BS demodulation requirements

- Proposals

  - Option 1: Do not consider ICI

  - Option 2: Consider ICI

  - Option 3: It is up to the BS implementation

  - Option 4: Consider ICI in the following cases

| SCS | MCS | 100 MHz | 400 MHz | 800 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | QPSK | No | No | - | - | - |
|  | 16 QAM | No | Yes | - | - | - |
|  | 64 QAM | Yes | Yes | - | - | - |
| 480 kHz | QPSK | - | No | No | No | - |
|  | 16 QAM | - | No | No | Yes | - |
|  | 64 QAM | - | No | Yes | Yes | - |
| 960 kHz | QPSK | - | No | No | No | No |
|  | 16 QAM | - | No | No | No | No |
|  | 64 QAM | - | No | No | No | No |

- Recommended WF

  - Please discuss your preferences and indicate if there is an option that can be compromised. For Option 3, how should we consider simulation alignment if large differences are observed among companies?

Issue 1-4-1b: Use of ICI for UE demodulation requirements

- Proposals

  - Option 1: Do not consider ICI

  - Option 2: Consider ICI

  - Option 3: It is up to the UE implementation

  - Option 4: Consider ICI in the following cases

| SCS | MCS | 100 MHz | 400 MHz | 800 | 1600 | 2000 |
| --- | --- | --- | --- | --- | --- | --- |
| 120 kHz | QPSK | No | No | - | - | - |
|  | 16 QAM | No | Yes | - | - | - |
|  | 64 QAM | Yes | Yes | - | - | - |
| 480 kHz | QPSK | - | No | No | No | - |
|  | 16 QAM | - | No | No | Yes | - |
|  | 64 QAM | - | No | Yes | Yes | - |
| 960 kHz | QPSK | - | No | No | No | No |
|  | 16 QAM | - | No | No | No | No |
|  | 64 QAM | - | No | No | No | No |

- Recommended WF

  - Please discuss your preferences and indicate if there is an option that can be compromised. For Option 3, how should we consider simulation alignment if large differences are observed among companies?

Issue 1-4-2: SNR limit

Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF, Issue 3-1: AWGN offset

- Proposals

  - Option 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget.

  - Option 2: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage.

- Recommended WF

  - Given the BS RF agreements, can we agree on Option 2?

### Sub-topic 1-5 Spec structure

Issue 1-5-1: Base station structure

- Proposals

  - Proposal 1a: Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in spectrum range of 24.25~52.6 GHz” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in spectrum range of  52.6 GHz~71 GHz”.

  - Proposal 1b: Define the FR2-2 requirements in the same clause as FR2-1 requirements but separate requirements tables. The legacy FR2-1 requirements table title should be updated to “xxx requirements for BS type 2-O operating in FR2-1” and the table title for FR2-2 requirements should be said as “xxx requirements for BS type 2-O operating in FR2-2”.

  - Proposal 2: Use Table 2.2 for TS 38.104 spec structure and Table 2.3 for TS 38.141-2 spec structure

- Proposed positions in TS 38.104 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 11.2.2.1 |
| PUCCH OTA performance requirements for format 0 | 11.3.2.2 |
| PUCCH OTA performance requirements for format 1 | 11.3.2.3 |
| PUCCH OTA performance requirements for format 2 | 11.3.2.4 |
| PUCCH OTA performance requirements for format 3 | 11.3.2.5 |
| PUCCH OTA performance requirements for format 4 | 11.3.2.6 |
| PRACH OTA performance requirements | 11.4.2.2 |

- Proposed positions in TS 38.141-2 for FR2-2 requirements

| Requirements | Sub-clause |
| --- | --- |
| CP-OFDM PUSCH performance requirements | 8.2.1 |
| PUCCH OTA performance requirements for format 0 | 8.3.1 |
| PUCCH OTA performance requirements for format 1 | 8.3.2 |
| PUCCH OTA performance requirements for format 2 | 8.3.3 |
| PUCCH OTA performance requirements for format 3 | 8.3.4 |
| PUCCH OTA performance requirements for format 4 | 8.3.5 |
| PRACH OTA performance requirements | 8.4.1 |

- Recommended WF

  - Please confirm if we can agree on Proposal 1a or 1b

  - If Proposal 2 is not agreeable, please comment which cases need to be revised.

## Companies views’ collection for 2nd round

### Open issues

Sub topic 1-2 Channel model

| Company | Comments |
| --- | --- |
| XXX | Issue 1-2-1: Channel model tap delay resolution Issue 1-2-2: Optionality of channel models Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz |
| Nokia | Issue 1-2-1: Channel model tap delay resolution Our preference if Option 2. We can compromise to option 1, 3 and 4. For the resolution, it was also the assumption when defining delay resolution that with wider bandwidths the resolution is increased. Considering that 480 kHz and 960 kHz have minimum channel bandwidth of 400 MHz, it is natural to assume that achievable delay resolution is increased 2 times in comparison to 200 MHz requirements. Issue 1-2-2: Optionality of channel models We are not sure if the proposal referred to the SCS being optional. If yes we would prefer to discuss Issue 1-3-1. Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz Our preference if either Option 2 or 3. We can also compromise with Option 1. Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz For option 1 we don’t understand why the TDLD has less taps. Our calculations are still giving different values than the ones shown here. We can agree on Option 3 as a baseline. However, in our view we need to agree on the taps itself to avoid mismatch on the understanding of TS 38.101-4. |
| Anritsu | Issue 1-2-2: Optionality of channel models The original proposal was considering the relationship with the issue 1-3-1. But if it is not the idea in the group and same channel models would be applied irrespective of SCS, the optionality discussion is not necessary. [Aug 23] Issue 1-2-1: Channel model tap delay resolution Considering the situation that Anritsu is the only company proposing the delay resolution 2.5ns, we are fine to compromise to 2.0 ns resolution with the condition which was agreed during GTW on 22nd. (i.e. For below or equal to 200MHz, 5ns delay resolution; for above 200MHz, 2ns delay resolution.) As for the tap number, we confirmed 16 can be supported. |
| Ericsson | Issue 1-2-1: Channel model tap delay resolution Need TE vendors input on feasible tap number and resolution. We prefer Option 2 or 4 at current stage. Issue 1-2-2: Optionality of channel models We don’t see the necessary of this issue. If the motivation is to pick tap delay resolution (5ns for 120kHz SCS and other value for 480k/960kHz SCS), we think current delay spread is only 10ns which is also too small for 5ns resolution. Issue 1-2-3: Doppler shift for demodulation requirements above 52.6 GHz We think the Doppler shift is channel model relevant not physical channel relevant, so we supporte Option 3 to cover typical scenarios in both outdoor and indoor deployments. Issue 1-2-4: Simplified TDLA model for demodulation requirements above 52.6 GHz Pending the discussion until we have conclusion on 1-2-1. |
| Rohde & Schwarz | Issue 1-2-1: Channel model tap delay resolution We still need more time to confirm the number of taps feasible, so we propose to go with [12] right now. We can update to 16 in the next meeting in case we can confirm. Update: We are ok, with the proposal by Ericsson in the WF, keep 12 Taps for the grids with 5ns and go with [16] in brackets for the 2ns grids, to be confirmed in the next meeting. |
| Huawei | Issue 1-2-1: Channel model tap delay resolution According to the agreement from GTW, 5ns for CBW smaller than 200MHz and 2ns for CBW larger than 200MHz. We wonder whether 5ns resolution can reflect channel model defined in 38.901 if 10ns RMS delay spread is used. We prefer 30ns RMS delay for 5ns resolution that simplified channel model can be reused and 10ns RMS delay spread for 2ns resolution. TE vendor can give feedback of number of taps. |

Sub topic 1-3 Channel bandwidth and SCS

| Company | Comments |
| --- | --- |
| XXX | Issue 1-3-1: SCS for demodulation requirements Issue 1-3-2: Channel bandwidth for demodulation requirements Issue 1-3-2a: Channel bandwidth for BS demodulation requirements Issue 1-3-2b: Channel bandwidth for UE demodulation requirements |
| Nokia | Issue 1-3-1: SCS for demodulation requirements Our preference if Option 1. We would like to consider all SCS which are defined for ext to 71 GHz. We can compromise to Option 3. We are willing to compromising making 960 kHz optional. This can be done adopting the same procedure as used for HST. Issue 1-3-2a: Channel bandwidth for BS demodulation requirements We agree with proposals 1 and 2. We understand that RF room has taken time to analyse feasibility of BS demodulation requirements, and there was no contribution preventing the requirements, and that the current agreement indicates that those requirements are feasible to be specified. Therefore, we prefer to test 400 MHz and maximum bandwidth for 480 kHz and 960 kHz SCS. Issue 1-3-2b: Channel bandwidth for UE demodulation requirements We agree with proposals 1 and 2 and can compromise to proposal 5. We understand that RF room has taken time to analyse feasibility of BS demodulation requirements, and there was no contribution preventing the requirements, and that the current agreement indicates that those requirements are feasible to be specified. We have also in the UE Demod thread a compromise proposal where we suggest for RAN4 to define two set of requirements (Added as proposal 5): - Define one set of requirements, which match the currently available legacy TE capabilities (480kHz with not too large CBW, and 960 KHz with non-full FDRA). - Define one set of requirements, which match the feasibility of 71GHz capable test equipment (480 and 960 kHz with larger CBW). - Define capability rules to allow that one of the 2 sets of the requirements are passed depending on what test equipment is available. Therefore, we prefer to test 400 MHz and maximum bandwidth for 480 kHz and 960 kHz SCS with the above mentioned limitations for each requirement set. |
| Anritsu | Issue 1-3-1: SCS for demodulation requirements Our preference is option 2 but we can compromise with option 3. |
| Ericsson | Issue 1-3-1: SCS for demodulation requirements We don’t quite get the point that “960kHz SCS as optional”. Does it mean a BS could skip the tests for 960kHz SCS even 960kHz SCS is declared to be supported? If yes, we don’t think it is a good way for the requirement. We still prefer Option 2. Issue 1-3-2a: Channel bandwidth for BS demodulation requirements We want to point out that the AWGN_offset could be the last way for link budget compensation. If we lower AWGN_offset to 0dB, the noise floor is equal to sensitivity level. Then the measured SNR will depend on the product sensitivity which would not really reflect the demodulation performance. Regarding there is no clear agreement on whole link budget for FR2-2 UL demod in RF session, we still prefer Proposal 3 at the current stage to align with Issue 1-4-2. Issue 1-3-2b: Channel bandwidth for UE demodulation requirements We suggest to delete this issue since it has been captured in thread [325] discussion. |
| Samsung | Issue 1-3-1: SCS for demodulation requirements We prefer option 2, regarding FR2-2, UE supported 120KHz, 480KHz, and 960KHz are optional feature, we donot think it is reasonable to define 960KHz as optional. |
| Huawei | Issue 1-3-1: SCS for demodulation requirements We support option 3 Issue 1-3-2a: Channel bandwidth for BS demodulation requirements We share the same views with Ericsson and not consider reduce the AWGN offset. |

Sub topic 1-4 Assumptions of processing

| Company | Comments |
| --- | --- |
| XXX | Issue 1-4-1a: Use of ICI for BS demodulation requirements Issue 1-4-1b: Use of ICI for UE demodulation requirements Issue 1-4-2: SNR limit |
| Nokia | Issue 1-4-1a: Use of ICI for BS demodulation requirements We prefer option 2, but can compromise with Option 4. We don’t understand how Option 3 would work on simulation alignment. It could happen that if a company has implemented ICI and the other don’t the results will differ significantly, and we will have problem converging to the requirements. Option 4 shows some examples of test cases where is expected to provide more benefits. Issue 1-4-1b: Use of ICI for UE demodulation requirements We prefer option 2, but can compromise with Option 4. We don’t understand how Option 3 would work on simulation alignment. It could happen that if a company has implemented ICI and the other don’t the results will differ significantly, and we will have problem converging to the requirements. Option 4 shows some examples of test cases where is expected to provide more benefits. Issue 1-4-2: SNR limit We prefer Option 2. We understand that the current agreement from RF indicates the feasibility of the requirements, and therefore we don’t need to keep the sentence indicating that we are waiting for RF decision. |
| Ericsson | Issue 1-4-1a: Use of ICI for BS demodulation requirements Different PN model would cause different performance on high MCS or larger CBW. The ICI benefit could also be different in that case. Issue 1-4-1b: Use of ICI for UE demodulation requirements We suggest to delete this issue since it has been captured in thread [325] discussion. Issue 1-4-2: SNR limit We slightly prefer Option 1. Should we align the view between 1-3-2 and 1-4-2? |
| Samsung | Issue 1-4-1a: Use of ICI for BS demodulation requirements We prefer to option2, whether ICI is up to BS implementation, pending on MCS and channel bandwidth, In the low MCS and small number of bandwidth, the ICI gain is limited, |
| Huawei | Issue 1-4-1: Use of ICI for demodulation requirements We prefer to not consider ICI compensation considering limited performance gain. Issue 1-4-2: SNR limit Option 1. Testable SNR should be derived from RF part. |
| Qualcomm | Issue 1-4-1b: Use of ICI for UE demodulation requirements @moderator: ICI for UE demodulation is discussed in Thread [325], we would like to encourage interested companies to comment on it there and not adding this to the WF for Thread [324] to avoid duplication. |

Sub topic 1-5 Spec structure

| Company | Comments |
| --- | --- |
| XXX | Issue 1-5-1: Base station structure |
| Nokia | Issue 1-5-1: Base station structure We prefer Proposal 1b in comparison to 1a. All proposals are agreeable in any case. There is no practical difference on both, so in our view it is more important to get to an agreement. |
| Ericsson | Issue 1-5-1: Base station structure Both Proposal 1a and 1b are fine for us. We slightly prefer 1b if “FR2-1” and “FR2-2” are allowed in the specification. |
| Samsung | Issue 1-5-1: Base station structure We support option 1, similar as the RF spec. |
| Huawei | Issue 1-5-1: Base station structure Option 1a |

# Topic #2: PUSCH requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Issue mapping |  |
| --- | --- | --- | --- | --- |
| R4-2212226 | Ericsson | Discussion on FR2-2 PUSCH demodulation requirements Proposal 1: Do not consider ICI for BS demodulation requirements. Proposal 2: RAN4 do not consider 960kHz SCS for FR2-2 BS demodulation requirements. Proposal 3: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget. Proposal 4: If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2 minimum CBW per SCS, MCS 4/16/20 for 1Tx and MCS 4/16 for 2Tx could be used for FR2-2 PUSCH demodulation requirements. Proposal 5: Do not define demodulation requirement for FR2-2 PUSCH repetition type A in Rel-17. Introduce the requirement in Rel-19 or higher release if necessary. | 2-3-1 2-1-1 2-3-2 2-4-2 2-4-1 |
| R4-2212229 | Ericsson | Simulation results for FR2-2 PUSCH demodulation requirements |  |
| R4-2212674 | Nokia, Nokia Shanghai Bell | Discussion on PUSCH demodulation requirements for the extension to 71 GHz Observation 1: Latest RAN4 agreements on number of PRBs cannot be used for transform-precoding test cases for which the number of PRBs have to be calculated considering powers of 2, 3, and 5 only. Proposal 1: Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as: SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 100; 64; 256; N/A; N/A; N/A. 480; N/A; 64; 120; 240; N/A. 960; N/A; 32; 60; 120; 144. Proposal 2: RAN4 to define demodulation requirements for FR2-2 including opportunity for AWGN_offset reduction as specified for FR2-1. Proposal 3: RAN4 to define demodulation requirements for 400 MHz CBW, 1600 MHz for 480 kHz SCS, and 2000 MHz for 960 kHz SCS. | 2-4-3 Thread [310] 2-1-2 |
| R4-2212675 | Nokia, Nokia Shanghai Bell | PUSCH simulation results for the extension to 71 GHz |  |
| R4-2213669 | Samsung | View on BS demodulation requirement for NR extended to 71GHz Proposal 1: 650Hz can be considered for TDL-A channel Proposal 2: RAN4 applies only 120KHz and 480 KHz SCS for UL requirements definition. | 2-2-1 2-1-1 |
| R4-2213812 | Huawei,HiSilicon | Discussion on FR2-2 PUSCH requirements Proposal 1: Define PUSCH performance requirements with MCS 4, 16 and 18. Proposal 2: Use 1T2R Low and 2T2R Low Proposal 3: Only consider 100MHz bandwidth for 120kHz SCS and 400MHz bandwidth for 480kHz SCS. Proposal 4: Define PUSCH requirements with 200Hz Doppler spread. Proposal 5: Define PUSCH requirements with 10ns RMS delay spread. Proposal 6: Use MCS 4 and 16 for TDL-A and MCS 18 for TDL-D. Proposal 7: Use Table 2-1 for FR2-2 PUSCH performance requirements definition. | 2-4-2 2-4-3 2-1-2 2-1-1 2-2-3 2-2-2 2-2-1 2-4-4 |
| R4-2213813 | Huawei,HiSilicon | Simulation results on FR2-2 PUSCH requirements |  |
| R4-2212150 | Intel Corporation | draftCR to 38104 on BS radiated requirements for PUSCH FR2-2 |  |
| R4-2212232 | Ericsson | Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements |  |
| R4-2212233 | Ericsson | Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements |  |
| R4-2213819 | Huawei,HiSilicon | Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2 |  |

## Open issues summary

### Sub-topic 2-1 Channel bandwidth and SCS

Issue 2-1-1: Sub-carrier spacings for PUSCH requirements

- Proposals

  - Option 1: 120 Hz and 480 kHz

  - Option 2: 120 kHz, 480 kHz and 960 kHz

- Recommended WF

  - Discuss your preferences among the options having in mind the parallel discussion in the general Issue 1-3-1

Issue 2-1-2: Channel bandwidth for PUSCH requirements

- Proposals

  - Option 1:

    - 120 kHz SCS with 100 MHz

    - 480 kHz SCS with 400MHz

  - Option 2:

    - 120 kHz SCS with 100 MHz and 400 MHz

    - 480 kHz SCS with 400MHz and 1600 MHz

    - [960 kHz SCS with 400 MHz and 2000 MHz]* pending decision on the inclusion of 960 kHz

- Recommended WF

  - Discuss your preferences among the options.

### Sub-topic 2-2 Channel model

Issue 2-2-1: TDL model used for PUSCH requirements

- Proposals

  - Option 1: Use MCS 4 and 16 for TDL-A and MCS 18 for TDL-D.

- Recommended WF

  - Please comment on your preferences

Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements

- Proposals

  - Option 1: Define PUSCH requirements with 10ns RMS delay spread..

- Recommended WF

  - Please comment if Option 1 is agreeable.

Issue 2-2-3: Doppler spread for PUSCH requirements

- Proposals

  - Option 1: 200 Hz

  - Option 2: 650 Hz

- Recommended WF

  - Please indicate your preference among options 1 and 2.

### Sub-topic 2-3 Assumptions for PUSCH demodulation requirements

Issue 2-3-1: Use of ICI for BS demodulation requirements

- Proposals

  - Option 1: Do not consider ICI for BS demodulation requirements

- Recommended WF

  - Please discuss if Option 1 is agreeable and indicate other options otherwise.

Issue 2-3-2: SNR limit

- Proposals

  - Proposal 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget.

- Recommended WF

  - Please discuss Proposal 1.

### Sub-topic 2-4 Configuration and test cases PUSCH demodulation requirements

Issue 2-4-1: Repetition type A

- Proposals

  - Option 1: Do not define demodulation requirement for FR2-2 PUSCH repetition type A in Rel-17. Introduce the requirement in Rel-19 or higher release if necessary.

- Recommended WF

  - Please discuss if Option 1 is agreeable.

Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements

- Proposals

  - Option 1:

    - Define PUSCH performance requirements with MCS 4, 16 and 18.

    - Use 1T2R Low and 2T2R Low

  - Option 2:

    - If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2 minimum CBW per SCS, MCS 4/16/20 for 1Tx and MCS 4/16 for 2Tx could be used for FR2-2 PUSCH demodulation requirements.

- Recommended WF

  - Please discuss options 1 and 2.

Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding

- Proposals

  - Option 1: Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 64 | 256 | N/A | N/A | N/A |
| 480 | N/A | 64 | 120 | 240 | N/A |
| 960 | N/A | 32 | 60 | 120 | 144 |

- Recommended WF

  - Please discuss Option 1 and comment if it is acceptable.

Issue 2-4-4: Parameters for PUSCH requirements

- Proposals

  - Option 1: Use the following table for FR2-2 PUSCH performance requirements definition

Proposed simulation assumptions for FR2-2 PUSCH

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |

- Recommended WF

  - Please comment on Option 1 and comment if it is acceptable.

  - Some entries in the table might need revision after reaching agreements on the other issues

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Prefer Option 1. Issue 2-1-2: Channel bandwidth for PUSCH requirements Support Option 1 at current stage. Make final decision until RF have conclusion on link budget issue. Issue 2-2-1: TDL model used for PUSCH requirements We can accept MCS4 and MCS16 for TDL-A and MCS20 for TDL-D. Regarding CPE has been agreed to be considered for the requirement and the degradation caused by PN is also small for MCS20 with TDL-D, we think it is still feasible for the requirements. Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements We can accept 10ns for the Delay spread. Issue 2-2-3: Doppler spread for PUSCH requirements Same comment as 1-2-3. Issue 2-3-1: Use of ICI for BS demodulation requirements We support Option 1. Issue 2-3-2: SNR limit We support Proposal 1. Issue 2-4-1: Repetition type A We support Option 1 to only consider normal requirements for FR2-2 PUSCH. Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements We support Option 2 at current stage. Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding We think Option 1 is OK but only smallest PRB for 120/480kHz SCS can be considered. Issue 2-4-4: Parameters for PUSCH requirements Depends on the agreements of topics above. |
| Nokia | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements We prefer Option 2. We could compromise with Option 2a, where requirements for 960 kHz are declared optional. Issue 2-1-2: Channel bandwidth for PUSCH requirements We prefer option 2, and review after decision from RF Issue 2-2-1: TDL model used for PUSCH requirements We agree with the intention of Option 1. We think it has to be reviewed the maximum MCS with 64 QAM Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements We are fine with Option 1. Issue 2-2-3: Doppler spread for PUSCH requirements We are fine using 650Hz for QPSK and 16 QAM. We prefer 200 Hz for 64 QAM Issue 2-3-1: Use of ICI for BS demodulation requirements We don’t agree with Option 1. We think ICI provides gains for high MCS and wide bandwidth. Therefore, we think we determine a set of requirements that need ICI For example we could say: MCS; 100 MHz; 400 MHz; 800; 1600; 2000. 120 kHz; QPSK; No ICI; No ICI; -; -; -. 16 QAM; No ICI; ICI required; -; -; -. 64 QAM; ICI required; ICI required; -; -; -. 480 kHz; QPSK; -; No ICI; No ICI; No ICI; -. 16 QAM; -; No ICI; No ICI; ICI required; -. 64 QAM; -; No ICI; ICI required; ICI required; -. 960 kHz; QPSK; -; No ICI; No ICI; No ICI; No ICI. 16 QAM; -; No ICI; No ICI; No ICI; No ICI. 64 QAM; -; No ICI; No ICI; No ICI; No ICI. Issue 2-3-2: SNR limit Agree with Proposal 1 Issue 2-4-1: Repetition type A We would prefer to have requirements. We are fine with the proposal. We think that the performance is already tested with current requiremeents, therefore no strong need for FR2-2 specific requirements. Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements We would prefer not to preclude MCS 20 before the conclusion on testability. Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding We agree with Option 1. Issue 2-4-4: Parameters for PUSCH requirements We agree on the configurations presented in the table, but wider bandwidths and 960 kHz SCS is missing. We would propose to wait for 2nd round before deciding on that table since it depends on many other issues. |
| Samsung | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements We support option 1, since all the SCS for FR2-2 is optional, we would like to prioritize the requirement for UE with supported SCS for both data and initial access. Issue 2-1-2: Channel bandwidth for PUSCH requirements We support opiton1, with minimum CBW for each SCS Issue 2-2-1: TDL model used for PUSCH requirements We are ok for MCS 4 and MCS16 for TDL-A for initial simulation, for MCS 18, The achievable SNR for 70% should be checked, whether it is higher that SNR limit, especially for 2 Tx condition if defined Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements We are ok with option 1 Issue 2-2-3: Doppler spread for PUSCH requirements Both option 1 and option 2 are fine for us, we can define the separated test cases to cover two Doppler spread. Eg, TDLA- 650Hz. TDL-D 200HZ Issue 2-3-1: Use of ICI for BS demodulation requirements ICI compensation is up to BS implementation, if the simulation results have a large gap among companies, the baseline assumption should be discussed for alignment purpose Issue 2-3-2: SNR limit Ok with option 1 Issue 2-4-1: Repetition type A Ok with option 1 Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements At current stage, we support option 2 Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding We prefer to apply the similar rule as Rel-15, only the minimum RB for DFT requirement, 64 PRB can be considered Issue 2-4-4: Parameters for PUSCH requirements For MCS 20, the feasibility of SNR under 2x2 should be checked, at current stage, we prefer only consider for 1Tx for MCS 20 if defined. |
| Moderator (Nokia) | Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF Issue 3-1: AWGN offset |
| Huawei | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Option 1 Issue 2-1-2: Channel bandwidth for PUSCH requirements Option 1. We don’t see any need to consider large CBW considering limited testable SNR Issue 2-2-1: TDL model used for PUSCH requirements We are OK with the proposal. For MCS with 64QAM, either MCS 18 or 20 are fine for us. Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements OK with option 1 Issue 2-2-3: Doppler spread for PUSCH requirements We don’t see any performance difference between these two Doppler spread. We prefer to choose single Doppler spread for all cases Issue 2-3-1: Use of ICI for BS demodulation requirements Support option 1 Issue 2-3-2: SNR limit OK with proposal 1 Issue 2-4-1: Repetition type A Option 1 Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements Option 1. For 64QAM, either MCS18 or 20 are fine to us. Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding We don't support define PUSCH requirements with transform precoding. That is also agreements from last meeting. Do we want to overturn the agreements from last meeting? Issue 2-4-4: Parameters for PUSCH requirements Support option 1 but we are also fine to change MCS20 to MCS18. Based on our simulation results, the required SNR for MCS20 is quite lower than 20dB, We think it is feasible to define requirments for MCS 20 |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212150 | Company A |
|  | Company B |
| R4-2212232 | Company A |
|  | Company B |
| R4-2212233 | Company A |
|  | Company B |
| R4-2213819 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Original proposals: - Option 1 (Ericsson, Samsung, Huawei): 120 Hz and 480 kHz - Option 2 (Nokia): 120 kHz, 480 kHz and 960 kHz Moderator remarks: Diverging views on the issue. Tentative agreements: - None Candidate options: - Option 1: 120 Hz and 480 kHz - Option 2: 120 kHz, 480 kHz and 960 kHz - Option 3: 120 kHz, 480 kHz and 960 kHz - 960 kHz requirements are declared optional Recommendations for 2nd round: - Continue discussion with the updated options. Issue 2-1-2: Channel bandwidth for PUSCH requirements Original proposals: - Option 1 (Ericsson, Samsung, Huawei): - 120 kHz SCS with 100 MHz - 480 kHz SCS with 400MHz - Option 2 (Nokia): - 120 kHz SCS with 100 MHz and 400 MHz - 480 kHz SCS with 400MHz and 1600 MHz - [960 kHz SCS with 400 MHz and 2000 MHz]* pending decision on the inclusion of 960 kHz Moderator remarks: Diverging view. In the meanwhile BS RF has reached agreements that should be considered for the group. Tentative agreements: - None Candidate options: - Same as 1st round Recommendations for 2nd round: - Continue discussion, considering agreements in the RF session. |
| Sub-topic#2-2 | Issue 2-2-1: TDL model used for PUSCH requirements Original proposals: - Option 1 (Huawei, : Use MCS 4 and 16 for TDL-A and MCS 18 for TDL-D. - Option 1b (Huawei, : Use MCS 4 and 16 for TDL-A and MCS 20 for TDL-D. Moderator remarks: Samsung, check MCS 18, if SNR limit is ok Nokia, review 64 QAM MCS Ericsson, MCS 20 Tentative agreements - TDL model used for PUSCH requirements - Use MCS 4 and 16 with TDL-A and 64 QAM MCS with TDL-D. Candidate options: - MCS for 64 QAM: - Option 1: MCS 18 - Option 2: MCS 20 Recommendations for 2nd round: - Confirm tentative agreement and continue discussing on MCS for 64 QAM requirement Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements Original proposals: - Option 1 (Ericsson, Nokia, Samsung, Huawei): Define PUSCH requirements with 10ns RMS delay spread.. Moderator remarks: All companies agree with Option 1. Tentative agreements: - Define PUSCH requirements with 10ns RMS delay spread.. Candidate options: - none Recommendations for 2nd round: - Confirm the tentative agreement. Issue 2-2-3: Doppler spread for PUSCH requirements Original proposals: - Option 1: 200 Hz - Option 2: 650 Hz Moderator remarks: No consensus on the Doppler spread. Samsumg suggested discussing separated for TDLA and TDLB Tentative agreements: - None Candidate options: - TDLA options - Option 1: 200 Hz - Option 2: 650 Hz - TDLD options - Option 1: 200 Hz - Option 2: 650 Hz Recommendations for 2nd round: - Please discuss it independently for TDLA and TDLD |
| Sub-topic#2-3 | Issue 2-3-1: Use of ICI for BS demodulation requirements Original proposals: - Option 1: Do not consider ICI for BS demodulation requirements Moderator remarks: No clear alignment on the views. Tentative agreements: - None Candidate options: - Same as 1st round Recommendations for 2nd round: - Continue discussion Issue 2-3-2: SNR limit Original proposals: - Proposal 1 (Huawei, Samsung, Nokia, Ericsson): Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget. Moderator remarks: All companies agree with the proposal. In the meanwhile RF room has reached agreements indicating feasibility of the tests Tentative agreements: Candidate options: - Option 1: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. - Option 2: Keep the agreement in the previous meeting that using the minimum CBW and 20dB SNR limit for discussion at current stage. Pending the decision until RF have agreements on the link budget. Recommendations for 2nd round: - Can we agree on Option 1 given the latest RF outcome? |
| Sub-topic#2-4 | Issue 2-4-1: Repetition type A Original proposals: - Option 1 (Ericsson, Nokia, Samsuing, Huawei): Do not define demodulation requirement for FR2-2 PUSCH repetition type A in Rel-17. Introduce the requirement in Rel-19 or higher release if necessary. Tentative agreements - Do not define demodulation requirement for FR2-2 PUSCH repetition type A in Rel-17. Introduce the requirement in Rel-19 or higher release if necessary.: Candidate options: - none Recommendations for 2nd round: - Confirm the tentative agreement Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements Original proposals: - Option 1 (Huawei): - Define PUSCH performance requirements with MCS 4, 16 and 18. - Use 1T2R Low and 2T2R Low - Option 2 (Samsung, Ericsson): - If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2 minimum CBW per SCS, MCS 4/16/20 for 1Tx and MCS 4/16 for 2Tx could be used for FR2-2 PUSCH demodulation requirements. Moderator remarks: No clear views. Issue must be split into MCS and number Tx combinations. Tentative agreements: Candidate options: - Same as 1st round Recommendations for 2nd round: - Continue discussion Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding Original proposals: - Option 1 (Ericsson, Nokia): Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as SCS (kHz); 100 MHz; 400 MHz; 800 MHz; 1600 MHz; 2000 MHz. NRB; NRB; NRB; NRB; NRB. 120; 64; 256; N/A; N/A; N/A. 480; N/A; 64; 120; 240; N/A. 960; N/A; 32; 60; 120; 144. Moderator remarks: One company comented that transform precoding was not agreed in the last meeting, but according to the WF R4-2210664 TP is agreed. In the second round lets try to align the understanding on the previous agreements Tentative agreements: Candidate options: - Option 1: same as 1st round - Option 2: only the minimum RB for DFT requirement, 64 PRB can be considered Recommendations for 2nd round: - Continue discussion Issue 2-4-4: Parameters for PUSCH requirements Original proposals: - Option 1: table in original issue Moderator remarks: Discussion on the table depends on the other issues. Tentative agreements - none Candidate options: - none Recommendations for 2nd round: - Continue discussion |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212150 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet |
| R4-2212232 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet |

## Discussion on 2nd round

### Sub-topic 2-1 Channel bandwidth and SCS

Issue 2-1-1: Sub-carrier spacings for PUSCH requirements

- Proposals

  - Option 1: 120 Hz and 480 kHz

  - Option 2: 120 kHz, 480 kHz and 960 kHz

  - Option 3: 120 kHz, 480 kHz and 960 kHz

    - 960 kHz requirements are declared optional

- Recommended WF

  - Continue discussion with the updated options having in mind the parallel discussion in the general Issue 1-3-1

Issue 2-1-2: Channel bandwidth for PUSCH requirements

Please consider GTW agreement from 17th August for thread [104-e][310] NR_exto71GHz_BSRF, Issue 3-1: AWGN offset

- Proposals

  - Option 1:

    - 120 kHz SCS with 100 MHz

    - 480 kHz SCS with 400MHz

  - Option 2:

    - 120 kHz SCS with 100 MHz and 400 MHz

    - 480 kHz SCS with 400MHz and 1600 MHz

    - [960 kHz SCS with 400 MHz and 2000 MHz]* pending decision on the inclusion of 960 kHz

- Recommended WF

  - Discuss your preferences among the options.

### Sub-topic 2-2 Channel model

Issue 2-2-1: TDL model used for PUSCH requirements

In the first round there was confusion on agreements of MCS ans TDL channel model, therefore the moderator attempted a tentative agreement for channel models, and the MCS is discussed on Issue 2-4-2.

Issue 2-2-2: Delay spread used for the channel model for PUSCH requirements

No more discussion needed.

Issue 2-2-3: Doppler spread for PUSCH requirements

- Proposals

  - TDLA options

    - Option 1: 200 Hz

    - Option 2: 650 Hz

  - TDLD options

    - Option 1: 200 Hz

    - Option 2: 650 Hz

- Recommended WF

  - Please indicate your preference among options 1 and 2 for TDLA and TDLD. Can we compromise with TDLA 10-650 and TDLD 10-200?

### Sub-topic 2-3 Assumptions for PUSCH demodulation requirements

Issue 2-3-1: Use of ICI for BS demodulation requirements

There is a parallel discussion on Issue 1-4-1a. The moderator suggests waiting conclusion on the main thread. You can indicate in the comments if PUSCH specific discussion is needed.

Issue 2-3-2: SNR limit

There is a parallel discussion on Issue 1-4-2. The moderator suggests waiting conclusion on the main thread. You can indicate in the comments if PUSCH specific discussion is needed.

### Sub-topic 2-4 Configuration and test cases PUSCH demodulation requirements

Issue 2-4-1: Repetition type A

Agreement reached in the first round; no more discussion needed.

Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements

- Proposals

  - Options for the 1st MCS

    - Option 1-1: MCS 4 with 1T2R Low and 2T2R Low

  - Options for the 2nd  MCS

    - Option 2-1: MCS 16 with 1T2R Low and 2T2R Low

  - Options for the 3rd MCS

    - Option 3-1: MCS 18 with 1T2R Low and 2T2R Low

    - Option 3-2: MCS 20 with 1T2R Low

      - Note: If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2

    - Option 3-3: MCS 20 with 1T2R Low and MCS 18 with 2T2R Low

      - Note: If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2

- Recommended WF

  - Please discuss the options for each MCS. We have clear view for MCS4 and MCS16. Can we agree on Option 3 for the 3rd MCS?

Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding

Moderator notes: It was commented that there was an agreement indicating transform precoding was not included. From the WF R4-2210664 we can find the following agreement, which we should have in mind when replying to this issue:

Issue 2-2-2: How to consider transform decoding? Specify requirements for FR2-2 PUSCH demodulation with transform precoding enabled.

- Proposals

  - Option 1: Adopt the number of PRBs for each SCS and CBW combination when transform precoding is enabled as

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 120 | 64 | 256 | N/A | N/A | N/A |
| 480 | N/A | 64 | 120 | 240 | N/A |
| 960 | N/A | 32 | 60 | 120 | 144 |
| Note: Configuration to be applied only for agreed SCS and CBW. |  |  |  |  |  |

- Recommended WF

  - Please comment on the option. Focus is not to agree on the inclusion of the SCS and CBW, but rather on the configuration with TP in case they are agreed. In order to clarify if there is a misunderstanding, a new issue is created at Issue 2-4-5.

Issue 2-4-4: Parameters for PUSCH requirements

- Proposals

  - Option 1: Use the following table for FR2-2 PUSCH performance requirements definition

Proposed simulation assumptions for FR2-2 PUSCH

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 120 | 100 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 16 | TDLA10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low |  |
|  |  |  |  | 2x2 Low |  |

- Recommended WF

  - Please comment on Option 1 and comment if it is acceptable.

  - Some entries in the table might need revision after reaching agreements on the other issues

Issue 2-4-5: Test cases for PUSCH requirements with transform precoding

Moderator notes: It was commented that there was an agreement indicating transform precoding was not included. From the WF R4-2210664 we can find the following agreement, which we should have in mind when replying to this issue:

Issue 2-2-2: How to consider transform decoding? Specify requirements for FR2-2 PUSCH demodulation with transform precoding enabled.

- Proposals

  - Option 1: Revise agreement from R4-2210664 and not include test cases with transform precoding.

  - Option 2: Define test cases with MCS 16, and minimum CBW for the agreed SCSs

  - Option 3: Define test cases with MCS 16 and same CBW as for PUSCH without transform precoding

- Recommended WF

  - Please comment on the options.

## Companies views’ collection for 2nd round

### Open issues

Sub topic 2-1 Channel bandwidth and SCS

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Issue 2-1-2: Channel bandwidth for PUSCH requirements |
| Nokia | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Our preference if Option 2. We would like to consider all SCS which are defined for ext to 71 GHz. We can compromise to Option 3. We are willing to compromising making 960 kHz optional. This can be done adopting the same procedure as used for HST. Issue 2-1-2: Channel bandwidth for PUSCH requirements We prefer Option 2. Considering the RF agreement we think there are no feasibility questions regarding requirements with wider bandwidths. |
| Ericsson | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Duplicated discussion with general part. We prefer Option 1. Issue 2-1-2: Channel bandwidth for PUSCH requirements Prefer Option 1 at current stage. |
| Huawei | Issue 2-1-1: Sub-carrier spacings for PUSCH requirements Option 1. The SNR limit should be derived from RF part. Before it is agreed, we should focus on minimum CBW for each SCS. Issue 2-1-2: Channel bandwidth for PUSCH requirements Option 1. The SNR limit should be derived from RF part. Before it is agreed, we should focus on minimum CBW for each SCS. |

Sub-topic 2-2 Channel model

| Company | Comments |
| --- | --- |
| XXX | Issue 2-2-3: Doppler spread for PUSCH requirements |
| Nokia | Issue 2-2-3: Doppler spread for PUSCH requirements For TDLA we prefer Option 2: 650 Hz. For TDLD we prefer Option 1: 200 Hz. We think this is a good compromise between the companies wanting 200 Hz and the ones wanting 650 Hz. |
| Ericsson | Issue 2-2-3: Doppler spread for PUSCH requirements Duplicated discussion with general part. We prefer TDLA10-650 and TDLD10-200. |
| Huawei | Issue 2-2-3: Doppler spread for PUSCH requirements Based on agreements from GTW, TDLA10-650 and TDLD10-200 are fine for us. |

Sub-topic 2-3 Assumptions for PUSCH demodulation requirements

| Company | Comments |
| --- | --- |
| XXX | Issue 2-3-1: Use of ICI for BS demodulation requirements Issue 2-3-2: SNR limit |
| Nokia | Issue 2-3-1: Use of ICI for BS demodulation requirements Issue 2-3-2: SNR limit We prefer agreeing in the general issue. |
| Ericsson | Duplicated discussion as general part. |
| Huawei | Same views as general part |

Sub-topic 2-4 Configuration and test cases PUSCH demodulation requirements

| Company | Comments |
| --- | --- |
| XXX | Issue 2-4-1: Repetition type A Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding Issue 2-4-4: Parameters for PUSCH requirements Issue 2-4-5: Test cases for PUSCH requirements with transform precoding |
| Nokia | Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements We agree with Option 1-1, Option 2-1 For the 64 QAM MCS, we think Option 3-3 is a good compromise. Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding We agree with Option 1. The calculated values are in line with maximum PRB allocation and restriction of number of PRBs for TP. Issue 2-4-4: Parameters for PUSCH requirements The table is fine in general. Pending decision on MCS, CBW and channel model/Doppler shift. Issue 2-4-5: Test cases for PUSCH requirements with transform precoding Option 2. We prefer to keep the current agreement and use the same approach as used for FR2-1 with 1 CBW per SCS. |
| Ericsson | Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements We prefer only 1T2R for 64QAM. There is no 2T2R requirement for FR2-1 64QAM. Based on our simulation, ideal SNR for 1T2R with MCS20 could reach to 15dB. It would be better to keep enough margin to 20dB SNR limit if impairment and misalignments between companies. Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding The allocated PRBs for DFT-s-OFDM requirements could be half of the numbers in the Option 1 which is used in FR2-1 FRC, i.e., 30 PRBs for 120kHz SCS. Issue 2-4-4: Parameters for PUSCH requirements Make decision until previous issues have agreements. Issue 2-4-5: Test cases for PUSCH requirements with transform precoding We prefer only define requirements for QPSK cases. |
| Huawei | Issue 2-4-2: MCS and number Tx/Rx branches for PUSCH requirements For MCS4 and MCS16, we are OK for both 1T2R and 2T2R. For 64QAM, we are OK to follow FR2-1 to use 1T2R Issue 2-4-3: Number of PRBs for PUSCH requirements with transform precoding Option 1 is OK Issue 2-4-4: Parameters for PUSCH requirements Generally we are OK for the parameters. For MCS20, we can only test 1T2R to follow Rel-15 test. Issue 2-4-5: Test cases for PUSCH requirements with transform precoding We suggest only test QPSK with minimum bandwidth. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212150 | Moderator: CR is not available. Should we change the status as withdrawn? |
|  | Company B |
| R4-2212232 | Nokia: To be confirmed with the outcome of Issue 2-4-5 and 2-4-3 From our calculation, we should use 64 PRBs for TP enabled with 480 kHz SCS/400 MHz and 120 kHz SCS/100 MHz. Some missing cases: - 120 kHz SCS with 100 MHz CBW - 480 k Hz with wider bandwidths. - 960 kHz SCS missing as wel. To be updated depending on the agreements. Ericsson: Thanks for the comments. We will update the PRB number for TP enabled cases. For CP-OFDM cases, the FRC for FR2-1 120kHz SCS 100MHz CBW could be reused for FR2-2. For 480kHz with larger CBW and 960kHz SCS cases, we would like to keep it open until we have agreement on it. |
|  | Company B |
| R4-2212233 | Nokia: To be confirmed with the outcome of Issue 2-4-5 and 2-4-3 From our calculation, we should use 64 PRBs for TP enabled with 480 kHz SCS/400 MHz and 120 kHz SCS/100 MHz. Some missing cases: - 120 kHz SCS with 100 MHz CBW - 480 k Hz with wider bandwidths. - 960 kHz SCS missing as well. To be updated depending on the agreements. Ericsson: Thanks for the comments. We will update the PRB number for TP enabled cases. For CP-OFDM cases, the FRC for FR2-1 120kHz SCS 100MHz CBW could be reused for FR2-2. For 480kHz with larger CBW and 960kHz SCS cases, we would like to keep it open until we have agreement on it. |
|  | Company B |

| R4-2213819 | Nokia: This is not how we understood the proposals we are discussing in 1-5-1. The CR has new clauses for the FR2-2 requirements, shouldn’t we have used the same clauses with different tables for FR2-2? This is also the agreement we had in RAN4 102 (R4-2207223): Issue 1-5-1: Implementation of FR2-2 requirements into specification Capture FR2-2 demodulation requirement into same section as FR2-1 but with different tables if possible. Additionally, the new titles have a spelling error, it should be “operating” not “opeating”. |
| --- | --- |
|  | Company B |

# Topic #3: PUCCH requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations | Issue mapping |
| --- | --- | --- | --- |
| R4-2212227 | Ericsson | Discussion on FR2-2 PUCCH demodulation requirements Proposal 1: Define FR2-2 normal PUCCH format 4 demodulation requirements for both DM-RS 1+0 and 1+1 configurations. Proposal 2: Take 16 PRB for FR2-2 multi-PRB PUCCH demodulation requirements. Proposal 3: Only define DM-RS 1+1 configuration for FR2-2 multi-PRB PUCCH format 4 demodulation requirements. | 3-2-5 3-1-3 3-2-5 |
| R4-2212230 | Ericsson | Simulation results for FR2-2 PUCCH demodulation requirements |  |
| R4-2212676 | Nokia, Nokia Shanghai Bell | Discussion on PUCCH demodulation requirements for the extension to 71 GHz Proposal 1: RAN4 to define PUCCH demodulation requirements using TDL-A 10-650 Low channel model. Observation 2: Enhanced PUCCH formats provide SNR gains in the range of 9 to 10 dBs in comparison to single PRB PUCCH formats 0, 1, and 4 Proposal 2: RAN4 to define requirements for enhanced PUCCH formats 0, 1, and 4 with 10 PRBs and 1 OFDM symbol. Proposal 3: RAN4 to define enhanced PUCCH format 0 requirements with 10 PRBs, and 1 OFDM symbol. Proposal 4: RAN4 to adopt the following parameters for enhanced PUCCH format 0 simulations: <Moderator> Table omitted due to formatting issues, shown only in the issue for discussion Proposal 5: RAN4 to define enhanced PUCCH format 1 requirements with 10 PRBs, and 2 UCI information bits. Proposal 6: RAN4 to adopt the following table of parameters for PUCCH format 1: <Moderator> Table omitted due to formatting issues, shown only in the issue for discussion Proposal 7: RAN4 to define PUCCH format 2 requirements with the following 2 cases -Case 1 with 4 PRBs, 1 OFDM symbol and 4 UCI bits, -Case 2 with 9 PRBs, 2 OFDM symbols and 22 UCI bits Proposal 8: RAN4 to adopt the following table of parameters for PUCCH format 2: <Moderator> Table omitted due to formatting issues, shown only in the issue for discussion Proposal 9: RAN4 to define PUCCH format 3 requirements with the following 2 cases -Case 1 with 1 PRBs, 14 OFDM symbols and 16 UCI bits, -Case 2 with 3 PRBs, 4 OFDM symbols and 16 UCI bits Proposal 10: RAN4 to define PUCCH format 3 requirements with and without additional DMRs for test case with 14 OFDM symbols. Proposal 11: RAN4 to adopt the following table of parameters for PUCCH format 3: <Moderator> Table omitted due to formatting issues, shown only in the issue for discussion Proposal 12: RAN4 to define enhanced PUCCH format 4 requirements with 10 PRBs, and 22 UCI information bits. Proposal 13: RAN4 to adopt the following table of parameters for PUCCH format 4: <Moderator> Table omitted due to formatting issues, shown only in the issue for discussion | 3-1-1 3-1-3 3-2-1 3-2-1 3-2-2 3-2-2 3-2-3 3-2-3 3-2-4 3-2-4 3-2-4 3-2-5 3-2-5 |
| R4-2212677 | Nokia, Nokia Shanghai Bell | PUCCH simulation results for the extension to 71 GHz |  |
| R4-2213814 | Huawei,HiSilicon | Discussion on FR2-2 PUCCH requirements Proposal 1: Consider following assumptions for FR2-2 PUCCH requirements definition: -1T2R -TDLA10-200 -120kHz/100MHz and 480kHz/400MHz Proposal 2: Use Table 2-1 for FR2-2 PF0 simulation assumptions. Proposal 3: Use Table 2-2 for FR2-2 PF1 simulation assumptions. Proposal 4: Use Table 2-3 for FR2-2 PF2 simulation assumptions. Proposal 5: Use Table 2-4 for FR2-2 PF3 simulation assumptions. Proposal 6: Use Table 2-5 for FR2-2 PF4 simulation assumptions. | 3-1-1 3-1-2 3-1-3 3-2-1 3-1-3 3-2-2 3-2-3 3-2-4 3-2-5 3-1-3 |
| R4-2213815 | Huawei,HiSilicon | Simulation results on FR2-2 PUCCH requirements |  |
| R4-2212151 | Intel Corporation | draftCR to 38141-2 on BS radiated requirements for PUCCH FR2-2 |  |
| R4-2213818 | Huawei,HiSilicon | Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104 |  |

## Open issues summary

### Sub-topic 3-1 General PUCCH

Issue 3-1-1: Channel model for PUCCH requirements

- Proposals

  - Option 1: TDL-A 10-650 Low channel model.

  - Option 2: TDLA10-200

  - Option 3: TDLA10-200 Low

- Recommended WF

  - Discuss among the options

Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements

- Proposals

  - Option 1: Consider following assumptions for FR2-2 PUCCH requirements definition:

    - 1T2R

    - 120kHz/100MHz and 480kHz/400MHz.

  - Option 2: Consider following assumptions for FR2-2 PUCCH requirements definition:

    - 1T2R

    - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz.

- Recommended WF

  - Discuss among the options 1 and 2 having in mind the parallel discussion in the general Issue 1-3-1

Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4

- Proposals

  - Option 1: 16 PRBs.

  - Option 2: 10 PRBs.

- Recommended WF

  - Discuss among the options 1 and 2

### Sub-topic 3-2 PUCCH configuration tables

Issue 3-2-1: PUCCH format 0

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 and 10 |
| Number of OFDM symbols: | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | Test metric: ACK missed detection rate < 1% |
| Phase noise model | TR 38.808 set 2 |

- Recommended WF

  - Please discuss your preferences among options 1 and 2

Issue 3-2-2: PUCCH format 1

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<0.1% |
|  |  |  |  | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Value |
| --- | --- |
| TR/RX | 1T2R |
| Number of UCI information bits | 2 |
| Number of PRBs | 1 and 10 |
| Number of OFDM symbols: | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Propagation condition | TDL channel model: TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% NACK to ACK (0.1 %) |

Recommended WF

  - Please discuss your preferences among options 1 and 2

Issue 3-2-3: PUCCH format 2

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-200 Low | 120 | 100 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  | 480 | 400 | 1 | 4 | 4 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | Prob(DTXACK)<1% Prob(ACK miss)<1% |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Case 1 | Case 2 |
| --- | --- | --- |
| TR/RX | 1T2R | 1T2R |
| Modulation order | QPSK | QPSK |
| Number of UCI information bits | 4 | 22 |
| Number of PRBs | 4 | 9 |
| OFDM symbols | 1 | 2 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | Disabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither | Neither |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 13 | 12 |
| Propagation condition | TDL-A 10-650 | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | ACK missed detection rate < 1% | UCI BLER < 1% |

Recommended WF

  - Please discuss your preferences among options 1 and 2

Issue 3-2-4: PUCCH format 3

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | 120 | 100 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  | 480 | 400 | 14 | 1 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |

  - Option 2: Use the following table for simulation assumptions

| Parameter | value |
| --- | --- |
| Number of UCI information bits | 16 bits |
| Number of PRBs Number of OFDM symbols: | 1 for 14 OFDM symbols |
|  | and |
|  | 3 for 4 OFDM symbols |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| First symbol | 13 |
| Modulation order | QPSK |
| DM-RS sequence generation | NID0=0 |
| Additional DMRS | With and without for 14 OFDM symbols |
|  | Without for 4 OFDM symbols |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

Recommended WF

  - Please discuss your preferences among options 1 and 2

Issue 3-2-5: PUCCH format 4

- Proposals

  - Option 1:

    - Define FR2-2 normal PUCCH format 4 demodulation requirements for both DM-RS 1+0 and 1+1 configurations.

    - Only define DM-RS 1+1 configuration for FR2-2 multi-PRB PUCCH format 4 demodulation requirements.

  - Option 2: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation Branches | Modulation order | Propagation conditions and correlation matrix | Length of the orthogonal cover code | Index of the orthogonal cover code | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-200 Low | n2 | n0 | 120 | 100 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 480 | 400 | 14 | 1 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | No additional DMRS | Prob(UCI BLER)<1% |

  - Option 3: Use the following table for simulation assumptions

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 22 bits |
| Number of PRBs | 1 and 10 (Enhanced) |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 0 |
| Number of OFDM symbols: | 14 |
| Additional DMRS | With and without |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz SCS = 960 kHz; CBW = 400 MHz |
| Testing metric | UCI BLER < 1% |

Recommended WF

  - Please discuss your preferences among options 1 and 2

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1-1: Channel model for PUCCH requirements We support Option 1. Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Depend on the discussion on PUSCH part. Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4 We can accept 16 PRBs if link budget is not an issue. Issue 3-2-4: PUCCH format 3 Issue 3-2-5: PUCCH format 4 The DM-RS configurations we prefer to consider both 1+0 and 1+1 for long OFDM symbol case to keep the same coverage as FR2-1, and an applicability rule could be added for the test. |
| Nokia | Issue 3-1-1: Channel model for PUCCH requirements We prefer Option 1 Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements We refer Option 2. Depending on the outcome of Issue 1-3-1, Option 1 is aceptable. Issue 3-2-1: PUCCH format 0 Issue 3-2-2: PUCCH format 1 Issue 3-2-3: PUCCH format 2 Issue 3-2-4: PUCCH format 3 Issue 3-2-5: PUCCH format 4 All parameter tables have differences that depend on decisions on Issues of sub-topic 3-1. I would suggest that we merge the tables and keep the values from the open issues as FFS. |
| Samsung | Issue 3-1-1: Channel model for PUCCH requirements Since QPSK is applied for PUCCH, we are ok with option 1, there is no different between option 2 and option 3 Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Ok with option 1 Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4 Either option 1 and option 2 are fine for us Issue 3-2-1: PUCCH format 0 Pending on agreed RB, channel model, and CBW, option1, can be regarded as starting point Issue 3-2-2: PUCCH format 1 Pending on agreed RB, channel model, and CBW, option1, can be regarded as starting point Issue 3-2-3: PUCCH format 2 Ok with option 2, excepting for SCS and CBW Issue 3-2-4: PUCCH format 3 OK with option 2, excepting for SCS and CBW Issue 3-2-5: PUCCH format 4 Pending on agreed channel model and number of multi-PRB, option1 can be regarded as starting point. |
| Huawei | Issue 3-1-1: Channel model for PUCCH requirements Option 3. We don’t see any difference between two Doppler spread considering large SCS. We want to only consider indoor scenario. Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Option 1 Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4 Option 1. Option 1 can better test whether BS support multi RB feature Issue 3-2-1: PUCCH format 0 Option 1. We prefer to configure different RB number into cases with different symbol number and don’t increase the test cases. Issue 3-2-2: PUCCH format1 Option 1 Issue 3-2-3: PUCCH format 2 Option 1. Issue 3-2-4: PUCCH format 3 Option 1 Issue 3-2-5: PUCCH format 4 Option 2. We don’t see any need to configure additional DMRS considering such low Doppler spread |

Example 2

Sub topic 1-1

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212151 | Company A |
|  | Company B |
| R4-2213818 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: Channel model for PUCCH requirements Original proposals: - Option 1 (Ericsson, Nokia, Samsung): TDL-A 10-650 Low channel model. - Option 2: TDLA10-200 - Option 3 (Huawei): TDLA10-200 Low Moderator remarks: - 3 companies agree on Option 1 while 1 company wants Option 3. Issue depends on the discussion on Doppler spread. Therefore the moderator suggests agreeing on TDLA 10-xx Low, and try to converge on Dopler spread in the second round. Tentative agreements: - TDLA 10 Candidate options: - Option 1: Adopt delay spread 650 for PUCCH requirements, i.e. TDLA 10-650 Low. - Option 2: Adopt delay spread 200 for PUCCH requirements, i.e. TDLA10-200 Low. Recommendations for 2nd round: - Confirm tentative agreement and proceed with the Doppler spread discussion Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Original proposals: - Option 1 (Huawei, Nokia?, , Samsung, Huawei), : Consider following assumptions for FR2-2 PUCCH requirements definition: - 1T2R - 120kHz/100MHz and 480kHz/400MHz. - Option 2 (Nokia): Consider following assumptions for FR2-2 PUCCH requirements definition: - 1T2R - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz. Moderator remarks: One company commented that decision on this issue depends on the discussion on the PUSCH part. Two companies agreed on Option 1 and 1 company agreed on Option 2. Therefore the moderator recommends agreeing on Rx/Tx for PUCCH and leave SCS/CBW discussion forf the second round discussion. Tentative agreements: - Consider 1T2R for FR2-2 PUCCH requirements definition Candidate options: - Option 1: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz and 480kHz/400MHz. - Option 2: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz. - Option 3: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz*. - Note: 960 kHz SCS is declared as optional. Recommendations for 2nd round: - Confirm the tentative agreements and discuss among the options. Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4 Original proposals: - Option 1 (Huawei, Samsung, Ericsson): 16 PRBs. - Option 2 (Samsung): 10 PRBs. Moderator remarks: Strong support for Option 1. Therefore it is agreeable. Tentative agreements: - Number of PRBs for enhanced PF0, PF1, PF4 - 16 PRBs Candidate options: - none Recommendations for 2nd round: - Confirm the tentative agreement |
| Sub-topic#3-2 | Issue 3-2-1: PUCCH format 0 Original proposals: - Option 1 (Samsung, Huawei), table contained in the original issue - Option 2 (no comments) table contained in the original issue Moderator remarks: Companies commenting the issue were fine with Option 1 as an starting point given the pending open issues. Considering the tentative agreements on Issues 3-1-1, 3-1-2, 3-1-3 the moderator suggests agreeing on Option 1 as a baseline removing the open points, which can be updated in the 2nd round. Tentative agreements: Number of TX antennas; Number of demodulation branches; Propagation conditions and correlation matrix; Number of bits; SCS (kHz); CBW (MHz); Number of OFDM symbols; Number of PRB; Hopping; Test metric. 1; 2; TDLA10-XXX Low; 1; 120; 100; 1; 1; Disabled; Prob(ACK miss)<1% Prob(DTX->ACK)<1%. 2; 16; Enabled; Prob(ACK miss)<1% Prob(DTX->ACK)<1%. 480; 400; 1; 1; Disabled; Prob(ACK miss)<1% Prob(DTX->ACK)<1%. 2; 16; Enabled; Prob(ACK miss)<1% Prob(DTX->ACK)<1%. - *To be revisited after agreements on Doppler spread and SCS. Candidate options: - Inclusion of 960 kHz SCS in the table - Other parameters not included in the table Recommendations for 2nd round: - Continue discussion on the Issue regarding SCS and Doppler shift, and update table accordingly Issue 3-2-2: PUCCH format 1 Original proposals: - Option 1 (Samsung, Huawei), table contained in the original issue - Option 2 (no comments) table contained in the original issue Moderator remarks: Companies commenting the issue were fine with Option 1 as an starting point given the pending open issues. Considering the tentative agreements on Issues 3-1-1, 3-1-2, 3-1-3 the moderator suggests agreeing on Option 1 as a baseline removing the open points, which can be updated in the 2nd round. Tentative agreements: Number of TX antennas; Number of demodulation branches; Propagation conditions and correlation matrix; Number of bits; SCS (kHz); CBW (MHz); Number of OFDM symbols; Number of PRB; Hopping; Test metric. 1; 2; TDLA10-XXX Low; 2; 120; 100; 14; 1; Enabled; Prob(NACK to ACK)<0.1%. 1; Enabled; Prob(ACK miss)<1%. 16; Enabled; Prob(NACK to ACK)<0.1%. 16; Enabled; Prob(ACK miss)<0.1%. 480; 400; 14; 1; Enabled; Prob(NACK to ACK)<0.1%. 1; Enabled; Prob(ACK miss)<1%. 16; Enabled; Prob(NACK to ACK)<0.1%. 16; Enabled; Prob(ACK miss)<1%. - *To be revisited after agreements on Doppler spread and SCS. Candidate options: - none Recommendations for 2nd round: - Continue discussion on the Issue regarding SCS and Doppler shift, and update table accordingly Issue 3-2-3: PUCCH format 2 Original proposals: - Option 1 (Huawei), table contained in the original issue - Option 2 (Samsung) table contained in the original issue Moderator remarks: Diverging preferences among Option 1 and Option 2. The main difference among option 1 and 2 is the test metric and the adoption of the 960 kHz SCS. Previous agreement from R4-2210664 on PUCCH format 2 was that we should reuse FR2-1 configuration for PRB and OFDM symbols, and that ACK missed detection rate and UCI BLER should be used. Tentative agreements: None Candidate options: - Option 1 Define PUCCH format 2 demodulation requirements for - Case 1: - 1 OFDM symbol, 4 PRBs, 4 bits, - Test metric Prob(DTXACK)<1% and Prob(ACK miss)<1% - Case 2: - 2 OFDM symbol, 9 PRBs, 22 bits, - Test metric Prob(DTXACK)<1% and Prob(ACK miss)<1% - Option 2 Define PUCCH format 2 demodulation requirements for - Case 1: - 1 OFDM symbol, 4 PRBs, 4 bits, - Test metric Prob(DTXACK)<1% and Prob(ACK miss)<1% - Case 2: - 2 OFDM symbol, 9 PRBs, 22 bits, - Test metric UCI BLER < 1% Recommendations for 2nd round: - Split the issue into for the discussion of the 2 test cases for each SCS. It is the moderator understanding that Option 2 is more aligned with the agreement from R4-2210664. So it is recommended to confirm that agreement and discuss if we can agree on Option 2. Issue 3-2-4: PUCCH format 3 Original proposals: - Option 1 (Huawei), table contained in the original issue - Option 2 (Samsung, Ericsson) table contained in the original issue Moderator remarks: One company requested discussion on applicability rule and consideration of both 1+0 and 1+1 for long OFDM symbol case. Continue discussion Tentative agreements: - none Candidate options: - Same as 1st round Recommendations for 2nd round: - Continue discussion based on the options - Create new Issue: Applicability rule for PUCCH format 3 - Create new Issue: DMRs configuration for PUCCH format 3 Issue 3-2-5: PUCCH format 4 Original proposals: - Option 1 (Samsung, Huawei), table contained in the original issue - Option 2 (Ericsson?) table contained in the original issue Moderator remarks: One company requested discussion on applicability rule and consideration of both 1+0 and 1+1 for long OFDM symbol case. Continue discussion Tentative agreements: - none Candidate options: - Same as first round Recommendations for 2nd round: - Continue discussion on the available options - Create new Issue: Applicability rule for PUCCH format 4 - Create new Issue: DMRs configuration for PUCCH format 4 |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212151 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet |
| R4-2213818 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet |

## Discussion on 2nd round

### Sub-topic 3-1 General PUCCH

Issue 3-1-1: Channel model for PUCCH requirements

Tentative agreement in the 1st round for TDLA 10, conclusion on Doppler spread still pending.

- Adopt TDLA 10 channel model for PUCCH requirements with

  - Option 1: Adopt Doppler spread 650 for PUCCH requirements, i.e. TDLA 10-650 Low.

  - Option 2: Adopt Doppler spread 200 for PUCCH requirements, i.e. TDLA10-200 Low.

- Recommended WF

  - Discuss among the options

Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements

- Proposals

  - Option 1: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition:

    - 120kHz/100MHz and 480kHz/400MHz.

  - Option 2: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition:

    - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz.

  - Option 3: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition:

    - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz*.

    - Note: 960 kHz SCS is declared as optional.

- Recommended WF

  - Discuss among the options 1 and 2 having in mind the parallel discussion in the general Issue 1-3-1

Issue 3-1-3: Number of PRBs for enhanced PF0, PF1, PF4

Discussion concluded in the 1st round.

### Sub-topic 3-2 PUCCH configuration tables

Issue 3-2-1: PUCCH format 0

Moderator remark: Continue discussion on the Issue regarding SCS and Doppler shift, and update table accordingly in the WF.

Issue 3-2-2: PUCCH format 1

Moderator remark: Continue discussion on the Issue regarding SCS and Doppler shift, and update table accordingly in the WF.

Issue 3-2-3: PUCCH format 2

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA10-xxx Low* | 120 | 100 | 1 | 4 | 4 | Enabled | TBD |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | TBD |
|  |  |  | 480 | 400 | 1 | 4 | 4 | Enabled | TBD |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | TBD |
| *To be revisited after agreements on Doppler spread and SCS. |  |  |  |  |  |  |  |  |  |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Case 1 | Case 2 |
| --- | --- | --- |
| TR/RX | 1T2R | 1T2R |
| Modulation order | QPSK | QPSK |
| Number of UCI information bits | 4 | 22 |
| Number of PRBs | 4 | 9 |
| OFDM symbols | 1 | 2 |
| First PRB prior to frequency hopping | 0 | 0 |
| Intra-slot frequency hopping | Disabled | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither | Neither |
| Hopping ID | 0 | 0 |
| Initial cyclic shift | 0 | 0 |
| First symbol | 13 | 12 |
| Propagation condition | TDL-A 10-xxx* | TDL-A 10-xxx* |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz [SCS = 960 kHz; CBW = 400 MHz]* | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz [SCS = 960 kHz; CBW = 400 MHz]* |
| Testing metric | TBD | TBD |
| *To be revisited after agreements on Doppler spread and SCS. |  |  |

Recommended WF

  - Please discuss your preference. Separate issue was created for test metric on Issue 3-2-6. Can we agree on Option 1?

Issue 3-2-4: PUCCH format 3

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-xxx Low* | 120 | 100 | 14 | 1 | 16 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  | 480 | 400 | 14 | 1 | 16 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | TBD | Prob(UCI BLER)<1% |
| *To be revisited after agreements on Doppler spread and SCS. |  |  |  |  |  |  |  |  |  |  |  |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 16 bits |
| Number of PRBs Number of OFDM symbols: | 1 for 14 OFDM symbols |
|  | and |
|  | 3 for 4 OFDM symbols |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 13 |
| Modulation order | QPSK |
| DM-RS sequence generation | NID0=0 |
| Additional DMRS | TBD* |
| Propagation condition | TDL-A 10-650 |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz [SCS = 960 kHz; CBW = 400 MHz]* |
| Testing metric | UCI BLER < 1% |
| *To be revisited after agreements on Doppler spread and SCS. |  |

Recommended WF

  - Please discuss your preferences among options 1 and 2. Please note options from 1st round were modified to make TBD on topics that are open. New issue created on 3-2-9 for DMRS discussion

Issue 3-2-5: PUCCH format 4

- Proposals

  - Option 1: Use the following table for simulation assumptions

| Number of TX antennas | Number of demodulation Branches | Modulation order | Propagation conditions and correlation matrix | Length of the orthogonal cover code | Index of the orthogonal cover code | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA10-xxx Low* | n2 | n0 | 120 | 100 | 14 | 1 | 22 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 480 | 400 | 14 | 1 | 22 | Enabled | TBD | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | TBD | Prob(UCI BLER)<1% |
| *To be revisited after agreements on Doppler spread and SCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Option 2: Use the following table for simulation assumptions

| Parameter | Value |
| --- | --- |
| Number of UCI information bits | 22 bits |
| Number of PRBs | 1 and 10 (Enhanced) |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| First symbol | 0 |
| Number of OFDM symbols: | 14 |
| Additional DMRS | TBD |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Propagation condition | TDL-A 10-xxx* |
| SCS and BW | SCS = 120kHz; CBW = 100 MHz SCS = 480 kHz; CBW = 400 MHz and 1600 MHz [SCS = 960 kHz; CBW = 400 MHz]* |
| Testing metric | UCI BLER < 1% |
| *To be revisited after agreements on Doppler spread and SCS. |  |

Recommended WF

  - Please discuss your preferences among options 1 and 2. Please discuss your preferences among options 1 and 2. Please note options from 1st round were modified to make TBD on topics that are open. New issue created on 3-2-10 for DMRS discussion.

Issue 3-2-6: Test metric for PUCCH format 2

Moderator remark: Previous agreement from R4-2207223

Issue 2-4-2c: Detailed PUCCH test setup for PUCCH format 2 Number of PRBs: - Option 1: 9 Number of OFDM symbols: - Option 1: 2 - Option 2: 1 and 2 Intra-slot frequency hopping: enabled Test metric: ACK missed detection rate < 1%, UCI BLER < 1%

- Proposals:

  - Option 1: Define PUCCH format 2 demodulation requirements for

    - Case 1:

      - 1 OFDM symbol, 4 PRBs, 4 bits,

      - Test metric Prob(DTX to ACK)<1% and Prob(ACK miss)<1%

    - Case 2:

      - 2 OFDM symbol, 9 PRBs, 22 bits,

      - Test metric Prob(DTX to ACK)<1% and Prob(ACK miss)<1%

  - Option 2: Define PUCCH format 2 demodulation requirements for

    - Case 1:

      - 1 OFDM symbol, 4 PRBs, 4 bits,

      - Test metric ACK missed detection rate < 1%

    - Case 2:

      - 2 OFDM symbol, 9 PRBs, 22 bits,

      - Test metric UCI BLER < 1%

- Recommended WF:

  - Please discuss the proposals, and indicate if previous agreement has to be revised.

Issue 3-2-7: Applicability rule for PUCCH format 3

- Proposals

  - Option 1: Define applicability rules for the test.

  - Option 2: Define applicability rule according to manufacturer declaration as for FR2-1 (i.e. D.103 and D.104)

- Recommended WF

  - Please discuss the options.

Issue 3-2-8: Applicability rule for PUCCH format 4

- Proposals

  - Option 1: Define applicability rules for the test.

  - Option 2: Define applicability rule according to manufacturer declaration as for FR2-1 (i.e. D.103 and D.105)

- Recommended WF

  - Please discuss the options.

Issue 3-2-9: DMRs configuration for PUCCH format 3

- Proposals

  - Option 1: Define PUCCH format 3 test cases with DMRS 1+0 and 1+1.

  - Option 2: Define PUCCH format 3 test cases with DMRS 1+0.

- Recommended WF

  - Please state your preferences.

Issue 3-2-10: DMRs configuration for PUCCH format 4

- Proposals

  - Option 1: Define PUCCH format 4 test cases with DMRS 1+0 and 1+1.

  - Option 2: Define PUCCH format 4 test cases with DMRS 1+0.

- Recommended WF

  - Please state your preferences.

## Companies views’ collection for 2nd round

### Open issues

Sub-topic 3-1 General PUCCH

| Company | Comments |
| --- | --- |
| XXX | Issue 3-1-1: Channel model for PUCCH requirements Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements |
| Nokia | Issue 3-1-1: Channel model for PUCCH requirements We prefer Option 1. Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Our preference if Option 2. We would like to consider all SCS which are defined for ext to 71 GHz. We can compromise to Option 3. We are willing to compromising making 960 kHz optional. This can be done adopting the same procedure as used for HST. |
| Ericsson | Issue 3-1-1: Channel model for PUCCH requirements We prefer Option 1. Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements We prefer Option 1. |
| Samsung | Issue 3-1-1: Channel model for PUCCH requirements We support option 1, since QPSK is only applicable for PUCCH, it should be ok with high doppler spread Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements We support option 1, |
| Huawei | Issue 3-1-1: Channel model for PUCCH requirements Option 2 Issue 3-1-2: Subcarrier spacing, TX/RX and CBW for PUCCH requirements Option 1 |

Sub-topic 3-2 PUCCH configuration tables

| Company | Comments |
| --- | --- |
| XXX | Issue 3-2-1: PUCCH format 0 Issue 3-2-2: PUCCH format 1 Issue 3-2-3: PUCCH format 2 Issue 3-2-4: PUCCH format 3 Issue 3-2-5: PUCCH format 4 Issue 3-2-6: Test metric for PUCCH format 2 Issue 3-2-7: Applicability rule for PUCCH format 3 Issue 3-2-8: Applicability rule for PUCCH format 4 Issue 3-2-9: DMRs configuration for PUCCH format 3 Issue 3-2-10: DMRs configuration for PUCCH format 4 |
| Nokia | Issue 3-2-3: PUCCH format 2 We can agree with the table format in Option 1 as a baseline. The format of Option 1 is also more in line with other agreements. To be revised after further agreements are reached. Issue 3-2-4: PUCCH format 3 We can agree with the table format in Option 1 as a baseline. The format of Option 1 is also more in line with other agreements. To be revised after further agreements are reached. Issue 3-2-5: PUCCH format 4 We can agree with the table format in Option 1 as a baseline. The format of Option 1 is also more in line with other agreements. To be revised after further agreements are reached. Issue 3-2-6: Test metric for PUCCH format 2 We prefer Option 2, which we believe is the previous agreement. Issue 3-2-7: Applicability rule for PUCCH format 3 We agree with Option 2. Reuse approach used for FR2-1. Issue 3-2-8: Applicability rule for PUCCH format 4 We agree with Option 2. Reuse approach used for FR2-1. Issue 3-2-9: DMRs configuration for PUCCH format 3 We prefer Option 1. 1+0 and 1+1 is the same test coverage that we have in FR2-1. So we prefer to keep both. Issue 3-2-10: DMRs configuration for PUCCH format 4 We prefer Option 1. 1+0 and 1+1 is the same test coverage that we have in FR2-1. So we prefer to keep both. |
| Ericsson | Issue 3-2-3: PUCCH format 2 We prefer Option 1 but the intra-slot hopping for 1 symbol cases should be N/A. Issue 3-2-4: PUCCH format 3 We prefer Option 1. Issue 3-2-5: PUCCH format 4 We prefer Option 1. Issue 3-2-6: Test metric for PUCCH format 2 We are a bit confused by the relationship between this issue and Issue 3-2-3. Could we merge them together? If only test metric is considered in this issue, we prefer Option 2 to use UCI BLER for case 2. Issue 3-2-7: Applicability rule for PUCCH format 3 We tend to Option 2. Issue 3-2-8: Applicability rule for PUCCH format 4 We tend to Option 2. Issue 3-2-9: DMRs configuration for PUCCH format 3 We tend to Option 1 with applicability rule for the tests. Issue 3-2-10: DMRs configuration for PUCCH format 4 We tend to Option 1 with applicability rule for the tests. |
| Huawei | Issue 3-2-3: PUCCH format 2 Option 1 Issue 3-2-4: PUCCH format 3 We prefer Option 1. Issue 3-2-5: PUCCH format 4 Option 1 Issue 3-2-6: Test metric for PUCCH format 2 . Option 2. Besides, the Prob(DTX-.ACK)<1% should also be considered for cases with 4 bits. Issue 3-2-7: Applicability rule for PUCCH format 3 Option 2 is OK. Issue 3-2-8: Applicability rule for PUCCH format 4 Option 2 Issue 3-2-9: DMRs configuration for PUCCH format 3 We can compromise to option 1 considering test coverage Issue 3-2-10: DMRs configuration for PUCCH format 4 We can compromise to option 1 considering test coverage |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212151 | Moderator: CR not available |
|  | Company B |
| R4-2213818 | Nokia: This is not how we understood the proposals we are discussing in 1-5-1. The CR has new clauses for the FR2-2 requirements, shouldn’t we have used the same clauses with different tables for FR2-2? This is also the agreement we had in RAN4 102 (R4-2207223): Issue 1-5-1: Implementation of FR2-2 requirements into specification Capture FR2-2 demodulation requirement into same section as FR2-1 but with different tables if possible. |
|  | Company B |

# Topic #4: PRACH requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company |  | Proposals / Observations | Issue mapping |
| --- | --- | --- | --- | --- |
| R4-2212228 | Ericsson | Discussion on FR2-2 PRACH demodulation requirements Propose 1: Take 0.12us as the time error tolerance of TDLA10-650 channel if it is agreed for the multi-path fading channel. Proposal 2: Take table 2-1 for FR2-2 PRACH preamble configuration. Table 2-1 Ncs for FR2-2 PRACH preamble Lra; 139; 571; 1151. Ncs; 69; 285; 575. Logical sequence index; 0; 0; 0. v; 0; 0; 0. SCS [kHz]; 120, 480, [960]; 120, 480; 120. | 4-2-2 Option 2 4-1-2 Proposal 1, 2 |
| R4-2212231 | Ericsson | Simulation results for FR2-2 PRACH demodulation requirements |  |  |
| R4-2212678 | Nokia, Nokia Shanghai Bell | Discussion on PRACH demodulation requirements for the extension to 71 GHz - RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model. - The NCS parameters used for FR2 requirements (Ncs=69) are the ones that provide the maximum coverage area for LRA=139. The LRA=139 and Ncs=69 provide a similar coverage area to LRA=571 and Ncs=285 and LRA=1151 and Ncs=575. RAN4 to define PRACH demodulation performance requirements using the following Ncs parameters: -SCS=120, LRA=139, NCS=69 -SCS=120, LRA=571, NCS=285 -SCS=120, LRA=1151, NCS=575 -SCS=480, LRA=139, NCS=69 -SCS=480, LRA=571, NCS=285 -SCS=960, LRA=139, NCS=69 The achievable maximum detectable timing offset for 480 kHz and 960 kHz SCS is 1.0 us and 0.5 us respectively. The Rel 15 requirements for PRACH include timing offset scheme with offset up to 0.8 us, which exceeds the maximum detectable offset for 960 kHz SCS. RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS. Rel 16 NR-U requirements are defined such that PRACH time error tolerance is larger or equal to TAC resolution. Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command. RAN4 to define time error tolerance of PRACH timing requirements as: SCS; Channel model; Time error tolerance (ns). 120; AWGN; 70. TDLA10-650Note 1; [120]Note 1. 480; AWGN; 18. TDLA10-650; [68] Note 1. 960; AWGN; 9. TDLA10-650; [59] Note 1. NOTE 1: To be revised after definition of channel taps used for TDLA 10.. Use the table below for the test cases and parameters related to PRACH demodulation requirements: <Moderator comment> table omitted due to formatting issues and shall be included directly in the issues for discussion. Please check original paper | 4-1-3 Option 1 4-1-1 Option 1 4-1-2 Option 2 4-2-3 4-2-1 Option 1 4-2-2 Option 3 |
| R4-2212679 | Nokia, Nokia Shanghai Bell | PRACH simulation results for demodulation requirements for the extension to 71 GHz |  |  |
| R4-2213816 | Huawei,HiSilicon | Discussion on FR2-2 PRACH requirements Proposal 1: Use following configuration for PRACH requirements: -SCS=120, LRA=139, NCS=69 -SCS=120, LRA=571, NCS=285 -SCS=120, LRA=1151, NCS=575 -SCS=480, LRA=139, NCS=69 -SCS=480, LRA=571, NCS=285 Proposal 2: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined. |  | 4-1-1 Option 2 4-1-2 Option 1 4-2-1 Proposal 1 |
| R4-2212680 | Nokia, Nokia Shanghai Bell | Draft CR 38.104: PRACH requirements for FR2-2 |  |  |
| R4-2212681 | Nokia, Nokia Shanghai Bell | Draft CR 38.141-2: PRACH requirements for FR2-2 |  |  |

## Open issues summary

### Sub-topic 4-1 PRACH configurations

Issue 4-1-1: SCS for PRACH requirements

- Proposals

  - Option 1: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS

  - Option 2: Define requirements for PRACH using 120 kHz and 480 kHz SCS.

- Recommended WF

  - Please discuss your preferences among Option 1 and Option 2 having in mind the parallel discussion in the general Issue 1-3-1

Issue 4-1-2: Ncs, logical sequence index combinations

Three companies brough proposals for this issue with the same configuration for 120 kHz and 480 kHz

- Proposals

  - Proposal 1: Use following configuration for PRACH requirements:

    - SCS=120, LRA=139, NCS=69

    - SCS=120, LRA=571, NCS=285

    - SCS=120, LRA=1151, NCS=575

    - SCS=480, LRA=139, NCS=69

    - SCS=480, LRA=571, NCS=285

  - Proposal 2: Use following configuration for PRACH requirements in addition to proposal 1:

    - SCS=960, LRA=139, NCS=69 if we defined 960 kHz SCS requirements

- Recommended WF

  - Considering 3 companies brought proposals covering Proposal 1. Please confirm if we can agree on Proposal 1.

  - Discuss if Proposal 2 can be agreed, considering that it will also depend on the outcome of Issue 4-1-1 regarding requirements with 960 kHz SCS.

Issue 4-1-3: Channel model

- Proposals

  - Option 1: RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model.

- Recommended WF

  - Please discuss if Option 1 is agreeable.

### Sub-topic 4-2 Accuracy and test procedure

Issue 4-2-1: PRACH time error tolerance general aspects

- Proposals

  - Proposal 1: Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command.

- Recommended WF

  - Please discuss if proposal 1 is agreeable

Issue 4-2-2: PRACH time error tolerance

- Proposals

  - Option 1: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined.

  - Option 2: Take 0.12us as the time error tolerance of TDLA10-650 channel if it is agreed for the multi-path fading channel.

  - Option 3: RAN4 to define time error tolerance of PRACH timing requirements as:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650Note 1 | [120]Note 1 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | [68] Note 1 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | [59] Note 1 |
| NOTE 1: To be revised after definition of channel taps used for TDLA 10. |  |  |

- Recommended WF

  - Please indicate your preferences among options 1, 2, and 3 having in mind that time error tolerance could have impact on simulation alignment

Issue 4-2-3: Maximum timing offset for PRACH tests

- Proposals

  - Option 1: RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS.

- Recommended WF

  - Please discuss if option 1 is agreeable

### Sub-topic 4-3 List of test cases

Issue 4-3-1: PRACH list of test cases

- Proposals

  - Proposal 1: Use the table below for the test cases and parameters related to PRACH demodulation requirements:

| Format | SCS | Prach sequence length | Ncs | Propagation condition | Frequency Offset (Hz) | Time estimation tolerance |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 139 | 69 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 139 | 69 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 571 | 285 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 571 | 285 | TDLA10-650 | 7100 | 120 ns |
| A2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| B4 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| C2 | 120kHz | 1151 | 575 | AWGN | 0 | 70 ns |
| A2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| B4 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| C2 | 120kHz | 1151 | 575 | TDLA10-650 | 7100 | 120 ns |
| A2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 139 | 69 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 139 | 69 | TDLA10-650 | 7100 | 68 ns |
| A2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| B4 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| C2 | 480kHz | 571 | 285 | AWGN | 0 | 18 ns |
| A2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| B4 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| C2 | 480kHz | 571 | 285 | TDLA10-650 | 7100 | 68 ns |
| A2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| B4 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| C2 | 960kHz | 139 | 69 | AWGN | 0 | 9 ns |
| A2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| B4 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |
| C2 | 960kHz | 139 | 69 | TDLA10-650 | 7100 | 59 ns |

- Recommended WF

  - Discuss table above. To be updated according to agreements on subtopics 2-1 and 2-2

## Companies views’ collection for 1st round

### Open issues

Example 1

| Company | Comments |
| --- | --- |
| Ericsson | Issue 4-1-1: SCS for PRACH requirements We prefer Option 2. Issue 4-1-2: Ncs, logical sequence index combinations We prefer Option 1. Issue 4-1-3: Channel model We support Option 1. Issue 4-2-1: PRACH time error tolerance general aspects We can support Option 1. Issue 4-2-2: PRACH time error tolerance We can accept Option 1 to pending the discussion for now. Do we need to check this with TE vendors? Issue 4-2-3: Maximum timing offset for PRACH tests Reduced maximum timing offset should be considered. Option 1 with 0.4us could be a candidate. We are open for further discussion. |
| Nokia | Issue 4-1-1: SCS for PRACH requirements We prefer Option 1, including 960 kHz Issue 4-1-2: Ncs, logical sequence index combinations We agree with Proposal 1 and Proposal 2 Issue 4-1-3: Channel model We agree with Option 1. Issue 4-2-1: PRACH time error tolerance general aspects We agree with Proposal 1 Issue 4-2-2: PRACH time error tolerance We think Option 3 captures well how the tolerance was calculated so far. @Ericsson would it be ok to agree on those values and update after TDL model is concluded? The Time error tolerance is anyway in square brackets Ericsson@Nookia: We read through the calculation in Nokia’s contribution and think the value here is OK if the channel model is agreed. We suggest companies could also check the value. Issue 4-2-3: Maximum timing offset for PRACH tests We agree with Option 1. As we analysed in our paper, 0.4 us is detectable delay for PRACH sequences using the new SCSs. Issue 4-3-1: PRACH list of test cases We agree with the list as a baseline, depends on outcome of other issues. |
| Samsung | Issue 4-1-1: SCS for PRACH requirements We support option 2, Issue 4-1-2: Ncs, logical sequence index combinations Ok with option 1 for simulation assumption for alignment Issue 4-1-3: Channel model Ok with option 1 Issue 4-2-1: PRACH time error tolerance general aspects Ok with option 1 Issue 4-2-2: PRACH time error tolerance Ok with option 1, since the simplified FR2-2 is not determined yet, Issue 4-2-3: Maximum timing offset for PRACH tests Ok with option 1 for 480KHz Issue 4-3-1: PRACH list of test cases ok for 120KHz and 480KHz, the time estimation tolerance should be pended on agreed simplified channel model |
| Huawei | Issue 4-1-1: SCS for PRACH requirements Option 1 Issue 4-1-2: Ncs, logical sequence index combinations Support option 1 Issue 4-1-3: Channel model OK with option 1 Issue 4-2-1: PRACH time error tolerance general aspects Ok with option 1 Issue 4-2-2: PRACH time error tolerance OK with option 1 Issue 4-2-3: Maximum timing offset for PRACH tests Ok with option 1 for 480KHz Issue 4-3-1: PRACH list of test cases Ok for 120KHz and 480KHz, the time estimation tolerance should be pended on agreed simplified channel model |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212680 | Company A |
|  | Company B |
| R4-2212681 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Issue 4-1-1: SCS for PRACH requirements Original proposals: - Option 1: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS - Option 2: Define requirements for PRACH using 120 kHz and 480 kHz SCS. Moderator remarks: Diverging views on the topic. Suggest discussing general SCS Issue 1-3-1 on GTW and try to converge on the SCS in general. Tentative agreements: - none Candidate options: - Option 1: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS - Option 2: Define requirements for PRACH using 120 kHz and 480 kHz SCS. - Option 3: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS - Define that 960 kHz is optional Recommendations for 2nd round: - Continue discussion on the candidate options Issue 4-1-2: Ncs, logical sequence index combinations Original proposals: - Proposal 1: Use following configuration for PRACH requirements: - SCS=120, LRA=139, NCS=69 - SCS=120, LRA=571, NCS=285 - SCS=120, LRA=1151, NCS=575 - SCS=480, LRA=139, NCS=69 - SCS=480, LRA=571, NCS=285 - Proposal 2: Use following configuration for PRACH requirements in addition to proposal 1: - SCS=960, LRA=139, NCS=69 if we defined 960 kHz SCS requirements Moderator remarks: Companies were all aligned on the Proposal 1. As for Proposal 2 it depends on agreement regarding 960 kHz. Tentative agreements: - Use following configuration for PRACH requirements with 120 kHz SCS: - SCS=120, LRA=139, NCS=69 - SCS=120, LRA=571, NCS=285 - SCS=120, LRA=1151, NCS=575 - Use following configuration for PRACH requirements with 480 kHz SCS: - SCS=480, LRA=139, NCS=69 - SCS=480, LRA=571, NCS=285 Candidate options: - Option 1: Use following configuration for PRACH requirements with 960 kHz SCS: - SCS=960, LRA=139, NCS=69 if we defined 960 kHz SCS requirements Recommendations for 2nd round: - Confirm tentative agreement and discuss Option 1 with Issue 4-1-1. Issue 4-1-3: Channel model Original proposals: - Option 1: RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model. Moderator remarks: All companies commenting the issue agree with Option 1. Tentative agreements: - RAN4 to define PRACH demodulation requirements using AWGN and TDL-A 10-650 channel model. Candidate options: - Not needed Recommendations for 2nd round: - Confirm the tentative agreement |
| Sub-topic#4-2 | Issue 4-2-1: PRACH time error tolerance general aspects Original proposals: - Proposal 1: Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command. Moderator remarks: All companies commenting the issue agree with Proposal 1. Tentative agreements: - Not to define time error tolerance that is smaller than the minimum possible step for the timing advance command. Candidate options: - none Recommendations for 2nd round: - Confirm the tentative agreement Issue 4-2-2: PRACH time error tolerance Original proposals: - Option 1: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined. - Option 2: Take 0.12us as the time error tolerance of TDLA10-650 channel if it is agreed for the multi-path fading channel. - Option 3: RAN4 to define time error tolerance of PRACH timing requirements as: SCS; Channel model; Time error tolerance (ns). 120; AWGN; 70. TDLA10-650Note 1; [120]Note 1. 480; AWGN; 18. TDLA10-650; [68] Note 1. 960; AWGN; 9. TDLA10-650; [59] Note 1. NOTE 1: To be revised after definition of channel taps used for TDLA 10.. Moderator remarks: 1 company asked to wait for definition of simplified TDLA model and two other companies agreed on Option 2 which can be revised after definition of TDLA channel taps. Tentative agreements: - None Candidate options: - Same as 1st round discussion Recommendations for 2nd round: - Continue discussion Issue 4-2-3: Maximum timing offset for PRACH tests Original proposals: - Option 1: RAN4 to specify PRACH performance with timing offset up to 0.4 us for 480 kHz and 960 kHz SCS. Moderator remarks: - One company agrees to reduce timing offset, but further discuss exact value. Two companies agree with Option 1. One company agrees on Option 1 for 480 kHz. It is moderator understanding that agreement on Option 1 does not imply agreement on 960 kHz SCS which is also being discussed in other issues. Therefore it is suggested to agree on the general principle for SCS≥480 kHz and discuss the exact value of the offset. Tentative agreements: - RAN4 to specify PRACH performance with smaller maximum timing offset for each agreed SCSs above 120kHz. Candidate options: - Option 1: Adopt maximum timing offset as 0.4 us for SCS ≥480 kHz. - Option 2: Other values Recommendations for 2nd round: - Option 1: Adopt maximum timing offset as 0.4 us for SCS ≥480 kHz. |
| Sub-topic#4-3 | Issue 4-3-1: PRACH list of test cases Original proposals: - Table shown in the original issue Moderator remarks: One company asked to keep time tolerance open and include only 120 and 480 kHz SCS. One company agrees as a baseline to be reviewed after agreement on other issues Tentative agreements: Format; SCS; Prach sequence length; Ncs; Propagation condition; Frequency Offset (Hz); Time estimation tolerance. A2; 120kHz; 139; 69; AWGN; 0; TBD. B4; 120kHz; 139; 69; AWGN; 0; TBD. C2; 120kHz; 139; 69; AWGN; 0; TBD. A2; 120kHz; 139; 69; TDLA10-650; 7100; TBD. B4; 120kHz; 139; 69; TDLA10-650; 7100; TBD. C2; 120kHz; 139; 69; TDLA10-650; 7100; TBD. A2; 120kHz; 571; 285; AWGN; 0; TBD. B4; 120kHz; 571; 285; AWGN; 0; TBD. C2; 120kHz; 571; 285; AWGN; 0; TBD. A2; 120kHz; 571; 285; TDLA10-650; 7100; TBD. B4; 120kHz; 571; 285; TDLA10-650; 7100; TBD. C2; 120kHz; 571; 285; TDLA10-650; 7100; TBD. A2; 120kHz; 1151; 575; AWGN; 0; TBD. B4; 120kHz; 1151; 575; AWGN; 0; TBD. C2; 120kHz; 1151; 575; AWGN; 0; TBD. A2; 120kHz; 1151; 575; TDLA10-650; 7100; TBD. B4; 120kHz; 1151; 575; TDLA10-650; 7100; TBD. C2; 120kHz; 1151; 575; TDLA10-650; 7100; TBD. A2; 480kHz; 139; 69; AWGN; 0; TBD. B4; 480kHz; 139; 69; AWGN; 0; TBD. C2; 480kHz; 139; 69; AWGN; 0; TBD. A2; 480kHz; 139; 69; TDLA10-650; 7100; TBD. B4; 480kHz; 139; 69; TDLA10-650; 7100; TBD. C2; 480kHz; 139; 69; TDLA10-650; 7100; TBD. A2; 480kHz; 571; 285; AWGN; 0; TBD. B4; 480kHz; 571; 285; AWGN; 0; TBD. C2; 480kHz; 571; 285; AWGN; 0; TBD. A2; 480kHz; 571; 285; TDLA10-650; 7100; TBD. B4; 480kHz; 571; 285; TDLA10-650; 7100; TBD. C2; 480kHz; 571; 285; TDLA10-650; 7100; TBD. Note 1: Pending agreement on Time tolerance and definition of requirements with 960 kHz SCS.. Candidate options: - Option 1: If 960 kHz SCS is agreed, use the table below for the test cases and parameters related to PRACH demodulation requirements: Format; SCS; Prach sequence length; Ncs; Propagation condition; Frequency Offset (Hz); Time estimation tolerance. C2; 480kHz; 571; 285; TDLA10-650; 7100. A2; 960kHz; 139; 69; AWGN; 0. B4; 960kHz; 139; 69; AWGN; 0. C2; 960kHz; 139; 69; AWGN; 0. A2; 960kHz; 139; 69; TDLA10-650; 7100. B4; 960kHz; 139; 69; TDLA10-650; 7100. C2; 960kHz; 139; 69; TDLA10-650; 7100. Recommendations for 2nd round: - Review table during WF drafting considering potential agreements on other issues. - Consider Option 1 in case 960 kHz SCS is agreed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2212680 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet- |
| R4-2212681 | “return to” No comments were received, but since the Draft CR may need revision due to agreements, the moderator suggests not to endorse the proposal yet- |

## Discussion on 2nd round

### Sub-topic 4-1 PRACH configurations

Issue 4-1-1: SCS for PRACH requirements

- Proposals

  - Option 1: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS

  - Option 2: Define requirements for PRACH using 120 kHz and 480 kHz SCS.

  - Option 3: Define requirements for PRACH using 120 kHz, 480 kHz and 960 kHz SCS

    - Define that 960 kHz is optional

- Recommended WF

  - Please discuss your preferences among Option 1, Option 2, and Option 3 having in mind the parallel discussion in the general Issue 1-3-1

Issue 4-1-2: Ncs, logical sequence index combinations

Agreements on the first round for 120 kHz and 480 kHz. Decision on 960 kHz pends decision on 4-1-1. Come back to this issue once decision is reached in 4-1-1.

No further discussion needed.

Issue 4-1-3: Channel model

Agreement reached on the 1st round.

No further discussion needed.

### Sub-topic 4-2 Accuracy and test procedure

Issue 4-2-1: PRACH time error tolerance general aspects

Agreement reached on the 1st round.

No further discussion needed.

Issue 4-2-2: PRACH time error tolerance

- Proposals

  - Option 1: Postponing discussion of time error tolerance until simplified FR2-2 channel model is determined.

  - Option 2: Take 0.12us as the time error tolerance of TDLA10-650 channel if it is agreed for the multi-path fading channel.

  - Option 3: RAN4 to define time error tolerance of PRACH timing requirements as:

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 120 | AWGN | 70 |
|  | TDLA10-650Note 1 | [120]Note 1 |
| 480 | AWGN | 18 |
|  | TDLA10-650 | [68] Note 1 |
| 960 | AWGN | 9 |
|  | TDLA10-650 | [59] Note 1 |
| NOTE 1: To be revised after definition of channel taps used for TDLA 10. |  |  |

- Recommended WF

  - Please indicate your preferences among options 1, 2, and 3 having in mind that time error tolerance could have impact on simulation alignment. For the sake of progress, can we move forward with Proposal 3 and revise values one the simplified TDLA model is decided?

Issue 4-2-3: Maximum timing offset for PRACH tests

- Proposals

  - Option 1: Adopt maximum timing offset as 0.4 us for SCS ≥480 kHz.

  - Option 2: Other values

- Recommended WF

  - Considering the tentative agreement in the 1st round, please discuss if option 1 is agreeable, otherwise please indicate to which value the timing offset should be reduced.

### Sub-topic 4-3 List of test cases

Issue 4-3-1: PRACH list of test cases

Tentative agreement reached in the 1st round.

Table to be updated in the WF if there is further progress on the time estimation tolerance and SCS.

## Companies views’ collection for 2nd round

### Open issues

Sub-topic 4-1 PRACH configurations

| Company | Comments |
| --- | --- |
| XXX | Issue 4-1-1: SCS for PRACH requirements |
| Nokia | Issue 4-1-1: SCS for PRACH requirements Our preference if Option 1. We would like to consider all SCS which are defined for ext to 71 GHz. We can compromise to Option 3. We are willing to compromising making 960 kHz optional. This can be done adopting the same procedure as used for HST. |
| Ericsson | Issue 4-1-1: SCS for PRACH requirements We prefer Option 2. |
| Huawei | Issue 4-1-1: SCS for PRACH requirements We prefer Option 2. |

Sub-topic 4-2 Accuracy and test procedure

| Company | Comments |
| --- | --- |
| XXX | Issue 4-2-2: PRACH time error tolerance Issue 4-2-3: Maximum timing offset for PRACH tests |
| Nokia | Issue 4-2-2: PRACH time error tolerance We think it would be good for progress to take Option 3 as a baseline. We can review the values after confirming the channel model. Difference on the time error tolerance could potentially make it harder to align simulations. SO if we take few meetings to converge on the channel model we could end up losing a lot of time on the simulation alignment. Issue 4-2-3: Maximum timing offset for PRACH tests We agree with Option 1. Our original proposal was having maximum offset that 960 kHz could use. We could also use larger value for 480 kHz SCS, like 0.6 us. If companies want to further check we can anyway postpone discussion for the next meeting, since this decision doesn’t stop us from having simulation alignment. |
| Ericsson | Issue 4-2-2: PRACH time error tolerance Option 2 and 3 are OK for us. Slightly prefer Option 3. Issue 4-2-3: Maximum timing offset for PRACH tests Option 1 is OK. |
| Samsung | Issue 4-1-1: SCS for PRACH requirements We support option 2, |
| Huawei | Issue 4-2-2: PRACH time error tolerance Option 3 is OK Issue 4-2-3: Maximum timing offset for PRACH tests Option 1 is OK |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2212680 | Company A |
|  | Company B |
| R4-2212681 | Company A |
|  | Company B |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on general aspects for FR2-2 demodulation requirements |  |  |
|  | WF on PUSCH demodulation requirements for FR2-2 | Nokia |  |
|  | WF on PUCCH demodulation requirements for FR2-2 | Ericsson |  |
|  | WF on PRACH demodulation requirements for FR2-2 | Samsung |  |
|  | Big CR FR2-2 BS demodulation requirements 38.104 | Intel |  |
|  | Big CR FR2-2 BS demodulation requirements 38.141-2 | Nokia |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2212673 |  | Draft CR - definition of FR2-2 | Nokia, Nokia Shanghai Bell | Return to |  |
| R4-2212150 |  | draftCR to 38104 on BS radiated requirements for PUSCH FR2-2 | Intel Corporation | Return to |  |
| R4-2212232 |  | Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Return to |  |
| R4-2212233 |  | Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Return to |  |
| R4-2213819 |  | Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2 | Huawei,HiSilicon | Return to |  |
| R4-2212151 |  | draftCR to 38141-2 on BS radiated requirements for PUCCH FR2-2 | Intel Corporation | Return to |  |
| R4-2213818 |  | Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104 | Huawei,HiSilicon | Return to |  |
| R4-2212680 |  | Draft CR 38.104: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Return to |  |
| R4-2211540 |  | Views on channel model for FR2-2 | Anritsu Corporation | Noted |  |
| R4-2212105 |  | On General for ext71GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212225 |  | Discussion on general issue on FR2-2 demodulation | Ericsson | Noted |  |
| R4-2213803 |  | Discussion on general issues for FR2-2 demodulation requirements | Huawei,HiSilicon | Noted |  |
| R4-2213804 |  | Discussion on spec structure for FR2-2 demodulation and CSI requirements | Huawei,HiSilicon | Noted |  |
| R4-2213967 |  | UE Demodulation Requirements for FR2-2 (up to 71GHz) | Qualcomm Incorporated | Noted |  |
| R4-2212226 |  | Discussion on FR2-2 PUSCH demodulation requirements | Ericsson | Noted |  |
| R4-2212674 |  | Discussion on PUSCH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212675 |  | PUSCH simulation results for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213669 |  | View on BS demodulation requirement for NR extended to 71GHz | Samsung | Noted |  |
| R4-2213812 |  | Discussion on FR2-2 PUSCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212227 |  | Discussion on FR2-2 PUCCH demodulation requirements | Ericsson | Noted |  |
| R4-2212676 |  | Discussion on PUCCH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212677 |  | PUCCH simulation results for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213814 |  | Discussion on FR2-2 PUCCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212228 |  | Discussion on FR2-2 PRACH demodulation requirements | Ericsson | Noted |  |
| R4-2212678 |  | Discussion on PRACH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212679 |  | PRACH simulation results for demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213816 |  | Discussion on FR2-2 PRACH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212229 |  | Simulation results for FR2-2 PUSCH demodulation requirements | Ericsson | Noted |  |
| R4-2213813 |  | Simulation results on FR2-2 PUSCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212230 |  | Simulation results for FR2-2 PUCCH demodulation requirements | Ericsson | Noted |  |
| R4-2213815 |  | Simulation results on FR2-2 PUCCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212231 |  | Simulation results for FR2-2 PRACH demodulation requirements | Ericsson | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2214655 |  | WF on general aspects for FR2-2 demodulation requirements | Huawei,HiSilicon | Agreeable |  |
| R4-2214388 |  | WF on PUSCH demodulation requirements for FR2-2 | Nokia | Agreeable |  |
| R4-2214500 |  | WF on PUCCH demodulation requirements for FR2-2 | Ericsson | Agreeable |  |
| R4-2214389 |  | WF on PRACH demodulation requirements for FR2-2 | Samsung | Agreeable |  |
| R4-2212150 |  | draftCR to 38104 on BS radiated requirements for PUSCH FR2-2 | Intel Corporation | Not pursued |  |
| R4-2212232 | R4-2215123 | Draft CR for TS38.104 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Endorsable |  |
| R4-2212233 | R4-2215124 | Draft CR for TS38.141-2 FRC tables for FR2-2 PUSCH demodulation requirements | Ericsson | Endorsable |  |
| R4-2213819 | R4-2215126 | Draft CR Introduction of FR2-2 PUSCH performance requirements in TS 38.141-2 | Huawei,HiSilicon | Endorsable |  |
| R4-2212151 |  | draftCR to 38141-2 on BS radiated requirements for PUCCH FR2-2 | Intel Corporation | Not pursued |  |
| R4-2213818 | R4-2215125 | Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104 | Huawei,HiSilicon | Endorsable |  |
| R4-2212680 |  | Draft CR 38.104: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsable |  |
| R4-2212681 |  | Draft CR 38.141-2: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsable |  |
| R4-2211540 |  | Views on channel model for FR2-2 | Anritsu Corporation | Noted |  |
| R4-2212105 |  | On General for ext71GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212225 |  | Discussion on general issue on FR2-2 demodulation | Ericsson | Noted |  |
| R4-2213803 |  | Discussion on general issues for FR2-2 demodulation requirements | Huawei,HiSilicon | Noted |  |
| R4-2213804 |  | Discussion on spec structure for FR2-2 demodulation and CSI requirements | Huawei,HiSilicon | Noted |  |
| R4-2213967 |  | UE Demodulation Requirements for FR2-2 (up to 71GHz) | Qualcomm Incorporated | Noted |  |
| R4-2212226 |  | Discussion on FR2-2 PUSCH demodulation requirements | Ericsson | Noted |  |
| R4-2212674 |  | Discussion on PUSCH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212675 |  | PUSCH simulation results for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213669 |  | View on BS demodulation requirement for NR extended to 71GHz | Samsung | Noted |  |
| R4-2213812 |  | Discussion on FR2-2 PUSCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212227 |  | Discussion on FR2-2 PUCCH demodulation requirements | Ericsson | Noted |  |
| R4-2212676 |  | Discussion on PUCCH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212677 |  | PUCCH simulation results for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213814 |  | Discussion on FR2-2 PUCCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212228 |  | Discussion on FR2-2 PRACH demodulation requirements | Ericsson | Noted |  |
| R4-2212678 |  | Discussion on PRACH demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2212679 |  | PRACH simulation results for demodulation requirements for the extension to 71 GHz | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2213816 |  | Discussion on FR2-2 PRACH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212229 |  | Simulation results for FR2-2 PUSCH demodulation requirements | Ericsson | Noted |  |
| R4-2213813 |  | Simulation results on FR2-2 PUSCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212230 |  | Simulation results for FR2-2 PUCCH demodulation requirements | Ericsson | Noted |  |
| R4-2213815 |  | Simulation results on FR2-2 PUCCH requirements | Huawei,HiSilicon | Noted |  |
| R4-2212231 |  | Simulation results for FR2-2 PRACH demodulation requirements | Ericsson | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents