3GPP TSG-RAN WG4 Meeting # 104-e 												      R4-2214330

Electronic Meeting, August 15 – August 26, 2022

Title:	WF on further RRM enhancement for NR and MR-DC - PUCCH SCell activation/deactivation requirements

Agenda item:			9.8.3

Source:	CATT

Document for:	Approval

Topic #1: PUCCH SCell activation/deactivation core requirements maintenance

Sub-topic 1-1 PUCCH SCell activation delay requirements

Issue 1-1-1: Whether the PL-RS will introduce extra delay time when the known condition is met in FR2 (the value of [X] in 8.3.12)?

Way forward:

- Option 1:

      - When PL-RS of target PUCCH SCell is known, the X=5 sample measurement time is always considered and no need to consider condition of ‘maintain’ or ‘not maintain’.

- Option 2: (Nokia)

      - Pathloss estimation for UL Tx power control is based on RS being configured for L1 measurement using narrow beam.

        - a)       PL-RS is based on the M L1-RSRP measurement samples.

      - How many samples (M) do we need to acquire an accurate L1 measurement of PL-RS?

        - a)        M=1/3 has been used in L1-RSRP measurement. M=1 is used in SCell activation delay for unknown DL SCell.

        - b)      Would M=[3] be sufficient to do L1 measurement in the same as in L1-RSRP?

      - M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

Issue 1-1-2: Update TFirst_available_CSI and TCSI_reporting_after in the PUCCH SCell activation delay requirements?

Way forward:

- Option 1: (Ericsson)

      - Existing requirements:

        - -	TFirst_available_CSI: the delay uncertainty in acquiring the first available downlink CSI reference resource.

        - -	TCSI_reporting_after: the delay uncertainty in acquiring the first available CSI reporting resource after T3

      - Update to:

        - TLast_Valid_CSI: the delay uncertainty in acquiring the downlink CSI reference resource which is the last CSI reference resource associated with the first available CSI report resource after T1+T2+T3.

        - TCSI_reporting_after is the delay uncertainty in acquiring the first available CSI reporting resource after T3

- Option 2:

      - Keep the existing requirements.

Issue 1-1-3: The conditions where the cross PUCCH group CSI reporting UE capability is required?

Agreements:

- The conditions where the cross PUCCH group CSI reporting UE capability is required shall be aligned with RAN4#101e agreement (only required when the beam information is needed to be indicated).

Issue 1-1-4: Clarification on previous working assumption on PL-RS?

Agreements:

- Based on RAN2 spec, there is no case that when pathlossReferenceRSs is not provided but PUCCH-SpatialRelationInfo is provided in FR2.

- The above agreement has no impact on existing RAN4 requirements.

- FFS: In FR1, when pathlossReferenceRSs is not provided, the PL-RS is assumed as the SSB for fine timing tracking during SCell activation. Thus, no uncertainty for PL-RS to be considered.

Issue 1-1-5: Clarification on the relation between the associated RS for TCI state, PL-RS and spatial relation indication?

Agreements:

- No need to have restrictions on the relation between the associated RS for TCI state, PL-RS and spatial relation indication in core requirements.

Issue 1-1-6: Clarification on the condition for PUCCH SCell activation with multiple Downlink Scells?

Agreements:

- Updated the condition to:  any to-be-activated unknown non-PUCCH Scell in a different band from to-be-activated PUCCH Scell has active serving cell(s) or known to-be-activated non-PUCCH Scell(s) on the same band.

Issue 1-1-7: Clarification on interruption length on UL transmission when colliding with PRACH transmission on PUCCH SCell?

Way forward:

- Option 1: (Huawei)

      - No need to define the interruption length of PRACH transmission when UE is not capable of parallelTxPRACH-SRS-PUCCH-PUSCH.

- Option 2: (Ericsson)

      - RAN4 to define 4 different interruption groups with each group having different interruption length.

      - For preambles format A1, A2, A3, B1, B2, B3, B4, C0 and C1, the interruption length is 0.5ms.

      - For preambles format 0 and 3, the interruption length is 1ms.

      - For preambles format 1, the interruption length is 3ms.

      - For preambles format 2, the interruption length is 5ms.

Topic #2: PUCCH Scell activation/deactivation performance requirements

Sub-topic 2-1 Test cases and test configuration

Issue 2-1-1: Whether to define test case for intra-band CA

Agreements:

- RAN4 to not define test cases for intra-band CA between dual PUCCH groups, i.e. PUCCH SCell should not be in the same band as the other PUCCH group.

Issue 2-1-2: For EN-DC test, whether PUCCH SCell and  PSCell can be in the same FR

Agreements:

- For EN-DC test, PUCCH SCell and PSCell cannot be in the same FR.

Issue 2-1-3: Whether to define test case for FR1 PCell/PSCell + FR2 PUCCH SCell

Agreements:

- Define test case for FR1 PCell/PSCell + FR2 PUCCH SCell

- TE determines the starting point of the test based on the activation command of which the HARQ ACK is successfully received

- For FR1 PCell/PSCell + FR2 PUCCH SCell, consider known PUCCH SCell with valid TA

- Introducing the unknown PUCCH SCell and invalid TA cases with applicability rule based on the testability study

- Updating the applicability when testability study progress

Issue 2-1-4: Test configuration for PUCCH SCell activation with multiple DL SCells in FR2 (SA TC 1-10: FR2 PUCCH SCell+FR2 SCell+FR2 PCell)

Agreements:

- For SA TC 1-10: FR2 PUCCH SCell+FR2 SCell+FR2 PCell

    - Configures a PCell and two SCells (one PUCCH Scell and one normal SCell) in different FR2 bands

    - The two SCells are intra-band contiguous to each other.

    - The two SCells belong to the same PUCCH group, i.e. the secondary PUCCH group

Issue 2-1-4a: whether to define test for the case when non-PUCCH SCell is in same PUCCH group as PCell, i.e. Primary PUCCH group

Agreements:

- For PUCCH SCell activation with multiple DL SCells in FR2, define test for the case when non-PUCCH SCell is in the same PUCCH group as PCell, i.e. Primary PUCCH group.

    - Configures a PCell and one PUCCH SCell in different FR2 bands

    - Configure a normal SCell on the same band as PCell.

    - The PCell and normal SCells belong to the same PUCCH group, i.e. the Primary PUCCH group

Reference

- R4-2214262 Email discussion summary for [104-e][210] NR_RRM_enh2_3, CATT, RAN4#104-e