3GPP TSG-RAN WG4 Meeting # 104-e											R4-2214343

Electronic Meeting, 15– 26 August 2022

Title: 	WF on RRM requirements for Rel-17 NB-IoT and LTE-MTC

Agenda Item:	9.24.4, 9.24.5

Source: 	Huawei, HiSilicon

Document for:	Approval

# Topic #2: Performance requirements for NB-IoT

### Sub-topic 2-1 Test cases for neighbour cell measurement for NB-IoT

Issue 2-1: Whether and how to define test cases for inter-frequency neighbour cell measurement for NB-IoT

Agreement:

  - Define inter-frequency neighbour measurement test cases with the serving cell in anchor carrier, and state in the test cases that the target carrier frequency information is provided to the UE to ensure that UE has the context of the target carrier frequency.

Issue 2-2: Test cases list

Agreement:

  - TC1:HD-FDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA/in-band/guard-band

  - TC2: TDD intra-frequency neighbour cell measurement under normal coverage in non-DRX in SA//in-band/guard-band

  - TC3:HD-FDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band

  - TC4:TDD inter-frequency neighbour cell measurement under normal coverage in DRX in SA SA//in-band/guard-band

UE is only required to be tested in one operation mode out of SA, in-band, guard-band.

# Reference

R4-2214281	Email discussion summary for [104-e][231] NB_IOTenh4_LTE_eMTC6_RRM