3GPP TSG-RAN WG4 Meeting # 104-e                                                            R4-2214369

Electronic Meeting, 15– 26 August 2022

Agenda item:			9.5.6

Source:	CATT

Title:	WF on NR repeater conformance test remaining issues

Document for:	Approval

- Introduction

This WF is to capture the agreements for the email discussion thread [104-e][306] NR_Repeater_RFConformance_Part2 discussion in RAN4#104e. The WF was drafted according the 1st round discussion summary [1] and was discussed in the 2nd round.

- Way Forward

  - FR1 RF conformance test

2.1.1 FR1 declaration open issues

Agreements:

- NC CA no needs to be declared.

- Group delay:

  - Declaration with values for all repeaters

    - It’s not precluded that 0 can be declared if repeater can fulfil transient time requirements

    - Further work on the specification draft

2.1.2 Rx spurious emission limits test set up

Agreement:

- Use the following setup and add “Optional” in signal generator box

2.1.3 Repeater Tx/Rx spurious emissions MU and TT

Agreements:

- RAN4 consider MU values for BS Rx spurious emissions as baseline for repeater Rx spurious emissions.

- 30 MHz ≤ f ≤ 4 GHz: ±2.0 dB

- 4 GHz < f ≤ 19 GHz: ±4.0 dB

- 19 GHz < f ≤ 26 GHz: ±4.5 dB

- RAN4 adopt 0dB as TT values for Tx and Rx spurious emissions.

2.1.4 ALCR BW in MU

Agreement:

- BW should be replaced by the nominal repeater channel bandwidth for ACLR requirements.

2.1.5 UL/DL timing for TDD FR1 (and FR2) repeater

Agreement:

- Add generic description into specification: “UL/DL timing can be provided to the repeater”

  - FR2 RF conformance test

    - FR2 declarations open issues

Agreement:

- NC CA and long group delay follow FR1 agreement

2.2.2 TRP measurement with input antenna rotating together with repeater

Agreements:

- Keep the 3GPP description of providing input signal to the repeater generic and allow reasonable uncertainty for it, allowing test engineers to use appropriate solutions in various different test systems.

- Further work on the drafting TP

2.2.3 TDD off power measurement

Agreements:

- Take same approach as BS for transmit antenna gain assumption during on/off period.

- In TDD ON/OFF power measurement, input signal generator does not transmit towards repeater during the OFF-power measurement.

2.2.4 OTA test procedure

Agreements:

- “Measurement impact from feeding test signal by generating a signal for repeater input with repeater to be turned off.  Verify measured result is enough below requirement limit”

- Further work on specification drafting

Reference

[1] R4-2214166, Email discussion summary for [104-e][306] NR_Repeater_RFConformance_Part2